|  |  | BHARAT HEAVY ELECTRICALS LIMITED Heavy Power Equipment Plant Hyderabad – 502032. INDIA Phones: 23182250/23020173 Fax No.s : 040-23021910 & 23021954,23186173 E-Mail : burre@bhelhyd.co.in |  | Date: Due Date: Supplier Qtn. No. : Date : |  |  | ___________________ ___________________ ___________________ ___________________ |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TENDER DOCUMENTS TO BE FILLED BY THE SUPPLIER FOR SUPPLY OF 12 TON BOTTOM POURING LADLE |  |  |  |  |  |  |  |  |
| ADDRESS OF THE SUPPLIER: |  |  |  |  |  |  |  |  |
| TELEPHONE NO.S : |  |  |  |  |  |  |  |  |
| FAX NO.S : |  |  |  |  |  |  |  |  |
| E MAIL ADDRESS : |  |  |  |  |  |  |  |  |
| Sl. No. | DESCRIPTION OF BHEL REQURIMENT |  | BHEL SPEC. |  | OFFERED | DEVIATIONS |  | REMARKS |
| 1.0 | PURPOSE Bottom pouring ladle of 12 Ton capacity is required to handle molten Steel and to carry out pouring operations in Foundry. |  |  |  |  |  |  |  |
| 2.0 | TECHINICAL DETAILS |  |  |  |  |  |  |  |
| 2.1 | Capacity of Ladle |  | 12 Ton |  |  |  |  |  |
| 2.2 | Quantity required |  | 1 No. |  |  |  |  |  |
| 2.3 | Type of Ladle |  | Bottom pouring ladle with gear box assembly |  |  |  |  |  |
| 2.4 | DIMENSIONS Supplier has to get the dimensions of the ladle wetted by M/s BHEL after placement of Order and adopt the necessary changes if any. The tentative dimensional details of the ladle can be as follows |  |  |  |  |  |  |  |
| 2.4.1 | Shell Dimensions |  |  |  |  |  |  |  |
|  | Inside Dia |  | 1830 mm |  |  |  |  |  |
|  | Bottom Dia |  | 1600 mm |  |  |  |  |  |
|  | Height |  | 1830 mm |  |  |  |  |  |
| 2.4.2 | Plate Thickness |  |  |  |  |  |  |  |
|  | Side |  | 16 mm |  |  |  |  |  |
|  | Bottom |  | 20 mm |  |  |  |  |  |
| 2.4.3 | Refractory Lining |  |  |  |  |  |  |  |
|  | Side |  | 165 mm |  |  |  |  |  |
|  | Bottom |  | 215 mm |  |  |  |  |  |
| 2.4.4 | Top Clearance |  | 390 mm |  |  |  |  |  |
| 2.5 | Details of Graphite Stopper Head Supplier has to Specify |  |  |  |  |  |  |  |

| 3.0 | FEATURES : |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 3.1 | Bottom pouring ladle shall be supplied with Stopper Rods and Nuts (2 No.s. ), operating lever arrangement, gear mechanism with hand wheel for tilting the ladle, trunions, bail-arm, heat shield, hook / shackle for lifting with EOT crane lop, legs, refractory lining ring, locking lever etc. |  |  |  |  |
| 3.2 | Bottom pouring ladle shall be supplied with 5 No.s of graphite stopper heads as optional item. |  |  |  |  |
| 4.0 | DOCUMENTATION: |  |  |  |  |
| 4.1 | Supplier shall furnish constructional details of the Ladle. i.e. General arrangement drawing, volume to weight calculations for molten steel along with the quotation. |  |  |  |  |
| 4.2 | Supplier shall furnish detailed dimensional sketches of stopper rod with nut, refractory insulating sleeve to be fitted on to the stopper rod, refractory nozzle and graphite stopper head. |  |  |  |  |
| 4.3 | Three sets of each of the following documents to be furnished along with the Ladle |  |  |  |  |
|  | Operation Manual | 3 sets |  |  |  |
|  | Maintenance Manual | 3 sets |  |  |  |
| 4.4 | List of customers for similar geared bottom pouring ladle shall be furnished along with the offer. |  |  |  |  |
| 5.0 | PRE INSPECTION : M/s BHEL Engineers will inspect the Ladle at suppliers works before dispatch. |  |  |  |  |
| 6.0 | GUARANTEE : The geared bottom pouring Ladle shall be guaranteed for a period of one year from date of commissioning. |  |  |  |  |
| 7.0 | SPECIAL REMARKS: |  |  |  |  |
| 7.1 | Vendor shall submit technical offer and price offer separately in two sealed covers |  |  |  |  |
| 7.2 | They shall be original equipment manufacturers |  |  |  |  |
| 7.3 | They shall be in line of manufacturing of similar Bottom pouring ladles with capacities greater than or equal to 5 Ton and up to 20 Ton for not less than 5 years. |  |  |  |  |