TECHNICAL SPECIFICATION FOR 45 KW MEDIUM FREQUENCY INDUCTION BRAZING MACHINE TECHNICAL SPECIFICATION NO. EMT - 6709 REV. 02

| S. N. | DESCRIPTION OF BHEL REQUIREMENT | Qty. | OFFERED | DEVIATION / REMARKS |
| --- | --- | --- | --- | --- |
| 1.0 | PURPOSE : |  |  |  |
|  | 45 KW Medium Frequency Induction Brazing machine will be used for brazing the joint of water box bottom part with hollow copper conductors as shown in Annexure - I. |  |  |  |
| 2.0 | SCOPE : |  |  |  |
|  | The scope of supply includes Design, Manufacturing, Supply, Erection, Commissioning and Job Proving of Performance of the machine at BHEL Haridwar works. | 01 no. |  |  |
| 3.0 | JOB DESCRIPTION, & TECHNICAL REQUIREMENT OF JOB : |  |  |  |
| 3.1 | Job Description : Brazing of water box bottom part (material : copper) with hollow copper conductors of Stator Winding Bar of large size Turbogenerator as per Annexure - I Refer Annexure II, for the detail dimensions of Water Box Bottom Part |  |  |  |
| 4.0 | TECHNICAL SPECIFICATION OF 45 KW MEDIUM FREQUENCY INDUCTION BRAZING MACHINE : |  |  |  |
| 4.1 | Main Parameters : Continuous output power – 45 kw Frequency Range : 10 to 30 KHz Medium Frequency Induction Brazing should consist of following major items like Frequency Converter, Co-axial Transformer, Brazing Inductor, Chilling Unit, Mobile Trolley, Controller, Radiation Pyrometer etc. as specified in sl. no. 4.2 to 4.7.4 below. |  |  |  |
| 4.2 | FREQUENCY CONVERTER : |  |  |  |
| 4.2.1. | Frequency Converter will vary the frequency range between 10 to 30 KHz automatically as per the job requirement |  |  |  |
| 4.2.2 | Front display board should indicate the following parameters of the machine – DC voltage and current, Output power, frequency, temperature of brazing joint through radiation pyrometer etc. |  |  |  |
| 4.2.3 | Frequency converter should be cooled by forced cooling water. NOTE : i) Normal drinking water supply available at the shop floor, will be used for this purpose. Vendor shall furnish concurrence on this point. ii) Suitable partition / barrier must be provided between the water cooled items and non water cooled items in the cubicle of frequency converter so that under the event of water splash / leakage, electrical items and other items are NOT DRENCHED / AFFECTED WITH WATER. |  |  |  |
| 4.3 | CO-AXIAL TRANSFORMER : |  |  |  |
| 4.3.1 | Light weight, CYLINDRICAL CONFIGURATION, water cooled Co-axial transformer easy to manoeuvre on and around the brazing joint, shall be supplied. Co-axial transformer will be connected to frequency converter by 10.0 mtr. Long and Light weight flexible cable. Size and weight of the Co-axial transformer and arrangement for holding / hanging the Co-axial transformer, shall be submitted in the offer. | . |  |  |
| 4.3.2 | NOTE : Normal drinking water supply available at the shop floor, will be used for this purpose. Vendor shall furnish concurrence on this point. |  |  |  |
| 4.3.3 | Suitable Mobile Stand and good quality, effective & operator friendly Spring Balancer shall be supplied for hanging the Co-axial transformer. |  |  |  |
| 4.3.4 | For fixing the brazing inductor (sl. no. 4.4) on the mouth of Co-axial transformer (sl. no. 4.3) screw type of joint will not be acceptable. |  |  |  |
| 4.4 | BRAZING INDUCTOR : | 01 no. |  |  |
| 4.4.1 | Water Cooled brazing inductor suitable for the brazing joint as per attached Annexure I and Annexure II, shall be supplied with the machine. Refer sl. no. 4.3.4 for fixing the brazing inductor at the mouth of Co-axial transformer |  |  |  |
| 4.4.2 | NOTE : Normal drinking water supply available at the shop floor, will be used for this purpose. Vendor shall furnish concurrence on this point. | . |  |  |
| 4.5 | CHILLING UNIT : |  |  |  |
| 4.5.1 | Chilling unit will consist of effective and reputed make chiller, high performance circulating pump, leak proof and quick end fitting coupling for incoming and outgoing water connection between the chiller and frequency converter, Make and technical specification of chilling unit and pump, shall be furnished in the offer. NOTE : Normal drinking water supply available at the shop floor, will be used for this purpose. |  |  |  |
| 4.5.2 | Suitable end fitting complete with leak proof stop cock and nozzle shall be supplied for the following utilities : i) First filling and Make up water ii) Overflow connection iii) Complete draining out water from storage tank |  |  |  |
| 4.5.3 | Water level indicator with marking for minimum and maximum level of water, shall be provided |  |  |  |
| 4.5.4 | The complete chilling unit along with all joints and fittings, must be sealed hermetically to eliminate leakage of water. |  |  |  |
| 4.5.5 | Overall dimensions of the complete chilling unit, shall be submitted in the offer. |  |  |  |
| 4.6 | MOBILE TROLLEY : |  |  |  |
| 4.6.1 | Separate mobile trolley for Frequency Converter (sl. no. 4.2) and Chilling Unit (sl. no. 4.5), shall be supplied. Each mobile trolley shall be complete with minimum 4 nos. good quality nylon wheels and suitable handle to steer the frequency converter and chilling unit at shop floor as and when required. Size of the mobile trolley shall be furnished in the offer. |  |  |  |
| 4.6.2 | Suitable lifting rod with eye / other arrangement shall be provided on the top base of the mobile trolley (4.6.1) to shift and transport the frequency converter (sl. no. 4.2) and chilling unit (sl. no. 4.5) without water, inside the work shop, with the help of overhead crane. |  |  |  |
| 4.7 | CONTROL : |  |  |  |
| 4.7.1 | One main controller with programmable temperature control for setting of time during heating mode, soaking mode, total time for brazing operation etc. shall be provided on the front panel of the cubicle of frequency converter. Make, Model and specification details of the controller, shall be submitted in the offer. |  |  |  |
| 4.7.2 | RADIATION PYROMETER : Make, Model and Specification details of Radiation Pyrometer, shall be submitted in the offer. Suitable arrangement shall be provided for fixing the radiation pyrometer on outer body of Co-axial transformer (sl. no. 4.3) for the indication and control of the temperature of the brazing joint during entire brazing process. |  |  |  |
| 4.7.3 | Switch button arrangement clamped on the Co-axial transformer (sl. no. 4.3) shall be provided, for making the power circuit ON and OFF by the operator as and when required. |  |  |  |
| 4.7.4 | FOOT PEDAL SWITCH : Additionally one number foot pedal switch is to be provided for switching ON and OFF the power circuit for brazing operation, in case the controller is not used. |  |  |  |
| 5.0 | GENERAL ARRANGEMENT DRAWING AND LAYOUT OF 45 KW MEDIUM FREQUENCY INDUCTION BRAZING MACHINE : |  |  |  |
| 5.1 | General arrangement drawing of the offered 45 kw medium frequency induction brazing machine incorporating the schematic sketch / technical leaflet of the main machine, chilling unit, mobile stand for hanging the Co-axial transformer, sketch / drawing of brazing inductor (sl. no. 4.4) to braze the component as per Annexure – I & II, total plan area, overall size and weight the frequency converter and chiller unit, total power requirement etc. shall be submitted by the vendor along with the offer. |  |  |  |
| 5.2 | General Arrangement Drawing and Layout as referred in sl. no. 5.1, shall also be submitted by the vendor for BHEL approval, within 4 weeks after placement of Purchase Order. |  |  |  |
| 6.0 | OPERATIONAL & MAINTENANCE MANUAL OF 45 KW MEDIUM FREQUENCY INDUCTION BRAZING MACHINE : |  |  |  |
| 6.1 | 4 sets of Operational & Maintenance manual in English language, will be supplied with 45 kw medium frequency induction brazing machine. Out of these 4 sets, one set manual is to be supplied on CD |  |  |  |
| 6.2 | NOTE : In addition to other information, O&M Manual should contain the following features also : |  |  |  |
| 6.2.1 | Erection Manual |  |  |  |
| 6.2.2 | Detail Operation Manual |  |  |  |
| 6.2.3 | Electrical wiring diagram indicating layout of the cables, plugs, junction box, terminal strips etc. with proper identification numbers |  |  |  |
| 6.2.4 | Alarm list and Fault Diagnostic manual having Description, Cause and Remedy |  |  |  |
| 6.2.5 | Operation and Maintenance Manual of all auxiliary systems / installations / equipments |  |  |  |
| 6.2.6 | Electrical wiring circuit of all auxiliary system / equipments along with the part list / bill of materials of all the components used in the machine |  |  |  |
| 6.2.7 | Safety instruction for the operation of 45 kw medium frequency induction brazing machine including other items such as Chiller Unit etc. |  |  |  |
| 6.2.8 | Operation and Maintenance Manual of all Bought Out items such as Controller (4.7.1), Radiation Pyrometer (4.7.2), Spring Balancer (4.3.3), Chilling Unit (4.5) etc. |  |  |  |
| 6.2.9 | List of “Dos” and “Donts” |  |  |  |
| 6.2.10 | Detailed equipment Parts List along with OEM address, shall be referred in O & M manual. |  |  |  |
| 7.0 | POWER SUPPLY AND ENVIRONMENT CONDITION : |  |  |  |
| 7.1 | BHEL will provide input power supply 3 phase, 3 wire, 415 + / - 15% volt, 50 + / - 3% Hz to Isolator Switch ( BHEL scope) nearby control panel of 45 kw medium frequency induction brazing machine. Cable connection from the isolator switch to the control panel and remaining cabling / wiring for 45 kw medium frequency induction brazing machine and other auxiliaries / equipment, will be in the scope of the vendor. There will be no neutral wire and earthing will be provided through shop structure column. |  |  |  |
| 7.2 | Environment Condition |  |  |  |
|  | 45 kw medium frequency induction brazing machine and other accessories & equipments etc. should be suitable for tropical condition with ambient temperature from 0 to 45 Degree Centigrade and relative humidity maximum 95 % |  |  |  |
| 7.3 | Noise Level |  |  |  |
| 7.3.1 | Maximum noise level of any supplied item, should be below 85dB(A) one meter away from the machine with correction factor for background noise. |  |  |  |
| 7.3.2 | Vendor will demonstrate the noise level as per International Standard like DIN 45636-16, if asked for. |  |  |  |
| 8.0 | OTHER ESSENTIAL ITEMS : |  |  |  |
| 8.1 | Full description along with the quantity and price (in Price Bid) not covered at sl. no. 4.1 to 4.7.4 above but are essential for the operation, control, installation and commissioning of 45 kw medium frequency induction brazing machine & its auxiliaries, shall be offered. Vendor shall submit the details of such items in the offer, if any. |  |  |  |
| 9.0 | PAINTING : |  |  |  |
| 9.1 | Control Panel shall be painted with RAL 7038 Grey paint or mutually agreed |  |  |  |
| 9.2 | NOTE : Vendor will repair the paint with original paint in case the same is damaged in transit / erection, before commissioning of 45 kw medium frequency brazing machine and chilling unit. |  |  |  |
| 10.0 | PRE-ACCEPTANCE OF 45 KW MEDIUM FREQUENCY INDUCTION BRAZING MACHINE AT VENDOR'S WORKS : |  |  |  |
| 10.1 | Pre-acceptance of 45 kw medium frequency induction brazing machine with brazing operation on test samples mutually agreed, will be witnessed by BHEL engineers deputed for training (sl. no. 11.0) at vendor’s works. |  |  |  |
| 10.2 | Test samples required for brazing operation during pre-acceptance test at vendor’s works, shall be arranged by the vendor. |  |  |  |
| 10.3 | Verification of the items of Spares as per Purchase Order |  |  |  |
| 10.4 | Completeness of Operational and Maintenance Manual as per sl. no. 6.1 to 6.2.10 |  |  |  |
| 11.0 | TRAINING : |  |  |  |
| 11.1 | Training of 3 BHEL engineers at vendor’s works, for 5 working days, each covering following fields. |  |  |  |
| 11.1.1 | One engineer for the Operation of 45 kw medium frequency induction brazing machine and other supplied items / auxiliaries. |  |  |  |
| 11.1.2 | One engineer for Mechanical maintenance of 45 kw medium frequency induction brazing machine and other supplied items / auxiliaries |  |  |  |
| 11.1.3 | One engineer for Electrical, Electronics maintenance of various items of 45 kw medium frequency induction brazing machine and other supplied items / auxiliaries |  |  |  |
| 12.0 | ERECTION AND COMMISSIONING OF 45 KW MEDIUM FREQUENCY INDUCTION BRAZING MACHINE : |  |  |  |
| 12.1 | Vendor shall take full responsibility for carrying out the erection, start up, commissioning, testing of each item of 45 kw medium frequency induction brazing machine and other supplied items / auxiliaries etc. |  |  |  |
| 12.2 | Service requirement like unskilled man, power, compressed air and water shall be provided by BHEL during erection and commissioning of 45 kw medium frequency induction brazing machine and other supplied items / auxiliaries etc. Details of these requirements should be informed / discussed by the vendor and agreed with BHEL in advance. |  |  |  |
| 12.3 | EOT crane 2 Ton capacity available in the shop where 45 kw medium frequency induction brazing machine will be installed. In case, higher capacity of crane is required, representative of vendor shall hire from local source for the higher capacity mobile crane. |  |  |  |
| 12.4 | Erection & Commissioning of Chilling unit and other accessories & equipments etc. with all electrical & mechanical connections, shall be the responsibility of the vendor. |  |  |  |
| 12.5 | Compressed air pressure available in BHEL shop, is in the order 4.0 kg / square centimeter, the same can be provided for the erection purpose. |  |  |  |
| 13.0 | JOB PROVING AND FINAL ACCEPTANCE OF 45 KW MEDIUM FREQUENCY INDUCTION BRAZING MACHINE AT BHEL HARIDWAR WORKS : |  |  |  |
| 13.1 | Complete and successful brazing operation of minimum 4 nos. brazing joint as per the attached Annexure – I, shall be taken up under the presence of vendor’s expert deputed for commissioning the machine at BHEL works. |  |  |  |
| 13.2 | Demonstration of all features of 45 kw medium frequency induction brazing machine, chilling unit and other supplied accessories, by the vendor’s representative, to the satisfaction of BHEL |  |  |  |
| 13.3 | Training of BHEL operators in the operation and maintenance of 45 kw medium frequency induction brazing machine including other supplied accessories etc. by the vendor's commissioning experts during the period of commissioning and job proving of 45 kw medium frequency induction brazing machine, at BHEL works |  |  |  |
| 14.0 | GUARANTEE : |  |  |  |
| 14.1 | Guarantee for complete 45 kw medium frequency induction brazing machine including other supplied accessories for 24 months from the date of final acceptance (sl. no. 13.0) of 45 kw medium frequency induction brazing machine at BHEL Works. |  |  |  |
| 14.2 | Any spare required during guarantee period shall have to be arranged by the vendor free of cost (up to BHEL Haridwar stores) and duty levied have to be borne by the vendor. |  |  |  |
| 15.0 | QUALITY PLAN : |  |  |  |
|  | Vendor shall submit Quality Plan within 6 weeks after issue of Purchase Order. |  |  |  |
| 16.0 | PACKING : |  |  |  |
| 16.1 | Sea worthy (in case of foreign vendor) and rigid packing shall be provided for all the items of 45 kw medium frequency induction brazing machine including other supplied items in wooden boxes to avoid any damage / loss in transit. All small and loose items and spares, should also be suitably packed in separate wooden boxes with proper identification tag. |  |  |  |
| 17.0 | GENERAL INFORMATION : |  |  |  |
|  | All the information attached with the tender documents, are exclusive property of BHEL Haridwar. Under any circumstance, the information as stated above, must not be passed on to any third party without prior permission of BHEL Haridwar and must not be used directly or indirectly detrimental to the interest of BHEL. |  |  |  |
| 18.0 | SPARES : |  |  |  |
| 18.1 | Set of recommended spares indicating the category of essential / optional and along with the quantity of the items required for 2 years trouble free operation of 45 kw medium frequency induction brazing machine and other accessories, shall be submitted in the offer. Following items shall also be included in the above referred list. : Fuse : Relays : Incandescent lamp : Electronic Cards |  |  |  |
| 18.2 | Price of each item unit wise, listed under the Spares, shall be furnished in Price Bid |  |  |  |
| 18.3 | Validity of offer for the spares : 2 years after Final Acceptance of 45 kw medium frequency induction brazing machine (refer sl. no. 13.1) at BHEL works |  |  |  |
| 19.0 | QUALIFYING CONDITION : |  |  |  |
| 19.1 | Offer of only those vendors will be considered who have supplied and commissioned at least one number 45 kw or above capacity medium frequency induction brazing machine for brazing of pieces of copper to copper and functionally similar for the brazing operation as described in sl. no. 1.0 of this Technical Specification, in last 5 years and the same is working satisfactorily for one year after commissioning the machine. Supporting documents as mentioned in sl. no. 19.3.1 to 19.3.4, must be submitted along with the offer. |  |  |  |
| 19.2 | Vendors shall submit Performance Certificate of Medium Frequency Induction Brazing machine of 45 kw and above for brazing of copper items. Performance certificate of supplied induction heating equipment for the processes such as Annealing, Hardening, Tempering, Bonding, Drying etc. are not acceptable and the offer will be ignored. |  |  |  |
| 19.3 | Name of the company / customer, where 45 kw and above capacity medium frequency induction brazing machine (s) has (have) been commissioned |  |  |  |
| 19.3.1 | Complete postal address of the company / customer |  |  |  |
| 19.3.2 | Name and designation of the contact person of the company / customer |  |  |  |
| 19.3.3 | Year of commissioning |  |  |  |
| 19.3.4 | Application for which the machine (s) has (have) been commissioned |  |  |  |