Annexure - I

Sub:- Preventive and Breakdown maintenance of critical machines and plants  in TCB

and UHV Transformer Block.

Scope of Work.

1.  	All activities of preventive maintenance including electrical inspection list and mechanical inspection list is to be done of critical machines & plants as per list enclosed. Preventive maintenance of 81 Nos. of Critical machines & plants of TCB and UHVTransformer Block to be done twice in a year (List enclosed).PM of CNC Cropping line,New CNC,Slitting line and new Slitting line has to be done four times in a year. Total nos of PMs in a year=170

2.	The details of work to be done is as listed below:

2.1	Work will be covered under two category namely electrical & mechanical activities which will be certified by the supervisor of the group. The activity called the “inspection list” consists of the following.

2.2	Electrical Inspection List:-

1)	General cleaning of all electrical components & control panels.

2)	Check / rectify all motors insulation using megger .

Check / rectify earth wiring.

Check / rectify main switch fuses & limit switches.

Clean contacts & relays using fine emery sheets and CTC.

Check / rectify control gear, starter, brushes etc.

Clean resistors, transformers etc.

Check / rectify bearings of all motors, rotating parts and use oil grease for proper lubrication.

- Check / rectify rectifiers.

- Check / rectify over load relays.

- Check / rectify loose connections and tighten them.

- Replaced damaged flexible conduits and cables.

- Check / rectify pendent control system.

- Check / rectify all clutches of feed and rapid drive.

- Check / rectify all safety interlocks.

- Check / rectify all brake drums and circuits.

- Replacement carbonized non operative contactor if necessary.

2.3	Mechanical Inspection List:-

- Clean thoroughly and do visual inspection.

- Check / rectify motor pulleys and bearings.

- Dismantle and refit diaphragm valves.

- Check / rectify all pneumatic and hydraulic valves for proper operation (Opening and closing as per operational requirements).

- Check / rectify gas regulating valves.

- Check / rectify operator complaints if any and rectify the same.

- Check / rectify machine for functional operation and verify with the operator.

- Check / rectify belt pulleys & tension of belts, replace the belt if necessary.

- Check / rectify for air & hydraulic leakage if any and rectify the same.

- Clean all hydraulic and air filters.

- Check / rectify water cooling line , pressure switch  and clean the line incase of any choking.

3. 	Lubrication

- Check / rectify reduction gear box oil and top up if necessary.

- Check / rectify hydraulic oil and maintain oil level. Replace the oil 	after every 2000 working hours.

- Use oil gun/grease to lubricate the same.

- Check / rectify all plumber block, chain motor & bearings for the 	lubrication & use grease to lubricate the same.

- Check / rectify all slides & lubricate the same.

- In case of large oil reservoirs, check the condition of oil. Get it tested 	and keep record of the same . run moisture filters and check for 	moisture coolant.

4.	Inspection of work

Work will be checked and supervised by BHEL 	Supervisors/Engineers.

5.	Keeping records

A separate register is to be maintained by the contractor / firm to keep the records mentioned above as well as the No. of machines done on each working day. This will be checked by BHEL Engineers / Supervisors every week. Apart from the above a preventive maintenance card issued by WE&S division, Form No. 4253 is to be filled and this is to be certified by shop in-charge at the completion of each machine / plant.

6	You have to attend the breakdown maintenance of  mechanical and 	electrical nature whatsoever, may be during the operation of the 	machines.

7.	Any suggestion / addition in the scheduled PM work is to be implemented by the contractor as desired by BHEL Engineers.

8.	Man power

The working group for the above preventive and breakdown maintenance activity should consists of minimum manpower of skilled workers(11 nos) , semi skilled (01)and unskilled workers(08 nos).

*The contractor may depute more than the minimum technicians as 	appropriate to carryout maintenance work of Preventive 	and 	breakdown Maintenance of critical machines and plants in TCB 	area.If required, staff are to be detained on extra time beyond 4.00 	PM or  Sunday or Holiday. For this BHEL will not pay any thing 	extra

9.	BHEL scope of supply

BHEL will supply consumables like lubricants & cotton waste, emery paper etc. Any spares required for the PM work will also be supplied by BHEL free of cost.

10.	Contractor should have all the maintenance tools and tackles such as set of spanners, allen keys, oil & grease gun, megger , multimeter, clamp meter etc. however special tools will be provided by BHEL.

11.    If required, personnel are to be detained on extra time beyond 4.00 PM or Sunday or Holiday. For this BHEL will not pay any thing extra.

DYMGR (FRX)