Annexure-I

Sample Report

Measurement Report

Date: 27.07.07

Time: 11:31:35

Component : Casing 500MW

Drg. No : 0-10307-41007

Project Name : Raichur

Machine: CNC Horizontal Borer 160 (Model name/Layout number of machine)

*******************************************************************************

*  Note: ‘0’ in Error denotes “Measured Values are within tolerance”          *

*******************************************************************************

Description          Nom.    Measured   Dev.from     Tol.    Tol.     Error

Size    Value      Nom.Size    upper   lower    (Out-Tol)

(+)            (-)

SLOTWIDTH:           40      40.0345    0.0345       0      -0.05     0.0345

SLOTDEPTH:           24.6    24.5548    -0.0451      0      -0.13     0

GROOVEWIDTH:         39      39.0755    0.0755       0.1    -0.1      0

---------           ---       ----       ---         ---     ---     ---

---------           ---       ----       ---         ---     ---     ---

---------           ---       ----       ---         ---     ---     ---

MAIN INNER BORE     121.98  122.1065    0.1265       0.04   -0.02    0.0865

OUTER TAPER ANGLE:    1.475   1.4943    0.0193       0.05   -0.05    0

COUNTER BORE:        34.847  34.9773    0.1303       0.04   -0.02    0.0903

(DET-1)

Note:

- The Nominal size means the nominal value as mentioned in the drawing without application of any tolerance or any adjustment for mid value.

- The Tolerance values should be specified in Upper and Lower tolerance as mentioned in the drawing and not in any other tolerance band with adjustment.