| fnukad ,oa gLrk{kj SIGN & DATE |  |  |  | PRODUCT PURCHASE SPECIFICATION |  |  |  |  | # Capital |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  | ## Ik`"B |  |  | ## 1 | ## dk |  | ## 2 |
|  |  |  |  |  |  |  |  |  | ## Page |  |  | 1 | of |  | 2 |
| SUPERSEDES INVENTORY NO. |  | CO2 Ambient Air Monitor - GENERAL: Equipment is required to measure CO2 in Ambient Air for Green House Gas, Industrial applications, ambient air monitoring of CO2 assessment. - SPECIFICATION OF EQUIPMENT: - Portable for carrying to site for measurement of CO2 in ambient air based on non- dispersive infra-red principle. - Measuring range of CO2: 0 - 5000 PPM - High resolution LCD Display - Built – in Sampling Pump - Visual and audible alarm - Integrated data logging capability - Accuracy : 1% , Linearity : 1% FSD - Air filter – Filtered sample line - Automated zero at regular intervals for correcting sample cell contamination, source and detector. - Data Logging: Built – in data logger, Minimum 512K battery backed RAM (1000 records). Data Down loading software, RS232 Data Transfer Cable etc - Power Supply : 220 V +- 10 V AC with option of battery operation and Battery Charger - Carrying bag - Portable calibration gas cylinder of known concentration. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| lkexzh lwph la[;k dks vf/kdzfer djrk gS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| COPYRIGHT AND CONFIDENTIAL The information on this documents is the property of Bharat Heavy Electrical Limited.It must not be used directly or indirectly in any way detrimental to the interest of the company |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| LoRokf/kdkj ,oa xksiuh; bl izys[k esa nh xbZ lwpuk Hkkjr gsoh bysfDVªdYl dh lEifRr gS bldk izR;{k ,oa vizR;{k :Ik ls fdlh Hkh rjg iz;ksx] tks fd daiuh ds fgr esa gkfudkjd gks u fd;k tk, A |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| gLrk{kj ,oa fnukad SIGN & DATE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | uke NAME |  | fnukad ,oa gLrk{kj SIGNATURE & DATE |  |  |  |  |  |
|  |  |  | A. Goel |  |  | vuqoknd TRANSLATED BY |  | Bhoj raj |  |  |  |  |  |  |  |
|  |  |  |  |  |  | fuekZ.kdrkZ WORKED BY |  | Bhoj Raj |  |  |  |  |  |  |  |
| Lkkexzh lwph la[;k INVENTORY NO. |  |  |  |  |  | tkapdrkZ CHECKED BY |  | A. Goel |  |  |  |  |  |  |  |
|  |  | Lger foHkkx AGREED DEPTT. | uke NAME |  | fnukad ,oa gLrk{kj DATE & SIGNATURE | i;Zos{k.kdrkZ SUPERVISED BY |  | A. Goel |  |  |  |  |  |  |  |
|  |  |  |  |  |  | Lohd`fr % APPROVED : |  |  |  |  |  |  |  |  |  |
|  |  | REV.NO. 1 |  |  |  | fuekZ.k % PREPARED : | T+kkjh % ISSUED : |  |  |  | fnukad % DATE : |  |  |  |  |
|  |  | Dt. 1.11.08 |  |  |  |  |  |  |  |  |  |  |  |  |  |

| *+- fnukad ,oa gLrk{kj SIGN & DATE |  |  |  | PRODUCT PURCHASE SPECIFICATION |  |  | # Capital |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | ## Ik`"B | ## 2 |  | ## dk | ## 2 |
|  |  |  |  |  |  |  | ## Page | 2 |  | of | 2 |
| SUPERSEDES INVENTORY NO. |  | 3.0 General Terms & Conditions 3.1 Installation & Commissioning: Instrument should be commissioned at PCRI, BHEL Haridwar (U. K.)) to the satisfaction of customer by vendor. 3.2 Document : (1)Three hard copies O&M manual in English language giving complete technical information for operation. (2) Calibration certificate from recognized agency / laboratory. 3.3 Guarantee: The equipment shall be guaranteed for any breakage / malfunctioning of the apparatus for a period of 18 months from the date of commissioning or 24 months from the date of dispatch, whichever is earlier. In case of any breakdown during this time period, the guarantee period will get extended accordingly. TC & GC certificate to be provided. 3.4 After Sales service: The supplier shall give along with the offer a list of components/boards that he feels that the user should maintain and cost of each item. For items not on this list. 3.5 Users : Along with the offer, the supplier shall give a list of users of similar equipment in India. It should contain detailed name, address & department, telephone number, e-mail address, model, year of supply etc. 3.6 Packing & Dispatch: The equipment / accessories shall be packed in a strong suitable water proof/vibration proof packing boxes capable of bearing road transit hazards. Packing box shall be properly identified and marked with BHEL's(India) Purchase Order Number. 3.7 All tools, tackles, gases, commissioning spares, consumables, calibration standards to suit Indian conditions, maintenance tool kit necessary for installation & commissioning & calibration of equipment at site be included in th eoffer. 4.0. Cross referred Document: Nil |  |  |  |  |  |  |  |  |  |
| lkexzh lwph la[;k dks vf/kdzfer djrk gS |  |  |  |  |  |  |  |  |  |  |  |
| COPYRIGHT AND CONFIDENTIAL The information on this documents is the property of Bharat Heavy Electrical Limited.It must not be used directly or indirectly in any way detrimental to the interest of the company |  |  |  |  |  |  |  |  |  |  |  |
| LoRokf/kdkj ,oa xksiuh; bl izys[k esa nh xbZ lwpuk Hkkjr gsoh bysfDVªdYl dh lEifRr gS bldk izR;{k ,oa vizR;{k :Ik ls fdlh Hkh rjg iz;ksx] tks fd daiuh ds fgr esa gkfudkjd gks u fd;k tk, A |  |  |  |  |  |  |  |  |  |  |  |
| gLrk{kj ,oa fnukad SIGN & DATE |  |  |  |  |  |  |  |  |  |  |  |
| Lkkexzh lwph la[;k INVENTORY NO. |  | REV.NO. 1 |  |  | fuekZ.kdrkZ WORKED BY | Bhoj Raj |  |  |  |  |  |
|  |  |  |  |  | TkkapdrkZ CHECKED BY | A. Goel |  |  |  |  |  |