TECHNICAL   SPECIFICATIONS                          STT-0037

HEAVY  DUTY  DOUBLE  ENDED  PEDESTAL  GRINDER

1. PURPOSE:- Double Ended Pedestal Grinders are used to sharpen high speed steel cutting tools and carbide tools used on the Lathes, Milling machines, Vertical boring and Horizontal boring machines..

2. TECHNICAL  DETAILS - Double Ended Pedestal Grinder should be for heavy duty work with the following details:-

A. Grinding wheel size                                                                        :  400x 50x 50.8 mm

B.Grinding wheel with medium grains on one side ( for HSS Tools) : vendor to confirm

and Carbide tip grinding wheel ( for Carbide Tools) on other side.

C. Dia of the shaft                                                      :                          40 mm approx.

D. Centre height from ground                                    :                           900 mm approx.

E. Distance between the wheels                                :                           600 mm approx

F. Space required for the installation                         :                         vendor to specify.

G. Electric Motor  with  its make ( single speed)       :  5HP, 3 Phase,  50 HZ, 400/440 Volt,

1500 rpm  approx. TEFC Induction motor

(It should be of reputed make e.g. L&T, Siemens,Crompton etc )

H.  Starter (suitable mounted on m/c near to work place) :   DOL  Push  Button starter of reputed

make.

I. Grinder should be fitted with all standard accessories like :- eye shield, wheel guard and heavy duty work table on either  side of grinder etc .

J. Grinder should be fitted on heavy cast iron pedestal (Height)  :   vendor to specify

k. Type, size, no and make of the bearing used                             :   vendor to specify

3.  ACCESSORIES  NEEDED WITH THE  GRINDER:

- Machine Light :- M/c  lamps with bulbs fitted for both  grinding wheel separately with flexible/ movable arrangement and  separate ON/OFF system for both lamps to operate on 230 V, single phase AC supply.

- 2 nos. antirust trays fitted below both the sides of grinding   wheels so that the used coolant/ dust can be collected and drained through flexible pipes. A container to provided for collection of coolant and dust partials, so that it should not spread over the ground.

- Erection and commissioning will be done by the supplier at BHEL Hardwar  .( without civil work). Foundation drawing, Foundation Bolts, nuts and washers will be supplied by the supplier.

- Vendor to quote for recommended spares item wise for two  year trouble free working including bearings and grinding wheels.

- Vendor to give tool kit with the supply.

- Offer should be complete in ready to operate condition.

- Vendor to give complete catalogue with the offer as well as separate complete catalogue , operational & maintenance manual   05 sets showing part drawings, part list & circuit diagrams along with the supply of  grinder.

- T/C, G/C required.

Guarantee should be from the date of commissioning.

- Vendor to give point wise reply as per above  specifications.