BHARAT HEAVY ELECTRICALS LIMITED

UNIT: JHANSI

(WORK. ENGG. & SERVICES DEPTT.)

EQUIPMENT	:	MOTORISED SLIDING DOORS

ITEM NO		:	LE-3/4/6258/1 & 2

LOCATION		:	LOCO AREA

QUANTITY		:	2 SETS

SPECIFICATION	:	10572/R-2

1.0  SPECIFICATION:

Supply and fixing of motorised sliding doors for shot blast plant at LOCO area.

2.0  FABRICATION:

The main gate for the entrance of size 6000 x 6200 mm height at plant shall be made in two segments of 3000 x 6200 mm each. Each segment shall slide side wise on a set of rollers provided at top of the doors. At the bottom side of the door a guide shall be provided for the smooth movement of the doors. The movement of the doors at top will be done in a specially designed beam capable of bearing the load of the doors. This beam will be mounted on two fabricated pillars provide with the system. The main gate shall be fabricated form M.S Sq. pipe and M.S sheets specially bend in ‘U’ shape to give extra strength to the gate and withstand wind pressure. A service door of size 750 mm x 2000 mm shall be provided in one segment of the door per set.

3.0  GATE MOVEMENT:

The sliding movement of the gate shall be effected by suitable capacity of motor coupled to the gearbox. Wire winding mechanism shall be mounted on top gate-sliding beam.

4.0  CONTROL PANEL:

The control panels for the system shall consist of contactors for motor, overloads relay, protection fuses, limit switches and other standard panel accessories. When OPEN push button is pressed the doors shall open side ways and shall stop as soon at the push button in released. If the “OPEN” push button is kept pressed, the doors shall stop by limit switches provided at the. Similarly, the doors shall close till the “CLOSE” button is pressed and shall stop the movement when the button is released. If the button is kept pressed the doors shall stop by limit switches provided at the centre. There shall be audio visual alarm during door movement.

5.0  GENERAL:

1.	Technical literature shall be submitted along with the offer.

2.	Details of motor shall be given with the offer.

3.        Make of the motors ( preferably M/s Alstom/ ABB/Crompton) & gear box ( preferably

M/s Redicon/ Greaves/ Laxmi/ shanthi).

4.         The gates should be painted with one coat of red oxide primer & two coats of finishing

paint before dispatch

5.	Four set of operation and maintenance manual shall be provided with the supply consisting of following.

- Operation and maintenance instruction and Do’s / Don’ts

- Electronic and electrical circuit diagram.

6.       Erection and Commissioning of door shall be done by supplier at BHEL, Jhansi. All

tools tackles & manpower shall be in suppliers scope.

6.0   DETAILS TO BE SUBMITTED WITH THE OFFER:

- Dimensioned outline general arrangement drawing

- Details of safety devices / guards provided.

- Electrical schematic diagram.

- Total electrical load.

- Quotation with item wise prices of spares for 2 years normal working.

- Details of standard & optional accessories offered.

7.0	QUALIFYING CONDITIONS

Only those vender, who have experience of manufacturing, supplying and commissioning such gates or any other installations having gates of similar or bigger dimensions will qualify to quote for this tender subject to the submission of sufficient documentary support to justify their claim.

| Date | Prepared By | Checked By | Approved By |  |
| --- | --- | --- | --- | --- |
|  | Dy. Mgr (WEX) | Mgr.(WEX) | DGM.(WEX) | AGM(WEX) |