BHARAT HEAVY ELECTRICALS LIMITED

UNIT: JHANSI

(WORK. ENGG. & SERVICES DEPTT.)

SCHEME                		 :           Phase-II

EQUIP	MENT			:	Elevating platform for Bushing Assy.

ITEM NO			:	LE- 1/7/6266

LOCATION			:	Power Assy. Shop (Bay-9)

QUANTITY			:	1 Nos

SPECIFICATION NO.	:	10577 /Rev-1

| SNO | DESCRIPTION FOR BHEL REQUIREMENT |  |  |  | SPECIFIED / TO BE CONFIRMED BY |
| --- | --- | --- | --- | --- | --- |
| 1.0 | PURPOSE & WORKPIECE MATERIAL |  |  |  |  |
| 1.1 | Purpose: |  |  |  | Vendor to confirm |
|  | Elevating platform is required for making high voltage connections during assembly and testing of power transformer, reactors, CTs, CVTs & Bushings. |  |  |  |  |
|  | Quantity : 1 No. |  |  |  |  |
| 1.2 | Job to be tested : Power transformers, Reactors, CTs, CVTs and Bushings upto 500 kV class |  |  |  | Vendor to confirm |
| 2.0 | SPECIFICATION: |  |  |  |  |
| 2.1 | (a) Maximum working height with full vertical lift :16 m (Lifting Height is 14.5 m, working height which operator can reach is 16m) (b) Length of working platform (fix) : 1.5 m (c) Width of the working platform : 1.1 m (d) Fencing height of working platform : 1.1 m (e) Adjustable height of working platform: 3.4 m to 14.5 m (f) Lifting/lowering speed of platform : 5 m/min |  |  |  | Vendor to confirm |
| 2.2 | Working radius of extendable platform( around the center of mast) : 1.6 to 4.5 m |  |  |  | Vendor to confirm |
| 2.3 | Capacity of main platform : 300 kg. |  |  |  | Vendor to confirm |
| 2.4 | Capacity of extendable platform : 150 kg. |  |  |  | Vendor to confirm |
| 2.5 | Minimum height from the ground level : 3.4m |  |  |  | Vendor to confirm |
| 2.6 | Total ground area of the base including the outer riggers : 10 m Sq.(max.) |  |  |  | Vendor to confirm |
| 2.7 | The following basic features should be in-built in the equipment |  |  |  |  |
| 2.7.1 | Platform |  |  |  | Vendor to confirm |
| 2.7.1.1 | Platform have capacity to carry three person |  |  |  | Vendor to confirm |
| 2.7.1.2 | Platform shall always remain horizontal with proper hand railing for safety |  |  |  | Vendor to confirm |
| 2.7.1.3 | One part of platform is capable of extending |  |  |  | Vendor to confirm |
| 2.7.1.4 | The platform shall have 360° continuous rotation in both clock wise and anti clock wise at all working heights (optional) |  |  |  | Vendor to confirm |
| 2.7.2 | Telescopic mast |  |  |  |  |
| 2.7.2.1 | Platform will be raised or lowered by telescopic mast |  |  |  | Vendor to confirm |
| 2.7.2.2 | Working : Hydraulic |  |  |  | Vendor to confirm |
| 2.7.2.3 | The mast shall be able to take platform to any working point with a radius of 3m up to the height of 16m |  |  |  | Vendor to confirm |
| 2.7.2.4 | Maximum vertical leaning (sway) when lifted to maximum height: 150 mm |  |  |  | Vendor to confirm |
| 2.7.3 | Infrastructure (Base of the Elevator) |  |  |  |  |
| 2.7.3.1 | Stay guides for rigidity and stability, to support platform, if applicable |  |  |  | Vendor to confirm |
| 2.7.3.2 | All equipments like hydraulic power pack electrical control equipments should be fixed on the infrastructure |  |  |  | Vendor to confirm |
| 2.7.3.3 | Panel with parallel controls should be provided on infrastructure where the movement of the platform can be controlled |  |  |  | Vendor to confirm |
| 2.7.3.4 | Manual controls provided at ground level |  |  |  | Vendor to confirm |
| 2.7.3.5 | Jacks/ Out riggers fitted on all four corners of infrastructure, if required |  |  |  | Vendor to confirm |
| 2.7.4 | Drive |  |  |  |  |
| 2.7.4.1 | Motorized |  |  |  | Vendor to confirm |
| 2.7.4.2 | Moving should be done through pneumatic/solid tyres |  |  |  | Vendor to confirm |
| 2.7.4.3 | Steering arrangement alongwith brakes to maneuver the movement |  |  |  | Vendor to confirm |
| 2.8 | Applicable standard : European CE - standards or equivalent |  |  |  | Vendor to confirm |
| 2.9 | SAFETY ARRANGEMENTS: |  |  |  |  |
|  | Following safety features in addition to other standard safety features should be provided on the system: |  |  |  | Vendor to confirm |
|  | 1.System should have adequate and reliable safety interlocks / devices to avoid damage to the system, work piece and the operator due to the malfunctioning or mistakes. |  |  |  | Vendor to confirm |
|  | 2. All electronics parts are housed in shielded enclosures. |  |  |  | Vendor to confirm |
|  | 3. Safety feature of the platform corresponding to German Regulation VBG14 or equipment. |  |  |  |  |
| 2.10 | ENVIRONMENTAL PERFORMANCE OF THE SYSTEM: |  |  |  |  |
|  | The Instrument shall conform to following factors related to environment : |  |  |  |  |
|  | If any safety / environmental protection enclosure is required it should be built in the system by the vendor. |  |  |  | Vendor to confirm |
| 3.0 | ACCESSORIES: |  |  |  |  |
| 3.1 | Mechanism for auto steering of the platform |  |  |  | Vendor to specify |
| 4.0 | SPARES: |  |  |  |  |
| 4.1 | All necessary spares for two years trouble-free operation. |  |  |  | Vendor to Confirm |
| 5.0 | DOCUMENTATION : 4 copies of each of the following shall be supplied along with the system |  |  |  | Vendor to Confirm |
| 5.1 | Instruction manual |  |  |  | Vendor to Confirm |
| 5.2 | Maintenance manual along with circuit diagrams and schematics |  |  |  | Vendor to Confirm |
| 5.3 | Test & Guarantee Certificate. |  |  |  | Vendor to Confirm |
| 5.4 | Calibration certificate |  |  |  | Vendor to Confirm |
| 5.5 | List of customer where equipment has been supplied earlier |  |  |  | Vendor to Confirm |
| 5.6 | Conformance certificate for safety standards |  |  |  | Vendor to Confirm |
| 6.0 | SYSTEM INSPECTION: |  |  |  |  |
| 6.1 | Inspection at suppliers works before dispatch |  |  |  | Vendor to Confirm |
| 6.2 | Demonstration of all features of the Elevating platform system and all Accessories. |  |  |  | Vendor to Confirm |
| 7.0 | TRAINING: |  |  |  |  |
| 7.1 | BHEL testing engineer and maintenance person should be trained at BHEL Works in the area of (a) Use of all system Features, programming for Measuring Systems & supplied accessories etc. (b) Electrical, Electronic & maintenance for system & other supplied equipments |  |  |  | Vendor to confirm |
| 7.2 | Competent, English speaking experts shall be arranged by the vendor during training for satisfactory & effective training of BHEL personnel. |  |  |  | Vendor to confirm |
| 8.0 | ERECTION & COMMISSIONING |  |  |  |  |
| 8.1 | Supplier to take full responsibility for carrying out the start up and commissioning |  |  | Vendor to confirm |  |
| 8.2 | Successful proving of BHEL components by the supplier shall be considered as part of commissioning. |  |  | Vendor to confirm |  |
| 8.3 | Demonstration of all features of the Elevating platform system & all accessories to the satisfaction of BHEL for their efficient and effective use. |  |  | Vendor to confirm |  |
| 8.4 | Demonstration by actual use of all supplied components and accessories to their full capacity. |  |  | Vendor to confirm |  |
| 8.5 | Commissioning spares, required for commissioning of the system within stipulated time, shall be brought by the supplier on returnable basis. |  |  | Vendor to confirm |  |
| 8.6 | Schedule of Commissioning shall be submitted with the offer. |  |  | Vendor to confirm |  |
| 8.7 | Charges, duration, terms & conditions for E&C should be furnished in detail separately by vendor along with offer. |  |  | Vendor to confirm |  |
| 9.0 | AMBIENT CONDITIONS & THERMAL STABILITY : |  |  |  |  |
| 9.1 | Total System and all supplied items should work trouble free and efficiently under following operating conditions and should give specified accuracies. Power Supply: Voltage: 415 V ,3-Phase Frequency: 50 Hz  1.5 Hz Operating Humidity : 35% to 80% Operating Temperature : 0°C to 45°C (Vendor to confirm that system is suitable for above and details of provisions made on the system for the same are to be furnished by Vendor. Alternate supply arrangement/ drive systems may also be quoted. |  |  | Vendor to confirm |  |
| 9.2 | Thermal Stability of the complete system keeping in view specified Ambient Conditions and accuracy requirements of BHEL components and trouble free operation of the system should be ensured by vendor. (Vendor to confirm that system is suitable for above and details of provisions on the system for the same should be furnished by Vendor) |  |  | Vendor to confirm |  |
| 10.0 | PACKING: |  |  |  |  |
| 10.1 | Air worthy/Sea worthy & rigid packing for all items of complete system, all Accessories and other supplied items to avoid any damage/loss in transit. When system is despatched in containers, all small loose items shall be suitably packed in boxes |  |  | Vendor to confirm |  |
| 11.0 | GENERAL : The vendor should submit the following information: |  |  |  |  |
| 11.1 | Equipment Model |  |  | Vendor to confirm |  |
| 11.2 | Total connected load (KVA): |  |  | Vendor to confirm |  |
| 11.3 | Total weight of the system |  |  | Vendor to confirm |  |
| 11.4 | Paint/ Colour of platform verdigris Green to IS 5/1961. |  |  | Vendor to confirm |  |
| 11.5 | Vendor to submit, along with offer, the reference list of customers where similar systems have been supplied mentioning the customer, Machine Model, major specifications of the supplied system, Year of Supply etc |  |  | Vendor to confirm |  |
| 11.6 | Detailed catalogues , sketch/ photographs of the m/c and accessories/ attachments should be submitted with the offer. |  |  | Vendor to confirm |  |
| 11.7 | GURANTEE:- 24 months from the date of acceptance of the machine irrespective of supply, erection & commissioning |  |  | Vendor to confirm |  |

12. MANDATORY DOCUMENTS REQUIRED TO BE SUBMITTED WITH THE OFFER-

- Dimensioned outline general arrangement drawing/catalogue  with write up of offered platform with list of standard accessories.

- Details of safety devices / guards provided.

- Electrical schematic diagram.

- Total electrical load.

5.   Quotation with item wise prices of spares for 2 years normal working.

- Details & list of standard & optional accessories offered. All standard accessories should be

supplied with the platform without extra cost.

- Catalogues / leaflets of machine offered .Operation & maintenance manual ( 3sets) & tool kit to be   provided with machine/platform.

8.  Make & details (rating) of the control panels & other items like motors, gears, pumps,

hydraulic systems etc.

9.  Charges for the Erection & commissioning of the machine/platform at BHEL Jhansi should be

quoted in offer separately.

10.  Make of bought out items- Bought out items of following make are to be provided &

mentioned in offer..

(a) Motor-                           Siemens/ Alstom / GEC/ABB/Crompton make

(b) Gear box-                      Greaves / Relicon / Shanti make

(c) Hydraulic system-          Rexroth / L& T make

(d) Switch gear-                   ABB/Siemens/L&T

(e)  Bearings-                      SKF/NBC

13. GENERAL CONDITIONS-

- The machine/platform should be painted with one coat of red oxide primer & two coats of finishing paint before dispatch. Colour of the finishing paint should be verdigris Green to IS 5/1961.

- Machine/platform will be inspected & tried by our engineer prior to dispatch. Provision is to be made for deputation of commissioning engineer for erection & commissioning of the machine/platform at BHEL , Jhansi.

- The performance of the machine/platform should be demonstrated by the supplier after its successful commissioning at our works. Training to our operators should be given by supplier’s representative.

4.	BHEL reserves the right to verify information submitted by vendor. In case the information is found to be false / incorrect , the offer shall be rejected.

14. QUALIFYING CONDITIONS

Only those vender, who have supplied and commissioned 1 No such  platform in the past ten years and the same is presently working satisfactorily for more than one year (more than six month if supplied to BHEL) after commissioning should quote. The following information is to be submitted by the vender about the companies where similar platforms have been supplied. This is required from all venders for qualifications of their offers.

- Name of the customer/ company where similar platforms  is installed

- Complete postal address of the customer

- Year of commissioning

- Name and designation of the contact person of the customer

- Phone , Fax No. and e-mail address of the contact person of the customer

- Performance certificate from the customer regarding satisfactory performance of  the platforms supplied to them

| Drafted By | Checked By | Approved By |  |  |
| --- | --- | --- | --- | --- |
| V. Kevre Dy.Mgr(WEX) | I. Mathur Mgr(WEX) | S.J.C. DGM(WEX) | A.K.S. AGM(FW) | V.K.A. AGM(TRM/TRP) |