SPECIFICATION OF VERTICAL MILLING MACHINE  EMT-6751

- PURPOSE:

The machine shall be required for machining of small components of various insulating materials such as epoxy based glass epoxy laminates, phenolic based cotton fabric laminates.

- MACHINE SPECIFICATIONS:

2.1      SPINDLE

2.1.1   Spindle position                              :  Swiveling type Vertical

Swiveling  angle +/- 45 deg.

2.1.2   Spindle taper                                  :  ISO 40

2.1.3   Spindle motor power                      :  5.5 KW or more

2.1.4   Spindle speed range                        :  35.5 to 1800 r.p.m. (approx.)

2.1.5   Vertical movement of quill             :  70 mm or more

2.1.6    No’s of speed                                 :  18

2.1.7    Min/Max distance from lower       :   65/535 mm

face of milling spindle to table

top face.

2.1.8     Distance from spindle centre to     :  350mm

column face.

2.2       TABLE

2.2.1   Overall dimension of  table              :  1520x310mm

( lengthxwidth )

2.2.1    Overall clamping area                     :  1350 mm x 310 mm approx.

2.2.2    Max. permissible load on table       :   350 kg max.

2.2.3    The table top should have longitudinal T-slots. The dimension of

T- slots is to be specified by supplier.

POWER OPERATED TRAVERSE

2.2.4    Longitudinal traverse                        :  800 mm or more

2.2.5   Vertical traverse                                :   400  mm or more

2.2.6    Cross traverse                                   :  265 mm or more

2.2.7     Minimum distance from left hand    :  250mm

table end to centre of spindle.

2.3                   FEED RATES

2.3.1    No’s of feed                                       :  18

2.3.2    Rapid traverse rate

Longitudinal & cross                         : 3200 mm/min or more

Vertical                                              :  800 mm or more

2.3.3    Feed rates

Longitudinal & cross                         :  16-1800 mm / min(approx.)

Vertical                                              :  4-200 mm/min(approx.)

- GENERAL FEATURES OF MACHINE:

  - The specification of feed drive motors with make , model no. max. & nominal torque should be specified.

  - Preferably the guideways of all axes should have telescopic type protective covers with       adequate sealing on joints to prevent seepage of glass dust generated during milling operation. Guide ways   should be of hardened steel with hardness HRC60 or more.

  - The machine should be capable of high stock removals with close tolerances on work

piece. For reference, Metal  removing rate in cc/min to be specified.

  - The machine must rigid streamlined and vibration free construction. All gears must      be case hardened and ground and all the slides should be rigid to withstand heavy stock

removal. It should have independent main drive and feed drive motors.

  - A suitable , preferably centralized lubricating system for lubrication of all moving /

rotating elements should be there preferably with interlock. Open lubrication parts

should be protected from glass dust.

  - Each axis should have electrical limit switches interlocked with feed / rapid drive as

well as mechanical stopper.

  - Each axis should have hydraulic clamps interlocked with feed / rapid drive.

- ATTACHMENT & ACCESSORIES

The following accessories should be quoted separately giving their individual prices. (Catalogs

of all these accessories should be submitted by the supplier along with the offer)

  - Machine vices with fixed base, width of jaws 160 mm.

  - Universal swivel vices with fixed base , width of jaw 160 mm.

4.3    Universal vice 160mm.

4.4     REDUCTION MILLING SOCKETS

4.5    TANG TYPE :

ISO40 / MT 2

ISO40/ MT 3

4.6      DRAW BOLT TYPE:

ISO 50 / MT 2

ISO 50 / MT 3

4.7        COLLET  ADOPTER

4.7.1    Collet Adopter ISO 40

4.7.2    Set of collets (metric) of sizes (4,6,8,10,12,16,20,25,30 & 32 mm)

4.8     STUB ARBORS WITH KEY                 ISO40X22X25

ISO40X32X25

4.9     Work piece illumination light.

4.10    Foundation parts.

4.12    Any other standard / optional attachment which will increase the productivity /

performance of the machine should be quoted with their individual prices.

5.0           POWER SUPPLY / ENVIRONMENT CONDITION

5.1  The machine should be suitable for operation in the following conditions:-

5.2   Power supply for the machine : 415 V + 10% / -15% , 3 phase, 50 Hz +/-3%

5.3  Temperature :     5 to 45 deg.C.

5.4   Relative humidity :    95% max.

5.5  Compressed air :  4 to 6 Kg / sq.cm

5.6    Other conditions: Similar to tropical country.

5.7    Dust laden atmosphere due to generation of glass dust during machining.

6              SPARES ( OPTIONAL )

6.1  Mechanical & hydraulic spare list should have followings:

6.1.1 One number of each type of the following items used in hydraulic

lubrication .

6.1.2 Pressure reducing & pressure relief valves.

6.1.3 Direction control & flow control valves

6.1.4 Pressure switches

6.1.5 Level switches

6.1.6 Flow switches

6.1.7 One complete set of Rubber hoses used in machine

6.1.8 Two numbers filter elements of regenerative type & ten numbers filter

elements of disposable type used in hydraulic lubrication circuit.

6.1.9 One set of each type of lead screw-lead nut used for feed drives of table,

cross & vertical axis.

6.1.10One set of wipers used in machine.

6.1.11One number of each type of worm gears & bushes used in machine and spindle

bush bearing.

6.1.12  spare part list of fast consuming items for for 5 years trouble free service.

6.2                       Electrical spares

.

6.2.1   All the electrical components should be of reputed make, e.g. Siemens,

ABB etc.

6.2.3   Spares should have 5 nos. of each type of indicating lamps and push buttons.

6.2.4   Spares should have 2 nos. of each type of relays and contractors.

6.2.5   Spares should have 2 nos. of each type of limit switches.

6.2.6    Spares should have 2 nos. of each of Electromagnets and electrical coils of

directional valves.

6.2.7     List of fast wear out items  should be mentioned.

6.2.8     The machine should have machine lamp of 220V AC supply.

8.0                  MANUALS / DOCUMENTS

8.1 5 sets of documents for operation, maintenance and servicing manuals for

machine and other equipment to be provided in English language only, out of

these 5 sets one set will be on CD.

8.2 Service manual should contain followings in addition to description

8.3 Assembly drawings of each unit with marked list of each component with its

specification & make.

8.4  lubrication arrangment with marked list of each component

with its specification & make.

8.5  Detail catalogue of bought out items with its specification & make.

8.6  Foundation drawing is to be submitted by supplier within 4 weeks of placement

of purchase order.

9                 GENERAL

9.1 Total power requirement of the machine to be specified by the supplier.

9.2 Overall space required for installation of the machine to be specified by the

supplier.

9.3  Pre-dispatch inspection may be carried out by BHEL representative at Party’s

works.    However it is at the discretion of BHEL and final acceptance will be

done at BHEL ,Hardwar after carrying out all the required accuracy tests etc.

9.4  Pre accept of machine training to operator , mechanical & electrical maintenance

should be provided free of cost at supplier works for 3 days.

9.5  The manufacturer shall take the full responsibility for supervising the erection

start up, testing of machine its controls and other supplied equipment, test piece

machining etc. at BHEL, Hardwar.

9.6  Colour of the machine should be apple green as per ISO.

9.7  The supplier is to submit a clause wise deviation/acceptance statement against

each clause of this technical specification along with the offer.

10                  GUARANTEE

10.1  The machine shall be guaranteed for 18 months after commissioning and job

proving at BHEL works.Any spares required during guarantee period shall have

to be arranged by the vendor, free of cost ( upto the BHEL  Haridwar stores )

and duty levied have to be borne by the supplier.

11                   PACKING

11.1   Rigid packing for all the items of the machine, all accessories and other supplied

items in wooden boxes to avoid any damage/loss in transit.

All small and loose items like spares should be suitably packed in wooden

boxes.