BHARAT HEAVY ELECTRICALS LIMITED

RAMACHANDRAPURAM::HYDERABAD – 502 032::502 032 INDIA

Phone No: 0091- 040- 23182250, 23020173

Fax No: 0091- 40- 23021910, 23020173

SPECIFICATION& SCOPE FOR PORTABLE

WELDING FUME & DUST EXTRACTOR

Qty: With 3 Mtrs. Hood Length 14 Nos.

- Application:-

-	To extract welding fumes at source of welding in constricted place,

Absorb all harmful elements in filtration system & release clear air.

- Welding fumes are required to be captured from various welding stations in different work centers.

- Technical Specification :-

i)	Net Extraction Capacity 	-	1250 m3 /hour

ii)	Motor Power			-	0.75 KW / (max)

iii)	Extraction Hood Length	-	3 mtrs.

- Cartridge type filtration system with proper surface treatment , having

Filter surface efficiency	-	99.8%

v)	Filter surface area		-	> 30 Sq. mtrs.

vi)        Free blowing capacity of fan	-	> 2200m3/hr.

vii)	Automatic filter cleaning with compressed air pulse & with set frequency of pulses.

viii)	Automotive Start /stop switching system.

ix)	Built in compressor for independent filter cleaning.

x)	Aluminum mesh (high grade) pre-filter to protect the cartridge from sparks.

xi)	Noise Level			-	within 69 dB (A).

xii)	Audio alarm for filter saturation and over heating.

xiii)	Input Supply 			-	220 V ± 10% Variation single phase

& 50 HZ ± 3% Variation

xiv)	Working Supply 		-	Ambient Temperature - 3°C to 48 °C

Relative humidity	- Upto 95 %

During rainy Season.

xv)	Filter Type			-	Self Cleaning

3.0	Technical Feature /Condition :-

i)	Steel, Welded Construction painted with polyester –epoxy, Powder paint.

- Door for easy access to the filter and for waste drawer. It should be tightly sealed with latches.

- Pre-filter, Metal filter to protect cartridge filter from spark & for first step of filtration.

- Cartridge –main filter on roll-out tray.

- Carbon filter.

- Handle

- Swivel Castor Wheel for mobility.

- Waste drawer mounted on roll-out rails.

- Built in Noise Abatement features.

- Built in safety feature for safe use as per National / International norms.

- Rigid construction for day to day shop floor use.

- Lifting eye for handling by crane.

- Dia of Extraction arm 		: 	> 200mm

- Scope of supply:-

- Portable fume extractor as per specifications and technical features.

- Fume extraction arm of 3 mtrs. length to capture fumes at a distance of 30-40 cm from the welding source with 360o rotatable hood and lever mechanism to set at required position by welder during welding.

- To be mounted on castor wheel base for portability.

- Input supply lead- 10.0 m long with plug.

- All connectors, clips etc.  required for hose and connection.

- Recommended consumables and spare for 2 year smooth operation.

- Commissioning and trials by vendor at BHEL works.

- Training to BHEL personnel

- Guarantee for 3 years smooth and trouble free working after successful

commissioning.

- Prompt after sales service and response time within 24 hours.

- 3 sets of operation and maintenance manuals consisting of following documents

With supply:-

- Functional details with construction details.

- Specification and makes of main components.

- Operating instructions.

- Maintenance instructions.

- Trouble shooting and remedies.

- All clarification to be given point wise as per above requirements and supporting

technical literatures to be submitted with offer.

- Provide names & addresses of satisfied customers using similar Welding Fume

Extraction System.

*********