SPECIFICATION FOR GIRTH GREASING SYSTEM

- PURPOSE :

These rules group the requirements and recommendations concerning the selection and purchase of an automatic grease spray type lubrication system for girth gear – pinion set used in ball tube mills.

The supply shall constitute a complete assembly in working order and ready for installation and connection.

- DESCRIPTION OF THE LUBRICATION

It consists of the following sub-assemblies:

__ 1 lubricant drum which acts as tank.

__ 1 Pumping station

__ 1 compressed air treatment system

__ 1 Spray panel

__ 1  Set of rigid and flexible pipes which connect the above sub-

assemblies.

__ 1 Set of control instruments with electric box

- ENVIRONMENT:

.Dusty atmosphere

.Presence of vibrations.

.Lubricant : Mobil make

Reference : Mobiltac c or similar

- SUB-ASSEMBLIES OF THE LUBRICATION STATION

  - - Pumping system

Pumping can be carried out by an electric pump.

It must be acceptable to the lubricant drum (Std oil drum – 210 lit)

.Supply a lifting system for replacing the drum

(See sketch in appendix F.)

. An electric level controller trips an alarm in case of low level.

.The grease flows which are necessary for running-in and normal

operation   periods are indicated in appendix-A.

4.1.1 SYSTEM WITH ELECTRIC PUMP

. The electric pump permits sufficiently fine pro-portioning of grease,

in order to meet the daily quantities used.

. The greased quantity atomized is adjusted by acting on the pump

(cam adjustment).

  - AIR TREATMENT SYSTEM

It is used for atomizing the grease on the gear teeth.

The equipment which forms this system is listed in appendix C or E.

  - SPRAY PANEL

It consists of a hinged door which is fixed on the girth gear housing (See general arrangement sketch appendix F )

It basically includes a lubricant distributing block as well as spray valves.

The equipment  that forms this system is listed in appendix C or E.

    - SPRAY VALVES

Spray valves position is indicated in sketch of appendix G.

The spray valves number depends on the width of the girth gear.

It is usually provided one spray valve for lubricating a girth gear width of 80 to 100 mm.

    - DISTRIBUTION BLOCK

It supplies the  spray valves with the quantities of proportioned lubricants by displacement of its pistons, these displacements are generated by the flow of grease discharged by the pump.

The piston of one of the elements of the distributor includes an index which acts on an electric contact.

Acting on this contact permits determining the quantity of grease fed within a given time, tripping timers.

    - CONTROL EQUIPMENT ON THE DISTRIBUTOR

SYSTEM A WITH ELECTRIC PUMP

.2  adjustable timers ensure:

- Grease delivery to the spray valves through the distribution group.

- Grease atomizing on the teeth through compressed air injection

The first timer controls the duration of atomization (adjustable from 0 to 10s) by opening the solenoid valve mark 8,sketch appendix B and is actuated by switching the electric contact mentioned in 4.3.2 .

The other timer controls the time spent between two atomizations (adjustable from 0 to 2mm), trips a “lubrication defect” alarm if the time lapsed is longer than the adjusted time.  This timer must be reset at each switching of the electric contact.

  - CONTROL INSTRUMENTS AND ELECTRIC BOX

    - CONTROL INSTRUMENTS

The instruments (alarm thresholds, valve opening)

Shall be shop-adjusted except for the timers and the impulse  meter whose setting values are determined at the site.

All instruments are provided with tight box, protection level IP655 and tropicalized contacts, if necessary.

    - ELECTRIC  BOX

The protection of this box is atleast  IP 55

It groups the terminal boxes, timers, meters relays.

FOR GIRTH GEAR GREASING SYSTEM WITH PNEUMATIC PUMP

5.1	PUMPING SYSTEM

.Pumping is carried out by a pneumatic pump.

.It must be adaptable to lubricant drum.

.supply  a lifting system for replacing the drum (see sketch in

appendix F.)

.An electrical level controller trips an alarm in case of low level.

.The grease flows which are necessary for running in and normal

operation periods are indicated in appendix A.

.Pneumatic pump does not permit sufficiently fine proportioning of

grease, in order to meet the daily quantities used.

.Therefore, the lubricating cycle must be periodically interrupted (for

a maximum of 10 minutes.)

  - AIR TREATMENT SYSTEM

It is used for atomizing the grease on the teeth and driving the pneumatic pump.

  - DISTRIBUTING BLOCK

Piston of one of the elements of the distributor includes an index which acts on the electrical contact.

Acting on this contact, the impulse meter is actuated.

CONTROL EQUIPMENT ON THE DISTRIBUTOR

SYSTEM WITH PNEUMATIC PUMP

.2 Adjustable timers and an impulse meter ensure:

Grease delivery to the spray valves through the distribution group.

Grease atomizing on the teeth through compressed air injection.

Periodically stop the operation of the pump.

The first timer periodically (period adjustable from 0 to 10 mm)

cause pump starting and air feeding to the spray valves

(opening of solenoid valves mark 7 and 8, sketch. Appendix D)

The impulse meter controls the number of cycles achieved by the distributor and therefore the grease quantity injected. When the number of impulses displayed is reached, the pump is stopped (closing of solenoid valve mark 7, sketch appendix D)

.The second timer controls stopping of the air injection to spray valves (closing of solenoid valve mark 8, sketch appendix D-3)

It also generates an alarm if the number of impulses displayed has not been reached in the time fixed (adjustable from 0 to 20mm).This timer is reset by the first timer).

6   DATA TO BE GIVEN ALONG WITH OFFER

  - Dimensional drawing of lube oil system should be submitted.

  - Instrumentation on lubrication system, lubrication schemes, and details about lubrication pump and motor.

  - Inspection procedures, detailed quality plan and test procedures.

  - Painting and packing details.

  - Reference list of lubrication system supplied earlier, with performance certificates.

  - Guarantee certificates.

7 	Following documents shall be supplied with in 4 months from the  placement of purchase order.

A. O&M Manual – Twenty copies

B. General arrangement drawing of lubrication

and piping system – 20 copies.

8 	INSPECTION :- The following inspection shall be performed on each  grease spraying system.

a)	Over all dimensions of each spray panel & pump assembly.

b) 	The list of items and rating

c)	As per approved quality plan and drawings.

| VAR NO | DESCRIPTION | MATL. CODE |
| --- | --- | --- |
| 01 | GIRTH GEAR GREASING SYSTEM WITH ELECTRICAL PUMP | BA9750051017 |
| 02 | GIRTH GEAR GREASING SYSTEM WITH PNEUMATIC PUMP | BA9750051025 |
| 03 | GIRTH GEAR GREASING SYSTEM WITH ELECTRICAL PUMP – FOR 3448 MILL 3 NOS SPRAY NOZZLES. | BA9750051033 |
| 04 | GIRTH GEAR GREASING SYSTEM WITH PNEUMATIC PUMP 4NOS SPRAY NOZZLES FOR BBD 3448 TUBE MILL | BA9750051041 |
| 05 | GIRTH GEAR GREASING SYSTEM SPRAY PANEL FOR BBD 4760/4772 MILLS (ELEC.PUMP). | BA9750051050 |

APPENDIX-A

| DATA SHEET | REV BY DATE |
| --- | --- |
|  | 00 |
| LUBRICATION SYSTEM FOR GIRTH GEAR AND PINION |  |
| MILL TYPE: |  |
| 10. SITE |  |
| 11.TEMPERATURE MIN: AVERAGE MAX: |  |
| 12. ATMOSPHERE (MOISTURE- STEAM- DUST-CHEMICAL) |  |
| 20. GENERALITIES |  |
| 21. GIRTH GEAR WIDTH: |  |
| 22. NUMBER OF SPRAY VALVES: |  |
| 23.VERSUS WITH ELECTRICAL PUMP : PNEUMATIC PUMP: |  |
| 30.GREASE |  |
| 31. QUALITY |  |
| 32.CONSUMPTION RUNNING IN PERIOD: NORMAL PERIOD: |  |
| 40. SERVICE AIR |  |
| 41 AIR FLOW FOR SPRAY VALVES: FOR PNEUMATIC PUMP (1): |  |
| 42. AIR PRESSURE ON SPRAY VALVES: |  |
| 50. COMPLEMENTARY INFORMATIONS |  |
| - FOR SYSTEM WITH PNEUMATIC PUMP LIE 22-41-42 ARE TO BE COMPLETED BY THE SUPPLIER. |  |

APPENDIX-C

LUBRICATION OF THE PINION-WHEEL ASSEMBLY WITH ELECTRIC PUMP

LIST OF EQUIPMENT

_________________________________________________________________________

MARK	    QUANTITY 	DESIGNATION		OBSERVATIONS

_ _______________________________________________________________________

1	1	ELECTRIC CONTROL PUMP		PRESSURE : 10 MPA

2	1	ELECTRIC MOTOR 0.55 KW,1500RPM							415V,3ǿ,50HZ.

3		    1	MECHANICAL AND HYDRAULIC

CONNECTING DEVICE

4		     1	COVER FOR OPEN DRUM		PROVIDED WITH 3

LIFTING RINGS.

5		     1      	SAFTY VALVE				.FOR PUMP  PROTECTION

.CALIBRATION PRESSURE.:10MPA

6		      1	PRESSURE  GAUGE WITH 1/4  TURN	BOX DIA 100mm FILLED

SHUT OFF VALVE			.WITH GLYCERINE

.SCALE  0-16 MPA.

7		      1     ELECTRIC LEVEL CONTROLLER

AIR TREATMENT SYSTEM

8		     1     PRESSURE REDUCER LUBRICATING 	FILTERATION : 50 MICRONS				FILTER WITH SHUTOFF VALVE	.AUTOMATIC DRAIN

9	                   1	SUPPLY SOLENOID VALVE

| VAR NO |  |
| --- | --- |
| 01 | 110 VAC |
| 02 | 240 VAC |

SPRAY PANEL

10		  1        DISTRIBUTOR BLOCK (N OUTLETS)

11		  1       CONTROL EQUIPMENT ON THE 		  ACCORDING TO DESCRIPTION

DISTRIBUTOR				  chap.4.2.3

12		  1	FILTER - STRAINER			  MESH  8 / 10

13		  n	SPRAY VALVES

14	  1   	PRESSURE GAUGE EQUIP PED		BOX DIA 100 MM FILLED

WITH A ¼ TURN SHUT-OFF VALVE	GLYCERIN

SCALE  0-1 MPA.

15	 1 	PRESSURE SWITCH			ADJUSTING RANGE 0-1 MPA

APPENDIX-D

TYPICAL LUBRCATION DIAGRAM FOR GIRTH GEAR ALTERNATIVE WITH PNEUMATIC PUMP

APPENDIX-E

LUBRICATION OF THE PINION-WHEEL ASSEMBLY ALTERNATIVE WITH

PNEUMATIC PUMP

LIST OF EQUIPMENT

_________________________________________________________________________

MARK	    QUANTITY 	DESIGNATION		OBSERVATIONS

_ _______________________________________________________________________

1	1	PNEUMATIC CONTROL PUMP		RATIO:45/1

2	1	COVER FOR OPEN DRUM		PROVIDED WITH 3 LIFTING

RINGS

3		    1	AIR PRESSURE REDUCER		ADJUSTING RANGE 0 – 1,2 MPA

4		     1	PRESSURE GUAGE EQUIPPED WITH	BOX DIA 100 MM FILLED

a ¼ TURN SHUTOFF VALVE	               WITH GLYCIRIN SCALE:0-16MPA

5		     1      ELECTRIC LEVEL CONTROLLER

AIR TREATMENT SYSTEM

6		      1	PRESSURE REDUCER LUBRICATING 	FILTERATION : 50 MICRONS

FILTER WITH SHUTOFF VALVE

7/8		    2     SOLENOID VALVES

| VAR NO |  |
| --- | --- |
| 01 | 110 VAC |
| 02 | 240 VAC |

SPRAY PANEL

9		  1        DISTRIBUTOR BLOCK (N OUTLET)

10		  1       CONTROL EQUIPMENT ON THE 		  ACCORDING TO DESCRIPTION

DISTRIBUTOR				  chap.4.2.3

11		  1	FILTER - STRAINER			  MESH  8 / 10

12		  0	SPRAY VALVES

13	  1   	PRESSURE GAUGE EQUIPPED		BOX DIA 100 MM FILLED

WITH A ¼ TURN SHUT-OFF VALVE	GLYCERIN

SCALE  0-1 MPA.

14	 1 	PRESSURE SWITCH			ADJUSTING RANGE 0-1 MPA

APPENDIX-F

APPENDIX-F

LAYOUT DIAGRAM OF SPRAY PANEL

d-e    The value depends on the spray valve conception.  It will be determined

by experiments as typical value   e: 150 mm.

RECORD OF REVISIONS

| REV.NO | DATE | REVISION DETAILS | REVISED | APPROVED |
| --- | --- | --- | --- | --- |
| 01 | 13.12.94 | VOLTAGES INCLUDED | T.M.R | J.G.K |
| 02 | 18.05.99 | 4 NOZZLE SPRAY SYSTEM FOR BBD-3448 MILL-ADDED | K.V.R | T.M.R |
| 03 | 29.10.01 | FOR SOLENOID AC OUTPUT VALVE VAR TABLE ADDED | T.M.R | J.G.K |
| 04 | 09.07.07 | VARIANT NO.04 ADDED,POINT NO.8 INSPECTION NOTES ADDED AND SPRAY PANEL DIMENSIONS SHOWN. | N.D.S | T.M.R |
| 05 | 17.01.09 | VAR.05 ADDED | PAVAN | T.M.R |