Ref.: BHEL/ PD/ SPEC/ BB/001

- SCOPE OF SUPPLY and WORKS FOR BOOM BARRIERS

| S.No | Item | Make | Suppliers Comments |
| --- | --- | --- | --- |
|  | Scope includes Dismantling of existing 4 nos. of Boom Barriers & Supply, Erection & Commissioning of 3 nos. of new Boom Barriers as detailed below. | -- |  |
| 1 | Dismantling of existing Boom Barriers - - At J Gate-1 No. - At Main Gate - 1 No. and - At Administrative Building - 2No. ________________________ Total Quantity: 4 nos. | -- |  |
| 2 | Supply, installation and commissioning, demonstration and training of three numbers BHEL personals on boom barriers. The details of length and location of boom barriers are as below - At Administrative Gate 1 - 1 no. – Length - 6m - At Administrative Gate 2 - 1 no. – Length - 5m - At Main gate - 1 no. - Length - 8m _________________________ Total Quantity – 3 nos. | Gunnebo / Elka/ Magnetic/ Tescon |  |
| 3 | The Boom barrier should be designed for Heavy duty operation with sleek and modern looks, an built anti-crush safety device to suspend the Motion of barrier should it meet any obstruction. Beam sensor should be provided for further protection to vehicles. Provision for dual speed of barrier to optimize time and maximum safety. The barrier should be activated by push button. Its electronic control panel should be designed to accept signals from various controls and safety accessories such as beam sensor , card readers, etc. Boom Barriers shall be manually operative as an additional facility. | -- |  |

B)  Technical Specification  for Boom barrier

| S.No | Item | Type / Make | Suppliers Comments |
| --- | --- | --- | --- |
| 1.0 | Specification- |  |  |
| 1.1 | Duty cycle – 100% | -- |  |
| 1.2 | Incoming power – 230C AC +/- 10% , 50 Hz | -- |  |
| 1.3 | Amb.Temp range – 5 to 50 deg C | -- |  |
| 1.4 | Boom Length – As indicated in point A-2 | -- |  |
| 1.5 | Opening time from full close to full open: 8 seconds (maximum) | -- |  |
| 1.6 | Control – Microprocessor based | -- |  |
| 1.7 | Housing material - Aluminium | -- |  |
| 1.8 | All mechanical parts to be Zinc coated and chromate | -- |  |
| 1.9 | Boom Bar – Rectangular boom with red reflecting warning tape and buffers. | -- |  |
| 1.10 | Protection class : IP55 | -- |  |
| 1.11 | Boom stand – The boom should rest on a stand of height 800mm from the ground | -- |  |
| 1.12 | Body Housing: The body housing of the barrier should be highly corrosion resistant | -- |  |
| 1.13 | Barrier Body Paint: The barrier should be powder coated in Blue/ white color and the boom bar should have reflective red/ orange stripes and "STOP FOR SECURITY CHECK" sign on both the sides in red/ orange at the centre of the boom. | -- |  |
| 1.14 | Wires and cables : Required cables and wires are in bidder’s scope. | Finolex/ Polycab/ Havells |  |
| 1.15 | UPS back up for minimum 30 minutes to be provided by the bidder, for boom barrier operation at the time of power failure. In case of power failure, UPS back up operation should be available. | Emerson /APC /Liebert |  |
| 1.16 | Remarks: Supply comprises boom barriers inclusive of drive mechanism, electric motor, boom, controller , switches, supply of necessary cables , conduits, optical sensors, etc,. Civil works for dismantling the existing 4 nos of barriers and installation of new barriers are all in bidders scope. The vendor has to take complete responsibility for installation and commissioning of boom barriers in BHEL premises. | -- |  |

C) General terms and conditions

| SL | Item | Type/ make | Supplier’s comment |
| --- | --- | --- | --- |
| 2.1 | Essential Spares: Vendor should quote essential spares required for 2 years of trouble free operation & it must include following items as minimum -: - Push buttons – 1 set for each barrier. - Gear Box - 1no. for each barrier. | a-Reputed b-same as that of boom barrier |  |
| 2.2 | THE PRICE OF SPARES TO BE QUOTED SEPERATELY AGAINST EACH ITEM. | ---- |  |
| 2.3 | The bidder shall depute their representative when ever called to attend to break down maintenance during guarantee period- Free of cost. | ---- |  |
| 2.4 | The equipment shall be offered for inspection before dispatch at Bidder’s works. Only after obtaining clearance in writing from BHEL , the boom barriers can be shipped. 1 week prior information has to be given for deputation BHEL personnel for inspection. | ---- |  |
| 2.5 | The control of Boom barriers shall be able to be integrated with Under Vehicle Scanning Systems and License plate recognition system (for future expansion). | --- |  |
| 2.6 | Bidder is required to select the make of items as mentioned above i.e. Gunnebo/ Elka/ Magnetic/ Tescon only. In case the Bidder is offering any other or equivalent make, the offer is liable for rejection. . | -- |  |

D. Pre-Qualification

| SL | Pre qualification criterion | Supplier’s comment |
| --- | --- | --- |
| 1. | The bidder shall have minimum 3 years experience in supply, installation and commissioning, maintenance of Boom Barriers to Industrial establishments like Power plants / Government Institutions/ research labs / Airports / Oil Terminals etc. Proof of experience shall be submitted. 1 no. of performance Certificate shall be enclosed as proof of successful completion of an order. |  |
| 2. | The bidder shall have local office with service center in Hyderabad. Service setup shall have trained personals for providing technical assistance and after sales services, whenever required. |  |
| 3. | If the bidder is not manufacturer of products quoted, the bidder should submit Authorization letter from the Original Equipment Manufacturer (OEM) for the offered Boom Barrier. Bids without OEM Authorization letters will be rejected. Authorization letters shall be addressed to, AGM/Purchase-CG,BHEL-R.C Puram, Hyderabad. |  |