Technical specification

Name of the equipment: Magnetic Particle Inspection Machine

| Sl no | Description of BHEL requirement |
| --- | --- |
| 1 | Purpose: The equipment shall be capable of conducting wet fluorescent and wet non-fluorescent MPI on turbine blades. Detailed brochure for the equipment shall be furnished along with the quotation. |
| 2 | The equipment shall be bench type with fixed head stock and movable tailstock, between which the blades shall be clamped and magnetized. The head stock and tail stock shall be copper braided. |
| 3 | The headstock and tailstock shall have synchronized rotation to facilitate examination of blades in clamped condition. |
| 4 | The equipment shall be capable of clamping different types of blades with complex airfoil and non-uniform cross-sections, with length varying between 50 and 800 mm, effective diameter varying between 15 and 175 mm and weight varying between 50 grams to 15 Kgs. |
| 5 | The magnetizing capacity of the equipment shall be more than or equal to 3000A Half Wave DC and 60,000 AT. |
| 6 | Allowable Magnetization time at 3000A shall be 5 seconds or more. |
| 7 | Allowable magnetizing current for continuous operation shall be 1000A or more. |
| 8 | Facility for independent current, flux and also combined magnetization shall be provided |
| 9 | Indicators for current, flux and demagnetization shall be provided. |
| 10 | Both push button and foot-switch shall be provided. |
| 11 | The spraying shall be manual. |
| 12 | The equipment shall have stationary UV flood lights. |
| 13 | Dark hood with suitable ventilation shall be provided. |
| 14 | There shall be built-in facility in the equipment for in-situ demagnetization after MPI. |
| 15 | Separate tunnel demagnetizer of 500 X 500 mm tunnel size and roller mounted withdrawal tray shall be supplied. |
| 16 | The following accessories and consumables shall be supplied. |
| 16.1 | Digital UV and white light meter – 1 no. |
| 16.2 | Pie gauge (as per ASTM Standard)– 1 no. |
| 16.3 | Portable handheld UV light – 1 no. |
| 16.4 | Gauss-meter (Range : -20 to +20 gauss, Resolution 1 Gauss) – 1 no. -DIGITAL TYPE |
| 16.5 | Centrifuge tube with stand – 1 no. |
| 16.6 | Fluorescent magnetic particle both water base and oil based - 2 Kg each. |
| 16.7 | Non-fluorescent magnetic particle (black) - 2 Kg |
| 16.8 | Corrosion inhibitors, wetting agents and anti-foaming agents - 2 Kg |
| 16.9 | Rubber pads with copper mesh – 4 nos. |
| 16.10 | Tool kit for maintenance - 1 set. |
| 17 | Mechanical, Electrical and Electronic Spares for 5 years’ trouble free operation with itemized break-up shall be quoted. |
| 18 | In addition to the above, the following items shall be quoted as spares. |
| 18.1 | UV lamp – 4 nos. |
| 18.2 | Pie gauge (as per ASTM Standard) - 2 no.s |
| 18.3 | Gauss-meter (Range : -20 to +20 gauss, Resolution 1 Gauss) – 2 nos.-DIGITAL TYPE. |
| 18.4 | Centrifuge tube – 3 no's |
| 18.5 | All indicators (for current, flux etc.) used on the equipment – 1 no. each |
| 19 | The power supply to the equipment shall be 415VAC / 50Hz. |
| 20 | Commissioning and demonstration of trouble-free operation of the equipment shall be done at BHEL |
| 21 | Pre-dispatch inspection shall be carried out at Supplier’s works |
| 22 | Training in operation & maintenance shall be provided at BHEL. |
| 23 | Operation and maintenance manual in English shall be supplied. |
| 24 | Calibration certificate for the equipment / accessories shall be supplied. |
| 25 | The equipment shall fulfill applicable safety regulations and guidelines. Supplier to furnish details. |