## PRECISION  SURFACE GRINDER
TECHNICAL SPECIFICATIONS

STT-0020

- PURPOSE :-

The machine must be capable of precision grinding of  blade roots & shrouds to meet requirements of blade assembly in ST Casings.

2.0 WORK PIECE MATERIAL

Various alloy steels including high temp. & creep resistant alloy steels of material grade 21Cr Mo V47, 17 Cr MoV511 etc., forged steels, cast iron, bearing material, non ferrous alloys and other similar materials used in the industry for manufacturing power plant equipment.

- BROAD SPECIFICATIONS

The machine offered must be completely independent unit in ready to operate condition & scope of supply should include coolant spray & filtration arrangement with splash guard, lamp for light for easy working, electromagnetic chuck suitable for the table of machine and equipped with all accessories , diamond dresser

  - TABLE

- Table Clamping Area				600x200mm approx

- No. and Size of  T-slot in table			Supplier to Indicate

- Longitudinal traverse            			More then 600mm                            
 -Cross  traverse           				More than 200mm.

- Vertical traverse                                                       350 mm approx

- Max. height of spindle centre above table               Supplier to indicate

- Table speed (Infinitely variable) 			up to 15 M/Min. approx.

- Feed for Longitudinal Traverse			Supplier to Indicate

- Feed for Cross Traverse 				Supplier to Indicate

- Transverse fine feed by hand / graduation		Supplier to Indicate

-  Least count of hand wheel for vertical Travel        Supplier to Indicate

-  Least count of hand wheel for Transverse Travel   Supplier to Indicate

- Rapid Vertical Traverse                                            Supplier to Indicate

-  Automatic continuous Transverse feed                   Supplier to Indicate

(mm/stroke

  - GRINDING WHEEL HEAD 
- Size of grinding wheel in mm     		D250X20XD76.2
- Grinding wheel speed				approx. 2400rpm

  - POWER
Grinding wheel head motor			2.0 KW approx.
Hydraulic Pump Motor (KW)			Supplier to Inicate
Coolant pump motor				1/8HP approx.

- ACCESSORIES
Following accessories are required with the machine offered.

-  Electromagnetic chuck with power supply & control unit (size 600x200mm)        1 No.

-  Electric Lamp mounted on m/c 220 Volts		                                          1 No.

- Grinding wheel 					                                           2 No

- Wheel  Flange				                                                        1 No.

- All Electricals suitable for 3ф, 415 volts, 50 Hz AC Supply and Control Voltage 220V.

-  Balancing Mandrel

- Hydraulic power pack.

- One set of standard tools for normal operations like spanners, allen keys etc.

5.0 GENERAL FEATURES

  - Machine will be complete with hydraulic equipment i.e. pump and motor with    closed loop recycling and cleaning arrangement for coolants.

  - There should be no leakage of hydraulic/ lubrication oil from the machine during operation or idle time.

  - No tandem pump should be used in hydraulic system and elsewhere.

  - M/c must be of rigid construction and vibration free column supporting the wheel spindle must also be of robust construction.

  - Motors, control panels, bearings and other parts to be of reputed make. Supplier to furnish details.

  - Overall size of the M/c, weight, standard colour and noise level to be indicated. Desired paint colour : Apple Green

  - All the electrical control equipments and switchgears should be of Siemens make.

6.0  SPARES 
Party must offer following items with price

6.1- One number of each type of pumps used in hydraulic lubrication  circuit.

6.2- Five number of each type filter elements used in hydraulic and  lubrication     
        circuit.

6.3- One number of each type direction control, pressure relief, pressure reducing and
         non return valves used in hydraulic and lubrication  circuit.

6.4- Complete one set of sealing used in machine.

6.5- Complete one set of hose pipes used in machine.

6.6- Complete one set of compressor/tension springs used in machine.

6.7- Five set of fuses (if less than 2 Amp) should be given.

6.8- One 3-Phase, 20 Amp. MCBDB should be attached to the control panel or base 
       (whichever fixed) having industrial socket.

7. 0  INSTALLATION & COMMISSIONING

Supplier’s representative to supervise the erection of the M/c at Hardwar.
Commissioning of the complete equipment supplied will be carried out by the supplier at our works. Machine, its controls and accessories will be tested as per agreed norms confirming to International/national standards.

8.0 MACHINE ACCEPTANCE:
Final acceptance shall be made by BHEL at Hardwar following successful erection & commissioning and achieving the specified accuracy.

9.0 FOUNDATION
Supplier to furnish details of foundation after 2 weeks of placement of PO. Foundation bolts, nuts, plates etc. will be supplied by the party.

10.0  TC & GC

TC & GC for the complete equipment to be furnished alongwith the supply. Equipment must be guaranteed against any design and manufacturing defect for a minimum period of 12 months from the date of commissioning.

12.0 MANUALS

3 Sets of operating and maintenance manual which must contains followings in addition to    
 their standard manual in English language.

12.1- Hydraulic, lubrication and coolant circuit diagram with description, specification and 
          make of each elements of each circuits.

12.2- Assly diagram of each subassemblies of machine with marked parts list with their 
          specification of each components.

12.3- Full document to be given on CD also.

12.4- Electrical manual should clearly describe the working of machine with systematic 
          functioning of interlocks provided.

13.0  AMBIENT CONDITIONS
           M/c is required to operate at following conditions:
         - Ambient Temp.				5 to 50 deg. Cen.
        – Relative Humidity			Upto 95 %
        - Power supply				415  10%, 3 phase, 50 Hz  3%

14.0  INSPECTION
         BHEL may carryout pre dispatch inspection at supplier’s works.

Note :-

a)- Product catalogue containing complete technical specification indicating the model

offered must be enclosed along with the offer.

b)- Point wise details must be furnished by the supplier.