QUALITY ASSURANCE REQUIREMENT

Indent No. T201/6/4150W  Dt: 23.09.06

Item: LP Inner Inner Casing

LP Inner Outer Casing

| A | - Filled in vendor registration form to be furnished alongwith necessary enclosures. - Bidder to furnish the list of manufacturing & testing facilities. This should include the fabrication, stress relieving & machining facilities. - Bidder to inform the name of their sub-vendor if any & furnish their credentials including facilities, QA system. - Bidder to inform the sub-vendor for materials alongwith credential for review/ approval by BHEL. - Bidder to submit the list of qualified welders & NDT personnel alongwith copy of their qualification certificate. - Bidder to furnish their experience w.r.t. same/ similar items giving description, size, weights, customer & year of supply. |
| --- | --- |
| B. | - Welding to be carried out by qualified/ approved welders as per ASME Sec-IX. - NDT to be carried out by qualified/ certified personnel as per (ASNTC-1A) ASNDT or equivalent. - Party to submit detailed material Test Certificate (TC), Stress Relieving (SR) Graph, NDT report, , dimension report, painting report and final clearance at the time of despatch in two hard copies & two CD ROM. - All material plates, pipes, rods, casting are to be procured alongwith detailed Test certificates meeting the requirements as per relevant BHEL specification. - Pre-heating requirements to be followed as below: a)For plates thickness from 25 mm but below 70 mm if Carbon equivalent (CE) (C+Mn/6) exceeds 0.39. b)For plates thickness70 mm & above if CE (C+Mn/6) exceeds 0.38. c)The temperature of preheat shall be minimum 100 degree centigrade. - Party to submit their Mfr. Plan detailing the sequence of operations for fabrication & machining for review by WT & STT. - Item is not allowed for Bhilai project as NTPC approval is not there. - Copy of QP to be followed is enclosed. Vendor to confirm. - BHEL team/ its authorized inspection agency will carry out inspection as per stages marked under column’B’. Minimum 45 days advance notice will be given for inspection. - CHP’s if any w.r.t. specific project shall be informed later on. - Logsheet for specific project will be provided by BHEL for compliance by vendor. - Preservation & packing shall be as per approved procedure to be submitted by vendor. - Non-conformity if any to be referred to BHEL for review/ acceptance. Format to be sent. - Despatch clearance will be given after review/ verification of inspection reports by BHEL. |

(S.S. Chauhan)

AGM (QAX)