SPECIFICATIONS

FOR

15 MT WELDING MANIPULATOR
1.0      
Type


-
Conventional Welding Manipulator

2.0               Technical Specifications
i)
Rated Loading
-
15 MT (maximum)


 

Capacity 

ii)
Rated Centre   
-
300 mm for 15 MT Load.

of Gravity

iii) 
Rated Ecentricity
-
300 mm for 15 MT Load.

iv)
Table Diameter
- 
4500 mm

Rotation speed
-
0.25 to 2 rpm  

(360 deg C)


(Infinitely variable)

vi) 
Tilting Speed
-
Fixed

vii)
Tilting angle
-
0 deg to 135 deg






(90 deg forward tilt

       






and 45 deg backward tilt)

viii) Table slots and
-
8 nos. M 24

Bolt Site


(Radial)

ix) Input Supply
-
415 V +/- 10% Variation







3 Phase, 50 Hz +/- 3% Variation

x) Table Earthing
-
Upto 1500 A

for Welding

Capacity 

xi) Rotation and
-
To be mentioned with calculation by 

Tilting motor 

vendor 

capacity

xii) Rotation and
-
To be mentioned with calculation by

Tilting Torque

vendor.

3.0

TECHNICAL FEATURES :

i) Concentric rings on table to facilitate fast and accurate

alignment and marking.

ii) Remote Control pendant with push buttons for all functions

/movements i.e. stop/start/inching/emergency stop/speed control/tilt angle etc.

iii) Height adjustment provision on Frame ( 3 steps)

iv) Lifting Hooks for Table Frame for height adjustment as per requirement.

v) Rigid and sturdy structure, no vibration during rotation/ tilting operation with full load.

vi) Complete with controls, starter, earthing of table, standard clamps/Bolts, operator's Tool Box, Foundation Bolts/ washers etc and any other recommended toolings/ fixtures.

vii) Motors, Bearings, Gears, electrical components should be of reputed make like:

a)  Motors
           
  - Crompton Greaves/ABB/Siemens

b)  Gears/Gear Box 
  - Elecon

c)   Bearings

  - SKF/FAG

d) Electrical components - Siemens/L & T

viii) Civil Foundation Drawings within two weeks from placement of order.

4.0
Recommended spares for 2 years smooth operation with identification nos. and individual price.

5.0               
5  copies  of  operation and maintenance manuals including 

following documents :-

i) Mechanial Assembly Diagram;

ii) Electrical and Electronic Circuit diagram with wire nos. and wiring diagram; 

iii) Operation and maintenance instructions;

iv) Do's and Don'ts;

v) Fault Diagnosis;

vi) Complete spare parts list with part nos.

6.0                 2 years  Guarantee  for smooth working after commissioning and
Prompt after sales services during   and   after   guarantee period.

7.0                  Installation and commissioning by vendor at BHEL works.

8.0                 Training of BHEL personnel at party's/BHEL works as required

 by BHEL.

9.0  

 Names  and  addresses of customers with contact person to whom 

 Similar welding manipulators have   been supplied and working

 Satisfactory. 


