ACS_annexure

Description of the Validation Procedure of the BHEL ACS

This document attempts to delineate all the logic that goes into answering a simple question:

"Should a particular cardholder who has just presented his/her identity at a reader be allowed to pass through?"

(PLEASE READ ‘swipe’ as the presentation of card at the reader and system recognition of the activity)

# Basic Checks

Before any further validation checks are made, each card is put through certain checks and if it fails any of these checks, the swipe is rejected outright. These checks are now documented.

2.1. The reference data for the card that includes the permanent and transient reference data, the shift timings of the employee and the card readers allowed for that employee are all retrieved. If either of these elements is not present in the database, the card is rejected. For instance, if the card is marked with shift   "-1" which doesn’t exist in the shift table, the card is rejected. Similarly if the card is not present in the card table, it too is rejected. In case, the reference data is not available for the card just swiped, the error message "INVALID CARD" is flashed on the reader.

2.2. The reader at which the card holder has swiped may have been placed in a locked mode (mode 0) in which case, the card is rejected with the code "Reader disabled".

2.3. If the reader at which the card holder has swiped is in the collect /timeout mode, then the swipe is accepted without any further validation process. If the card no is available in CR memory as invalid card, then the swipe is rejected even in collect/timeout mode.

2.4 Now, if the card is among the suspended cards (see 1.1), then the swipe is rejected without any further processing with the message "SUSPENDED".

2.5 If the card belongs to a priority 0 employee and the swipe has been made in work areas that are permitted to that cardholder, he is allowed to move without any further checks of any type.

2.6 If the card belongs to a priority 0 employee and the swipe has NOT been made in any of the work areas permitted to that cardholder, then the FREE MOVMENT key should

have been pressed to allow the cardholder to pass through. The cardholder is barred from moving in other cases.

2.7 If the cardholder's priority is 2 and the Anti-Pass back feature is enabled, then the In-swipes will be rejected if the person is already inside BHEL and the out swipes will be rejected if the person is already outside BHEL (as per the transient data, see section 1.2). There will not be any anti pass back check for the first in swipe in employee’s own shift.

2.8 Once an employee shift is finished, all this transient reference data has to be reset back to normal state. This happens when at the time the employee swipes in for the first time after his previous shift has ended. The previous shift is considered as finished under the following circumstances:

2.8.1 When the employee earlier left BHEL with a regular swipe after the shift end and is now trying to enter BHEL again after the shift's end time has passed.

2.8.2 When the employee earlier left BHEL on official, training or special movement key before the shift had ended and is now entering BHEL again after the four hours if he had left for training and or after eight hours if he had left on official or special movement.

- Shift times calculations

Before proceeding with any further validation checks, the DAS has to calculate the precise dates and times of various times associated with the current shift of the cardholder.

  - Each shift specifies three or four time bands within which an employee can move through the gates. Each shift has an in-time, an out-time and one or two break times. All these times are provided to the ACS in 24 hours format. It is very important that these times maintain strict relationship with each other. For instance, when a period is stated to start at 2345 hours and end at 0015 hours, the period should be considered as ending in the next calendar day and not the same day.

  - In order to ensure that given a swipe time, all the shift timings for that swipe are properly calculated, the following logic is applied to the date and time calculations of the shift:

    - The time of end of the Shift start period is always after the start of the Shift start period.

    - The time of start of the lunch period is always after the end of the Shift start period.

    - The time of end of the lunch period is always after the start of the Shift lunch period.

    - The time of start of the Shift end period is always after the end of the Shift lunch period.

    - The time of end of the Shift end period is always after the start of the Shift end period.

  - At times, a second break is also provided for the convenience of those who are working overtime. In this case, the second break is always after the shift end start time. The integrity rules for the second break timings are:

    - The time of the start of the second break period is always after the start of the Shift end period.

    - The time of end of the second break period is always after the start of the second break period.

    - The time of end of the shift end time is always after the end of the second break period.

  - The current shift is supposed to be the one that has already started. The current swipe is always assumed to have occurred after the start of shift start period (when employees are allowed in).

  - There can be cases when an entry (IN) swipe is made after the shift end time and before the start of the next shift. In such cases, the shift times of the shift about to begin are used to validate the swipe. The only cases where the previous shift's times are used to validate are those when either the official key, the training pass or special movement keys have been used.

With the calculation of the dates of all the important times of the current shift in place, now we are all set to do the actual validations.

Validation of Regular Swipes

A regular swipe is the one that which is made without any key press. These swipes regularly occur at the time of shift start, end and the breaks. All the validation rules expressed here are only applicable if the card has been cleared by the basic checks of section 2.

4.1 Valid regular swipes are permitted only within that employee's own work areas.

4.2 An employee can, enter his own work areas during the shift in times. The first in-

swipe and the in-swipe time of the employee is recorded in the transient data.

4.3 An employee can also make the first swipe of the shift during the first break (lunch

break) as he maybe joining only during the second session. Thus, a first swipe is also

accepted during the break 1.

4.4 An employee is allowed to go out anytime during the first break.

4.5 If the flex lunch is enabled for a particular employee’s shift , he is allowed in with

regular swipe only within forty minutes of the time he first left during the current

lunch break. This means that an employee may go out and come back several times

during the lunch break. However, all his movements should take place only within any

forty minutes period of the total break 1.

4.6 An employee is permitted to leave the factory at the shift end period with a regular

swipe.

4.7 If the second break is also specified in the shift of a cardholder, then if he leaves

within the break2 period (which may also be well beyond the shift end start time), the

employee is considered to have gone out for break 2.  An employee may also return

within the break 2 (if he had earlier left within the break 2) with a regular swipe. An

employee is considered to have ended the shift if he leaves at any time after the shift

end.

4.8 For all the shift end  periods, special grace of 15 minutes is given to those employees

who have to attend to the crèche, those who are visually impaired and those who are

physically handicapped.

4.9 In all other cases, regular swipes are not validated and the message "NOT

ALLOWED" is displayed.

Employees working in HRDC or Transport & welfare departments will be allowed to move into and out of administrative building between 1145 and 1240 for enabling them to go to staff canteen.

Check for compatibility of keypad type is limited to his own work area only.

Free movement

The Free movement key is permitted under certain circumstances. All the validation rules expressed here are only applicable if the card has been cleared by the basic checks of section 2.  The free movement key is accepted only for the priority 0 employees and it allows them to move through any work area at anytime of the day.  No one else is validated through the Free movement key.

Official Pass

When an employee leaves his own work area on an official pass, he presses the official key. All the validation rules expressed here are applicable if the card has been cleared by the basic checks of section 2.

  - An employee can leave his own work area on official pass only after 10:00 AM and during his own shift timings and if he has made his normal in swipe.

  - An employee is permitted back into his own work area only if his last movement had been to leave his own work area on official pass. He cannot get back on his official pass if more than 8 hours have elapsed since the shift end start time (This is already documented earlier in section 2).

  - An employee is permitted to come inside with official pass through his own work are if he has made normal in swipe and his last movement is with no key.

  - An employee is permitted to enter into work areas other than his own within his own shift timings. He may also enter and leave other work areas with official up to 8 hours after his own shift ends.

  - If an employee is working on a new shift with the shift change/emergency key then also he can move with official pass.

Medical Pass

An employee is allowed to go out on medical pass through the factory work area during his shift timings only and  if  he has already come inside with normal swipe or on shift change or on emergency pass. All the validation rules expressed here are only applicable if the card has been cleared by the basic checks of section 2.

  - The employee is permitted to re-enter BHEL on medical pass only if he had gone out on medical pass and he is coming back within the same shift through factory or his own work area.

  - In case of Second and Third shift employees they can go out through their own work area also. But they should come back with in two hours from the left time.

  - In all the other cases, he is not allowed to pass through with Medical pass key.

Personal Pass

An employee is allowed use personal pass during his shift only and only if he has already come inside with normal swipe or on shift change or on emergency pass. All the validation rules expressed here are only applicable if the card has been cleared by the basic checks of section 2.

  - An employee is permitted to enter BHEL with personal pass only if he had left the same shift with personal pass.

  - An employee is permitted to come inside with personal pass through his own work area if he has made normal in swipe and his last movement is with no key

Union Pass

The Union pass key is allowed only on those cards that are marked as issued with union passes. All the validation rules expressed here are only applicable if the card has been cleared by the basic checks of section 2.

  - Swipes with Union key press are rejected for everybody who is not marked with Union flag in the Reference Data.

  - When an employee uses union pass to enter into his own work area, he should have left his own work area with the Union pass earlier in the same shift else, he is not allowed to enter BHEL with Union pass.

  - Other than his own work area, a union pass holder with the union flag set is allowed to enter the Factory or the Administrative work areas if he has joined the day's shift.

Shift Change

Shift change is used to allow an employee to come in a shift other than that entered against his record in the reference data. All the validation rules expressed here are only applicable if the card has been cleared by the basic checks of section 2.

  - In order to allow the employees to change shifts easily, a special shift called D1 (Dummy) is defined. The employee is permitted to enter and leave his own work areas at all the four intervals described in the D1 shift. Please note that only the times  given in the D1 are compared. The complex behavior of dates of the shift periods as documented in Section 3 is applicable to the timing validations. This is because, the shift change cannot predict which shift the employee is joining or leaving as a number of overlapping shifts timings are present in the system.

  - If the current swipe indicates that the employee is coming in with shift change, the swipe is taken as the first swipe of the new shift of the employee.

  - An employee can use shift change to move during the breaks and at the end of the shift only if he has began his shift with Shift Change key.

D1 Shift Timings :  0500 – 0900; 1100 – 1300; 1400 – 1630; 1830 – 2359

## Late Entry

An employee is permitted to make a late entry into his own work area during his shift timings under certain circumstances. All the validation rules expressed here are only applicable if the card has been cleared by the basic checks of section 2.

  - If an employee presses the late entry key through he is well within the shift period. It is accepted as a regular first in swipe of the shift.

  - If an employee remains out of his own work area during the flex lunch period for more than 40 minutes, he cannot enter back with a regular swipe. He can use the Late Entry key.

  - If an employee is coming in for the first time in this shift in the first session (after the end of shift start period and before the break1), he can use the Late entry key.

  - If an employee is coming in for the first time in this shift in the second session  (after the end of lunch break and before the shift end), he can use the Late entry key.

  - If an employee has already start his shift in the first session but is returning late for his second session.

## Emergency Key

An employee maybe permitted to enter only his own work area with Emergency key. All the validation rules expressed here are only applicable if the card has been cleared by the basic checks of section 2.

  - An employee is not permitted entry into BHEL within his own shift timings with this key.

  - An employee is permitted to enter into BHEL outside his own shift timings if there was no previous in swipe made by the employee since the end of the last shift or the last entry after the shift end was also made on emergency key. This is required so that on some emergency work, an employee should be able to enter BHEL any number of times after his shift ends.

  - An employee can go out with Emergency if he came in side with Emergency.

## Sports Key

An employee maybe permitted to enter only his own work area with Sport key during his own shift. All the validation rules expressed here are only applicable if the card has been cleared by the basic checks of section 2.

  - An employee who is a sports pass holder is allowed to enter his own work area (before 10 AM.),only after having entered his own shift with in allowed shift start timing. The sports key cannot be used to enter the shift for the first time.

  - Under no circumstances can the Sports key be used swiping out.

### Exit on leave

An employee may leave BHEL with Exit on Leave key. All the validation rules expressed here are only applicable if the card has been cleared by the basic checks of section 2.

  - Employees are allowed to use the Exit on Leave key only from their own work areas.

  - The Exit on leave maybe used only to swipe out and not swipe in.

### Special Movement

Special movement is enabled for certain cardholders to move on official work, in and out of work areas. All the validation rules expressed here are only applicable if the card has been cleared by the basic checks of section 2.

  - Only those cardholders that are enabled with Special Movement are allowed to use this key, the rest are rejected.

  - Within the cardholders own work areas, the special movement key may be used to make an in swipe anytime within the shift and up to eight hours after the end of the regular shift time if he went out with Special Movement.

  - An employee can use the special movement key to get back into his own work area only if he had last left with the special movement key.

  - An employee can use the special movement key to enter and leave any work area other his own during his shift and up to eight hours after the end of his shift. There is no check for matching the swipe keys for work areas other than his own.

  - An employee can also come with special movement into his own work area if he had normal first swipe and his last movement is with no key.

### Training Key

### Special training key can be used by trainees to move in and out of BHEL any time during their assigned shift and within their work areas. All the validation rules expressed here are only applicable if the card has been cleared by the basic checks of section 2.

  - The cardholder is allowed to swipe in or out at any time within his shift.

  - A cardholder who wants to get in with training key (with in 4 hours from shift end start time) should have made the last swipe out during the same shift with Training key.

### Extra ordinary Key

The Extra ordinary key maybe used at any time to enter or exit any work area at any time. The Extra ordinary key works only if the card has been cleared by the basic checks of section 2.

This key is intended to meet exceptions/emergencies during system stabilization period . This will be withdrawn once system gets stabilized.