ANNEXURE -I

SPECIFICATIONS FOR ACCESS  CONTROL AND ATTENDANCE RECORDING SYSTEM

INTRODUCTION:

At present, the employee swipes card on a semi-proximity card reader which activates the turnstile and after the employee passes through the turnstile his attendance gets recorded in the system. . The data so recorded is used for Payroll as well as for generating number of Management Information Reports.

The semi-proximity cards are issued to all employees and others for entering factory premises which serve as identity cards as well as access control badges.

There are half height tripod type turnstiles controlling the access and gates are manned by CISF.

This activity is now proposed to be done with Biometric solution for Access Control and Attendance Recording.

The proposed solution is to provide for 10000 persons per day. Each person will have number of transactions based on the work requirements (A transaction means either an entry or an exit).

There are 8 entry gates to the factory and each gate is provided with different number of turnstiles. Access to the factory is through these gates.

Other areas are connected through modems and the data is used for Attendance and Management Information reporting.

REQUIREMENTS:

- The Biometric cards readers should operate turnstiles used for access control.

- The card readers should be weather proof and rugged. Since this is an industrial environment, the readers should be enclosed in casing so that they can be protected from water, heat and dust etc.

- These card readers should activate turnstiles after authenticating the person. The authentication should be done after the person shows his biometric identification i.e. finger print. The turnstile should be activated and move in forward direction and the attendance of the person should be recorded in to the system.

- The exit should be permitted through exit reader after showing of identity card. The out entry of the employee should be recorded in the system and this should be used for his attendance purpose.

- Employee can enter through any gate/turnstile and leave through any other gate/turnstile. This should be permissible. However, permissions for entry /exit should also be restricted as per business rules.

- The proximity type cards should have facility to store finger print data in the card itself. There should be provision for storing 3 finger print data in the card. The readers should have the capability of reading the card from a minimum distance of 4 cms.

- Preparation of Identification badges with finger printing activity and lanyard attachment will be undertaken by the vendor.

- Normally, one finger data is used for verification purpose. In case of any problem with the finger, the other impressions shall be used for checking through keypad operation. Vendor should specify the procedure for using other finger print for authentication. However, the response time should be below 3 seconds.

- The identity cards will also be used on proximity card readers used for different access control points in BHEL as per permissions given to the card holder. The proximity reader should activate Door locks for facilitating access.

- All exits will be allowed through exit readers and the attendance will be recorded in the system.

- Existing turnstiles will be replaced with new turnstiles.

We have campus wide network. The solution should consider the connectivity and data transfers across LAN.

- In case of any network non availability the system should be able to retain all transaction data in the local reader and the PC to which these readers are connected.

- The attendance reports generated from the system should be transferred to existing Payroll application for processing i.e. the data should be in soft form.

- It should be possible to integrate the data generated with ORACLE databases or SAP (as the case may be).

- The system should generate many Management Information Reports like Late Attendance, Departmental attendance, Extra hours working, Overtime hours, late night hours, Availability etc. However, reports shall have to be made available on need basis during the entire contractual period.

- The data should be available online for monitoring purposes.

- Business Logic for different shifts, change of shifts, overtime bookings and permissions for attendance, Specific area restrictions, special permissions, allowed hours, permissions for exit etc will be discussed and finalized at the time of placement of order.

- Visitor passes should also contain authentication methodology and system should transfer the instructions to the areas of visit for ease of access and proper identification.

- The card should also integrate with canteen management system

The system should be maintained by the vendor during the contractual period which includes Hardware, Software, Turnstiles and other related equipment. (The vendor is the solution provider for the total contract period).

- The software development and maintenance will be responsibility of vendor.

- The source code of software programs written for BHEL will be property of BHEL.

- All software used for access control or for attendance recording will be licensed software with permission for BHEL to use.

- The cards should have synchronization with the readers so that duplication of cards is not possible. Even if some one tries to duplicate the card with finger printing, the system should reject such cards i.e. any card forgery should not be possible and access should be denied.

- In case an employee forgets his card system should be able to match the finger print pattern with those stored in the server and permit access to the employee.

- There should also be a facility in the software to show the stored photograph in the HR system in case it is needed.

- The card holder should be permitted to access defined locations only

- The data on the server should be capable of storing at least 5 years transactions.

- The services should be available 24x7.

Scheme of Turnstiles at different gates is attached as Annexure-A.

The specifications should be read along with the detailed specifications (Annexure-B).