# ANNEXURE -___

## Specification for dry type Power Distribution Transformer Capacity 3000 KVA, 11/0.415 KV

| Sl. No. | Description | Unit | Requirement |
| --- | --- | --- | --- |
| 1 | Type of Transformer |  | Dry Type |
| 2 | Installation |  | Indoor |
| 3 | Reference Standards |  | IS:11171 /1965,IEC-725 |
| 4 | Class of insulation |  | F |
| 5 | Encloser |  | IP 23 |
| 6 | Type of coil |  | Cast Resin |
| 7 | Winding temperature protection relay be provided |  | As per Standard |
| 8 | Service |  | Continuos |
| 9 | No. of phases |  | 3 |
| 10 | Rated Power | KVA | 3000 |
| 11 | No load voltage | KV | 11 /0.415 |
| 12 | Frequency | HZ | 50 |
| 13 | Vector Group |  | DYN 11 |
| 14 | Type of cooling |  | ANAN |
| 15 | Impedance volt at rated current | % | As per Standard |
| 16 | Tappings (OFF Circuit on HV)selected by bolts and link provided on each phase |  | As per Standard |
| 17 | Impulse Voltage Withstand Level | KVP | As per Standard |
| 18 | Power Frequency Withstand Voltage - H.V.Winding - L. V.Winding | KV (RMS) | As per Standard |
| 19 | Losses (a)No load losses at rated frequency & 100% rated voltage (b)Losses on load |  | As per Standard |
| 20 | Tests (a)Type Test (b)Routine Test |  | As per Standard |