Annexure-A

MPR No. 6230/06/31 Date 09.12.2006

Specification of Mobile Crane 75Ton Capacity

Mobile Crane, 75Ton capacity mounted on pneumatic wheels

- 4 section Trapezoidal boom extending from 11M to 34.6M. - Max. Lifting capacity 75tonnes at 3M radius in blocked condition, with basic boom of 11M. - 4 X 4 wheel drive, 4 wheel steer. - Suitable water cooled diesel engine of adequate HP. - Engine mounted power shift transmission with multi-speeds forwards & reverse. - 4 Nos. hydraulic outriggers. - Fully enclosed 360 degree rotating operator/drives cabin. - Electronic load movement indicator with anti-two block system. - Control levers for all crane motions. - Full hydraulic dual braking system, operating on all wheels and spring applied hydraulically released parking brake operating on front axle. - Suitable earthmover pneumatic tyres. - Alongwith maintenance manual, operation manual, spare parts catalogue, tool kit, one spare wheel recommended fast moving spares for next two years and standard accessories. - Training to operator for proper operation at supplier works required. - Equipment to be load tested & commissioned at BHEL, Haridwar.