BHEL - JHANSI

TENDER DOCUMENT FOR CURING OVEN  , Qty -2 Nos.

Enquiry No. E7160027, DUE ON   27.01.07, Tender Cost: Rs. 500/-

C  O  N  T  E  N  T  S

Page No.

1.         Item Description & Technical Details                                         2-5

2.	Instructions for submission of Tender		                         6

3.	Commercial Terms & Conditions		                                   7-9

4.	Techno-Commercial Bid          		                                    10

( A.K.Banerjee  )		              ( P.K.Pathak )                                    ( Y K Verma )

DGM(MM)	                           Sr. PO (MM)                                      Sr. PO (MM)

1. Item Description & Technical Details :

Equipment		:	Curing oven

Department		:	Transformer Mfg. (Dry Type)

Spec. NO		:	7088 Rev.2

Item No		:	ME-1/4/2574 & 2575

Quantity		:	Two Nos.

CURING OVEN

The oven is to be used for curing of coil of Dry type transformer before & after encapsulation with epoxy resin.

The shall of the oven to be double walled made out of mild steel and suitable reinforced with rolled steel sections. The design to be such that the internal specified dimensions will not be reduced by Air ducts etc.As far as possible, all ducts should be above the floor level and the main structure should be free standing as avoid excavation of existing floor. Internal down comer hot air circulating ducts are also provided with adjustable grills for uniform thermal loss. The oven to be suitably insulated against thermal loss.  The rise in body temperature should not exceed 10 degree centigrade above ambient under maximum operating temperature for comfortable working all around the oven. The oven is to be designed & constructed for continuous operation.

The door is to be fully insulated and provided with clamps for quick clamping. The door is to be vertical sliding type and suitable counter balanced. The door is to be designed so as to ensure easy manual operation. The door is to be interlocked with heating elements through an automatic switch for the safety of operator, cutting off automatically the power supply to the heating elements as soon as door is opened.

1 No. pedestrian door of size 750 mm wide x 1800 mm high on one side of the oven is to be provided. The pedestrian door is to be provided with suitable handles and to be made to operate from both inside & outside of the oven. Suitable access ladder, platform with safety rails are to be provided on the top of the oven for doing maintenance work on blowers and exhaust system.

The oven is to be made suitable for end loading & 2 Nos. perforated top platform trolleys moving on rails for loading & unloading of charge load of 5 MT. Suitable rails to be laid inside the oven & extended outside to minimum one trolley length fitted flush with floor level. Loaded trolleys are to be easily pushed-in & pulled out after the change is heated up. Provision is to be made for mechanically propelling the trolleys into & out of the oven. Suitable hooks to be provided for pulling/pushing the trolley inside /out side of the oven and handle the trolley with EOT crane.

The stainless steel tubular type heating elements of adequate rating are to be provided as to ensure long working life and placed in such a way that inside chamber is evenly heated. The heating elements shall  be divided in 4 wattage circuits controlled through 4 triple pole rotary switches for selection of different rate of heating for 80/130/160/200 not be loaded mare than 75% of rated capacity for the specified duty. The heaters are to be placed in such a way that they can be removed easily for replacement or maintenance work without the necessity of removal of insulation.  The leads for heaters are to be brought out through insulated porcelain bushing & connected to insulated terminal plate. The terminal plate to have easy accessibility for connection & maintenance. Heaters to be connected through Solid State Relays for noise free operation.

Interior is to be illuminated by using bulk head type light fittings mounted on side panels & roof. Two Nos. insulated terminals suitable for 5 KV are to be provided on either side of the oven & mounted at 1200mm from floor level for megger & tan delta measurements.

The oven is to be provided with two Nos. Air circulating fans of suitable rating for rapid and equidistribution of heat inside the oven. Detailed specification of the Air circulating fan is to be furnished with the offer. The air circulating fan is be interlocked with heating elements in such a way that heating elements will not be energized unless Air circulating fan is running and alternatively failure of Air circulating fan will automatically cutoff the supply to heating elements. The oven is to be continuous bleeding” type which can be adjusted from 5% to 100% or for all fresh air makeup air facility. Suitable extraction equipment with ducting on discharge side is to be provided.

The temperature inside the oven is to be uniform with precise temperature control. The temperature is to be thermo statically controlled by sensing units placed at suitable locations. Thermocouples with fail-safe device is to be provided with suitable length of flexible compensating leads for connection to temperature controllers, indicators & recorders temperature indicators & controllers suitable for per setting of any desired temperature for automatic control from 50 to 200 degree centigrade is to be provided. A safety in the event of main thermostat / temperature controller failure with facility to set the safety thermostat at 10 degree centigrade above the preset thermostat on the temperature controller. Provision is to be made for audio visual alarm in the event of over shooting of temperature over the preset value & cutoff of power to the heating elements. A dial type thermometer of size 300 mm dia with least cont of 2 degree centigrade or less for indicating the inside temperature of the oven is to be provided at a convenient location for easy reading. Also , multipoint temperature recorder is to be provided to record the temperature at least at 4 locations. In addition to the above , at least 6 Nos. flexible thermocouples should be available to record temperatures of individual coils being processed in the oven. The temperature recorder is to be operated electrically by means of a synchronous motor.

The oven is also to be provided with a microprocessor based programmable temperature controller with facility for measurement, display, control, alarm, and annunciation. Detailed data sheet & write-up to be submitted with the offer.

All switches, relays, indicating lamps, contactors, fuses, main switch, thermometers & temperature recorder, overload protection device etc. are to be grouped on a control panel with hinged front & back access doors suitable for floor mounting is to be provided. The control panel may be erected within a distance of 3 meters from the oven. The control panel is to be complete with all internal wiring, inter-connection between the oven & control cubicle etc.

All electrical equipments are to be suitable for operation in an ambient temperature varying between 5 to 50 degree centigrade & 95% R.H. all electrical equipments are to be suitable for operation on 415 Volts, 3 phase, 4 wire, 50 HZ, A.C. supply.

The furnace is to be painted with one coat of red oxide primer & finished with 2 at site after commissioning.

Provision is to be made the services of a test engineer to set all the controls & demonstrate the operation of the oven at site.

Specification:

- Effective internal width of oven 	: 	2000 mm

- Effective internal depth of oven	:	5000 mm

- Effective inter height of oven		:	2500 mm

- Dimensions of trolley’s           height	:	230 mm

Width:	1800 mm

Length:           2400 mm

No. of trolleys:           2 Nos.

5.	Charge weight 				:

Copper		:	4000 kgs.

Epoxy 	resin	:	1000 kgs.

Structural steel:	5000 kgs.

6.	Rail level from floor 			:	Flush with floor

7.	Operating temperature range		:	40 to 200 degree centigrade

8.	Temperature regulation range		:	± 2 degree centigrade

9.	Time required to reach 200 degree

Centigrade from 50 degree centigrade

With charge 				:	3 hrs

10.	Time required to reach 160

Degree centigrade from 50 degree 	:	2 hrs

Centigrade with charge

Details to be submitted with the offer:

- Dimensioned outline general arrangement drawing & weight.

- Design, rating, material & make of heating elements used.

- Calculation details of rating of heating elements & mounting arrangement.

- Make, type, capacity & write-up on temperature indicator, temperature controller, temperature recorder, and microprocessor based programmable temperature controller used.

- Make, type, capacity & write-up on air circulating fans & exhaust fan with their specifications.

- Drawing showing method of bleeding, air extraction & intake system.

- Details of safety devices incorporated.

- Electrical schematic diagram.

- Schematic diagram of control panel.

- The temperature recorder & control circuit diagram.

- Total electrical load.

- Temperature rise/hour.

- Thickness of M.S. sheet used for walls.

- Special flooring required, if any,

- Quotation with itemized prices for with duration.

- Election & commissioning charges with duration.

- Alternative offer for raising temperature to reach 200 degree centigrade from ambient in 4 hrs.

- A list of customers with their addresses to whom similar/identical equipments have been supplied in last 5 years.

QUALIFYING CONDITIONS

Only those vender, who have supplied and commissioned at least two curing oven for similar application in the past ten years and such oven is presently working satisfactorily for more than one year (more than six month if supplied to BHEL) after commissioning should quote. The following information is to be submitted by the vender about the companies where similar ovens have been supplied. This is required from venders for qualifications of their offers.

- Name of the customer/ company where similar machine/plant is installed

- Complete postal address of the customer

- Year of commissioning

- Name and designation of the contact person of the customer

- Phone , Fax No. and e.mail address of the contact person of the customer

- Performance certificate from the customer regarding satisfactory performance of the machine/ plant supplied to them

2.INSTRUCTIONS FOR SUBMISSION OF TENDER

- The offer against the enquiry shall be submitted in two parts.  The first part called Techno-commercial bid i.e. Part - A shall be un-priced and will be submitted in line with given details in commercial terms and conditions .  The second part called Price-Bid  i.e. Part-B, in this you will have to mention the basic prices, taxes duties etc for basic equipment & Standard / optional accessories if any.

- The vendors shall submit Part-A and Part-B of their bids in two different sealed envelopes duly super-scribing Enquiry No. and Part No. of the bid.  These two parts of the bid shall be kept together in a third envelope super-scribing Enquiry No., due date and time of opening as indicated in the tender document.

- Part-A of the bid alone shall be opened on the due date in presence of the tenderers in the first instance.   This part of the bid shall be examined in detail.  Part B of the bid shall be opened only for such of the tenderers whose Techno-Commercial offers are found acceptable by BHEL.  Such tenderers shall be advised the date and time of opening of Part B i.e. price part of their bids.  Tenderers must carefully go through all the instructions and conditions before filling in Part B of the bid.  Any type of mistake in submission of the bids will make the tender liable for rejection.

- The bids shall be submitted only in the format given in the tender documents. The tenderer is  required to sign and stamp each and every page of the tender document clearly bring out deviations, if any.

- Tenders not complying with the above and other requirements of the tender shall be liable for rejection.  Tenderers are advised to read all the documents carefully before submission of the bids and ensure the fulfillments of all the technical and commercial parameters.

- No over-writing in any Part-A or Part-B is permitted.  Corrections in the bid should normally be avoided. However, when these become essential, bidder shall score out the incorrect information and put the correct information duly endorsing each correction with his signatures.

- Tender Cost (if any) is to submitted in the Form of  DD in favour of BHEL-Jhansi  payable at

Jhansi along with Techno –Commercial offer. This DD should not be kept in Sealed Price Bid.

3.COMMERCIAL TERMS & CONDITIONS FOR PURCHASE ENQUIRY

- Offer is invited in two bids, outer cover, inner cover-I (Techno-Commercial bid) and inner cover II (Price-Bid) must bear our tender no., due date and bidders name.  Offers received in single bid will be bypassed.

- The offer should reach this office on or before 13.15 hrs on the due date.  The tender can also be dropped in the Tender Box kept at Administrative Building Gate.  The tenders will be opened after 1400 hrs on the due date in presence of all bidders who will be present.

- Offer should be free from overwriting or erasures.  Corrections or additions, if any should be attested.

- No enhancement of rate for whatever cause is allowed once the offer is accepted and the order is placed.  Withdrawal from the quotation after it is accepted or failure to make the supply within the scheduled period will entail cancellation of order, forfeiture of earnest money if any and/or imposition of risk & cost purchase.

- Material offered should be of required quality and conforming strictly to our specifications / drawings or to the specifications in the drawings.  Any deviation must be clearly stated in the offer with particular clause /detail in our specification / drawing for which deviation is quoted.  All required documents/details  e.g. QA Plan, type TCs, drawings & data sheet etc. must be submitted along with the offer.

- Irrespective of whether it is mentioned in our specification, all machines / equipments are required to be calibrated in metric units and certificates must be issued where applicable.

- Work test  certificates / inspection certificates/guarantee certificate/ compliance certificate, etc. will be required to be submitted along with supplies in case an order is placed.

- Required Delivery  time for affecting supplies should be indicated in the offer.

- Failure to supply by the time specified on the order will make the supplier liable to pay unconditional penalty of ½% of the prices of goods in arrear per week subject to a maximum of 10% for Indian suppliers and max 5% for Foreign suppliers of the order value unless agreed otherwise.

- The quantities for each item to be purchased may vary from the tendered quantity according to our actual requirement at the time of placing orders.

- Bharat Heavy Electricals Ltd does not bind itself to accept the lowest of any tender, but reserves to itself the right to accept or reject all or a part of any tender at its discretion following BHEL Purchase Policy & Government guidelines.

- In all cases of disputes, the matter shall be referred for arbitration.  The arbitrator  shall be nominated by the Unit Head, Bharat Heavy Electricals Ltd., Jhansi or any other officer nominated by him.  The award of Arbitrator shall be final and binding on both the parties.

- In case of any suit or other legal proceedings under or relating to this tender, the courts at Jhansi shall have full jurisdiction.

- Total delivered cost at BHEL Jhansi Railway siding/Works shall be the criterion for deciding the tender.

- Unless otherwise specified, offer should be on ex-works basis (for Indian Suppliers) with freight & insurance to be borne by BHEL.

- Your quotation against this enquiry must be valid for ordering up to minimum of 120 days from the date of tender opening.

- ADDITIONAL FOR INDIAN SUPPLIERS:

I)        If any material is booked to selves or if it is not consigned to BHEL Jhansi, any warfage or

demurrage occurring from these shall be to the account of supplier.  In case of road transport,

material shall be booked only through our rate contract transporters mentioned in the purchase

order.

- Where there is provision for payment of Sales Tax extra, it will be paid only if the registration number both under State Sales Tax and Central Sales Tax are specifically mentioned on the bill/invoice.  Your Sales Tax Registration number should be mentioned in the offer positively.

- Excise Duty Gate Pass will be required in original.  Applicable taxes & duties must be included in the offer.

- ADDITIONAL FOR FOREIGN SUPPLIERS:

i)	For documentation and correspondence language shall be English only.

- Rates quoted should be on FOB basis.  All despatches are to be made by sea  by IPBC conference vessels only unless otherwise stated in purchase order.

- Agency commission, if any, will be paid out of the FOB value after receipt & acceptance of material at BHEL Jhansi in Indian Rupees.  TT Buying exchange rate prevailing on the tender opening date or the letter of credit date or date of purchase order whichever is lowest will be considered for calculating the agency commission.

- The rates quoted should be inclusive of charges of packing and delivery of material up to the port of shipment on FOB terms.  The material is required to be packed suitably in all respects considering the peculiarity of the material for normal transport by Sea/air and subsequently by Rail/Road in India suitably protected against the effect of tropical salt laden atmosphere in the event of consignment being delayed at Indian Ports before outward clearance.  You should also furnish details of shipping specification, sizes, volume, weight, number of cases etc. When articles are packed with packaging material of plant origin, phytosanitary certificate will be required.

- Our standard terms of payment is 100% payment through bank against despatch documents or 100% through sight irrevocable & unconfirmed letter of credit.

19.   Security deposit (SD) cum Performance Bank Guarantee  (PBG) for 10% basic value of the order (excluding taxes &

duties) for  execution of the purchase order (PO) & satisfactory Performance of the equipment before release of  PO

will be required to submit which will be valid from the date of issue of LOI to 24 months after acceptance of material by

BHEL with a further claim period of 3 months. PO will be issued only after receipt of SD cum PBG.

20. For fully imported equipments / plant/ machinery (no part or sub-assembly of Indian origin) :

a. No SD cum PBG will be applicable for basic order value up to Rs. 25.0 Lacs on foreign supplier.

b. No SD cum PBG will be applicable for basic order value up to Rs. 10.0 Lacs on Indian stockist / agent of

foreign manufacturer.

21.   B.G. shall be from Scheduled Banks/Public Financial Institutions as defined in the Companies Act.

22. The equipment shall be guaranteed for 24 months from the date of acceptance by BHEL irrespective of date of supply,

erection and commissioning.

23. Our standard terms of payment (for Indian Suppliers) is 80% of P.O. value be paid after 60 days of acceptance of the

equipment at BHEL Jhansi works and balance 20% after successful commissioning & handing over of the equipment

(where applicable).

24. Amount of Service Tax / Education Cess / any other Govt. duty ( as applicable on the date of offer) for erection or any

other activities associated with the scope of work in the tender must be indicated in Techno-commercial bid of the offer.

25 . Blank Price Bid (After erasing prices) must be submitted along with Techno commercial bid.

26.  If any goods are rejected, BHEL shall be at liberty to take action as per following at the

risk and cost of supplier:-

    - Allow the supplier, wherever mutually agreed, to rectify the rejected goods at BHEL's works within reasonable time as fixed by BHEL.

    - Allow the supplier to make free replacement within a specified period, rejected goods can be lifted by the supplier thereafter.

OR

Allow the supplier to refund the full amount paid to him by BHEL by a Demand Draft  before lifting the rejected goods.  Fresh replacement shall be regularized as per terms & conditions of the original purchase order.

OR

Take alternative  procurement action from elsewhere and recover the difference in cost, if any incurred by BHEL in this regard from the supplier.  The supplier shall not be entitled to any gain on purchase.

    - Terminate the contract either in part or in whole at the  discretion of BHEL and recover the loss, if any, from the supplier.

27. For any clarification on our tender terms and conditions please contact Sr. Purchase Officer (Capital Items)

before due date.

28. Inspection may be done at supplier works by BHEL.

## 4. TECHNO COMMERCIAL BID  PART-A

( Supplier should fill up this bid by putting the tick in accepted / non accepted column)

SUPPLIER’S NAME & ADDRESS

1.          Offered Equipment with complete technical details  :                      As per attached details

2.           Packing & Forwarding Charges, FOB charges (if any)                                                               ….%

3.   	Present rate of Excise duty (if any)         		                                                              ….%

4.     	Present rate of sales tax			 		                                                   %

(UPTT/CST, Not applicable in case of Foreign suppliers)

5.	Delivery period in  No. of  days from  date of LOI / PO                                                                                                                    6.	Taxes and duties shall be extra as applicable on 		              Accepted / Not accepted

the date of actual delivery.

7.	Penalty clause as well as other conditions given			Accepted / Not accepted

in general Terms & conditions and Standard Enquiry

terms & conditions are acceptable to us.

8.           Rates quoted in the price bid are on Ex Works / F.O.R. 		---------------------------------

Jhansi .basis.

9.	Validity of the rates quoted is 120 days from the 			Accepted / Not accepted

date of opening of price bid.

10.          Security deposit (SD) cum Performance Bank Guarantee                      Accepted / Not accepted

(PBG) for 10% basic value of the order (excluding taxes & duties)

for  execution of the purchase order (PO) & satisfactory

Performance of the equipment up to Guarantee period

of 24 months  before release of PO will be submitted.

11.        The equipment shall be guaranteed for 24 months from                            Accepted / Not accepted

the date of acceptance by BHEL irrespective of date of

supply, installation.

Note :- If any of the above conditions are not accepted

the offer shall be liable for rejection.