BHEL - JHANSI

TENDER DOCUMENT FOR DIESEL ELECTRIC MOBILE CRANE, Qty -1 No.

Enquiry No. E7160029, DUE ON   27.01.07, Tender Cost: Rs. 1,000/-

C  O  N  T  E  N  T  S

Page No.

1.         Item Description & Technical Details                                         2-5

2.	Instructions for submission of Tender		                         6

3.	Commercial Terms & Conditions		                                   7-9

4.	Techno-Commercial Bid          		                                    10

( A.K.Banerjee  )		              ( P.K.Pathak )                                     ( Y K Verma )

DGM(MM)	                           Sr. PO (MM)                                      Sr. PO (MM)

1. Item Description & Technical Details :

Equipment		:	Material handling

Spec NO		:	10549

Item No		:	MH-3/40/9610

Quantity		:	1 No

Title			:	Diesel Hydraulic Mobile Crane 15 Ton

General

Diesel operated hydraulic mobile crane of lifting capacity 15.00 tons at 3.5 mtrs. radius in blocked condition consist of 3 section hydraulic fully powered, fully synchronized telescopic boom extending from 8.80 M to 21.2 mtr.,

Road travel speed	:	30 Km/hr

The crane: should incorporate hydraulic out riggers self re-setting limit switches to hoist and derrick motions, automatic electro-mechanical brakes on hoist. It should have derrick and slewing motions, automatic safe load indicator, it should have overload warring system with positive cut outs,

Steering – Steering axle should be mounted solidly on the steel beam to the chassis frame it should have automatic type steering wheel and hydraulic powered.

Operator’s cab – It should be totally enclosed steel construction with glass windows lockable sliding door, electric fan & electric horn etc. it should incorporate of the crane controls for derricking motions, slewing motions and housing motions it should have foot paddle brake control & all other travel controls. 360 degree revolving operator cabin should be provided It should incorporate all the required instrumentations like gauges for air, pressure load etc.

Following detail to be provided in technical offer.

1. Chassis along with tyre, size, nos. & make etc.

2. Slewing arrangement detail electrical /hydraulic/ mechanical.

3. Hoisting and lowering arrangement elect./hydraulic/mechanical.

4. Hoisting/lowering slewing brake arrangement.

5. Hoisting speeds, lowering speeds & slewing speeds.

6. Maximum& Minimum speed of crane (A) Without load

(B) With full load

7. Details of prime mover engine

8. Details of hyd.circuit diagram and hydraulic items fitted on crane

9. Detail of out rigger arrangement

10. Details of operator cabin and operator control panel

MISCELLANEOUS:

- The offer should be complete with dimensioned outline general arrangement DRAWING WITH complete details of power transmission system, hydraulic system with circuit diagram and write up explaining special features.

- List of standard accessories supplied.

- List of optional accessories offered with item wise prices.

- Recommended spares for 2 year normal working to be quoted with item wise prices

- Optional equipment to be quoted extra for environmentals protection.

- A list of customers to whom similar/identical equipments have been supplied in last 3 year with their names and addresses.

The equipment shall be tested/inspected by BHEL engineers at supplier’s works as per test chart in accordance to IS for all accuracies & performance before dispatch. Equipment shall be guaranteed for its satisfactory performance for a period of 12 months from the data of commissioning along with guarantee certificate four copies of test & maintenance manuals, foundation/general arrangement & other drawing. Schematic circuit diagram, spares parts catalogues with make must be supplied before dispatch of the equipment.

Not with standing anything, the detail this specification, if anything else is required for it’s functioning, the same shall be specifically brought out & indicated. The tenderer is to submit complete details of the equipment in the first instance. Offers without complete details are liable to be rejected.

QUALIFYING CONDITIONS

Only those vender, who have supplied and commissioned at least two hydraulics mobile crane for similar application in the past ten year and such crane is presently working satisfactorily for more than one year (more then six month if supplied to BHEL ),after commissioning should quote. The following information is to be submitted by the vender about the companies where similar crane have been supplied. This is required from all venders for qualifications of their offers.

- Name of the customer/company where similar crane installed

- Complete postal address of the customer

- Year of commissioning

- Name and designation of the contact person of the customer

- Phone, Fax No. and e. mail address of the contact person of the customer

- Performance certificate from the customer regarding satisfactory performance of the crane supplied to them

For general requirement: Refer annexure I& II

ANNEXURE- I

General Conditions for Machines/Equipments:

ELECTRIC SUPPLY & CONTROL :

All electricals to be suitably tropicalised and designed to work on 415 volts, 3 Phase, 4 Wires, 50 Cycles A.C. supply provided in the circuit. Suitable step down transformers must be included in the offer to provide low voltage system. All switches, relays, controls etc. are to be grouped conveniently. Electric Motors  to confirm to BSS:2613/ relevant Indian Standards and shall have 'B' class insulation and of a reputable make like M/S SIEMENS, CROMPTON, KIRLOSKAR, AEI, GEC, NGEF etc. Starters to be incorporated.

SPARES :

Recommended spares are to be quoted for two years normal working with item wise price, details and relevant catalogues should be supplied, wherever applicable.

LUBRICATION :

Automatic centralized Lubrication system with suitable lubricating points should be provided. Details with type and quantities of lubricants are to be furnished including their source of supply. Lubrication points are to be displayed  on the body of machine / equipment.

PAINTING :

Machine / Equipment is to be painted with two coats of primer and two coats of finished paint. Colour of the finished coat shall be Verdigris green paint confirm to relevant I.S. . All bright steel components to be coated with rust preventive easily removable chemical  before dispatch.

DETAILS TO BE INCLUDED IN THE OFFER  :

- General arrangement drawing., foundation drawing., weight etc.

- Technical details , relevant catalogues, circuit diagram etc.

- Details of motor, starter , pump, hydraulics, pneumatics, controls etc. like type, make, model, rating etc.

- Types and details of various parts subject to wear & tear, such as bearings, gears etc., should be indicated together.

- Requirement of power water, air steam etc. with details like quantity and pressure.

- List of standard accessories to be supplied with the equipment / machine.

- List of optional accessories with item wise price and details.

- List of customers to whom similar / identical equipments have been supplied. Size / capacity of equipment, date of supply and customers full address are also to be indicated.

MISCELLANEOUS :

- Entire machine / equipment to be completely tested and inspected and four copies of the test / inspection  & guarantee certificates to be supplied with the equipment / machine. BHEL reserves the right to inspect and witnessed all tests. In the event of inspection / test not witnessed by BHE, a written approval should be obtained before commencement of dispatch.

- It is to be guaranteed  for 24 months from date of commissioning. Any parts of unit of the machine / equipment as a whole found damaged or defective during this period have to be replaced free of cost. Free after sales services is to be provided during guarantee period.

- Four copies of technical / descriptive catalogues, operating and service manuals, diagrams etc.have to be supplied with the machine / equipment.

- Not withstanding anything the details in this specification if any thing else is required for its efficient functioning, the same shall be specially brought out and included in the offer.

IMPORTANT NOTE :

- Complete unit including motors etc. shall be of adequate capacity for the duties, it is required to perform.

- All components / materials / controls used shall be in metric system confirming to relevant Indian / International standard.

- Installation & commissioning charges with duration , wherever applicable, shall be quoted separately.

ANNEXURE - II

General condition of supply of spares & maintenance manual.

Following details are required along with offer:

- Complete spare parts list of the machine. This list must include all manufactured  as well as bought out items of suppliers. For bought out items complete purchase specification and full address of supplier must be given.

- Cross sectional drawing for all sub assemblies to be submitted.

- Hydraulic / Pneumatic circuit diagram along with sequence of function to be described.

- Electrical circuit diagram to be supplied along with its working description in relation with hydraulic / pneumatic circuit.

- Details along with purchase specifications of all rubber 'O' rings, oil seals, wiper seals, hoses, solenoid valve etc. to be given. Dimensional details of all rubber parts and hoses with material specification to be given.

- Party to ensure that spares for the offered machine shall be available with them for another next ten years.

- All contactors and relays shall be of Siemens / L&T  make.

- Main isolator shall be G.E. power control make with GE power control make HRC fuses.

- All hydraulics shall be of Vickers sperry / Rexroth make only.

- Pneumatic timer shall be of BCH make .

- All bearing should be of SKF /NBC / Fag make only.

- All fastener shall be of Unbrako / GKW / TVS amake high tensile steel.

- All pneumatics shall be of Festo / Nucon / Dancal make only.

- All gear boxes shall be of Elicon / Redicon / Shanti make only.

2.INSTRUCTIONS FOR SUBMISSION OF TENDER

- The offer against the enquiry shall be submitted in two parts.  The first part called Techno-commercial bid i.e. Part - A shall be un-priced and will be submitted in line with given details in commercial terms and conditions .  The second part called Price-Bid  i.e. Part-B, in this you will have to mention the basic prices, taxes duties etc for basic equipment & Standard / optional accessories if any.

- The vendors shall submit Part-A and Part-B of their bids in two different sealed envelopes duly super-scribing Enquiry No. and Part No. of the bid.  These two parts of the bid shall be kept together in a third envelope super-scribing Enquiry No., due date and time of opening as indicated in the tender document.

- Part-A of the bid alone shall be opened on the due date in presence of the tenderers in the first instance.   This part of the bid shall be examined in detail.  Part B of the bid shall be opened only for such of the tenderers whose Techno-Commercial offers are found acceptable by BHEL.  Such tenderers shall be advised the date and time of opening of Part B i.e. price part of their bids.  Tenderers must carefully go through all the instructions and conditions before filling in Part B of the bid.  Any type of mistake in submission of the bids will make the tender liable for rejection.

- The bids shall be submitted only in the format given in the tender documents. The tenderer is  required to sign and stamp each and every page of the tender document clearly bring out deviations, if any.

- Tenders not complying with the above and other requirements of the tender shall be liable for rejection.  Tenderers are advised to read all the documents carefully before submission of the bids and ensure the fulfillments of all the technical and commercial parameters.

- No over-writing in any Part-A or Part-B is permitted.  Corrections in the bid should normally be avoided. However, when these become essential, bidder shall score out the incorrect information and put the correct information duly endorsing each correction with his signatures.

- Tender Cost (if any) is to submitted in the Form of  DD in favour of BHEL-Jhansi  payable at

Jhansi along with Techno –Commercial offer. This DD should not be kept in Sealed Price Bid.

3.COMMERCIAL TERMS & CONDITIONS FOR PURCHASE ENQUIRY

- Offer is invited in two bids, outer cover, inner cover-I (Techno-Commercial bid) and inner cover II (Price-Bid) must bear our tender no., due date and bidders name.  Offers received in single bid will be bypassed.

- The offer should reach this office on or before 13.15 hrs on the due date.  The tender can also be dropped in the Tender Box kept at Administrative Building Gate.  The tenders will be opened after 1400 hrs on the due date in presence of all bidders who will be present.

- Offer should be free from overwriting or erasures.  Corrections or additions, if any should be attested.

- No enhancement of rate for whatever cause is allowed once the offer is accepted and the order is placed.  Withdrawal from the quotation after it is accepted or failure to make the supply within the scheduled period will entail cancellation of order, forfeiture of earnest money if any and/or imposition of risk & cost purchase.

- Material offered should be of required quality and conforming strictly to our specifications / drawings or to the specifications in the drawings.  Any deviation must be clearly stated in the offer with particular clause /detail in our specification / drawing for which deviation is quoted.  All required documents/details  e.g. QA Plan, type TCs, drawings & data sheet etc. must be submitted along with the offer.

- Irrespective of whether it is mentioned in our specification, all machines / equipments are required to be calibrated in metric units and certificates must be issued where applicable.

- Work test  certificates / inspection certificates/guarantee certificate/ compliance certificate, etc. will be required to be submitted along with supplies in case an order is placed.

- Required Delivery  time for affecting supplies should be indicated in the offer.

- Failure to supply by the time specified on the order will make the supplier liable to pay unconditional penalty of ½% of the prices of goods in arrear per week subject to a maximum of 10% for Indian suppliers and max 5% for Foreign suppliers of the order value unless agreed otherwise.

- The quantities for each item to be purchased may vary from the tendered quantity according to our actual requirement at the time of placing orders.

- Bharat Heavy Electricals Ltd does not bind itself to accept the lowest of any tender, but reserves to itself the right to accept or reject all or a part of any tender at its discretion following BHEL Purchase Policy & Government guidelines.

- In all cases of disputes, the matter shall be referred for arbitration.  The arbitrator  shall be nominated by the Unit Head, Bharat Heavy Electricals Ltd., Jhansi or any other officer nominated by him.  The award of Arbitrator shall be final and binding on both the parties.

- In case of any suit or other legal proceedings under or relating to this tender, the courts at Jhansi shall have full jurisdiction.

- Total delivered cost at BHEL Jhansi Railway siding/Works shall be the criterion for deciding the tender.

- Unless otherwise specified, offer should be on ex-works basis (for Indian Suppliers) with freight & insurance to be borne by BHEL.

- Your quotation against this enquiry must be valid for ordering up to minimum of 120 days from the date of tender opening.

- ADDITIONAL FOR INDIAN SUPPLIERS:

I)        If any material is booked to selves or if it is not consigned to BHEL Jhansi, any warfage or

demurrage occurring from these shall be to the account of supplier.  In case of road transport,

material shall be booked only through our rate contract transporters mentioned in the purchase

order.

- Where there is provision for payment of Sales Tax extra, it will be paid only if the registration number both under State Sales Tax and Central Sales Tax are specifically mentioned on the bill/invoice.  Your Sales Tax Registration number should be mentioned in the offer positively.

- Excise Duty Gate Pass will be required in original.  Applicable taxes & duties must be included in the offer.

- ADDITIONAL FOR FOREIGN SUPPLIERS:

i)	For documentation and correspondence language shall be English only.

- Rates quoted should be on FOB basis.  All despatches are to be made by sea  by IPBC conference vessels only unless otherwise stated in purchase order.

- Agency commission, if any, will be paid out of the FOB value after receipt & acceptance of material at BHEL Jhansi in Indian Rupees.  TT Buying exchange rate prevailing on the tender opening date or the letter of credit date or date of purchase order whichever is lowest will be considered for calculating the agency commission.

- The rates quoted should be inclusive of charges of packing and delivery of material up to the port of shipment on FOB terms.  The material is required to be packed suitably in all respects considering the peculiarity of the material for normal transport by Sea/air and subsequently by Rail/Road in India suitably protected against the effect of tropical salt laden atmosphere in the event of consignment being delayed at Indian Ports before outward clearance.  You should also furnish details of shipping specification, sizes, volume, weight, number of cases etc. When articles are packed with packaging material of plant origin, phytosanitary certificate will be required.

- Our standard terms of payment is 100% payment through bank against despatch documents or 100% through sight irrevocable & unconfirmed letter of credit.

19.   Security deposit (SD) cum Performance Bank Guarantee  (PBG) for 10% basic value of the order (excluding taxes &

duties) for  execution of the purchase order (PO) & satisfactory Performance of the equipment before release of  PO

will be required to submit which will be valid from the date of issue of LOI to 24 months after acceptance of material by

BHEL with a further claim period of 3 months. PO will be issued only after receipt of SD cum PBG.

20. For fully imported equipments / plant/ machinery (no part or sub-assembly of Indian origin) :

a. No SD cum PBG will be applicable for basic order value up to Rs. 25.0 Lacs on foreign supplier.

b. No SD cum PBG will be applicable for basic order value up to Rs. 10.0 Lacs on Indian stockist / agent of

foreign manufacturer.

21.   B.G. shall be from Scheduled Banks/Public Financial Institutions as defined in the Companies Act.

22. The equipment shall be guaranteed for 24 months from the date of acceptance by BHEL irrespective of date of supply,

erection and commissioning.

23. Our standard terms of payment (for Indian Suppliers) is 80% of P.O. value be paid after 60 days of acceptance of the

equipment at BHEL Jhansi works and balance 20% after successful commissioning & handing over of the equipment

(where applicable).

24. Amount of Service Tax / Education Cess / any other Govt. duty ( as applicable on the date of offer) for erection or any

other activities associated with the scope of work in the tender must be indicated in Techno-commercial bid of the offer.

25 . Blank Price Bid (After erasing prices) must be submitted along with Techno commercial bid.

26.  If any goods are rejected, BHEL shall be at liberty to take action as per following at the

risk and cost of supplier:-

    - Allow the supplier, wherever mutually agreed, to rectify the rejected goods at BHEL's works within reasonable time as fixed by BHEL.

    - Allow the supplier to make free replacement within a specified period, rejected goods can be lifted by the supplier thereafter.

OR

Allow the supplier to refund the full amount paid to him by BHEL by a Demand Draft  before lifting the rejected goods.  Fresh replacement shall be regularized as per terms & conditions of the original purchase order.

OR

Take alternative  procurement action from elsewhere and recover the difference in cost, if any incurred by BHEL in this regard from the supplier.  The supplier shall not be entitled to any gain on purchase.

    - Terminate the contract either in part or in whole at the  discretion of BHEL and recover the loss, if any, from the supplier.

27. For any clarification on our tender terms and conditions please contact Sr. Purchase Officer (Capital Items)

before due date.

28. Inspection may be done at supplier works by BHEL.

## 4. TECHNO COMMERCIAL BID  PART-A

( Supplier should fill up this bid by putting the tick in accepted / non accepted column)

SUPPLIER’S NAME & ADDRESS

1.          Offered Equipment with complete technical details  :                      As per attached details

2.           Packing & Forwarding Charges, FOB charges (if any)                                                               ….%

3.   	Present rate of Excise duty (if any)         		                                                              ….%

4.     	Present rate of sales tax			 		                                                   %

(UPTT/CST, Not applicable in case of Foreign suppliers)

5.	Delivery period in  No. of  days from  date of LOI / PO                                                                                                                    6.	Taxes and duties shall be extra as applicable on 		              Accepted / Not accepted

the date of actual delivery.

7.	Penalty clause as well as other conditions given			Accepted / Not accepted

in general Terms & conditions

and Standard Enquiry terms & conditions are

acceptable to us.

8.           Rates quoted in the price bid are on Ex Works / F.O.R. 		---------------------------------

Jhansi .basis.

9.	Validity of the rates quoted is 120 days from the 			Accepted / Not accepted

date of opening of price bid.

10.          Security deposit (SD) cum Performance Bank Guarantee                      Accepted / Not accepted

(PBG) for 10% basic value of the order (excluding taxes & duties)

for  execution of the purchase order (PO) & satisfactory

Performance of the equipment up to Guarantee period

of 24 months  before release of PO will be submitted.

11.        The equipment shall be guaranteed for 24 months from                            Accepted / Not accepted

the date of acceptance by BHEL irrespective of date of

supply, installation.

Note :- If any of the above conditions are not accepted

the offer shall be liable for rejection.