BHEL - JHANSI

TENDER DOCUMENT FOR VACUUM DRYING PLANT FOR BAY-8, Qty -1 No.

Enquiry No. E7160030, DUE ON   27.01.07

C  O  N  T  E  N  T  S

Page No.

1.         Item Description & Technical Details                                         2-6

2.	Instructions for submission of Tender		                         7

3.	Commercial Terms & Conditions		                                 8-10

4.	Techno-Commercial Bid          		                                    11

( A.K.Banerjee  )		   ( P.K.Pathak )                                  ( Y K Verma )

DGM(MM)	                           Sr. PO (MM)                                      Sr. PO (MM)

1. Item Description & Technical Details :

##### ITEM					:	Vacuum Drying Vessel

ITEM NO.				:	ME- 1 / 6/2576

SPECIFICATION NO.		:	7096A

LOCATION				:	BAY-8 EXTENDED

QUANTITY			:	1 No.

VACUUM DRYING VESSEL

GENERAL  : -

- The vacuum drying vessel is to be used for processing and oil impregnation of ESP Transformers under vacuum. The vessel is to be designed to take a pay load of 40MT which is comprised of following materials : -

i)	Copper				:	10 MT

ii)	Steel					:	25MT

- Insulation ( Fuller board,

craft paper and parma wood)

including moisture content of 10 %

to 15% 				:	5 MT

The above pay load does not include weight of trolley on rails and oil. The preliminary and final drying of transformers is to be carried out under vacuum of 0.5 mbar .

2	VESSEL DETAILS : -

2.1	The vessel is to be mounted in a shallow pit to have top of rail inside vacuum vessel in level with factory floor level . The vessel is to be of rectangular construction, suitable for operating under vacuum as stated above. It is to be fabricated from tested quality mild steel to IS: 226 and suitably reinforced from out side to with stand the vacuum and pay load on trolley.

2.2	The payload is to be fed in to the vessel through a moving trolley on rails. Rails are to extend inside the vessel and 10 meters out side on the factory floor. Trolleys and suitable rails are also to be supplied with the equipment. The trolley is to be fabricated from standard steel sections and designed to carry a distributed load of 40 M.T. It is to be designed in such a way that height above floor level is minimum and does not exceed in any case 400 m.m., The dimension of the trolley shall be LENGTH 5900mm, WIDTH 3500mm , HEIGHT not more than 400mm Suitable provision is to be made either for mechanically propelling the trolley  into and out of the vessel  with the help of a proper winch fixed out side the vessel ort he trolley can be self propelled type.

  - The vessel is to be provided with a front door with side sliding opening, by hydraulically / Electrically and mechanically operated mechanism controlled by push buttons. The door after closing is to be sealed against rim of the vessel which is having two gaskets of circular cross section placed in the machined rim having dovetailed grooves on vessel rim. The rim of the vessel and door is to be machined to an accuracy of +/ - 0.75 mm across diagonals and ground finished to ensure perfect sealing. The gasket sealing material is to be of best quality VITON, oil resistant and suitable to with stand continuous temp of 120 degree centigrade. Vacuum tightening of sliding door over the opening of vessel is to be done by hydraulic cylinders clamps from all four sides. The hydraulic cylinders clamping is to be operated with push buttons and appropriate hydraulic system.

  - Saturated steam shall be provided at 6kg/sq.cm in our works. The pressure reducing station is to included in the supply to reduce the steam pressure to 3kgs/sq.cm. for heating the vessel . All accessories in the steam and condensate line like steam separator, strainer, pressure reducing valve with bypass arrangement, control valve, safety valve, pressure gauges, steam traps, non return valves etc. are to be  included in the offer. steam pipe line system and accessories should of IBR quality and IBR clearance from CIB ,UP is to obtained by the  supplier.

Air circulating fans are to be provided for rapid and equi - distribution an thorough heating of chart inside the vessel. The air circulating fans are to be designed for operation at 100 deg C and shall be fixed in such a way that there will be no lose of vacuum from the vessel. The vessel is to be provided with a relief valve to facilitate emergency blowing off in case of sudden bursting of steam line .

2.5	The vessel is to be complete with all the accessories and the whole equipment is to be tested 	for leak rate of 5 torr ltrs/

2.6	The following pipe connections are to be provided on the vessel by means of flanges which are to be drilled as per relevant Indian standard.:

2 Nos. 65 mm dia drain pipe at the maximum depth of the vessel to drain out any accumulated oil with control valve operated from out side.

20 Nos. 50 mm dia  oil entry pipes fix to he sides at equal distance 350 mm  below the top of the vessel. The ports are to be flanged externally and internally to connect flexible pipe connections to the transformer, which are  to be impregnated . 50 mm nominal bore X 2 mtr. long seamless metallic helically envoluted flexible pipe complete with adopter and sockets are to be provided for each oil entry points and fitted with control valves operated from out side.

20 Nos. 50 mm dia oil return pipes located at about 250/300 mm above the bottom floor in line to the oil entry pipes as called above. The ports are to be flanged externally and internally to connect flexible pipe connections to the transformer, which are  to be impregnated . 50 mm nominal bore X 2 mtr. long seamless metallic helically envoluted flexible pipe complete with adopter and sockets are to be provided for each oil entry points and fitted with control valves operated from out side.

Two Nos. blank ports with 8 terminals each having epoxy cast bushings for megger / tan delta measurements located 1200 mm from base of the vessel on side. details of the terminal board shall be submitted for approval from BHEL. These terminals are to be suitable for application of 5KV.

Tap off points from the main supply lines running through columns for supply of different services like steam, condensate, oil supply, oil return, water. Compressed air etc. needed for the safe operation of vacuum vessel are to be provided by the supplier.

2.7	The following opening / accessories are to be provided and located suitably on sides at convenient position:

-	4 Nos. sight glasses for viewing inside of the vessel placed suitably and two nos. sight glasses on sliding door.

-	4 Nos. suitable light fittings on the side of the vacuum drying vessel for illuminating inside area to facilitate visual inspections.

-	Vacuum pipe connections of adequate size near the top edge of the vessel and he canal between two gaskets around the door flange is also to be connected with vacuum line.

-	Dry hot air entry and exit ports located suitably on the vessel for the supply of dry air for scavenging the humid air during heating phase and also to break the vacuum.

2 Nos. blank ports with 8 terminals each having epoxy cast bushing for thermo couple connections positioned on each side of the vessel.

-	All seals, gaskets, light fitting , site glass assembly and wiring should be able to with stand variation of pressure and temperature.

2.8	EFFECTIVE INTERNAL DIMENSIONS OF VESSEL :

LENGTH /  DEPTH				: 	6000 M.M.

WIDTH					:	4000M.M.

HEIGHT					:	3500M.M.

- TEMPERATURE RECORDING AND CONTROL:

The vessel shall be designed for working on steam and capable of maintaining any temperature between 70 to 120 deg.C to an accuracy of +_ 2 deg.C throughout the vessel with transformers in side. The temperature inside the vessel is to be thermostatically controlled by sensing units fitted in the heating medium, controlling automatically.

VACUUM PUMPING & MEASURING EQUIPMENTS:

Two stage vacuum pumping unit is to be designed and capacity determined in such  manner that at the end of drying total pressure of approximate 0.5 m bar is existing in the chamber and that it draws / achieved absolute vacuum in the vessel in about 90 minutes when vessel is empty with out charge. The capacity of the whole pumping system should be related to volume of vessel taking in to consideration of leakage factor and moisture content in the insulation of job.

The vacuum pumping is to be consists of a rotary oil sealed pump with air ballast and mechanical booster pump with automatic switching on / cut off device. The rotary oil seal pump is to be provided with metallic steel bellows on inlet side.  Also, provision shall be made to include dust filter on inlet side and oil mist eliminator on exhaust line of rotary oil  sealed pump. Immediately after the vacuum connection from vacuum vessel, a water cooled trap cum vapour condenser of adequate capacity shall be provided. Another vapour condenser shall be provided between mechanical booster vacuum pump and rotary oil sealed pump. Condensers shall have automatic condensate drain off facility into condensate receivers having adequate holding capacity. The receivers shall have graduated gauge glass to indicate the water level of condensate in the tank at any time. The vacuum pumping unit shall be complete with all necessary devices as well as flanges and counter flanges. The capacities of each equipment in the group should be suitably matched and details furnished with the offer. The vacuum pumping system shall be package type , mounted on an open type frame work. All equipments are to be mounted in such a manner that they can be taken out easily when required for maintenance. Seperate offers are to be made for pumping units of various make like "Pfieffer", Leybold , TOHO, Etc. The vacuum pumping unit shall be complete with vacuum breaking valves, automatic shut off device etc.

5.0	VACUUM RELEASE AND RELIEF VALVE:

Vacuum is to be released before opening the vessel by admitting dry air from air dryer which should be capable of vacuum release within 30 Minutes.  The air drying unit is also to be used for supplying dry air for scavenging. The air dryer shall be provided with hygrometer, temperature gauge, timer etc. for regeneration system for Silica gel.

6.0 	LAGGING AND CLADING OF THE VESSEL

The vessel is to be lagged by a suitable heat insulating material so as to keep the temperature in the neighborhood of vessel not to exceed 5degree centigrade over the ambient and finally covered with Aluminium cladding.

7.0 	CONTROL PANEL AND GUAGES

A control panel carrying various contactors, relays, signal lights, push buttons, dial type vacuum gauge, Mecleod gauge, Pirani vacuum gauge with two measuring gauge and a six point recorder to record temperature and vacuum is to be included. Temperature controlling unit are to be incorporated with over riding control devices.

Control panel is to be equipped with Ammeter and voltmeter with selector switches for measuring. Illuminated Mimic diagram is to be provided on the control panel. Panel door shall be dust proof.

All cables inside control panel and out side control cable shall be of copper conductor of suitable rating. The colour of control cable should be red and all cables for power shall be RED/Yellow/Blue for phases and green for neutral.

All Electrics used in the plant shall be suitable for operation on 415Volts, 3 Phase , 4 wire , 50HZ A.C. Supply.

8.0 	INSTALLATION

The vessel is to be installed on our prepared foundation details of which will be furnished by manufacturer. Suitable ladders and platforms should be provided to reach top of the vessel and various equipments fitted at height.

9.0 	ERECTION AND COMMISSIONING

The offer should include the charges for complete transportation, erection and commissioning of the vessel and ancillary equipments with duration. Your commissioning engineer's services should also be available at site for setting all controls while demonstrating the working of the plant. Leak rate test shall be carried out for the empty vessel.

10.0 	LOAD TEST

Load test at full load shall be carried out for complete one cycle of processing of ESP transformer and various parameter shall be checked like achieving final vacuum, time taking to achieve the final vacuum, Time taken to complete the cycle of processing, Measuring of various parameter in transformer which have been processed and final filling the oil.

- MISCELLANEOUS

  - The offer should be descriptive and complete. Details of all equipments and accessories like Vacuum pumping system, Vessel and door heating system, door opening, sliding and sealing arrangement, Job loading and movement system with general arrangement drawings and catalogues to be enclosed with the offer. Offer with out complete details are liable to be rejected.

  - Firm should give breakup prices of all major items and activities.

  - The following drawings and information are to be furnished along with the offer:

- General arrangement drawing of vacuum vessel along with vacuum pumping system and accessories.

- Details and drawing of door supporting, door sliding and sealing arrangement including clamping of door.

- Details of proposed Hydraulic system / electromechanical drive.

- Details of piping layout of steam, condensate, oil and vacuum.

- Details of various controls (Electric/ Hydraulic/ Mechanical), instrumentation, safety devices etc.

- Electrical control and power schematic diagram.

- To including chilling and cooling water requirement.

QUALIFYING CONDITIONS

Only those vender, who have supplied and commissioned at least two Vacuum drying Vessel  for similar application in the past ten years and such oven is presently working satisfactorily for more than one year (more than six month if supplied to BHEL) after commissioning should quote. The following information is to be submitted by the vender about the companies where similar Vessels have been supplied. This is required from venders for qualifications of their offers.

- Name of the customer/ company where similar machine/plant is installed

- Complete postal address of the customer

- Year of commissioning

- Name and designation of the contact person of the customer

- Phone , Fax No. and e.mail address of the contact person of the customer

- Performance certificate from the customer regarding satisfactory performance of the machine/ plant supplied to them

2.INSTRUCTIONS FOR SUBMISSION OF TENDER

- The offer against the enquiry shall be submitted in two parts.  The first part called Techno-commercial bid i.e. Part - A shall be un-priced and will be submitted in line with given details in commercial terms and conditions .  The second part called Price-Bid  i.e. Part-B, in this you will have to mention the basic prices, taxes duties etc for basic equipment & Standard / optional accessories if any.

- The vendors shall submit Part-A and Part-B of their bids in two different sealed envelopes duly super-scribing Enquiry No. and Part No. of the bid.  These two parts of the bid shall be kept together in a third envelope super-scribing Enquiry No., due date and time of opening as indicated in the tender document.

- Part-A of the bid alone shall be opened on the due date in presence of the tenderers in the first instance.   This part of the bid shall be examined in detail.  Part B of the bid shall be opened only for such of the tenderers whose Techno-Commercial offers are found acceptable by BHEL.  Such tenderers shall be advised the date and time of opening of Part B i.e. price part of their bids.  Tenderers must carefully go through all the instructions and conditions before filling in Part B of the bid.  Any type of mistake in submission of the bids will make the tender liable for rejection.

- The bids shall be submitted only in the format given in the tender documents. The tenderer is  required to sign and stamp each and every page of the tender document clearly bring out deviations, if any.

- Tenders not complying with the above and other requirements of the tender shall be liable for rejection.  Tenderers are advised to read all the documents carefully before submission of the bids and ensure the fulfillments of all the technical and commercial parameters.

- No over-writing in any Part-A or Part-B is permitted.  Corrections in the bid should normally be avoided. However, when these become essential, bidder shall score out the incorrect information and put the correct information duly endorsing each correction with his signatures.

- Tender Cost (if any) is to submitted in the Form of  DD in favour of BHEL-Jhansi  payable at

Jhansi along with Techno –Commercial offer. This DD should not be kept in Sealed Price Bid.

3.COMMERCIAL TERMS & CONDITIONS FOR PURCHASE ENQUIRY

- Offer is invited in two bids, outer cover, inner cover-I (Techno-Commercial bid) and inner cover II (Price-Bid) must bear our tender no., due date and bidders name.  Offers received in single bid will be bypassed.

- The offer should reach this office on or before 13.15 hrs on the due date.  The tender can also be dropped in the Tender Box kept at Administrative Building Gate.  The tenders will be opened after 1400 hrs on the due date in presence of all bidders who will be present.

- Offer should be free from overwriting or erasures.  Corrections or additions, if any should be attested.

- No enhancement of rate for whatever cause is allowed once the offer is accepted and the order is placed.  Withdrawal from the quotation after it is accepted or failure to make the supply within the scheduled period will entail cancellation of order, forfeiture of earnest money if any and/or imposition of risk & cost purchase.

- Material offered should be of required quality and conforming strictly to our specifications / drawings or to the specifications in the drawings.  Any deviation must be clearly stated in the offer with particular clause /detail in our specification / drawing for which deviation is quoted.  All required documents/details  e.g. QA Plan, type TCs, drawings & data sheet etc. must be submitted along with the offer.

- Irrespective of whether it is mentioned in our specification, all machines / equipments are required to be calibrated in metric units and certificates must be issued where applicable.

- Work test  certificates / inspection certificates/guarantee certificate/ compliance certificate, etc. will be required to be submitted along with supplies in case an order is placed.

- Required Delivery  time for affecting supplies should be indicated in the offer.

- Failure to supply by the time specified on the order will make the supplier liable to pay unconditional penalty of ½% of the prices of goods in arrear per week subject to a maximum of 10% for Indian suppliers and max 5% for Foreign suppliers of the order value unless agreed otherwise.

- The quantities for each item to be purchased may vary from the tendered quantity according to our actual requirement at the time of placing orders.

- Bharat Heavy Electricals Ltd does not bind itself to accept the lowest of any tender, but reserves to itself the right to accept or reject all or a part of any tender at its discretion following BHEL Purchase Policy & Government guidelines.

- In all cases of disputes, the matter shall be referred for arbitration.  The arbitrator  shall be nominated by the Unit Head, Bharat Heavy Electricals Ltd., Jhansi or any other officer nominated by him.  The award of Arbitrator shall be final and binding on both the parties.

- In case of any suit or other legal proceedings under or relating to this tender, the courts at Jhansi shall have full jurisdiction.

- Total delivered cost at BHEL Jhansi Railway siding/Works shall be the criterion for deciding the tender.

- Unless otherwise specified, offer should be on ex-works basis (for Indian Suppliers) with freight & insurance to be borne by BHEL.

- Your quotation against this enquiry must be valid for ordering up to minimum of 120 days from the date of tender opening.

- ADDITIONAL FOR INDIAN SUPPLIERS:

I)        If any material is booked to selves or if it is not consigned to BHEL Jhansi, any warfage or

demurrage occurring from these shall be to the account of supplier.  In case of road transport,

material shall be booked only through our rate contract transporters mentioned in the purchase

order.

- Where there is provision for payment of Sales Tax extra, it will be paid only if the registration number both under State Sales Tax and Central Sales Tax are specifically mentioned on the bill/invoice.  Your Sales Tax Registration number should be mentioned in the offer positively.

- Excise Duty Gate Pass will be required in original.  Applicable taxes & duties must be included in the offer.

- ADDITIONAL FOR FOREIGN SUPPLIERS:

i)	For documentation and correspondence language shall be English only.

- Rates quoted should be on FOB basis.  All despatches are to be made by sea  by IPBC conference vessels only unless otherwise stated in purchase order.

- Agency commission, if any, will be paid out of the FOB value after receipt & acceptance of material at BHEL Jhansi in Indian Rupees.  TT Buying exchange rate prevailing on the tender opening date or the letter of credit date or date of purchase order whichever is lowest will be considered for calculating the agency commission.

- The rates quoted should be inclusive of charges of packing and delivery of material up to the port of shipment on FOB terms.  The material is required to be packed suitably in all respects considering the peculiarity of the material for normal transport by Sea/air and subsequently by Rail/Road in India suitably protected against the effect of tropical salt laden atmosphere in the event of consignment being delayed at Indian Ports before outward clearance.  You should also furnish details of shipping specification, sizes, volume, weight, number of cases etc. When articles are packed with packaging material of plant origin, phytosanitary certificate will be required.

- Our standard terms of payment is 100% payment through bank against despatch documents or 100% through sight irrevocable & unconfirmed letter of credit.

19.   Security deposit (SD) cum Performance Bank Guarantee  (PBG) for 10% basic value of the order (excluding taxes &

duties) for  execution of the purchase order (PO) & satisfactory Performance of the equipment before release of  PO

will be required to submit which will be valid from the date of issue of LOI to 24 months after acceptance of material by

BHEL with a further claim period of 3 months. PO will be issued only after receipt of SD cum PBG.

20. For fully imported equipments / plant/ machinery (no part or sub-assembly of Indian origin) :

a. No SD cum PBG will be applicable for basic order value up to Rs. 25.0 Lacs on foreign supplier.

b. No SD cum PBG will be applicable for basic order value up to Rs. 10.0 Lacs on Indian stockist / agent of

foreign manufacturer.

21.   B.G. shall be from Scheduled Banks/Public Financial Institutions as defined in the Companies Act.

22. The equipment shall be guaranteed for 24 months from the date of acceptance by BHEL irrespective of date of supply,

erection and commissioning.

23. Our standard terms of payment (for Indian Suppliers) is 80% of P.O. value be paid after 60 days of acceptance of the

equipment at BHEL Jhansi works and balance 20% after successful commissioning & handing over of the equipment

(where applicable).

24. Amount of Service Tax / Education Cess / any other Govt. duty ( as applicable on the date of offer) for erection or any

other activities associated with the scope of work in the tender must be indicated in Techno-commercial bid of the offer.

25 . Blank Price Bid (After erasing prices) must be submitted along with Techno commercial bid.

26.  If any goods are rejected, BHEL shall be at liberty to take action as per following at the

risk and cost of supplier:-

    - Allow the supplier, wherever mutually agreed, to rectify the rejected goods at BHEL's works within reasonable time as fixed by BHEL.

    - Allow the supplier to make free replacement within a specified period, rejected goods can be lifted by the supplier thereafter.

OR

Allow the supplier to refund the full amount paid to him by BHEL by a Demand Draft  before lifting the rejected goods.  Fresh replacement shall be regularized as per terms & conditions of the original purchase order.

OR

Take alternative  procurement action from elsewhere and recover the difference in cost, if any incurred by BHEL in this regard from the supplier.  The supplier shall not be entitled to any gain on purchase.

    - Terminate the contract either in part or in whole at the  discretion of BHEL and recover the loss, if any, from the supplier.

27. For any clarification on our tender terms and conditions please contact Sr. Purchase Officer (Capital Items)

before due date.

28. Inspection may be done at supplier works by BHEL.

## 4. TECHNO COMMERCIAL BID  PART-A

( Supplier should fill up this bid by putting the tick in accepted / non accepted column)

SUPPLIER’S NAME & ADDRESS

1.          Offered Equipment with complete technical details  :                      As per attached details

2.           Packing & Forwarding Charges, FOB charges (if any)                                                               ….%

3.   	Present rate of Excise duty (if any)         		                                                              ….%

4.     	Present rate of sales tax			 		                                                   %

(UPTT/CST, Not applicable in case of Foreign suppliers)

5.	Delivery period in  No. of  days from  date of LOI / PO                                                                                                                    6.	Taxes and duties shall be extra as applicable on 		              Accepted / Not accepted

the date of actual delivery.

7.	Penalty clause as well as other conditions given			Accepted / Not accepted

in general Terms & conditions

and Standard Enquiry terms & conditions are

acceptable to us.

8.           Rates quoted in the price bid are on Ex Works / F.O.R. 		---------------------------------

Jhansi .basis.

9.	Validity of the rates quoted is 120 days from the 			Accepted / Not accepted

date of opening of price bid.

10.          Security deposit (SD) cum Performance Bank Guarantee                      Accepted / Not accepted

(PBG) for 10% basic value of the order (excluding taxes & duties)

for  execution of the purchase order (PO) & satisfactory

Performance of the equipment up to Guarantee period

of 24 months  before release of PO will be submitted.

11.        The equipment shall be guaranteed for 24 months from                            Accepted / Not accepted

the date of acceptance by BHEL irrespective of date of

supply, installation.

Note :- If any of the above conditions are not accepted

the offer shall be liable for rejection.