BHEL - JHANSI

TENDER DOCUMENT FOR EOT CRANES , QTY –01 NO. EACH, TOTAL-03 NOS.

Enquiry No. E7160035, DUE ON   27.01.07, Tender Cost: Rs. 1,000/-

C  O  N  T  E  N  T  S

Page No.

1.         Enq Item No.1, Description & Technical Details                     2-11

2.         Enq Item No.2, Description & Technical Details                     12-20

3.         Enq Item No.3, Description & Technical Details                     21-31

4.	Instructions for submission of Tender		                         32

5.	Commercial Terms & Conditions		                                 33-35

6.	Techno-Commercial Bid          		                                    36

7.	Reverse Auction Details                                                             37-39

8.	Process Compliance form  : Annexure-IV                                   40

( A.K.Banerjee  )		     ( P.K.Pathak )                                  ( Y K Verma )

DGM(MM)	                           Sr. PO (MM)                                      Sr. PO (MM)

1. Item Description & Technical Details :

##### A. ENQUIRY ITEM No.1

##### ITEM Description                      :	Electric Over head Traveling Crane

Class II duty as per IS: 3117/ 1965

ITEM NO.			:		SP/CRJ/9559

SPECIFICATION NO.	:		CRJ/9559

LOCATION			:		BAY-5	 EXT.

QUANTITY			:		1 No.

1.0	GENERAL   :

Electric Over Head crane of capacity indicated below is required for loading, unloading of unfinished, finished components from Machine Tools as well as assembly of Transformers in transformer manufacturing shop. The crane will also be used for handling of dry type moulds and coils. The same has to work on a prepared gantry in transformer shop.

2.0	Capacity and leading dimensions are given hereunder:

2.1	Safe Working Load	:

Main Hoist				:	5  Tones

Aux. Hoist				:	3   Tones

2.2	Duty as per I.S.3177/1965			:	Class 2

2.3	(a)	Normal Span				:	24  Meters

(b)	Rail Center to Center			:	22.7  Meters (Provided)

2.4	Gantry Rail height above floor level		:	08.00  Meters (Provided)

2.5	Length of Bay					:	168 Meters

2.6	Type of Hook					:	Shank Type

2.7	Available Head above Gantry Rail 		:	2.8  Meters

2.8	Available side clearance from			:	0.45  Meters

Gantry Rail Center

2.9	Operation					:	Cabin & Radio Remote

2.10	Location					:	Indoor

2.11	Hook lift above floor level to be provided	:	8.00  meters

2.12	Electric Power Supply				:	415V, 3 Phase, 50 C/S, A.C.Supply

2.13	Gantry Rail					:	95 lbs (Provided)

2.14	Full Geared Speed  :

a)	Main Hoist		:	7.62 M /Min.

b)	Aux.   Hoist		:	7.62 M/Min.

b)	Traverse		:	21.32 M / Min.

c)	Travel			:	76.20 M / Min.

2.15	All the speed should be controlled in four steps both in forward & reverse direction. All four steps should have gradual increase in speed from minimum to maximum. Speed specified above are maximum speeds.

2.16	Speed control system shall be elaborately and clearly mentioned by the tenderer in their offer. Minimum speed,  speed at various steps to be clearly mentioned.

2.17	Speed control for hoisting / lowering shall be Thruster creep brake system. Creep speed shall be provided in the 1st step of both hoist and lowering motion.

2.18	Creep speed to be mentioned in the offer.

2.19	Firm to indicate following explicitly in their quotation:

(a)	Weight of each bridge (2 Nos.)

(b)	Weight of each carriage. 9 2 Nos)

(c)	Weight of complete trolley.( 1 No.)

(d) 	Weight of complete assembled crane

(e)	No. of wheels on LT & CT,  Wheel diameter and material specification.

(f  )	Wheel Load on each wheel

- Horse Power of various motors, their duty cycles, rating , make etc.

- Make, Type, size of thrusters and E.M. Brakes.

- Make of all electrical items like relays, contactors, resistance,  timer, limit switches etc.

- Size of Brake drum for each motion.

- Complete specification of wire rope for both motion.

- Number of falls of wire rope in main hoist and auxiliary Hoist.

- Erection & commissioning charges to be quoted separately.

3.00	GENERAL	:

3.1	The crane shall be generally of  box type construction and to be designed for class 2 duty in accordance with BSS: 466/1960 and or IS:3117/1965. Material shall be of tested quality as per relevant I. S.

3.2	BRIDGE CONSTRUCTION   :

    - The bridge should comprise of two main girders with two out riggers and the end carriage built up from mild steel channel with adequate diaphragms or stiffing plates. Each bridge girder shall be fabricated and dispatched without any spliced joints.

3.2.2	Bridge girder should have revetted / bolted connection with end carriages.

3.2.3	M.S Chequered plate platform shall be provided along with full length on both side of the crane along with toe board.

3.2.4	Hand railing shall be fitted on both sides of the end carriages and through out the length of the platform on the bridges. Size of the hand rail shall not be less than 25 mm. bore heavy gauge steel pipe. The vertical pipes to have flanges  at the bottom for proper support.

4.0	TRACK WHEEL:

4.1 	The end carriages shall be of approved design, substantial construction and of ample stiffness built up of M.S. section securely riveted or welded with each other. They shall be projected over the run way rails and shall be so arranged that in the event of  derailment the 	drop should not exceed 25 mm.

4.2	The end carriage of the crane should be mounted on double flanged  wheels of the approved diameter in accordance with B.S.S.466.  Each wheel shall be  fitted with two numbers roller bearings SKF/NBC/FHP make mounted on a fixed steel sleeve and carried by a hard steel pin. Grease nipple should be provided at suitable place for convenient greasing and the grease be retained, dirt excluded by means of suitable seals.

- TRAVELLING MECHANISM:

5.1	One wheel in each carriage is to be driven by a machine out forged steel gears by a motor mounted on or adjacent to the carriage. Motors of both the end carriage travel would be connected to one controller but the rotor circuit should be entirely independent to avoid synchronization and to permit the variation of speed, which is necessary to run the crane freely and smoothly on a track which deviates slightly from perfect straightness.

6.0	TRAVELLING MOTION BRAKE

6.1 	A thruster brake to be fitted on each individual drive of the track wheels. These brakes should 	be arranged to be operated from the control desk as well as remote control. The brakes to be of post type with hinged shoes, lined with good quality reinforced bonded brake lining and arranged for easy adjustment for the necessary wear on lining time to time.

6.2	An independent additional mech. operating hand brake is to be provided in LT motion to stop the crane immediately in case of power failure. This should also be operated from Radio remote on emergency condition.

7.0	CRABE FRAME :

7.1 	This should be built up of steel sections and plates to proper thickness and the upper surface of the frame should be plated over wherever required in order to give great lateral rigidly and maintenance facility. The lubricating points for all moving parts should be provided at the top conveniently for easy maintenance below the surface of the trolley i.e. balancing sheaves etc.

8.0	CRABE TRAVERSE :

8.1	The wheel should be of forged steel material of approved diameters duly secured to hard shaft steel axles and to run on sturdy roller bearing of SKF	make. The bearings to be enclosed in a housing and arrangement for grease lubrications should be provided. One axle should be driven by a motor through an enclosed good design harden spur gears of good quality. A thruster brake should be provided.

8.2	The cross travel gear box should be well secured at least by four bolts two on each side opposite the each other.

9.0       HOISTING MECHANISM :

9.1 	This shall consist of heavy duty cast iron drum driven by totally enclosed gearing. The drum shall be of ample diameter and of sufficient length to take full amount of rope without overlapping when the hook is in the highest position. When the hook is in the lowest position, there should be at least two full turn of rope remaining on drum. The method of attachment of the rope shall be such that in the event of rope un-winded completely there shall be no danger of its becoming freed from the drum. The drum should have right and left hand spiral grooves machined to suit the required size of wire rope, should be spigotted and bolted to the final driving gear. the whole gear box should be of standard design, having SKF/NBC/FHP make bearings and should be smooth and noiseless in operation.

9..2	The gearing should be of heat treated alloy steel pinions and carbon steel gears the larger gears having forged carbon steel rim shrunk and secured by bolts on heavy duty  cast iron or cast steel.  All gears will have machine cut teeth.  High speed gears must be of helical type teeth and all gears should be free from noise and with smooth operation. The drum shaft and pinion shaft shall be mounted on ball & roller bearing of SKF/NBC/FHP make and complete gear train shall be enclosed in a heavy duty cast iron gear box. The gear box must having jointing surface machined and should be made leak proof. All the extended shaft and end covers on gear box must be provided with good quality oil seal to prevent any leakage.

10.0	HOIST BRAKE  :

10.1	A thruster creep brake to be fitted on the high speed shaft. The brake to be of the post type with hinged shoes.  Shoe must be lined with best quality reinforced bonded asbestos industrial brake lining and arranged for easy adjustment. The brake should be normally held on by a balance weight and released by a thruster in the circuit of hoist motor.

11.0	OTHER BRAKES & LIMIT SWITCHES

  - There should be heavy duty double limit switch  for each hoisting  and lowering motions.

  - Gravity type limit switch must be provided for hoisting operation in order to provide additional safety.

  - Series type limit switches with single arm spring loaded allen west type should be provided for traverse motion.

  - Anti - collision type limit switches should be connected only in interlock circuits.

11.5 	All limit switches should be connected only in interlock circuits.

12.0	ROPE HOOK AND SHEAVES:

  - The hoist shall have the factor of safety being not less than 6.  Main hook for all  load should be supported on 4 falls.

  - Sheaves are to be of heavy duty cast iron mounted on Ball bearings. Guards to be fitted to prevent the rope leaving the sheaves, even when the hook if lowered to the rest on the ground and rope allowed to fall slack. The hook guard should be secured in such a manner that there should be no bolt protruding in the inner side of the hook.

13.0	MOTORS:

13.1	The all motors should be one hour rated crane duty. These should be totally enclosed type fitted with Ball / Roller bearings.

13.2	 The motor should have class 'B'/'F' insulation and to comply with IS: 325 according to   manufacture standard. All motor should be slip ring induction type and suitable for use of 415 V, 3 phase, 50 cycles, AC supply. The motor should be of GEC Alsthom/ crompton make only.

14.0.	CONTROL GEAR

14.1 	The hoist motor, long travel motors and cross travel motor should be controlled by semi automatic reversing contactor panel with a cam operated master controller.

15.0	CRANE PROTECTIVE PANEL:

15.1.	The design & construction of this panel must be of high standard. The equipment should comprise of contactors circuit breakers and magnetic oil dashpot type over current relay connected in all the phases of each motor. An emergency stop and a reset push button and an indicating pilot lamp should be incorporated. All connections internal & external should be accessible from front of the panel for inspection & maintenance. The contactor should be of SIEMENS,/ L&T overload relay of electrogear make and timer of CUTTLER - HAMMER.

  - The relay & contactors should be of robust design. The contactor should be suitable for class IV duty. Over load relay should be magnetic type provided with oil dashpot arrangement with adjustable current setting and with self reset mechanism. Auxiliary contacts of magnetic overload should be robust in construction.

  - Isolator switch fuse unit should be capable of load breaking and mechanically robust. Isolation to be of quick brake type and it should be of GE Control (EE) make. Main Isolating switch should be mounted ion the operator's cabin.

  - All fuses should be of HRC / GE Control (EE) make only.

  - Suitable rating control transformer should be provided for the control of the circuit.

16.0 	RESISTANCE BOX

  - All resistances should be of 10 minutes rating. The resistance should be rust less unbreakable S.S. grid type.

  - All resistance box should be mounted on the crane bridge.

17.0	CONTROL CABIN:

17.1 	The depth of crane cabin should be limited to 2150 mm. Opening to be cage on the platform should be approximately 750 mm x 750 mm. A ladder should be provided between the cage and the inspection platform of the crane. The controllers should be so positioned as to facilitate easy operation & maintenance. A warning bell (Electrically operated industrial type with 6" gong) and Fire extinguisher of 4.5 litre capacity with long discharge nozzle should be provided in operator's cabin. One more bell is to be provided strudy conventional pendulum type made of good bell metal and to be well secured inside the cabin in a convenient position for easy operation. A revolving type cushioned seat is to be provided in the cabin for the crane operator. A 2 feet 20 watt fluorescent tube fitting, one 16" sweep cabin fan with guard should be provided.

  - An inspection lamp with 20 meter 3 core flexible cable is to be provided with 240 V plug point in the cabin.

  - The total depth of control cabin should be such that it should not interfere with any other structure / semi gantry crane / machines in the entire bay. For this supplier must check all the available clearances at the side before finalizing/ designing of the crane dimension.

18.0 	RADIO REMOTE CONTROL  SYSTEM:

18.1	Radio remote control system shall be provided to operate the crane from shop floor in all motion and control. The radio remote control system shall consist of a portable transmitter and receiver along with the necessary relay logic for interfacing to the crane motors. The transmitter 	shall consist of push buttons.  A selector switch shall be provided in the operator cabin for remote / manual operation.

18.2	Details of Radio remote control system to be furnished with the offer.

18.3 	The system shall be following designated controls:

Main Power 	ON/OFF

Hoist creep 		UP/DOWN			1 -  0 -  1

Hoist 			UP/DOWN			1 -  0 -  1

Aux. Hoist creep 	UP/DOWN			1 -  0 -  1

Aux. Hoist 		UP/DOWN			1 -  0 -  1

CT			FORWARD/REVERSE	1 –  0 -  1

LT			LEFT / RIGHT (SLOW)	1 –  0 -  1

LT			LEFT / RIGHT (HIGH)	1 – 0 - 1

Alarm			ON/OFF

Emergency stop	ON/OFF

Under bridge lights	ON/OFF

19.0	CRANE BRIDGE LIGHTING :

19.1 	Four Nos. 300 watt flood lights with fitting or four Nos. 100 Watt HELOGEN  lights with fitting should be provided below the bridge girder.

19.2 	Separate ON/OFF switches for all four lights shell be provided in the operator's cabin.

19.3 	ON/OFF switch shall be provided for bridge light in Radio remote.

19.3 	A lighting transformer of suitable rating shall be provided for lights, fan etc.

20.0	WIRING:

20.1	The crane shall be supplied complete with all wiring and materials. Collectors on the crane bridge and on the crab shall be provided where applicable. Moveable type flexible cable with necessary trolleys and festooned system shall be provided for supply of trolley. LONGITUDNAL GANTRY LEADS ARE ALREADY PROVIDED IN THE BAYS.

20.2	All wiring shall be with PVC insulated and or armored cables having copper conductor. Wire and cable should be of 1100V grade in trunking.  Trunking should be used instead of conduit pipe.

20.3	All electrical wiring and installation should strictly confirm to IS Act, 1910 and I.E.E. Rules. 1956 and other standard practice as adopted by the I.S.I.

20.4	All control terminations should be ferruled such that the numbers are clearly visible from out side, crimped terminations should be provided for control cables and as far as possible for main cables. A set of suitable current collector shoe should be provided to suit the angle iron down shop leads. Details of existing down shop lead and current collector system can be checked at site before finalizing the supply of current collector system.

21.0	MAINTENANCE:

  - We would stress the need of adequate maintenance facility for all gearboxes and working mechanism which shall be located above platform level and shall be easily approachable for good maintenance.

22.0	SPARES :

22.1	Recommended spares for two years normal working of the equipment is to be furnished with the estimate, separately quoted for each items. Details of bearings, oil seals used should be supplied along with location of each item.

23.0	MISCELLANEOUS :

  - All gear boxes should have oil level indicator to show the level and condition of the oil in gear box.

  - All material used should be of the best of their respective kinds and should of the appropriate I.S. Specification & all workman ship should be of best quality.

  - The structural steel work in the bridge and the crab frame should be given one coat of suitable primer during fabrication. Gear boxes and similar parts to be given coat of oil paint. After erection at site, the crane should be given two coats of light orange synthetic enamel paint.

  - First fill oil, grease etc. for the commissioning of the crane shall be provided by the supplier.

  - Tool box with standard tools shall be supplied with the crane.

  - List of bought out items should be furnished with  names of makers and complete literature about the items should be supplied.

  - Crane to be fabricated for its satisfactory performance for a period of 24 months from the date of commissioning with guarantee certificate.

  - Lifting hook, wire rope and crane to be tested to IS standard and test certificate to be provided (4 copies each). Test certificate of hook should from a recognized Govt. Test House.

  - Four copies of manual of Operation and servicing to be with erection drawings. G.A. drawing, wiring diagram etc. to be supplied along with the crane.

  - Four copies of structural drawing, wheel assembly, maximum crane wheel load etc. H.P. rating of motor, hook approach, gear box details, weight of crane to be supplied.

  - Mechanical drawings, electrical wiring and schematic diagram should be sent in triplicate for approval before finalizing the design. Make and technical specification of all items is also to be submitted for approval.

  - Supplier shall inform BHEL in advance to witness assembly and load test.

  - The crane is to be erected by the supplier at site with their own labour, tools and tackles, winch. The crane is to be commissioned and tested before handing over.

  - All motions at various steps of speed shall be checked along with current being taken by respective motion motors.

  - After test but before crane is put to use, it shall be tested with overload of 125 percent of the S.W.L. when load is located at the center of span. During testing the overload test each motion shall be maneured in both direction and the crane shall sustain the load under full control.

  - Erection / commissioning charges to be quoted separately.

24.	QUALIFYING CONDITIONS

Only those vender, who have supplied and commissioned up to 10 T or higher Capacity EOT crane for similar application in the past ten years and such crane is presently working satisfactorily for more than one year (more than six month if supplied to BHEL) after commissioning should quote. The following information is to be submitted by the vender about the companies where similar cranes have been supplied. This is required from all venders for qualifications of their offers.

- Name of the customer/ company where similar machine is installed

- Complete postal address of the customer

- Year of commissioning

- Name and designation of the contact person of the customer

- Phone , Fax No. and e.mail address of the contact person of the customer

- Performance certificate from the customer regarding satisfactory performance of the machine supplied to them

25.0	DETAILS MENTIONED IN ANNEXURE –I SHALL BE PROVIDED COMPULSORILY ALONG WITH TECHNICAL BID.

ANNEXURE -  I

SPECIFICATION NO. CRJ 9559

DETAILS MENTIONED BELOW SHALL BE PROVIDED COMPULSORILY ALONG WITH TECHNICAL BID.

| S.No. | Description | Requirement | Offer | Remarks |
| --- | --- | --- | --- | --- |
| 1 | Hook lift above floor level | 8.00 Meter |  |  |
| 2 | Main hoist speeds | Max. 7.62 Meter / Min |  |  |
| 3 | Hoist creep speed on 1st step from master controller & remote control | 10% of Max. speed |  |  |
| 3 | Aux. Main hoist speeds | Max. 7.62 Meter / Min |  |  |
| 4 | Aux. Hoist creep speed on 1st step from master controller & remote control | 10% of Max. speed |  |  |
| 5 | Cross Traverse speed | Max. 21.32 Meter/ Min. |  |  |
| 6 | Long Travel speed | Max. 76.2 Meter / Min. |  |  |
| 7 | Weight of each bridge (two Nos. ) | Supplier to quote |  |  |
| 8 | Weight of each carriage ( two Nos.) | Supplier to quote |  |  |
| 9 | Weight of compete trolley | Supplier to quote |  |  |
| 10 | Weight of compete assembled crane | Supplier to quote |  |  |
| 11 | Over all width of crane | Supplier to quote |  |  |
| 12 | Wheel load on each L.T.wheel | Supplier to quote |  |  |
| 13 | HP of various motor, duty cycle , rating , make & speed | Supplier to quote |  |  |
| 14 | Size of brake drum Hoist CT LT | Supplier to quote |  |  |
| 15 | Wheel dia LT CT | Supplier to quote |  |  |
| 16 | Size and capacity of thruster brakes Hoist CT LT | Supplier to quote |  |  |
| 17 | No. of falls in hoist | 4 Fall |  |  |
| 18 | Size, make & Specification of wire rope | Supplier to quote |  |  |
| 19 | Maximum hook approach from LT rail center Cabin end Far end | Supplier to quote |  |  |
| 20 | Type & make of main hook | Shank type make to quote |  |  |

B. ENQUIRY ITEM No.2

##### ITEM				:	Electric Over head Traveling Crane

Class II duty as per IS: 3117/ 1965

ITEM NO.			:		SP/CRJ/9562

SPECIFICATION NO.	:		CRJ/9562

LOCATION			:		BAY-3

QUANTITY			:		1 No.

1.0	GENERAL   :

Electric Over Head crane of capacity indicated below is required for loading, unloading of unfinished, finished components from Machine Tools as well as assembly of Transformers in transformer manufacturing shop. The crane will also be used for mounting and removing of dry type coils and moulds. The same has to work on a prepared gantry in transformer shop.

2.0	CAPACITY AND LEADING DIMENSIONS ARE GIVEN HEREUNDER:

2.1	Safe Working Load	Main Hoist		:	5  Tones

2.2	Duty as per I.S.3177/1965			:	Class 2

2.3	(a)	Normal Span				:	18  Meters

(b)	Rail Center to Center			:	16.8  Meters (Provided)

2.4	Gantry Rail height above floor level		:	08.00  Meters (Provided)

2.5	Length of Bay					:	168 Meters

2.6	Type of Hook					:	Shank Type

2.7	Available Head above Gantry Rail 		:	2.8  Meters

2.8	Available side clearance from			:	0.45  Meters

Gantry Rail Center

2.9	Operation					:	Cabin & Radio Remote

2.10	Location					:	Indoor

2.11	Hook lift above floor level to be provided	:	8.00  meters

Hook lift below floor level			:	-------------

2.12	Electric Power Supply				:	415V, 3 Phase, 50 C/S, A.C.Supply

2.13	Gantry Rail					:	95 lbs (Provided)

2.14	Full Geared Speed  :

a)	Main Hoist		:	7.62 M /Min.

b)	Traverse		:	21.32 M / Min.

c)	Travel			:	76.20 M / Min.

2.15	All the speed should be controlled in four steps both in forward & reverse direction. All four steps should have gradual increase in speed from minimum to maximum. Speed specified above are maximum speeds.

2.16	Speed control system shall be elaborately and clearly mentioned by the tenderer in their offer. Minimum speed,  at various steps to be clearly mentioned.

2.17	Speed control for hoisting / lowering shall be Thruster creep brake system. Creep speed shall be provided in the 1st step of both hoist and lowering motion of main and auxiliary hoists.

2.18	Creep speed to be mentioned in the offer.

2.19	Firm to indicate following explicitly in their quotation:

(a)	Weight of each bridge (2 Nos.)

(b)	Weight of each carriage.( 2 Nos)

(c)	Weight of complete trolley.( 1 No.)

(d) 	Weight of complete assembled crane

(e)	No. of wheels on LT & CT,  Wheel diameter and material specification.

(f  )	Wheel Load on each wheel

- Horse Power of various motors, their duty cycles, rating, make etc.

- Make Type, size of thrusters and E.M. Brakes.

- Make of all electrical items like relays, contactors, resistance, timer, limit switches etc.

- Size of Brake drum for each motion.

- Complete specification of wire rope for both motions.

- Number of falls of wire rope in main hoist and auxiliary Hoist.

- Erection & commissioning charges to be quoted separately.

3.00	GENERAL	:

3.1	The crane shall be generally of  box type construction and to be designed for class 2 duty in accordance with BSS: 466/1960 and or IS:3117/1965. Material shall be of tested quality as per relevant I. S.

3.2	BRIDGE CONSTRUCTION   :

    - The bridge should comprise of two main girders with two out riggers and the end carriage built up from mild steel channel with adequate diaphragms or stiffing plates. Each bridge girder shall be fabricated and dispatched without any spliced joints.

3.2.2	Bridge girder should have riveted / bolted connection with end carriages.

3.2.3	M.S Chequered plate platform shall be provided along with full length on both side of the crane along with toe board.

3.2.4	Hand railing shall be fitted on both sides of the end carriages and through out the length of the platform on the bridges. Size of the hand rail shall not be less than 25 mm. bore heavy gauge steel pipe. The vertical pipes to have flanges at the bottom for proper support.

4.0	TRACK WHEEL:

4.1 	The end carriages shall be of approved design, substantial construction and of ample stiffness built up of M.S. section securely riveted or welded with each other. They shall be projected over the run way rails and shall be so arranged that in the event of derailment the drop should not exceed 25 mm.

4.2	The end carriage of the crane should be mounted on double flanged wheels of the approved diameter in accordance with B.S.S.466.  Each wheel shall be fitted with two numbers roller bearings SKF/NBC/FHP make mounted on a fixed steel sleeve and carried by a hard steel pin. Grease nipple should be provided at suitable place for convenient greasing and the grease be retained, dirt excluded by means of suitable seals.

- TRAVELLING MECHANISM:

5.1	One wheel in each carriage is to be driven by a machine out forged steel gears by a motor mounted on or adjacent to the carriage. Motors of both the end carriage travel would be connected to one controller but the rotor circuit should be entirely independent to avoid synchronization and to permit the variation of speed, which is necessary to run the crane freely and smoothly on a track which deviates slightly from perfect straightness.

6.0	TRAVELLING MOTION BRAKE

6.1 	A thruster brake to be fitted on each individual drive of the track wheels. These brakes should 	be arranged to be operated from the control desk as well as remote control. The brakes to be of post type with hinged shoes, lined with good quality reinforced bonded brake lining and arranged for easy adjustment for the necessary wear on lining time to time.

6.2	An independent additional mech. operating hand brake is to be provided in LT motion to stop the crane immediately in case of power failure. This should also be operated from Radio remote on emergency condition.

7.0	CRABE FRAME :

  - This should be built up of steel sections and plates to proper thickness and the upper surface of the frame should be plated over wherever required in order to give great lateral rigidly and maintenance facility. The lubricating points for all moving parts should be provided at the top conveniently for easy maintenance below the surface of the trolley i.e. balancing sheaves etc.

8.0	CRABE TRAVERSE :

8.1	The wheel should be of forged steel material of approved diameters duly secured to hard shaft steel axles and to run on sturdy roller bearing of SKF	make. The bearings to be enclosed in a housing and arrangement for grease lubrications should be provided. One axle should be driven by a motor through an enclosed good design harden spur gears of good quality. A thruster brake should be provided.

8.2	The cross travel gear box should be well secured at least by four bolts two on each side opposite the each other.

9.0       HOISTING MECHANISM :

9.1 	This shall consist of heavy duty cast iron drum driven by totally enclosed gearing. The drum shall be of ample diameter and of sufficient length to take full amount of rope without overlapping when the hook is in the highest position. When the hook is in the lowest position, there should be at least two full turn of rope remaining on drum. The method of attachment of the rope shall be such that in the event of rope un-winded completely there shall be no danger of its becoming freed from the drum. The drum should have right and left hand spiral grooves machined to suit the required size of wire rope, should be spigotted and bolted to the final driving gear. the whole gear box should be of standard design, having SKF/NBC/FHP make bearings and should be smooth and noiseless in operation.

9..2	The gearing should be of heat treated alloy steel pinions and carbon steel gears the larger gears having forged carbon steel rim shrunk and secured by bolts on heavy duty  cast iron or cast steel.  All gears will have machine cut teeth.  High speed gears must be of helical type teeth and all gears should be free from noise and with smooth operation. The drum shaft and pinion shaft shall be mounted on ball & roller bearing of SKF/NBC/FHP make and complete gear train shall be enclosed in a heavy duty cast iron gear box. The gear box must having jointing surface machined and should be made leak proof. All the extended shaft and end covers on gear box must be provided with good quality oil seal to prevent any leakage.

10.0	HOIST BRAKE  :

10.1	A thruster creep brake to be fitted on the high speed shaft. The brake to be of the post type with hinged shoes.  Shoe must be lined with best quality reinforced bonded asbestos industrial brake lining and arranged for easy adjustment. The brake should be normally held on by a balance weight and released by a thruster in the circuit of hoist motor.

11.0	OTHER BRAKES & LIMIT SWITCHES

  - There should be heavy duty double limit switch  for each hoisting  and lowering motions.

  - Gravity type limit switch must be provided for hoisting operation in order to provide additional safety.

  - Series type limit switches with single arm spring loaded allen west type should be provided for traverse motion.

  - Anti - collision type limit switches should be connected only in interlock circuits.

11.5 	All limit switches should be connected only in interlock circuits.

12.0	ROPE HOOK AND SHEAVES:

  - The hoist shall have the factor of safety being not less than 6.  Main hook for all  load should be supported on 4 falls.

  - Sheaves are to be of heavy duty cast iron mounted on Ball bearings. Guards to be fitted to prevent the rope leaving the sheaves, even when the hook if lowered to the rest on the ground and rope allowed to fall slack. The hook guard should be secured in such a manner that there should be no bolt protruding in the inner side of the hook.

13.0	MOTORS:

13.1	The all motors should be one hour rated crane duty. These should be totally enclosed type fitted with Ball / Roller bearings.

13.2	 The motor should have class 'B'/'F' insulation and to comply with IS: 325 according to   manufacture standard. All motor should be slip ring induction type and suitable for use of 415 V, 3 phase, 50 cycles, AC supply. The motor should be of GEC Alsthom/ crompton make only.

14.0.	CONTROL GEAR

14.1 	The hoist motor, long travel motors and cross travel motor should be controlled by semi automatic reversing contactor panel with a cam operated master controller.

15.0	CRANE PROTECTIVE PANEL:

15.1.	The design & construction of this panel must be of high standard. The equipment should comprise of contactors circuit breakers and magnetic oil dashpot type over current relay connected in all the phases of each motor. An emergency stop and a reset push button and an indicating pilot lamp should be incorporated. All connections internal & external should be accessible from front of the panel for inspection & maintenance. The contactor should be of SIEMENS,/ L&T overload relay of electrogear make and timer of CUTTLER - HAMMER.

  - The relay & contactors should be of robust design. The contactor should be suitable for class IV duty. Over load relay should be magnetic type provided with oil dashpot arrangement with adjustable current setting and with self reset mechanism. Auxiliary contacts of magnetic overload should be robust in construction.

  - Isolator switch fuse unit should be capable of load breaking and mechanically robust. Isolation to be of quick brake type and it should be of GE Control (EE) make. Main Isolating switch should be mounted ion the operator's cabin.

  - All fuses should be of HRC / GE Control (EE) make only.

  - Suitable rating control transformer should be provided for the control of the circuit.

16.0 	RESISTANCE BOX

  - All resistances should be of 10 minutes rating. The resistance should be rust less unbreakable S.S. grid type.

  - All resistance box should be mounted on the crane bridge.

17.0	CONTROL CABIN:

17.1 	The depth of crane cabin should be limited to 2150 mm. Opening to be cage on the platform should be approximately 750 mm x 750 mm. A ladder should be provided between the cage and the inspection platform of the crane. The controllers should be so positioned as to facilitate easy operation & maintenance. A warning bell (Electrically operated industrial type with 6" gong) and Fire extinguisher of 4.5 litre capacity with long discharge nozzle should be provided in operator's cabin. One more bell is to be provided strudy conventional pendulum type made of good bell metal and to be well secured inside the cabin in a convenient position for easy operation. A revolving type cushioned seat is to be provided in the cabin for the crane operator. A 2 feet 20 watt fluorescent tube fitting, one 16" sweep cabin fan with guard should be provided.

  - An inspection lamp with 20 meter 3 core flexible cable is to be provided with 240 V plug point in the cabin.

  - The total depth of control cabin should be such that it should not interfere with any other structure / semi gantry crane / machines in the entire bay. For this supplier must check all the available clearances at the side before finalizing/ designing of the crane dimension.

18.0 	RADIO REMOTE CONTROL  SYSTEM:

18.1	Radio remote control system shall be provided to operate the crane from shop floor in all motion and control. The radio remote control system shall consist of a portable transmitter and receiver along with the necessary relay logic for interfacing to the crane motors. The transmitter 	shall consist of push buttons.  A selector switch shall be provided in the operator cabin for remote / manual operation.

18.2	Details of Radio remote control system to be furnished with the offer.

18.3 	The system shall be following designated controls:

Main Power 		ON/OFF

Hoist creep 		UP/DOWN			1 -  0 -  1

Hoist 			UP/DOWN			1 -  0 -  1

CT			FORWARD/REVERSE	1 –  0 -  1

LT			LEFT / RIGHT (SLOW)	1 –  0 -  1

LT			LEFT / RIGHT (HIGH)	1 – 0 - 1

Alarm			ON/OFF

Emergency stop	ON/OFF

Under bridge lights	ON/OFF

19.0	CRANE BRIDGE LIGHTING :

19.1 	Four Nos. 300 watt flood lights with fitting or four Nos. 100 Watt HELOGEN  lights with fitting should be provided below the bridge girder.

19.2 	Separate ON/OFF switches for all four lights shell be provided in the operator's cabin.

19.3 	A lighting transformer of suitable rating shall be provided for lights, fan etc.

20.0	WIRING:

20.1	The crane shall be supplied complete with all wiring and materials. Collectors on the crane bridge and on the crab shall be provided where applicable. Moveable type flexible cable with necessary trolleys and festooned system shall be provided for supply of trolley. LONGITUDNAL GANTRY LEADS ARE ALREADY PROVIDED IN THE BAYS.

20.2	All wiring shall be with PVC insulated and or armored cables having copper conductor. Wire and cable should be of 1100V grade in trunking. Trunking should be used instead of conduit pipe.

20.3	All electrical wiring and installation should strictly confirm to IS Act, 1910 and I.E.E. Rules. 1956 and other standard practice as adopted by the I.S.I.

20.4	All control terminations should be ferruled such that the numbers are clearly visible from out side, crimped terminations should be provided for control cables and as far as possible for main cables. A set of suitable current collector shoe should be provided to suit the angle iron down shop leads. Details of existing down shop lead and current collector system can be checked at site before finalizing the supply of current collector system.

21.0	MAINTENANCE:

21.1	We would stress the need of adequate maintenance facility for all gearboxes and working mechanism which shall be located above platform level and shall be easily approachable for good maintenance.

22.0	SPARES :

22.1	Recommended spares for two years normal working of the equipment is to be furnished with the estimate, separately quoted for each items. Details of bearings, oil seals used should be supplied along with location of each item.

23.0	MISCELLANEOUS :

23.1	All gear boxes should have oil level indicator to show the level and condition of the oil in gear box.

  - All material used should be of the best of their respective kinds and should of the appropriate I.S. Specification & all workman ship should be of best quality.

  - The structural steel work in the bridge and the crab frame should be given one coat of suitable primer during fabrication. Gear boxes and similar parts to be given coat of oil paint. After erection at site, the crane should be given two coats of light orange synthetic enamel paint.

  - First fill oil, grease etc. for the commissioning of the crane shall be provided by the supplier.

  - Tool box with standard tools shall be supplied with the crane.

  - List of bought out items should be furnished with  names of makers and complete literature about the items should be supplied.

  - Crane to be fabricated for its satisfactory performance for a period of 24 months from the date of commissioning with guarantee certificate.

  - Lifting hook, wire rope and crane to be tested to IS standard and test certificate to be provided (4 copies each). Test certificate of hook should from a recognized Govt. Test House.

  - Four copies of manual of Operation and servicing to be with erection drawings. G.A. drawing, wiring diagram etc. to be supplied along with the crane.

  - Four copies of structural drawing, wheel assembly, maximum crane wheel load etc. H.P. rating of motor, hook approach, gear box details, weight of crane to be supplied.

  - Mechanical drawings, electrical wiring and schematic diagram should be sent in triplicate for approval before finalizing the design. Make and technical specification of all items is also to be submitted for approval.

  - Supplier shall inform BHEL in advance to witness assembly and load test.

  - The crane is to be erected by the supplier at site with their own labour, tools and tackles, winch. The crane is to be commissioned and tested before handing over.

  - All motions at various steps of speed shall be checked along with current being taken by respective motion motors.

  - After test but before crane is put to use, it shall be tested with overload of 125 percent of the S.W.L. when load is located at the center of span. During testing the overload test each motion shall be maneured in both direction and the crane shall sustain the load under full control.

  - Erection / commissioning charges to be quoted separately.

24.	QUALIFYING CONDITIONS

Only those vender, who have supplied and commissioned up to 10 T or higher Capacity EOT crane for similar application in the past ten years and such crane is presently working satisfactorily for more than one year (more than six month if supplied to BHEL) after commissioning should quote. The following information is to be submitted by the vender about the companies where similar cranes have been supplied. This is required from all venders for qualifications of their offers.

- Name of the customer/ company where similar machine is installed

- Complete postal address of the customer

- Year of commissioning

- Name and designation of the contact person of the customer

- Phone , Fax No. and e.mail address of the contact person of the customer

- Performance certificate from the customer regarding satisfactory performance of the machine supplied to them

24.0	DETAILS MENTIONED IN ANNEXURE –II SHALL BE PROVIDED COMPULSORILY ALONG WITH TECHNICAL BID.

ANNEXURE -  II

SPECIFICATION NO. CRJ 9562

DETAILS MENTIONED BELOW SHALL BE PROVIDED COMPULSORILY ALONG WITH TECHNICAL BID.

| S.No. | Description | Requirement | Offer | Remarks |
| --- | --- | --- | --- | --- |
| 1 | Hook lift above floor level | 8.00 Meter |  |  |
| 2 | Main hoist speeds | Max. 7.62 Meter / Min |  |  |
| 3 | Hoist creep speed on 1st step from master controller & remote control | 10% of Max. speed |  |  |
| 4 | Cross Traverse speed | Max. 21.32 Meter/ Min. |  |  |
| 5 | Long Travel speed | Max. 76.2 Meter / Min. |  |  |
| 6 | Weight of each bridge (two Nos. ) | Supplier to quote |  |  |
| 7 | Weight of each carriage ( two Nos.) | Supplier to quote |  |  |
| 8 | Weight of compete trolley | Supplier to quote |  |  |
| 9 | Weight of compete assembled crane | Supplier to quote |  |  |
| 10 | Over all width of crane | Supplier to quote |  |  |
| 11 | Wheel load on each L.T.wheel | Supplier to quote |  |  |
| 12 | HP of various motor, duty cycle , rating , make & speed | Supplier to quote |  |  |
| 13 | Size of brake drum Hoist CT LT | Supplier to quote |  |  |
| 14 | Wheel dia LT CT | Supplier to quote |  |  |
| 15 | Size and capacity of thruster brakes Hoist CT LT | Supplier to quote |  |  |
| 16 | No. of falls in hoist | 4 Fall |  |  |
| 17 | Size, make & Specification of wire rope | Supplier to quote |  |  |
| 18 | Maximum hook approach from LT rail center Cabin end Far end | Supplier to quote |  |  |
| 19 | Type & make of main hook | Shank type make to quote |  |  |

##### C. ENQUIRY ITEM No.3

##### ITEM				:	Electric Over head Traveling Crane

Class II duty as per IS: 3117/ 1965

ITEM NO.			:		SP/CRJ/9560

SPECIFICATION NO.	:		CRJ/9560

LOCATION			:		NEW INSULATION SHOP

QUANTITY			:		1 No.

1.0	GENERAL   :

Electric Over Head crane of capacity indicated below is required for loading, unloading of unfinished, finished components from Machine Tools as well as assembly of Transformers in transformer manufacturing shop. The same has to work on a prepared gantry in transformer shop.

2.0	Capacity and leading dimensions are given hereunder:

2.1	Safe Working Load	:

Main Hoist				:	5  Tones

2.2	Duty as per I.S.3177/1965			:	Class 2

2.3	(a)	Normal Span				:	10  Meters

(b)	Rail Center to Center			:	8.8  Meters (Provided)

2.4	Gantry Rail height above floor level		:	08.00  Meters (Provided)

2.5	Length of Bay					:	96 Meters

2.6	Type of Hook					:	Shank Type

2.7	Available Head above Gantry Rail 		:	2.8  Meters

2.8	Available side clearance from			:	0.45  Meters

Gantry Rail Center

2.9	Operation					:	Cabin & Radio Remote

2.10	Location					:	Indoor

2.11	Hook lift above floor level to be provided	:	8.00  meters

2.12	Electric Power Supply				:	415V, 3 Phase, 50 C/S, A.C.Supply

2.13	Gantry Rail					:	90 lbs (Provided)

2.14	Full Geared Speed  :

a)	Main Hoist		:	7.62 M /Min.

b)	Traverse		:	21.32 M / Min.

c)	Travel			:	76.20 M / Min.

2.15	All the speed should be controlled in four steps both in forward & reverse direction. All four steps should have gradual increase in speed from minimum to maximum. Speed specified above are maximum speeds.

2.16	Speed control system shall be elaborately and clearly mentioned by the tenderer in their offer. Minimum speed,  speed at various steps to be clearly mentioned.

2.17	Speed control for hoisting / lowering shall be Thruster creep brake system. Creep speed shall be provided in the 1st step of both hoist and lowering motion.

2.18	Creep speed to be mentioned in the offer.

2.19	Firm to indicate following explicitly in their quotation:

(a)	Weight of each bridge (2 Nos.)

(b)	Weight of each carriage. 9 2 Nos)

(c)	Weight of complete trolley.( 1 No.)

(d) 	Weight of complete assembled crane

(e)	No. of wheels on LT & CT,  Wheel diameter and material specification.

(f  )	Wheel Load on each wheel

- Horse Power of various motors, their duty cycles, rating , make etc.

- Make, Type, size of thrusters and E.M. Brakes.

- Make of all electrical items like relays, contactors, resistance,  timer, limit switches etc.

- Size of Brake drum for each motion.

- Complete specification of wire rope for both motion.

- Number of falls of wire rope in main hoist and auxiliary Hoist.

- Erection & commissioning charges to be quoted separately.

3.00	GENERAL	:

3.1	The crane shall be generally of  box type construction and to be designed for class 2 duty in accordance with BSS: 466/1960 and or IS:3117/1965. Material shall be of tested quality as per relevant I. S.

3.2	BRIDGE CONSTRUCTION   :

    - The bridge should comprise of two main girders with two out riggers and the end carriage built up from mild steel channel with adequate diaphragms or stiffing plates. Each bridge girder shall be fabricated and dispatched without any spliced joints.

3.2.2	Bridge girder should have revetted / bolted connection with end carriages.

3.2.3	M.S Chequered plate platform shall be provided along with full length on both side of the crane along with toe board.

3.2.4	Hand railing shall be fitted on both sides of the end carriages and through out the length of the platform on the bridges. Size of the hand rail shall not be less than 25 mm. bore heavy gauge steel pipe. The vertical pipes to have flanges  at the bottom for proper support.

4.0	TRACK WHEEL:

4.1 	The end carriages shall be of approved design, substantial construction and of ample stiffness built up of M.S. section securely riveted or welded with each other. They shall be projected over the run way rails and shall be so arranged that in the event of  derailment the 	drop should not exceed 25 mm.

4.2	The end carriage of the crane should be mounted on double flanged  wheels of the approved diameter in accordance with B.S.S.466.  Each wheel shall be  fitted with two numbers roller bearings SKF/NBC/FHP make mounted on a fixed steel sleeve and carried by a hard steel pin. Grease nipple should be provided at suitable place for convenient greasing and the grease be retained, dirt excluded by means of suitable seals.

- TRAVELLING MECHANISM:

5.1	One wheel in each carriage is to be driven by a machine out forged steel gears by a motor mounted on or adjacent to the carriage. Motors of both the end carriage travel would be connected to one controller but the rotor circuit should be entirely independent to avoid synchronization and to permit the variation of speed, which is necessary to run the crane freely and smoothly on a track which deviates slightly from perfect straightness.

6.0	TRAVELLING MOTION BRAKE

6.1 	A thruster brake to be fitted on each individual drive of the track wheels. These brakes should 	be arranged to be operated from the control desk as well as remote control. The brakes to be of post type with hinged shoes, lined with good quality reinforced bonded brake lining and arranged for easy adjustment for the necessary wear on lining time to time.

6.2	An independent additional mech. operating hand brake is to be provided in LT motion to stop the crane immediately in case of power failure. This should also be operated from Radio remote on emergency condition.

7.0	CRABE FRAME :

7.1 	This should be built up of steel sections and plates to proper thickness and the upper surface of the frame should be plated over wherever required in order to give great lateral rigidly and maintenance facility. The lubricating points for all moving parts should be provided at the top conveniently for easy maintenance below the surface of the trolley i.e. balancing sheaves etc.

8.0	CRABE TRAVERSE :

8.1	The wheel should be of forged steel material of approved diameters duly secured to hard shaft steel axles and to run on sturdy roller bearing of SKF	make. The bearings to be enclosed in a housing and arrangement for grease lubrications should be provided. One axle should be driven by a motor through an enclosed good design harden spur gears of good quality. A thruster brake should be provided.

8.2	The cross travel gear box should be well secured at least by four bolts two on each side opposite the each other.

9.0       HOISTING MECHANISM :

9.1 	This shall consist of heavy duty cast iron drum driven by totally enclosed gearing. The drum shall be of ample diameter and of sufficient length to take full amount of rope without overlapping when the hook is in the highest position. When the hook is in the lowest position, there should be at least two full turn of rope remaining on drum. The method of attachment of the rope shall be such that in the event of rope un-winded completely there shall be no danger of its becoming freed from the drum. The drum should have right and left hand spiral grooves machined to suit the required size of wire rope, should be spigotted and bolted to the final driving gear. the whole gear box should be of standard design, having SKF/NBC/FHP make bearings and should be smooth and noiseless in operation.

9..2	The gearing should be of heat treated alloy steel pinions and carbon steel gears the larger gears having forged carbon steel rim shrunk and secured by bolts on heavy duty  cast iron or cast steel.  All gears will have machine cut teeth.  High speed gears must be of helical type teeth and all gears should be free from noise and with smooth operation. The drum shaft and pinion shaft shall be mounted on ball & roller bearing of SKF/NBC/FHP make and complete gear train shall be enclosed in a heavy duty cast iron gear box. The gear box must having jointing surface machined and should be made leak proof. All the extended shaft and end covers on gear box must be provided with good quality oil seal to prevent any leakage.

10.0	HOIST BRAKE  :

10.1	A thruster creep brake to be fitted on the high speed shaft. The brake to be of the post type with hinged shoes.  Shoe must be lined with best quality reinforced bonded asbestos industrial brake lining and arranged for easy adjustment. The brake should be normally held on by a balance weight and released by a thruster in the circuit of hoist motor.

11.0	OTHER BRAKES & LIMIT SWITCHES

  - There should be heavy duty double limit switch  for each hoisting  and lowering motions.

  - Gravity type limit switch must be provided for hoisting operation in order to provide additional safety.

  - Series type limit switches with single arm spring loaded allen west type should be provided for traverse motion.

  - Anti - collision type limit switches should be connected only in interlock circuits.

11.5 	All limit switches should be connected only in interlock circuits.

12.0	ROPE HOOK AND SHEAVES:

  - The hoist shall have the factor of safety being not less than 6.  Main hook for all  load should be supported on 4 falls.

  - Sheaves are to be of heavy duty cast iron mounted on Ball bearings. Guards to be fitted to prevent the rope leaving the sheaves, even when the hook if lowered to the rest on the ground and rope allowed to fall slack. The hook guard should be secured in such a manner that there should be no bolt protruding in the inner side of the hook.

13.0	MOTORS:

13.1	The all motors should be one hour rated crane duty. These should be totally enclosed type fitted with Ball / Roller bearings.

13.2	 The motor should have class 'B'/'F' insulation and to comply with IS: 325 according to   manufacture standard. All motor should be slip ring induction type and suitable for use of 415 V, 3 phase, 50 cycles, AC supply. The motor should be of GEC Alsthom/ crompton make only.

14.0.	CONTROL GEAR

14.1 	The hoist motor, long travel motors and cross travel motor should be controlled by semi automatic reversing contactor panel with a cam operated master controller.

15.0	CRANE PROTECTIVE PANEL:

15.1.	The design & construction of this panel must be of high standard. The equipment should comprise of contactors circuit breakers and magnetic oil dashpot type over current relay connected in all the phases of each motor. An emergency stop and a reset push button and an indicating pilot lamp should be incorporated. All connections internal & external should be accessible from front of the panel for inspection & maintenance. The contactor should be of SIEMENS,/ L&T overload relay of electrogear make and timer of CUTTLER - HAMMER.

  - The relay & contactors should be of robust design. The contactor should be suitable for class IV duty. Over load relay should be magnetic type provided with oil dashpot arrangement with adjustable current setting and with self reset mechanism. Auxiliary contacts of magnetic overload should be robust in construction.

  - Isolator switch fuse unit should be capable of load breaking and mechanically robust. Isolation to be of quick brake type and it should be of GE Control (EE) make. Main Isolating switch should be mounted ion the operator's cabin.

  - All fuses should be of HRC / GE Control (EE) make only.

  - Suitable rating control transformer should be provided for the control of the circuit.

16.0 	RESISTANCE BOX

  - All resistances should be of 10 minutes rating. The resistance should be rust less unbreakable S.S. grid type.

  - All resistance box should be mounted on the crane bridge.

17.0	CONTROL CABIN:

17.1 	The depth of crane cabin should be limited to 2150 mm. Opening to be cage on the platform should be approximately 750 mm x 750 mm. A ladder should be provided between the cage and the inspection platform of the crane. The controllers should be so positioned as to facilitate easy operation & maintenance. A warning bell (Electrically operated industrial type with 6" gong) and Fire extinguisher of 4.5 litre capacity with long discharge nozzle should be provided in operator's cabin. One more bell is to be provided strudy conventional pendulum type made of good bell metal and to be well secured inside the cabin in a convenient position for easy operation. A revolving type cushioned seat is to be provided in the cabin for the crane operator. A 2 feet 20 watt fluorescent tube fitting, one 16" sweep cabin fan with guard should be provided.

  - An inspection lamp with 20 meter 3 core flexible cable is to be provided with 240 V plug point in the cabin.

  - The total depth of control cabin should be such that it should not interfere with any other structure / semi gantry crane / machines in the entire bay. For this supplier must check all the available clearances at the side before finalizing/ designing of the crane dimension.

18.0 	RADIO REMOTE CONTROL  SYSTEM:

18.1	Radio remote control system shall be provided to operate the crane from shop floor in all motion and control. The radio remote control system shall consist of a portable transmitter and receiver along with the necessary relay logic for interfacing to the crane motors. The transmitter 	shall consist of push buttons.  A selector switch shall be provided in the operator cabin for remote / manual operation.

18.2	Details of Radio remote control system to be furnished with the offer.

18.3 	The system shall be following designated controls:

Main Power 		ON/OFF

Hoist creep 		UP/DOWN			1 -  0 -  1

Hoist 			UP/DOWN			1 -  0 -  1

CT			FORWARD/REVERSE	1 –  0 -  1

LT			LEFT / RIGHT (SLOW)	1 –  0 -  1

LT			LEFT / RIGHT (HIGH)	1 – 0 - 1

Alarm			ON/OFF

Emergency stop	ON/OFF

Under bridge lights	ON/OFF

19.0	CRANE BRIDGE LIGHTING :

19.1 	Four Nos. 300 watt flood lights with fitting or four Nos. 100 Watt HELOGEN  lights with fitting should be provided below the bridge girder.

19.2 	Separate ON/OFF switches for all four lights shell be provided in the operator's cabin.

19.3 	A lighting transformer of suitable rating shall be provided for lights, fan etc.

20.0	WIRING:

20.1	The crane shall be supplied complete with all wiring and materials. Collectors on the crane bridge and on the crab shall be provided where applicable. Moveable type flexible cable with necessary trolleys and festooned system shall be provided for supply of trolley. LONGITUDNAL GANTRY LEADS ARE TO BE PROVIDED IN THE SHPO FOR 96 METERS LENGTH.

20.2	All wiring shall be with PVC insulated and or armored cables having copper conductor. Wire and cable should be of 1100V grade in trunking.  Trunking should be used instead of conduit pipe.

20.3	All electrical wiring and installation should strictly confirm to IS Act, 1910 and I.E.E. Rules. 1956 and other standard practice as adopted by the I.S.I.

20.4	All control terminations should be ferruled such that the numbers are clearly visible from out side, crimped terminations should be provided for control cables and as far as possible for main cables. A set of suitable current collector shoe should be provided to suit the angle iron down shop leads. Details of existing down shop lead and current collector system can be checked at site before finalizing the supply of current collector system.

21.0	MAINTENANCE:

  - We would stress the need of adequate maintenance facility for all gearboxes and working mechanism which shall be located above platform level and shall be easily approachable for good maintenance.

22.0	SPARES :

22.1	Recommended spares for two years normal working of the equipment is to be furnished with the estimate, separately quoted for each items. Details of bearings, oil seals used should be supplied along with location of each item.

23.0	MISCELLANEOUS :

23.1	All gear boxes should have oil level indicator to show the level and condition of the oil in gear box.

  - All material used should be of the best of their respective kinds and should of the appropriate I.S. Specification & all workman ship should be of best quality.

  - The structural steel work in the bridge and the crab frame should be given one coat of suitable primer during fabrication. Gear boxes and similar parts to be given coat of oil paint. After erection at site, the crane should be given two coats of light orange synthetic enamel paint.

  - First fill oil, grease etc. for the commissioning of the crane shall be provided by the supplier.

  - Tool box with standard tools shall be supplied with the crane.

  - List of bought out items should be furnished with  names of makers and complete literature about the items should be supplied.

  - Crane to be fabricated for its satisfactory performance for a period of 24 months from the date of commissioning with guarantee certificate.

  - Lifting hook, wire rope and crane to be tested to IS standard and test certificate to be provided (4 copies each). Test certificate of hook should from a recognized Govt. Test House.

  - Four copies of manual of Operation and servicing to be with erection drawings. G.A. drawing, wiring diagram etc. to be supplied along with the crane.

  - Four copies of structural drawing, wheel assembly, maximum crane wheel load etc. H.P. rating of motor, hook approach, gear box details, weight of crane to be supplied.

  - Mechanical drawings, electrical wiring and schematic diagram should be sent in triplicate for approval before finalizing the design. Make and technical specification of all items is also to be submitted for approval.

  - Supplier shall inform BHEL in advance to witness assembly and load test.

  - The crane is to be erected by the supplier at site with their own labour, tools and tackles, winch. The crane is to be commissioned and tested before handing over.

  - All motions at various steps of speed shall be checked along with current being taken by respective motion motors.

  - After test but before crane is put to use, it shall be tested with overload of 125 percent of the S.W.L. when load is located at the center of span. During testing the overload test each motion shall be maneured in both direction and the crane shall sustain the load under full control.

  - Erection / commissioning charges to be quoted separately.

24.	QUALIFYING CONDITIONS

Only those vender, who have supplied and commissioned at least 5 Nos. EOT cranes of up to 10 Ton or higher Capacity for similar application in the past ten years and such crane is presently working satisfactorily for more than one year (more than six month if supplied to BHEL) after commissioning should quote. The following information is to be submitted by the vender about the companies where similar cranes have been supplied. This is required from all venders for qualifications of their offers.

- Name of the customer/ company where similar machine is installed

- Complete postal address of the customer

- Year of commissioning

- Name and designation of the contact person of the customer

- Phone , Fax No. and e.mail address of the contact person of the customer

- Performance certificate from the customer regarding satisfactory performance of the crane supplied to them

25.0	DETAILS MENTIONED IN ANNEXURE –III SHALL BE PROVIDED COMPULSORILY ALONG WITH TECHNICAL BID.

ANNEXURE - III

SPECIFICATION NO. CRJ 9553 B

DETAILS MENTIONED BELOW SHALL BE PROVIDED COMPULSORILY ALONG WITH TECHNICAL BID.

| S.No. | Description | Requirement | Offer | Remarks |
| --- | --- | --- | --- | --- |
| 1 | Hook lift above floor level | 8.00 Meter |  |  |
| 2 | Main hoist speeds | Max. 7.62 Meter / Min |  |  |
| 3 | Hoist creep speed on 1st step from master controller & remote control | 10% of Max. speed |  |  |
| 4 | Cross Traverse speed | Max. 21.32 Meter/ Min. |  |  |
| 5 | Long Travel speed | Max. 76.2 Meter / Min. |  |  |
| 6 | Weight of each bridge (two Nos. ) | Supplier to quote |  |  |
| 7 | Weight of each carriage ( two Nos.) | Supplier to quote |  |  |
| 8 | Weight of compete trolley | Supplier to quote |  |  |
| 9 | Weight of compete assembled crane | Supplier to quote |  |  |
| 10 | Over all width of crane | Supplier to quote |  |  |
| 11 | Wheel load on each L.T.wheel | Supplier to quote |  |  |
| 12 | HP of various motor, duty cycle , rating , make & speed | Supplier to quote |  |  |
| 13 | Size of brake drum Hoist CT LT | Supplier to quote |  |  |
| 14 | Wheel dia LT CT | Supplier to quote |  |  |
| 15 | Size and capacity of thruster brakes Hoist CT LT | Supplier to quote |  |  |
| 16 | No. of falls in hoist | 4 Fall |  |  |
| 17 | Size, make & Specification of wire rope | Supplier to quote |  |  |
| 18 | Maximum hook approach from LT rail center Cabin end Far end | Supplier to quote |  |  |
| 19 | Type & make of main hook | Shank type make to quote |  |  |
| 20 | Down shop leads for 96M length | supplier to quote |  |  |

2.INSTRUCTIONS FOR SUBMISSION OF TENDER

- The offer against the enquiry shall be submitted in two parts.  The first part called Techno-commercial bid i.e. Part - A shall be un-priced and will be submitted in line with given details in commercial terms and conditions .  The second part called Price-Bid  i.e. Part-B, in this you will have to mention the basic prices, taxes duties etc for basic equipment & Standard / optional accessories if any.

- The vendors shall submit Part-A and Part-B of their bids in two different sealed envelopes duly super-scribing Enquiry No. and Part No. of the bid.  These two parts of the bid shall be kept together in a third envelope super-scribing Enquiry No., due date and time of opening as indicated in the tender document.

- Part-A of the bid alone shall be opened on the due date in presence of the tenderers in the first instance.   This part of the bid shall be examined in detail.  Part B of the bid shall be opened only for such of the tenderers whose Techno-Commercial offers are found acceptable by BHEL.  Such tenderers shall be advised the date and time of opening of Part B i.e. price part of their bids.  Tenderers must carefully go through all the instructions and conditions before filling in Part B of the bid.  Any type of mistake in submission of the bids will make the tender liable for rejection.

- The bids shall be submitted only in the format given in the tender documents. The tenderer is  required to sign and stamp each and every page of the tender document clearly bring out deviations, if any.

- Tenders not complying with the above and other requirements of the tender shall be liable for rejection.  Tenderers are advised to read all the documents carefully before submission of the bids and ensure the fulfillments of all the technical and commercial parameters.

- No over-writing in any Part-A or Part-B is permitted.  Corrections in the bid should normally be avoided. However, when these become essential, bidder shall score out the incorrect information and put the correct information duly endorsing each correction with his signatures.

- Tender Cost (if any) is to submitted in the Form of  DD in favour of BHEL-Jhansi  payable at

Jhansi along with Techno –Commercial offer. This DD should not be kept in Sealed Price Bid.

BHEL shall go for REVERSE AUCTION and finalize order on the basis of rate obtained  through reverse auction. During reverse auction, if no bid is received within specified time, BHEL, at its discretion, may decide to revise opening price/ scrap the reverse auction process/ proceed with conventional mode of tendering (opening of price bid Part B submitted by the tenderers earlier along with techno-commercial bid).

- General terms  and conditions of the REVERSE AUCTION shall be generally as given later in Tender document. However final terms & condition shall be intimated after techno-commercial evaluation of the tender.

3.COMMERCIAL TERMS & CONDITIONS FOR PURCHASE ENQUIRY

- Offer is invited in two bids, outer cover, inner cover-I (Techno-Commercial bid) and inner cover II (Price-Bid) must bear our tender no., due date and bidders name.  Offers received in single bid will be bypassed.

- The offer should reach this office on or before 13.15 hrs on the due date.  The tender can also be dropped in the Tender Box kept at Administrative Building Gate.  The tenders will be opened after 1400 hrs on the due date in presence of all bidders who will be present.

- Offer should be free from overwriting or erasures.  Corrections or additions, if any should be attested.

- No enhancement of rate for whatever cause is allowed once the offer is accepted and the order is placed.  Withdrawal from the quotation after it is accepted or failure to make the supply within the scheduled period will entail cancellation of order, forfeiture of earnest money if any and/or imposition of risk & cost purchase.

- Material offered should be of required quality and conforming strictly to our specifications / drawings or to the specifications in the drawings.  Any deviation must be clearly stated in the offer with particular clause /detail in our specification / drawing for which deviation is quoted.  All required documents/details  e.g. QA Plan, type TCs, drawings & data sheet etc. must be submitted along with the offer.

- Irrespective of whether it is mentioned in our specification, all machines / equipments are required to be calibrated in metric units and certificates must be issued where applicable.

- Work test  certificates / inspection certificates/guarantee certificate/ compliance certificate, etc. will be required to be submitted along with supplies in case an order is placed.

- Required Delivery  time for affecting supplies should be indicated in the offer.

- Failure to supply by the time specified on the order will make the supplier liable to pay unconditional penalty of ½% of the prices of goods in arrear per week subject to a maximum of 10% for Indian suppliers and max 5% for Foreign suppliers of the order value unless agreed otherwise.

- The quantities for each item to be purchased may vary from the tendered quantity according to our actual requirement at the time of placing orders.

- Bharat Heavy Electricals Ltd does not bind itself to accept the lowest of any tender, but reserves to itself the right to accept or reject all or a part of any tender at its discretion following BHEL Purchase Policy & Government guidelines.

- In all cases of disputes, the matter shall be referred for arbitration.  The arbitrator  shall be nominated by the Unit Head, Bharat Heavy Electricals Ltd., Jhansi or any other officer nominated by him.  The award of Arbitrator shall be final and binding on both the parties.

- In case of any suit or other legal proceedings under or relating to this tender, the courts at Jhansi shall have full jurisdiction.

- Total delivered cost at BHEL Jhansi Railway siding/Works shall be the criterion for deciding the tender.

15.  Offer should be on F.O.R.  Jhansi basis (Including Freight charges but excluding Insurance charges).

Insurance will be done by BHEL.

- Your quotation against this enquiry must be valid for ordering up to minimum of 120 days from the date of tender opening.

- ADDITIONAL FOR INDIAN SUPPLIERS:

I)        If any material is booked to selves or if it is not consigned to BHEL Jhansi, any warfage or

demurrage occurring from these shall be to the account of supplier.  In case of road transport,

material shall be booked only through our rate contract transporters mentioned in the purchase

order.

- Where there is provision for payment of Sales Tax extra, it will be paid only if the registration number both under State Sales Tax and Central Sales Tax are specifically mentioned on the bill/invoice.  Your Sales Tax Registration number should be mentioned in the offer positively.

- Excise Duty Gate Pass will be required in original.  Applicable taxes & duties must be included in the offer.

- ADDITIONAL FOR FOREIGN SUPPLIERS:

i)	For documentation and correspondence language shall be English only.

- Rates quoted should be on FOB basis.  All despatches are to be made by sea  by IPBC conference vessels only unless otherwise stated in purchase order.

- Agency commission, if any, will be paid out of the FOB value after receipt & acceptance of material at BHEL Jhansi in Indian Rupees.  TT Buying exchange rate prevailing on the tender opening date or the letter of credit date or date of purchase order whichever is lowest will be considered for calculating the agency commission.

- The rates quoted should be inclusive of charges of packing and delivery of material up to the port of shipment on FOB terms.  The material is required to be packed suitably in all respects considering the peculiarity of the material for normal transport by Sea/air and subsequently by Rail/Road in India suitably protected against the effect of tropical salt laden atmosphere in the event of consignment being delayed at Indian Ports before outward clearance.  You should also furnish details of shipping specification, sizes, volume, weight, number of cases etc. When articles are packed with packaging material of plant origin, phytosanitary certificate will be required.

- Our standard terms of payment is 100% payment through bank against despatch documents or 100% through sight irrevocable & unconfirmed letter of credit.

19.   Security deposit (SD) cum Performance Bank Guarantee  (PBG) for 10% basic value of the order (excluding taxes &

duties) for  execution of the purchase order (PO) & satisfactory Performance of the equipment before release of  PO

will be required to submit which will be valid from the date of issue of LOI to 24 months after acceptance of material by

BHEL with a further claim period of 3 months. PO will be issued only after receipt of SD cum PBG.

20. For fully imported equipments / plant/ machinery (no part or sub-assembly of Indian origin) :

a. No SD cum PBG will be applicable for basic order value up to Rs. 25.0 Lacs on foreign supplier.

b. No SD cum PBG will be applicable for basic order value up to Rs. 10.0 Lacs on Indian stockist / agent of

foreign manufacturer.

21.   B.G. shall be from Scheduled Banks/Public Financial Institutions as defined in the Companies Act.

22. The equipment shall be guaranteed for 24 months from the date of acceptance by BHEL irrespective of date of supply,

erection and commissioning.

23. Our standard terms of payment (for Indian Suppliers) is 80% of P.O. value be paid after 60 days of acceptance of the

equipment at BHEL Jhansi works and balance 20% after successful commissioning & handing over of the equipment

(where applicable).

24. Amount of Service Tax / Education Cess / any other Govt. duty ( as applicable on the date of offer) for erection or any

other activities associated with the scope of work in the tender must be indicated in Techno-commercial bid of the offer.

25 . Blank Price Bid (After erasing prices) must be submitted along with Techno commercial bid.

26.  If any goods are rejected, BHEL shall be at liberty to take action as per following at the

risk and cost of supplier:-

    - Allow the supplier, wherever mutually agreed, to rectify the rejected goods at BHEL's works within reasonable time as fixed by BHEL.

    - Allow the supplier to make free replacement within a specified period, rejected goods can be lifted by the supplier thereafter.

OR

Allow the supplier to refund the full amount paid to him by BHEL by a Demand Draft  before lifting the rejected goods.  Fresh replacement shall be regularized as per terms & conditions of the original purchase order.

OR

Take alternative  procurement action from elsewhere and recover the difference in cost, if any incurred by BHEL in this regard from the supplier.  The supplier shall not be entitled to any gain on purchase.

    - Terminate the contract either in part or in whole at the  discretion of BHEL and recover the loss, if any, from the supplier.

27. For any clarification on our tender terms and conditions please contact Sr. Purchase Officer (Capital Items)

before due date.

28. Inspection may be done at supplier works by BHEL.

## 4. TECHNO COMMERCIAL BID  PART-A

( Supplier should fill up this bid by putting the tick in accepted / non accepted column)

SUPPLIER’S NAME & ADDRESS

1.          Offered Equipment with complete technical details  :                      As per attached details

2.           Packing & Forwarding Charges, FOB charges (if any)                                                               ….%

3.   	Present rate of Excise duty (if any)         		                                                              ….%

4.     	Present rate of sales tax			 		                                                   %

(UPTT/CST, Not applicable in case of Foreign suppliers)

5.	Delivery period in  No. of  days from  date of LOI / PO                                                                                                                    6.	Taxes and duties shall be extra as applicable on 		              Accepted / Not accepted

the date of actual delivery.

7.	Penalty clause as well as other conditions given			Accepted / Not accepted

in general Terms & conditions

and Standard Enquiry terms & conditions are

acceptable to us.

8.           Rates quoted in the price bid are on  F.O.R. 		                                      Yes/ No

Jhansi basis (Including Freight charges but excluding

Insurance charges which will be done by BHEL).

9.	Validity of the rates quoted is 120 days from the 			Accepted / Not accepted

date of opening of price bid.

10.          Security deposit (SD) cum Performance Bank Guarantee                      Accepted / Not accepted

(PBG) for 10% basic value of the order (excluding taxes & duties)

for  execution of the purchase order (PO) & satisfactory

Performance of the equipment up to Guarantee period

of 24 months  before release of PO will be submitted.

11.        The equipment shall be guaranteed for 24 months from                            Accepted / Not accepted

the date of acceptance by BHEL irrespective of date of

supply, installation.

12.         BHEL may finalize order on the basis of rate			             Accepted / Not accepted                                    Reverse Auction or obtained either through sealed price bid.

12.0	GENERAL/ MANDATORY PART

(For reverse auction, following details are required to be furnished which are mandatory in nature)

12.1	E-mail ID							…………@………………..

12.2	Mobile Phone No.							…………………………….

Land Line  Phone No. with STD Code		                             …………………………….

12.3	Name of Contact Person						…………………………….

12.4	Fax Number							…………………………….

Note :- If any of the above conditions are not accepted

the offer shall be liable for rejection.

5. REVERSE AUCTION DETAILS:

| BUYER NAME | BHARAT HEAVY ELECTRICALS LIMITED, JHANSI |
| --- | --- |
| AUCTION TO BE CONDUCTED BY | C1 INDIA PVT. LTD. D-5, DEFENCE COLONY, NEW DELHI 110 024 https://auctions.gate2biz.net |
| DATE & TIME OF AUCTION. | TO BE INTIMATED LATER ON |
| DOCUMENTS ATTACHED | - BUSINESS RULES FOR REVERSE AUCTION - REVERSE AUCTION TERMS & CONDITIONS - CONTACT INFORMATION - PROCESS COMPLIANCE FORM |

A. Business Rules for Reverse Auction for BHEL - JHANSI

BHEL proposes to finalise the above tender through reverse auction mode. BHEL has made arrangement with M/s C1 India, Delhi, who shall be BHEL’s authorized service provider for the same. Please go through the guidelines given below and submit your acceptance to the same.

- Computerized reverse auction shall be conducted by BHEL while the vendors shall be quoting from their own offices/ place of their choice. Internet connectivity shall have to be ensured by each agency themselves. In extreme case of failure of Internet connectivity, (due to unforeseen circumstances- not power failure), fax communication shall have to be made immediately. BHEL/M/s C1 India may decide to extend the bidding time, at their discretion, but not as your right.

- M/s C1 India shall arrange to demonstrate/ train your nominated person(s), without any cost to you. They shall also explain you, all the Rules related to the Reverse Auction/ Business Rules Document to be adopted along with bid manual. You are required to give your compliance on it before start of bid process.

- Price basis shall be Landed Net of MODVAT Cost to BHEL, Jhansi i.e Landed Cost = Basic Rate + Custom / Excise Duty + Sales Tax + any other Taxes – MODVAT.

B. REVERSE AUCTION TERMS & CONDITIONS

Auction shall be in two parts-	 i) Initial Sealed Auction &

ii) Dynamic Reverse Auction.

    - Initial auction in which the vendors shall be asked to enter the price on Landed Cost, Net of Modvat basis, The Initial Auction shall be carried out initially for 30 minutes so that all the participating bidders can enter their data correctly. In this type of Auction, Vendors shall be allowed to quote once only and will not be able to see the Bids of other Bidders as well as the Lowest Bid for any Line Item.

    - Dynamic Reverse Auction: BHEL will declare its Opening Price (OP), which shall be visible to the all vendors during the start of the Dynamic Auction. You shall be required to start bidding now from this Opening Price.

    - The bid decrement amount shall be intimated later.

2.	All the vendors will be required to submit the final price confirmation and price break up indicating Basic Price, ED, Sales Tax , after the completion of Auction to BHEL, duly signed and stamped as token of acceptance without any new condition other than those already agreed to before start of auction.

3.	Dynamic Auction shall be for a period of 120 minutes. If a bidder places a Bid in the last 5 minutes of Closing of the Auction, the auction shall get extended automatically for another 5 minutes. In case, there is no Bid in the last 5 minutes of closing of Auction, the Auction shall get closed automatically without any extension.

4.	Your bid will be taken as an offer to sell. Bids once made by you, cannot be cancelled/withdrawn and you shall be bound to sell the material as mentioned above at your final bid price. Should you back out and not make the supplies as per the rates quoted, BHEL shall take action as appropriate.

5.	You shall be assigned a Unique User Name & Password by M/s C1 India Pvt.  Limited. It is mandatory to change the password after first login to ensure  confidentiality. All bids made from the Login ID given to you will be deemed to have been made by your company.

6.	You shall be able to view the following on your screen along with the necessary fields in the Dynamic Auction:

  - Leading Bid in the Auction

  - Bid Placed by you

  - Opening Price.

7.	At the end of the Reverse Auction, BHEL will decide upon the winner. BHEL’s decision on award of Contract shall be final and binding on all the Bidders.

8.	BHEL shall be at liberty to call other bidder/s for L1 matching / cancel the reverse auction process / tender at any time, before ordering, without assigning any reason.

9.	BHEL shall not have any liability to bidders for any interruption or delay in access to the site irrespective of the cause.

10.	Other terms and conditions shall be as per your techno-commercial offers and other correspondences till date.

11.	You are required to submit your acceptance to the terms/ conditions/ modality given above before participating in the reverse auction.

12. .	BHEL may finalize order  on the basis of rate	 obtained either through Reverse Auction  or through sealed price bid.

13.          You  will have to submit the completely filled up Process Compliance Form given as Annexure-II.

C.CONTACT INFORMATION:

| C1 INDIA PVT. LTD., NEW DELHI | BHARAT HEAVY ELECTRICLAS LIMITED, JHANSI |
| --- | --- |
| Akshaya Rath Sr. Account Manager Cell: +91-9350251043 C1 India Pvt. Ltd. D-5, Defence Colony New Delhi-110024 Tel: +91-11-24629600 Ext. 207 Fax: +91-11-24655186 Website: www.c1india.com | A.K.Banerjee, DGM-MM Phone: 0510- 2412431 Fax: 0510 2412120,2770161 E mail: akbanerjee@bheljhs.co.in P.K.Pathak Sr. Purchase Officer Phone: 0510-2412576 |
| Mansi Kumar Mob: 09810006951 Phone: 011 24629600 Ext 212 E mail : mansi.kumar@c1india.com | Y.K.Verma Sr. Purchase Officer Phone: 0510-2412569 E mail: ykverma@bheljhs.co.in |

Annexure-IV

Process   Compliance   Form

Date :

DGM (MM)

Bharat Heavy Electricals Limited

Jhansi

Sir,

Ref:	Your Tender Enquiry number E7160035 on Terms and Conditions for the Reverse Auction of

This letter is to confirm that –

- We will change the password provided to us.

- The undersigned is an authorised representative of the company mentioned below.

- I have studied and understood the terms and conditions as mentioned in your above referred tender enquiry.

- I agree, on behalf of my company (and principals if any) to these terms and conditions which would govern the Reverse Auction Process.

As such I request you to allow me to participate in this Reverse Auction

With regards,

Signature with company seal

Name –

Designation

Company / Organisation –

Address of Company / Organisation –

Seal of The Company