BHEL - JHANSI

TENDER DOCUMENT FOR COIL ENCAPSULATION PLANT, Qty -1 No.

Enquiry No. E7160036, DUE ON   27.01.07

C  O  N  T  E  N  T  S

Page No.

1.         Item Description & Technical Details                                         2-16

2.	Instructions for submission of Tender		                         17

3.	Commercial Terms & Conditions		                                 18-20

4.	Techno-Commercial Bid          		                                    21

( A.K.Banerjee  )		     ( P.K.Pathak )                                  ( Y K Verma )

DGM(MM)	                           Sr. PO (MM)                                      Sr. PO (MM)

1. Item Description & Technical Details :

| SPECIFICATION CUM COMPLIANCE CERTIFICATE FOR 01 NO. VACUUM RESIN CASTING PLANT |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Specification No. | 7083 | Item no. | ME-1/4/2573 | Location | Bay - 5 |
| Note : |  |  |  |  |  |  |
|  | 1. Vendor must submit complete information against clause No. 20 ( Qualifying condition). The offer meeting this clause would only be processed. |  |  |  |  |  |
|  | 2. The " offered" Column and where applicable, the " Deviations" & " Remarks " Column of this format shall be filled in by the Vender and submitted along with the offer. Inadequate / incomplete, ambiguous, or unsustainable information against any of the clauses of the specification / requirements shall be treated as non - compliance. |  |  |  |  |  |
|  | 3. The offer and all documents enclosed with the offer should be in English language only. |  |  |  |  |  |
| SCOPE : SUPPLY , ERECTION & COMMISSIONING OF 15 TONNE VERTICAL WINDING MACHINE COMPYING WITH SPECIFICATIONS AS BELOW: |  |  |  |  |  |  |
| S.NO. | DESCRIPTION FOR BHEL REQUIREMENT |  | SPECIFIED / TO BE CONFORMED BY | OFFERED | DEVIATIONS | REMARKS |
| 1 | PURPOSE & WORK PIESE MATERIAL |  |  |  |  |  |
| 1.1 | The Vacuum Resin Casting Plant is required for the manufacturing of cast resin coils with quartzs filler and insulation class of F & H both for Dry Type Transformers manufacturing. |  | VENDOR TO CONFIRM |  |  |  |
| 1.2 | work piece material: The vacuum compact casting equipment for batch wise preparation of liquid epoxy resin with abrasive filler and to cast transformer coils under vacuum in a casting tank with the prepared mixture. The casting plant is to be designed for automatic continuous dry, degassing and dosing of liquid epoxy resin , hardner and quartz filler system in single companents, under vacuum. |  | VENDOR TO CONFIRM |  |  |  |
| 2.0. | SPECIFICATION: |  |  |  |  |  |
| 2.1 | PLANT CONFIGURATION :-The Vacuum resin casting plant consist of following major items: |  | VENDOR TO CONFIRM |  |  |  |
| 2.1.1 | VACUUM CASTING CHAMBER |  |  |  |  |  |
| 2.1.1.1 | It is to be of welded construction size 3500 mm dia, 5000 mm long with doors on both end, designed for vacuum of 0.01 Torr and pressure of 3 KG per cm sq. |  |  |  |  |  |
| 2.1.1.2 | The chamber is to be designed for payload of 5 MT ( excluding weight of trolley and trolley rails.) |  |  |  |  |  |
| 2.1.1.3 | It is to be provided with heating coils for heating through electrical heating via oil heating system |  |  |  |  |  |
| 2.1.1.4 | The heating system should be capable of maintaining temperature between 60 degree C to 150 degree C, including electronic control, over temperature protection, accuracy of +/- 2 degree C and temperature display on the control panel. |  |  |  |  |  |
| 2.1.1.5 | Rails are to be provided on the bottom of the tank |  |  |  |  |  |
| 2.1.1.6 | A suitable trolley of 5MT capaity is to be provided to carry moulds |  |  |  |  |  |
| 2.1.1.7 | The wall is to be provided with 6 windows, and the lid with two sight glasses along with illuminating deivce for observation of the casting process. |  |  |  |  |  |
| 2.1.1.8 | The tank is to be provided with flange connection which is fitted with measuring current bushing for connecting of 6 X PT100. |  |  |  |  |  |
| 2.1.1.9 | The casting tank shall be equipped with all require pipe connections for vacuum and cast resin compound. |  |  |  |  |  |
| 2.1.1.10 | The casting chamber shall be equipped with all requires pipe connections for vacuum and cast resin compound. |  |  |  |  |  |
| 2.1.1.11 | The casting chamber shall have a provision of 16 nos.resin discharge points with suitable valves.. |  |  |  |  |  |
| 2.1.1.12 | The chamber shall be provided with air circulating fans for rapid and euidistribution, through heating of charge and circulation inside the chamberl. |  |  |  |  |  |
| 2.1.1.13 | The air circulating fans shall be suitable for operation at 100 deg.C and to be fixed un such a way that there will be no lose of vacuum while evacuating the chamber . |  |  |  |  |  |
| 2.1.1.14 | The chamber shall be heat insulated and covered with galvanised steel sheet. |  |  |  |  |  |
| 2.1.1.15 | The chamber shall be equipped with the following: - 2 Nos., 8 Pole testing current bushing . - 2 Resisitance thermometers PT100 to record the temperature of the tank. - 8 Resisitance thermometer PT100 to record the temperature of the part to be cast. - 1 Hnad operated vacuum isolating valve . - 1 Hand operated air adimitance valve. - 1 air admitance with nozzle to onnect a measuring gauge. - 1 Vacuum gauge DIVAC. |  |  |  |  |  |
| 2.1.1.16 | The leakage rate of the vessel shall be equal or less than 0.01 m bar ltr / sec. |  |  |  |  |  |
| 2.1.2 | DEGASSING MIXTURE FOR RESIN A thin film degassing mixture for the resin / flexibiliser / colour / quartz powder premixture of suitable volume (to make final quantity of 300 Liters inclusive of thin film degasing mixture for hardner/ accelerator/ quartz filler item no. 2.1.3) ,complete with mixing, degassing and homogenizing device, vacuum rotary seal and multiple sealing along with necessary drive. |  |  |  |  |  |
| 2.1.2.1 | Thin film degassing mixture shall be provided with the following: - Sight glass with illuminating device. - Resistance thermometer for recording for temperature of premixture.Temperature shall be digitally shown on the monitor. - Heating system by heat transfer medium in forged circulation through a double wall. - Probe for recording the minimum filling level - Probe for recoding the maximum filling level. - Three electro pneumatically operated valves for resin, hardener and flexibliser. - Air admission valve with nozzle. - Gauge tube with measuring cable to display the vacuum value on the monitor. - Vacuum isolating valve. - Water cooled seperater to protect the vacuum pump. |  |  |  |  |  |
| 2.1.2.2 | Heating unit for indirect heating of resin / filler constituent using hot water in forced circulation ,designed for a supply temperature of max. 90 degree C.optimally adapted to the mixer to be heated. |  |  |  |  |  |
| 2.1.2.3 | The Heating unit mainly consist of: -1 Circulating pump with suitable feeding capacity - Filling and emptying valve -1Electric flow heater -Thermostatic Temp. control -Temperature protection -Temp. &Pressure indication - Diaphram expansion vessel |  |  |  |  |  |
| 2.1.3 | DEGASSING MIXTURE FOR HARDNER A thin film degassing mixture for the hardner / accelerator quartz filler pre mixer designed as mentioned at s.no 2.1.2 |  |  |  |  |  |
| 2.1.3.1 | Heating unit for indirect heating of hardner / accelerator as mentioned at s.no 2.1.2.2 |  |  |  |  |  |
| 2.1.4 | VACUUM PUMP A rotary vacuum pumpof suitable capacity with ultimate vacuum with gas ballast in the range of 0.01Torr complete with the following: - Complete drive - Inlet filter to protect pump - Electropneumatically operated isolation valve for vacuum regulation - Air admission valve with nozzle - vacuum measuring gauge - One additional stand by vacuum pump. |  |  |  |  |  |
| 2.1.5 | DOSING PUMP Dosing Pump Unit for feeding and dosing of the two pre- mixtures, basically consist of following: - 2 numbers dosing pump one each for resin/filler/flexibilizer/colour premxture and other for hardner /filler/accelerator , designed for abrasive media, with dossing piston and sleeve made of ceramic material and additional fitting gap cooling. |  |  |  |  |  |
| 2.1.5.1 | The dossing pump unit shall consist of following: -2 Nos. non return valve -Hydraulic synchronizing device of dossing pump drive with hydraulic cylinder to ensure at exact dossing for every stroke. - One solinoid valve for cooling water |  |  |  |  |  |
| 2.1.5.2 | The dossing pump shall be mounted directly to the out side of the mixtures in their own housing well accessible. The hydraulic system is used as basic drive for the high driving power required. |  |  |  |  |  |
| 2.1.6 | STATIC FLOW MIXER A static flow mixer for a homogenously mixing of the two pre mixtures, made of stainless steel, with heating / cooling pockets. The mixer can be used again and cleaned, even if material has cured with out intention. A static flow mixer shall have the following: - 4 Electropneumatically operated valves for inlet into static mixer as well as for circulation - Resistance thermameter for recording the cast resin temperature and releasing the casting process, if temperature is with in a preset value |  |  |  |  |  |
| 2.1.7 | AUTOMATIC CASTING VALVES |  |  |  |  |  |
| 2.1.7.1 | 16 Numbers automatic casting valve in special heatable / coolable design, programme controlled shall be provided |  |  |  |  |  |
| 2.1.8 | HEATING & COOLING UNIT |  |  |  |  |  |
| 2.1.8.1 | Heating and cooling unit for heating and cooling of static flow mixer(2.1.6) up to the casting nozzles and dosing pumps (2.1.5),using hot water in forced circulation, designed for supply temperature of max. 90degree C. and mimimum 15degree C using cooling water unit |  |  |  |  |  |
| 2.1.8.2 | Heating and cooling unit shall have the following: -1 Circulating pump with suitable high feeding capacity (capacity to mention by supplier) - 1Filling and emptying valve -1Electric flow heater with thermostatic control, temp. protction. - 1 Thermostaic temp. control - 1Temp. protection - 1 Temp. & pressure indicator - 1 Overpressuresafety unit - 1 Diaphram Expansion vessel - 1Heat exchanger - Necessary cooling water arrangement. |  |  |  |  |  |
| 2.1.9 | VACUUM PUMPING UNIT FOR CASTING TANK |  |  |  |  |  |
| 2.1.9.1 | A vacuum pumping unit of adequate capacity for operating vacuum of 1mbar and ultimate vacuum in the range of 0.01 Torr, equipped with rotary vacuum pump, Root pumps preferable of Piffer Germany make |  |  |  |  |  |
| 2.1.9.2 | The vacuum pumping system shall consist of : - Rotary Vacuum pump ( capacvity make and model to specify) - Roots pump (capacity make and model to specify) - Special condenser and seperator - All necessary vacuum valves, isolation valve etc. |  |  |  |  |  |
| 2.1.10. | ELECTRIC CONTROL CABINET: The electrical control system shall work on 415+10% - 10% 50HZ + / - 3% , 3 PhaseA.C. (4WIRE SYSTEM WITH NETURAL), power source will be provided by BHEL at a single point near the equipment as per lay out recommend by supplier. All type of cable connection circuit brakers required for connecting BHEL power supply point to different parts of machine / Control cabinets shall be responsibility of supplier. Details for requirement of grounding / earthing with required construction shall be provided by supplier. |  |  |  |  |  |
| 2.1.10.1 | Electric control cabinet shall be equiped with: - 43 IP Protection - Contactors, main switch, protective motor switches, Fuses, pushbuttons - All electrical components in the cabinet should be mounted on DIN rail. |  |  |  |  |  |
| 2.1.10.2 | The control cabinet shall also include: - 1 Siemans PLC - PC with all accessories( hard disc, cd drive, pen drive, key board, mouse, TFT Colour monitor) for communication with the control in the control cabinet |  |  |  |  |  |
| 2.1.10.3 | The PC control shall offer the following : - Programme controlled formulating and casting - different casting programme should be selectable automatically - TFT - based process supervision - Automatic pot life supervision - High flexibility of equipment at changing process parameters |  |  |  |  |  |
| 2.1.10.4 | The following parameters can easily be changed with the help of above system: - Formulation recipe - Casting programmes - casting volumes - casting speeds - Vacuums - Times - Counters |  |  |  |  |  |
| 2.1.10.5 | A modem shall be installed to ensuring optimum assistance by remote diagnosis/ data communication.The modem integrated in to control cabinet shall carryout following function: - Analysis of malfunction and their elimination - Modification and optimization of control - Remote analyses shall be carried out during gurantee period as well as the elimination of malfunctions by remote assistance |  |  |  |  |  |
| 2.1.11 | Automatic dosing device for accelerator: It shall consist of glass /VA dosing vessel of suitable volume with scale and adjustable level controller and automatic inlet and out valves |  |  |  |  |  |
| 2.1.12 | Automatic Dosing device for highly viscous colour paste for processing directly out of drum. It shall consist of pneumaticlly operatd pile press |  |  |  |  |  |
| 2.1.13 | Filler Emptying station for dust free emptying of SiO2 filler sacks and transfer of filler into the two mixers (2.1.2 & 2.1.3) |  |  |  |  |  |
| 2.1.13.1 | Filler emptying Station shall consist of: - Venting device - Vibrating device - Conveyor screw - Y pipe with automatic valves - Electronically controlled plateform balance |  |  |  |  |  |
| 2.2 | Following safety features in addition to other standard safety features should be provided on the machine: |  | VENDOR TO CONFIRM |  |  |  |
| 2.2.1 | Equipment should have adequate and reliable safety interlocks/ deviccs to avoid damage to the machine, work piece and the operator due to the malfunctionong or mistakes. Machine functions should be continuously monitored and alarm/warning indications through lights/ alarm number with messages ( on panels ) should be available . |  | VENDOR TO CONFIRM |  |  |  |
| 2.2.2 | All the pipes. Cables etc . On the machine should be well supported and protected . |  | VENDOR TO CONFIRM |  |  |  |
| 2.2.3 | All the rotating parts used on machine should be statically & dynamically balanccd to avoid unduc vibrations . |  | VENDOR TO CONFIRM |  |  |  |
| 2.2.4 | ELCB to be provided in the the cicuit to protect the equipment for Earth fault. |  |  |  |  |  |
| 2.2.5 | Emergency Switches at suitable locations as per International Norms are to be proviede. |  | VENDOR TO CONFIRM |  |  |  |
| 2.3 | ENVIRONMENTAL PERFORMANCE TO THE MACHINE : The Machine shall conform to followong factors related to environment : |  | VENDOR TO CONFIRM |  |  |  |
| 2.3.1 | Maximum noisc levcl shall be 85 dB (A) at normal load condition, this will be measured as per international standards like DIN 45635-16. Supplier to demonstrate compliancc to noise level . if so required. |  | VENDOR TO CONFIRM |  |  |  |
| 2.3.2 | If any safety / environmental protection enclosure is required it should be built in the machine by the vendor |  | VENDOR TO CONFIRM |  |  |  |
| 2.3.3 | Paint of the machine should be oil / coolant resistant and should not peel off. |  | VENDOR TO CONFIRM |  |  |  |
| 3.0. | PNEUMATIC SYSTEM |  |  |  |  |  |
| 3.1 | COMPRSSED AIR POINTS : |  |  |  |  |  |
| 3.1.1 | Compressed Air Point with manual ON/OFF Valve and flexible of suitable for work piece cleaning. FRL, automatic dram trap valve condenste receiver for pncumatic system and suitable air compressor to give 8 bar pressure to operate machine . Compressor (reputed make ) shall be supplied by the vendor.( Capacity to be specified ). |  | VENDOR TO CONFIRM |  |  |  |
| 4.0. | TOOLING : |  |  |  |  |  |
| 4.1 | Necessary tooling (supplier to specify) shall be quoted. |  | VENDOR TO CONFIRM |  |  |  |
| 5.0. | LEVELLING & ANCHORING SYSTEM |  |  |  |  |  |
| 5.1 | Complete anchoring system including foundation bolts , anchoring materials ,fixators,leveling shoes etc, shall be supplied for the Equipment . |  | VENDOR TO CONFIRM |  |  |  |
| 6.0. | TOOLS FOR ERECTON, OPERATION & MAINTENANCE : |  |  |  |  |  |
| 6.1 | Special tools and equipment required for erection of the machine shall be breought by the vendor. Neccssary tools like Torque Wrench, Spanncrs,Keys, grease guns etc for operation and maintenancc of the machine should be supplied. List of such tools should be submitted with offer . |  | VENDOR TO CONFIRM |  |  |  |
| 7.0. | ACCESSORIES : Any accessories required for proper functioning shall be offerd |  | VENDOR TO CONFIRM |  |  |  |
| 8.0. | SPARES : |  |  |  |  |  |
| 8.1 | Itemwised break - up of mechanical, hydraulic, electrical and electronic spares used on the equipment in sufficient quantity as per recommendation of Vendor for 2 years of trouble frec operation on three shifts continuous runing basis should be offered by vender. The list to include following, in addition to other recommended spares :( Unit Price of each item of spare should be offered ) |  | VENDOR TO CONFIRM |  |  |  |
|  | a) Mechanical & Hydraulic Spares : All types of Valves, all types of pressure switches/transducers, all types of filters, all types of seals. |  | VENDOR TO CONFIRM |  |  |  |
|  | b) Electrical/Electronic Spares: All types of Relays , contactors, MCB's, Proximity Switches, Push Buttons, Indicating Lamps , Semicondrctor Frses, Secial Fuses, Circuit Breakes, Main Power Switch, Encoders, MMC module, Operator"s paned woth Disply Uint, Floppy Disk Unit . I/O Cards for PLC , Power Module & Control Cards for Drives . |  | VENDOR TO CONFIRM |  |  |  |
| 8.2 | All types of spares for total machine and accessories should be available for at least ten years after supply of the machine . If machine or control is likely to become obsolete in this period , the vendor should inform BHEL sufficiently in advance amd provide drawings of parts/ details of spares & suppliers to enable BHEL to procure these in advanece, if required |  | VENDOR TO CONFIRM |  |  |  |
| 8.3 | Recommended set of spares for all attchments are to be offered with details . |  | VENDOR TO CONFIRM |  |  |  |
| 8.4 | Vendor to confirm that complete list of spares for machine and accessries along with specification / type / model, and name & address of the spare supplier shall be furnished along with documentation to be supplied with the equipment |  | VENDOR TO CONFIRM |  |  |  |
| 9.0. | DOCUMENTATION : Five sets of following documents ( hard copies) in english language should be supplied along with the machine. |  | VENDOR TO CONFIRM |  |  |  |
| 9.1 | Operating manual of machine |  | VENDOR TO CONFIRM |  |  |  |
| 9.2 | Programming manuals of machine |  | VENDOR TO CONFIRM |  |  |  |
| 9.3 | Detailed maintenance manual of machine with all drawings of machine assemblies / sub assemblies / parts including electrical/ pneumatic / hydraulic circuit diagrames. All assemblies / sub- assemblies drawings shall be supplied with the part list also. |  | VENDOR TO CONFIRM |  |  |  |
| 9.4 | Catalogues , O&M manuals of all bought out items including drawings where ever applicable. |  | VENDOR TO CONFIRM |  |  |  |
| 9.5 | Detailed specification of all rubber items and hydraulic / lube fittings. |  | VENDOR TO CONFIRM |  |  |  |
| 9.6 | PLC programm print out with comments in english |  | VENDOR TO CONFIRM |  |  |  |
| 9.7 | PLC programm and programming software on CD. |  | VENDOR TO CONFIRM |  |  |  |
| 9.8 | Complete master list of parts used in the machine shall be submitted by the vender |  | VENDOR TO CONFIRM |  |  |  |
| 10.0 | TRAINING |  |  |  |  |  |
| 10.1 | BHEL persons should be trained at supplier works for mutually agreed period in the area of a - Mechanical, Electrical and electronic maintenance for machine and other supplied equipments Operator for machine operation |  | VENDOR TO CONFIRM |  |  |  |
| 10.2 | Air- fare , boarding and lodging for the trainees shall be borne by BHEL. |  | VENDOR TO CONFIRM |  |  |  |
| 10.3 | Vender to quote for training on man/ week basis. |  | VENDOR TO CONFIRM |  |  |  |
| 10.4 | Vender should commit to orgnised and quote for tranning of electronic engineer at manufacturer works for advance features and specilised training if so required by BHEL. |  | VENDOR TO CONFIRM |  |  |  |
| 11.0 | FOUNDATION |  |  |  |  |  |
| 11.1 | vendor shall submit the preliminay layout drawing for getting BHEL"s approval within one month from th date of Letter of Intent ( LOI) / P.O. , whichever is carlier . Soil condition data will be furnished by BHEL along with the approval. Complete foundation Design including details, like Static/Dynamic load details etc . and final layout Drawings shall be submitted by the supplier within one month after geting BHEL"s approval . The Layout should consist of all requirements pertaining to complete machine and all accessories including space requirement. BHEL shall construc complete foundation for machine & supplier shall check & approve the same before etection. Vendor should arrange equipment required for the testing of foundation, if required. The vendor shall also indicate detailed specifications of grouting compound and grouting proccdure etc. for foundation bolts of the machine. |  | VENDOR TO CONFIRM |  |  |  |
| 12.0. | ERECTION & COMMISSIONING |  |  |  |  |  |
| 12.1 | Supplier to take full responsibility for carrying out the erection, start up , testing of machine, it's control system & all types of other supplied equipment, machining of test pieces etc .Service requirement like power, air & water shall be provided by BHEL at only one point to be indicated by supplier in their foundation/ layout drawings . Other requiremnts like crane, lifting tackles etc. shall also be provided by BHEL. Details of these requirements should be informed by vendor in advance. |  | VENDOR TO CONFIRM |  |  |  |
| 12.2 | Successful proving of BHEL, components by the supplier shall be considered as part of commissioning .All tests, as mentioned at clause 16 ( Machine Acceptance shall form part of the commissionong activity . |  | VENDOR TO CONFIRM |  |  |  |
| 12.3 | Test mandrel for checking run-out/taper & alignment should be supplide by the Supplier. |  | VENDOR TO CONFIRM |  |  |  |
| 12.4 | Tools, Tackles, Test Mandrels, instruments and other necessary equipment including Laser erquipment required to carry out all above cativities should be brought by the supplier. |  | VENDOR TO CONFIRM |  |  |  |
| 12.5 | Commissionong spares required for commissining of the machine within stipulated time, shall be brought by the supplier on rerturnable basis. |  | VENDOR TO CONFIRM |  |  |  |
| 12.6 | All Cover plates required for the machine and its peripherals including pits, if any, shall be supplied and installed by the vendor. The plates should be suorced form India |  | VENDOR TO CONFIRM |  |  |  |
| 12.7 | Portion, if any , of the machine, acccssories and other supplied items where paint has rubbed off or peeled during transit or erection should be repainted and merged with the original surrounding paint by the vendor, For this purpose, the vendoe should supply sufficient quantity of touch-up paint of various colours of paint used. |  | VENDOR TO CONFIRM |  |  |  |
| 12.8 | Schedule of Erection smd Commissioning shall be submited with the offer. |  | VENDOR TO CONFIRM |  |  |  |
| 12.9 | Charges, duration terms & conditions for E&C should be furnished in detail separatcly by vendor along with offer. |  | VENDOR TO CONFIRM |  |  |  |
| 13.0. | ACCURACY TESTS: |  |  |  |  |  |
| 13.1 | GEOMETRICAL ACCURACIES : |  |  |  |  |  |
| 13.1.1 | Geometrical Accuracies Tests shall be in accordance with standard chart or applicable standard. Detailed Test charts for the same, clearly showing the accuracies to be achieved on the machine, shall also be submitted with the offer. |  | VENDOR TO CONFIRM |  |  |  |
| 13.1.2 | All the cbove accuraies to be demonstrated to BEHL engineers during pre-acceptancc tests at Suppliers works and during Erection & Commissioning at BHEL, works. |  | VENDOR TO CONFIRM |  |  |  |
| 14.0. | AMBIENT CONDITIONS & THERMAL STABILTY |  |  |  |  |  |
| 14.1 | Total machine including PLC system and all supplied items should work trouble free and efficiently under following operating conditions and should give specified accuracies.Power Supply: Voltage : 415 V +/-10%, Frequency : 50 Hz +/-1.5Hz .No . of phases = 4 wire system Ambient conditions : Temperature = 5 to 50 degree Celsius Relative Humidity = 95% max .(Vendor to confirm that machine is suitable for above and details of provisions on the machine for the same are to be furnished by Vendor ) |  | VENDOR TO CONFIRM |  |  |  |
| 14.2 | Weather conditions are tropical, Atmosphere may be dust laden during some part of the year. Machine shall be kept in the normal shop floor condition. Max.Temperature variation is up to 25 deg Celsius in 24 hours.( Vendor to confirm that machine is suitable for above and details of provisions on the machine for the same are to be furnished by Vendor |  | VENDOR TO CONFIRM |  |  |  |
| 14.3 | Thermal.Stability of the conplete machine keeping in view specifed Ambient Conditionds and accuracy requirements of BHEL components and trouble free operation of the machine should be ensured by vendor . ( Vendor to confirm that machine is suitable for above and details of provisions on the machine for the same should be furnished by Vendor ) |  | VENDOR TO CONFIRM |  |  |  |
| 14.4 | The machine , including attachments and accessories, should be suitable for 24 Hrs. Continuous operation to its full capacity for 24 Hrs. a day and 7 days a week throughout. Vendor to ensure and confirm the same . |  | VENDOR TO CONFIRM |  |  |  |
| 15.0. | PROVE OUT OF BHEL COMPONENTS: |  |  |  |  |  |
| 15.1 | Complete prove out of transformer coils shall be done by Vendor at BHEL works to the specified design accuracy. Materilal for the prove out components shall be provided by BEHL. Vendor shall be fully responsible for prove out of transformer coil as per drawing and other requirements specified by BHEL to the full satisfaction of BHEL. Clarifications, if any required by vendor, regarding accuracy requirements of the prove out transformer coil, whether specified or not, should be discussed and cleared by vendor during initial technical discussions. |  | VENDOR TO CONFIRM |  |  |  |
| 15.2 | Vendor shall be respnsible for any deviation/ rejecton in prove out transformer coil due to wrong opertion or malfuctioning of the machine during prove out operation and also for the delay in operation due to improper recommended tooling etc. The cost of such deviation / rejection, if any , shall be refunded by the vendor to BHEL |  | VENDOR TO CONFIRM |  |  |  |
| 16.0. | MACHINE ACCEPTANCE: ( Tests / Acctivities TO be performed by Vendor ) |  | VENDOR TO CONFIRM |  |  |  |
| 16.1 | Tests/Activities should be carrid out at supplier's works on the machine before dispatch : |  | VENDOR TO CONFIRM |  |  |  |
| 16.1.1 | Geometrical Accuracy Tests as per test chart . |  | VENDOR TO CONFIRM |  |  |  |
| 16.1.2 | The machine should be tested for continuous runing of 8hrs. If any break down occurs during this test, the test should be repcated for 8 hrs from that time. |  | VENDOR TO CONFIRM |  |  |  |
| 16.1.3 | Demonstration of all features of the machine , PLC system and all Acccssories. |  | VENDOR TO CONFIRM |  |  |  |
| 16.2 | Test to be carried out at BHEL works while commissioning the machine : |  |  |  |  |  |
| 16.2.1 | Geometrical Accuracy Tests as per test chart . |  | VENDOR TO CONFIRM |  |  |  |
| 16.2.2 | The plant should be tested for continuous running of 24 hrs. If any break down occurs during this test, the should be repeated for 24hrs from that time . |  | VENDOR TO CONFIRM |  |  |  |
| 16.2.4 | Demonstration of all features of the plant & all accessories to the satisfaction of BHEL for their efficient and effective use. |  | VENDOR TO CONFIRM |  |  |  |
| 16.2.5 | Demonstration by actual use of all supplied attachments and accessorice to their full capacity . |  | VENDOR TO CONFIRM |  |  |  |
| 16.2.6 | Job prove out. Minimum three batch trial will be taken as per BHEL specifications. |  | VENDOR TO CONFIRM |  |  |  |
| 16.2.7 | Two weeks supervision of independent operation of plant by BHEL after job prove out . |  | VENDOR TO CONFIRM |  |  |  |
| 16.2.8 | Training of BHEL plant operators in operation of complete plant & accessories etc.by the supllier's experts/ engineers during commissioning at BHEL works . |  | VENDOR TO CONFIRM |  |  |  |
| 17.0. | PACKING : |  |  |  |  |  |
| 17.1 | Standard & rigid packing for all items of complete machine, PLC System, all Accessories and other supplied items to avoid any damage/ loss in transit. When machine is despatched in containers, all small loose items shall be suitably packed in boxes |  | VENDOR TO CONFIRM |  |  |  |
| 18.0. | GUARANTEE : |  |  |  |  |  |
| 18.1 | 24 months from the date of acceptance of the machine. |  | VENDOR TO CONFIRM |  |  |  |
| 19.0. | GENERAL : The vendor should submit the following information : |  |  |  |  |  |
| 19.1 | Plant model |  | VENDOR TO CONFIRM |  |  |  |
| 19.2 | Total connected load ( KVA ) : |  | VENDOR TO CONFIRM |  |  |  |
| 19.3 | Floor area requred ( Length, Width, Height ) for complete plant & accessoeies. |  | VENDOR TO CONFIRM |  |  |  |
| 19.4 | Painting of plant / Electrical panels: RAL 6011 Apple green ( Polyurethane paint) |  |  |  |  |  |
| 19.5 | Weight of heaviest part of the plant |  |  |  |  |  |
| 19.6 | Weight of the heaviest assembly / sub assembly of the plant |  |  |  |  |  |
| 19.7 | Dimensions of largest part / sub assembly / assembly of the plant. |  |  |  |  |  |
| 19.8 | Detailed catalogues, sketch/ photograph of the plant and accessories/ attachments should be submitted with the offer. |  |  |  |  |  |
| 19.9 | Hydraulic, pneumatic and oil piping should be preferably matellic except places where flexible piping are eccential. All the pipes required for the same shall be included in the standard scope of the plant. |  |  |  |  |  |
| 20.0 | QUALIFYING CONDITIONS |  |  |  |  |  |
| 20.1 | Only those vender, who have supplied and commissioned atleast two plants for similar application and capacity in the past five years and sutch plant is presently working satisfactorily for more than one year ( more than six month if supplied to BHEL) after commissioning should quote. The following information is to be submitted by the vender about the companies where similar plants have been supplied. This is required from all the venders for qualifications of their offers. |  |  |  |  |  |
|  | 1. Name of the customer/ company where similar plant/ machine is installed |  |  |  |  |  |
|  | 2. Complete postal address of the customer |  |  |  |  |  |
|  | 3. Year of commissioning |  |  |  |  |  |
|  | 4. Application for which the machine is supplied with details of accuracies achieved on the job |  |  |  |  |  |
|  | 5. Name and designation of the contact person of the customer |  |  |  |  |  |
|  | 6. Phone , Fax No. and e.mail address of the contact person of the customer |  |  |  |  |  |
|  | 7. Performance certificate from the customer regarding satisfactry performance of the plant/ machine supplied to them |  |  |  |  |  |

2.INSTRUCTIONS FOR SUBMISSION OF TENDER

- The offer against the enquiry shall be submitted in two parts.  The first part called Techno-commercial bid i.e. Part - A shall be un-priced and will be submitted in line with given details in commercial terms and conditions .  The second part called Price-Bid  i.e. Part-B, in this you will have to mention the basic prices, taxes duties etc for basic equipment & Standard / optional accessories if any.

- The vendors shall submit Part-A and Part-B of their bids in two different sealed envelopes duly super-scribing Enquiry No. and Part No. of the bid.  These two parts of the bid shall be kept together in a third envelope super-scribing Enquiry No., due date and time of opening as indicated in the tender document.

- Part-A of the bid alone shall be opened on the due date in presence of the tenderers in the first instance.   This part of the bid shall be examined in detail.  Part B of the bid shall be opened only for such of the tenderers whose Techno-Commercial offers are found acceptable by BHEL.  Such tenderers shall be advised the date and time of opening of Part B i.e. price part of their bids.  Tenderers must carefully go through all the instructions and conditions before filling in Part B of the bid.  Any type of mistake in submission of the bids will make the tender liable for rejection.

- The bids shall be submitted only in the format given in the tender documents. The tenderer is  required to sign and stamp each and every page of the tender document clearly bring out deviations, if any.

- Tenders not complying with the above and other requirements of the tender shall be liable for rejection.  Tenderers are advised to read all the documents carefully before submission of the bids and ensure the fulfillments of all the technical and commercial parameters.

- No over-writing in any Part-A or Part-B is permitted.  Corrections in the bid should normally be avoided. However, when these become essential, bidder shall score out the incorrect information and put the correct information duly endorsing each correction with his signatures.

- Tender Cost (if any) is to submitted in the Form of  DD in favour of BHEL-Jhansi  payable at

Jhansi along with Techno –Commercial offer. This DD should not be kept in Sealed Price Bid.

3.COMMERCIAL TERMS & CONDITIONS FOR PURCHASE ENQUIRY

- Offer is invited in two bids, outer cover, inner cover-I (Techno-Commercial bid) and inner cover II (Price-Bid) must bear our tender no., due date and bidders name.  Offers received in single bid will be bypassed.

- The offer should reach this office on or before 13.15 hrs on the due date.  The tender can also be dropped in the Tender Box kept at Administrative Building Gate.  The tenders will be opened after 1400 hrs on the due date in presence of all bidders who will be present.

- Offer should be free from overwriting or erasures.  Corrections or additions, if any should be attested.

- No enhancement of rate for whatever cause is allowed once the offer is accepted and the order is placed.  Withdrawal from the quotation after it is accepted or failure to make the supply within the scheduled period will entail cancellation of order, forfeiture of earnest money if any and/or imposition of risk & cost purchase.

- Material offered should be of required quality and conforming strictly to our specifications / drawings or to the specifications in the drawings.  Any deviation must be clearly stated in the offer with particular clause /detail in our specification / drawing for which deviation is quoted.  All required documents/details  e.g. QA Plan, type TCs, drawings & data sheet etc. must be submitted along with the offer.

- Irrespective of whether it is mentioned in our specification, all machines / equipments are required to be calibrated in metric units and certificates must be issued where applicable.

- Work test  certificates / inspection certificates/guarantee certificate/ compliance certificate, etc. will be required to be submitted along with supplies in case an order is placed.

- Required Delivery  time for affecting supplies should be indicated in the offer.

- Failure to supply by the time specified on the order will make the supplier liable to pay unconditional penalty of ½% of the prices of goods in arrear per week subject to a maximum of 10% for Indian suppliers and max 5% for Foreign suppliers of the order value unless agreed otherwise.

- The quantities for each item to be purchased may vary from the tendered quantity according to our actual requirement at the time of placing orders.

- Bharat Heavy Electricals Ltd does not bind itself to accept the lowest of any tender, but reserves to itself the right to accept or reject all or a part of any tender at its discretion following BHEL Purchase Policy & Government guidelines.

- In all cases of disputes, the matter shall be referred for arbitration.  The arbitrator  shall be nominated by the Unit Head, Bharat Heavy Electricals Ltd., Jhansi or any other officer nominated by him.  The award of Arbitrator shall be final and binding on both the parties.

- In case of any suit or other legal proceedings under or relating to this tender, the courts at Jhansi shall have full jurisdiction.

- Total delivered cost at BHEL Jhansi Railway siding/Works shall be the criterion for deciding the tender.

- Unless otherwise specified, offer should be on ex-works basis (for Indian Suppliers) with freight & insurance to be borne by BHEL.

- Your quotation against this enquiry must be valid for ordering up to minimum of 120 days from the date of tender opening.

- ADDITIONAL FOR INDIAN SUPPLIERS:

I)        If any material is booked to selves or if it is not consigned to BHEL Jhansi, any warfage or

demurrage occurring from these shall be to the account of supplier.  In case of road transport,

material shall be booked only through our rate contract transporters mentioned in the purchase

order.

- Where there is provision for payment of Sales Tax extra, it will be paid only if the registration number both under State Sales Tax and Central Sales Tax are specifically mentioned on the bill/invoice.  Your Sales Tax Registration number should be mentioned in the offer positively.

- Excise Duty Gate Pass will be required in original.  Applicable taxes & duties must be included in the offer.

- ADDITIONAL FOR FOREIGN SUPPLIERS:

i)	For documentation and correspondence language shall be English only.

- Rates quoted should be on FOB basis.  All despatches are to be made by sea  by IPBC conference vessels only unless otherwise stated in purchase order.

- Agency commission, if any, will be paid out of the FOB value after receipt & acceptance of material at BHEL Jhansi in Indian Rupees.  TT Buying exchange rate prevailing on the tender opening date or the letter of credit date or date of purchase order whichever is lowest will be considered for calculating the agency commission.

- The rates quoted should be inclusive of charges of packing and delivery of material up to the port of shipment on FOB terms.  The material is required to be packed suitably in all respects considering the peculiarity of the material for normal transport by Sea/air and subsequently by Rail/Road in India suitably protected against the effect of tropical salt laden atmosphere in the event of consignment being delayed at Indian Ports before outward clearance.  You should also furnish details of shipping specification, sizes, volume, weight, number of cases etc. When articles are packed with packaging material of plant origin, phytosanitary certificate will be required.

- Our standard terms of payment is 100% payment through bank against despatch documents or 100% through sight irrevocable & unconfirmed letter of credit.

19.   Security deposit (SD) cum Performance Bank Guarantee  (PBG) for 10% basic value of the order (excluding taxes &

duties) for  execution of the purchase order (PO) & satisfactory Performance of the equipment before release of  PO

will be required to submit which will be valid from the date of issue of LOI to 24 months after acceptance of material by

BHEL with a further claim period of 3 months. PO will be issued only after receipt of SD cum PBG.

20. For fully imported equipments / plant/ machinery (no part or sub-assembly of Indian origin) :

a. No SD cum PBG will be applicable for basic order value up to Rs. 25.0 Lacs on foreign supplier.

b. No SD cum PBG will be applicable for basic order value up to Rs. 10.0 Lacs on Indian stockist / agent of

foreign manufacturer.

21.   B.G. shall be from Scheduled Banks/Public Financial Institutions as defined in the Companies Act.

22. The equipment shall be guaranteed for 24 months from the date of acceptance by BHEL irrespective of date of supply,

erection and commissioning.

23. Our standard terms of payment (for Indian Suppliers) is 80% of P.O. value be paid after 60 days of acceptance of the

equipment at BHEL Jhansi works and balance 20% after successful commissioning & handing over of the equipment

(where applicable).

24. Amount of Service Tax / Education Cess / any other Govt. duty ( as applicable on the date of offer) for erection or any

other activities associated with the scope of work in the tender must be indicated in Techno-commercial bid of the offer.

25 . Blank Price Bid (After erasing prices) must be submitted along with Techno commercial bid.

26.  If any goods are rejected, BHEL shall be at liberty to take action as per following at the

risk and cost of supplier:-

    - Allow the supplier, wherever mutually agreed, to rectify the rejected goods at BHEL's works within reasonable time as fixed by BHEL.

    - Allow the supplier to make free replacement within a specified period, rejected goods can be lifted by the supplier thereafter.

OR

Allow the supplier to refund the full amount paid to him by BHEL by a Demand Draft  before lifting the rejected goods.  Fresh replacement shall be regularized as per terms & conditions of the original purchase order.

OR

Take alternative  procurement action from elsewhere and recover the difference in cost, if any incurred by BHEL in this regard from the supplier.  The supplier shall not be entitled to any gain on purchase.

    - Terminate the contract either in part or in whole at the  discretion of BHEL and recover the loss, if any, from the supplier.

27. For any clarification on our tender terms and conditions please contact Sr. Purchase Officer (Capital Items)

before due date.

28. Inspection may be done at supplier works by BHEL.

## 4. TECHNO COMMERCIAL BID  PART-A

( Supplier should fill up this bid by putting the tick in accepted / non accepted column)

SUPPLIER’S NAME & ADDRESS

1.          Offered Equipment with complete technical details  :                      As per attached details

2.           Packing & Forwarding Charges, FOB charges (if any)                                                               ….%

3.   	Present rate of Excise duty (if any)         		                                                              ….%

4.     	Present rate of sales tax			 		                                                   %

(UPTT/CST, Not applicable in case of Foreign suppliers)

5.	Delivery period in  No. of  days from  date of LOI / PO                                                                                                                    6.	Taxes and duties shall be extra as applicable on 		              Accepted / Not accepted

the date of actual delivery.

7.	Penalty clause as well as other conditions given			Accepted / Not accepted

in general Terms & conditions

and Standard Enquiry terms & conditions are

acceptable to us.

8.           Rates quoted in the price bid are on Ex Works / F.O.R. 		---------------------------------

Jhansi .basis.

9.	Validity of the rates quoted is 120 days from the 			Accepted / Not accepted

date of opening of price bid.

10.          Security deposit (SD) cum Performance Bank Guarantee                      Accepted / Not accepted

(PBG) for 10% basic value of the order (excluding taxes & duties)

for  execution of the purchase order (PO) & satisfactory

Performance of the equipment up to Guarantee period

of 24 months  before release of PO will be submitted.

11.        The equipment shall be guaranteed for 24 months from                            Accepted / Not accepted

the date of acceptance by BHEL irrespective of date of

supply, installation.

Note :- If any of the above conditions are not accepted

the offer shall be liable for rejection.