BHEL - JHANSI

TENDER DOCUMENT FOR 15 TON VERTICAL  WINDING MACHINE, Qty -3 Nos.

Enquiry No. E7160043, DUE ON   06.02.07

C  O  N  T  E  N  T  S

Page No.

1.         Item Description & Technical Details                                         2-15

2.	Instructions for submission of Tender		                         16

3.	Commercial Terms & Conditions		                                 17-19

4.	Techno-Commercial Bid          		                                    20

( A.K.Banerjee  )		     ( P.K.Pathak )                                  ( Y K Verma )

DGM(MM)	                           Sr. PO (MM)                                      Sr. PO (MM)

1. Item Description & Technical Details :

| SPECIFICATION CUM COMPLIANCE CERTIFICATE OF 15 TON VERTICAL WINDING MACHINE |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | CONTACT PERSON: |  | Specification No. | 6158R1 |  |  |
|  |  |  | Item No. | MT-1/3/1409 to MT-1/3/1411 |  |  |
|  |  |  | Quantity | 3 NOS. |  |  |
| Note : |  |  |  |  |  |  |
|  | 1. Vendor must submit complete information against clause No. 20 ( Qualifying condition). The offer meeting this clause would only be processed. |  |  |  |  |  |
|  | 2. The " offered" Column and where applicable, the " Deviations" & " Remarks " Column of this format shall be filled in by the Vender and submitted along with the offer. Inadequate / incomplete, ambiguous, or unsustainable information against any of the clauses of the specification / requirements shall be treated as non - compliance. |  |  |  |  |  |
|  | 3. The offer and all documents enclosed with the offer should be in English language only. |  |  |  |  |  |
| NAME & ADDRESS OF THE SUPPLIER: |  |  |  |  |  |  |
| TELEPHONE NOS. : |  |  |  |  |  |  |
| FAX NOS. : |  |  |  |  |  |  |
| E.MAIL ADDRESS : |  |  |  |  |  |  |
| SCOPE : SUPPLY , ERECTION & COMMISSIONING OF 15 TONNE VERTICAL WINDING MACHINE COMPILING WITH SPECIFICATIONS AS BELOW: |  |  |  |  |  |  |
| S.NO. | DESCRIPTION FOR BHEL REQUIREMENT |  | SPECIFIED / TO BE CONFORMED BY | OFFERED | DEVIATIONS | REMARKS |
| 1 | PURPOSE & WORK PIECE MATERIAL |  |  |  |  |  |
| 1.1 | 15 Tonne vertical winding machine is a special purpose machine used to make transformer coil made of paper insulated copper conductor in parallel. No. of conductors vary as per design. These conductors are wound circular on a mould mounted on face plate of machine. |  | VENDOR TO CONFIRM |  |  |  |
| 1.2 | Work piece material: Machine used paper covered continuously transposed copper conductor (CTC) incorporating a number of individual annealed copper strips insulated with a poly vinyl based enamel. copper conductors as per IS:3855 (partIII)-1978&-18-1990 and PICC to CPS AA28125. |  | VENDOR TO CONFIRM |  |  |  |
| 2.0. | SPECIFICATION: |  |  |  |  |  |
| 2.1 | MACHINE CONFIGURATION :-The 15 tonne vertical winding machine comprises of rotary table moving around vertical axes at required rpm in clockwise and anti clockwise direction . complete assembly of rotary table (face plate )moves upon & down in a pit on three vertical screws by an electrical motor and gear drive. The top portion of pit is covered by a dia diaphragms cover having variable opening .Transformer coils are made of a bunch of insulated copper conductors on the ground floor by the operator. The control cabinet and portable control panel/foot pedal switch is mounted on floor level. Copper conductors are mounted on bobbin stand and pass through a conductor tensioning arrangements to generate tension while winding, a bending tool is required for copper conductors transposition. |  | VENDOR TO CONFIRM |  |  |  |
| 2.2 | TECHNICAL SPECIFICATION OF MACHINE:- |  |  |  |  |  |
| 2.2.1 | Capacity of the M/C 15 tonnes. |  | VENDOR TO CONFIRM |  |  |  |
| 2.2.2 | Coil diameters 550-2700 mm |  | VENDOR TO CONFIRM |  |  |  |
| 2.2.3 | Coil height 3200 mm |  | VENDOR TO CONFIRM |  |  |  |
| 2.2.4 | Face plate dia of rotary table 2250 mm |  | VENDOR TO CONFIRM |  |  |  |
| 2.2.5 | Min/Max Ht.of the face plate from ground 0-3200 mm |  | VENDOR TO CONFIRM |  |  |  |
| 2.2.6 | Vertical movement of rotary table 3600 mm |  | VENDOR TO CONFIRM |  |  |  |
| 2.2.7 | Tension applicable 2000 KPat 3000 mm |  | VENDOR TO CONFIRM |  |  |  |
| 2.2.8 | RPM/ Torque reqd for winding (torque required to overcome friction is excluded.) 0 to 6.5 rpm at 1250KPm torque, 6.5 to 15 rpm at-850 KPm torque, 15 to 30 at 725KPm torque, 30 to 40 rpm at 500 KPm torque. |  | VENDOR TO CONFIRM |  |  |  |
| 2.2.9 | Aperture dia. 700-3000 mm |  |  |  |  |  |
| 2.2.10 | Acceleration of rotary table 0.47 radian/sec.2 |  | VENDOR TO CONFIRM |  |  |  |
| 2.2.11 | Retardation of rotary table 0.2 - 0.7 radian/sec.2 |  | VENDOR TO CONFIRM |  |  |  |
| 2.3 | ROTARY TABLE |  |  |  |  |  |
| 2.3.1 | Face plate dia of rotary table 2250 mm |  | VENDOR TO CONFIRM |  |  |  |
| 2.3.2 | Load carrying capacity of rotary table. 15 Tonnes. |  | VENDOR TO CONFIRM |  |  |  |
| 2.3.3 | Face plate rotary table to be provided three rows of 25 mm tapped holes of 8 Nos.in each row at PCD of 1200, 1500,1700 & 2000 mm |  | VENDOR TO CONFIRM |  |  |  |
| 2.3.4 | Drive for rotary table .(Siemens/ABBmake ). all speed control mechanism either through operator panel or foot switch, to be incorporated using digital logie in the drive -40 KW /(suitable) AC Motor with PLC control & disc brake. |  | VENDOR TO CONFIRM |  |  |  |
| 2.3.5 | Forward & reverse motion speed of rotary table using preset speed & control through foot switch. |  | VENDOR TO CONFIRM |  |  |  |
| 2.3.6 | Vertical speed of rotary table 0.72 m/Min.(Approx) |  | VENDOR TO CONFIRM |  |  |  |
| 2.3.7 | Drive for vertical movement of rotary table Suitable AC motor with gear box. |  | VENDOR TO CONFIRM |  |  |  |
| 2.3.8 | Support for vertical movement of table -it shall of one lifting stage with 3 guiding pillars on which the machine support is mounted & driven by a gear motor and 3 worm gear screw jacks. |  | VENDOR TO CONFIRM |  |  |  |
| 2.3.9 | Diaphragm aperture platform with 4 Nos radial moving plates of electrically operated. |  | VENDOR TO CONFIRM |  |  |  |
| 2.3.10 | Digital pulse encoder for rotary motion |  | VENDOR TO CONFIRM |  |  |  |
| 2.3.11 | Speed control of motor for rotary table shall be through digital drive with encoder feedback- Digital drive and PLC control of Siemens/ Allen Bradley make. |  | VENDOR TO CONFIRM |  |  |  |
| 2.3.12 | Type of gear box- worm gear box |  | VENDOR TO CONFIRM |  |  |  |
| 2.3.13 | Make - Redicon/ standard make |  | VENDOR TO CONFIRM |  |  |  |
| 2.3.14 | Lubrication system Centralised lubrication |  | VENDOR TO CONFIRM |  |  |  |
| 2.4 | CONDUCTOR TENSIONING SYSTEM |  |  |  |  |  |
|  | The insulated copper conductors are passed through a tensioning device which consist of 2 Face - ground steel brake plate pressed together by the brake clamp. Clamping force is accomplished by means of a pressure bellow pneumatically operated. A finger guide in front of the tensioning device gathers all conductors in to a bundle. The tensioning unit is mounted over a horizontal beam fixed on the ground floor. |  | VENDOR TO CONFIRM |  |  |  |
| 2.4.1 | Size of brake plates 600X 500 mm |  | VENDOR TO CONFIRM |  |  |  |
| 2.4.2 | Diaphragm / pneumatic cylinders 2 Nos. |  | VENDOR TO CONFIRM |  |  |  |
| 2.4.3 | Air working pressure 4 bar |  | VENDOR TO CONFIRM |  |  |  |
| 2.4.4 | Maximum load on brake plates 5 Tonnes |  | VENDOR TO CONFIRM |  |  |  |
| 2.4.5 | Brake system shall be sliding manually over horizontal beam by rack and pinion arrangement |  | VENDOR TO CONFIRM |  |  |  |
| 2.4.6 | Height of horizontal beam above floor level 800 mm |  | VENDOR TO CONFIRM |  |  |  |
| 2.4.7 | Maximum N. of copper conductor used for winding 16 Nos. |  | VENDOR TO CONFIRM |  |  |  |
| 2.5 | ADAPTOR PLATE ( to be mounted on rotary table for making transformer coil. Drawing will be supplied by BHEL) |  |  |  |  |  |
| 2.5.1 | Out side dia of fabricated structural steel adaptor plate without extension attachment 750 mm |  | VENDOR TO CONFIRM |  |  |  |
| 2.5.2 | Out side dia of fabricated structural steel adaptor plate (shall be 1200 mm) with four Nos channel structure extension welded on the outer periphery at 90 degree - with T slot size of 44 * 27 mm and the adaptor shall be suitable to bear load of the 15 T coil along with the mould weight. |  | VENDOR TO CONFIRM |  |  |  |
| 2.5.3 | Height 500 mm |  | VENDOR TO CONFIRM |  |  |  |
| 2.5.4 | Centre through hole at top and bottom 75 mm dia |  | VENDOR TO CONFIRM |  |  |  |
| 2.6 | BENDING TOOL( FOR TRANSPOSITION OF COPPER CONDUCTOR) |  |  |  |  |  |
|  | Bending tool is required for bending copper conductors for making cross over bends. The bending operation shall be performed with minimal stress on the insulation material. It is a hydraulic operated tool. |  | VENDOR TO CONFIRM |  |  |  |
| 2.6.1 | Hydraulic pressure required 300 bar |  | VENDOR TO CONFIRM |  |  |  |
| 2.6.2 | Copper conductor size: 1.5 to 4mm thick,& 6 mm to 18 mm wide |  | VENDOR TO CONFIRM |  |  |  |
| 2.6.3 | Tool support swing type spring balancer |  | VENDOR TO CONFIRM |  |  |  |
| 2.6.4 | Operation of bend tool - Hydraulic and manual |  | VENDOR TO CONFIRM |  |  |  |
| 2.6.5 | Hydraulic pressure generated by Hydro - pneumatic magnifier - 1 Nos. of 300 bar capacity |  | VENDOR TO CONFIRM |  |  |  |
| 2.6.6 | Maximum stroke range of tool 20 mm |  | VENDOR TO CONFIRM |  |  |  |
| 2.7 | APERTURE |  |  |  |  |  |
| 2.7.1 | Motor for aperture drive ( Bhart bijlee, Simens , ABB make) - 1.5 HP , 415 volt 3 phase 50 HZ , 950 RPM / Suitable |  | VENDOR TO CONFIRM |  |  |  |
| 2.7.2 | Gear box - 1/960 ratio / suitable |  | VENDOR TO CONFIRM |  |  |  |
| 2.7.3 | Type of gear box - Double worm reduction |  | VENDOR TO CONFIRM |  |  |  |
| 2.7.4 | Aperture speed - 1000 mm / Min. |  | VENDOR TO CONFIRM |  |  |  |
| 2.7.5 | Type of aperture - 4 segment construction |  | VENDOR TO CONFIRM |  |  |  |
| 2.7.6 | Minimum/ maximum opening of aperture - 500mm / 3000 mm |  | VENDOR TO CONFIRM |  |  |  |
| 2.8 | BOBBIN & WIRE DRUM STAND |  |  |  |  |  |
|  | One No bobbin & wire drum stand made of framed structure steel with wood bobbin.Max length 400 mm long suitable for accommodating 4x2 bobbin of 700 mm dia (each 250 kg weight) or with suitable brackets for mounting three bobbins of 500 mm dia x 1000 mm with(1ton each) complete with necessary mounting shafts, rollers with bearing, rails etc. |  | VENDOR TO CONFIRM |  |  |  |
| 2.9 | CONSTRUCTION: |  |  |  |  |  |
| 2.9.1 | Vendor to furnish details of material, hardness & constructional detail material of rotary table structure -HT steel plate as is 2062 / cast iron grade 26. material of vertical lead screws - forged steel as per IS 2004 material of other components/structure-HT steel plates as per IS 2062. |  | VENDOR TO CONFIRM |  |  |  |
| 2.9.2 | Video images on cd including hard copy explaining the features Literature with photographs, drawings explaining the technical features should be enclosed with the offer. |  | VENDOR TO CONFIRM |  |  |  |
| 2.10 | OPERATION AND CONTROL SYSTEM: |  |  |  |  |  |
| 2.10.1 | OPERATORS PANEL: |  |  |  |  |  |
| 2.10.1.1 | All switches on the operators panel including that for table rotation should be within reach of operator of average height (Indian) for convenient, efficient & safe operation. All display / indications should also be conveniently placed accordingly. Layout showing complete details of the panel should be submitted. All the control(MMI)operations to be provided through operator panel interface to the drive & PCL through a suitable communication link |  | VENDOR TO CONFIRM |  |  |  |
| 2.10.1.2 | An auxiliary pendant, which can be taken to the table to the for job setting and similar other purposes, should be provided. Indication of RPM,Coil / Disc no.etc.should be indicated on the operator panel. |  | VENDOR TO CONFIRM |  |  |  |
| 2.10.1.3 | Foot pedal switch suitable for keeping it or down side for left foot or right foot operation with two switches one in the front&one in the side. Front switch for operating of machine at preset RPM &side switch to over -ride speed & to reach max.speed Afoot switch shall be dead man stop. |  | VENDOR TO CONFIRM |  |  |  |
| 2.11 | MANUAL CONTROL: |  |  |  |  |  |
| 2.11.1 | Complete manual control of machine with required switches / Keys should be provided on operators panel for selection of required axis direction Diagram/ Sketches for switch /keys shall be provided through operator panel interface on operators pendant. |  |  |  |  |  |
| 2.12 | HYDRAULIC SYSTEM: |  |  |  |  |  |
| 2.12.1 | The system should be centralised. Hydraulic tank shall be located near the bending tool. |  | VENDOR TO CONFIRM |  |  |  |
| 2.12.2 | Make rexroth/ Vickers sperry or equivalent from a reputed manufacturer. (Details to be submitted) |  | VENDOR TO CONFIRM |  |  |  |
| 2.12.3 | First filling of all required oils & grease etc. |  | VENDOR TO CONFIRM |  |  |  |
| 2.13 | ELECTRICAL SYSTEM |  |  |  |  |  |
| 2.13.1 | 415 +/- 10%, 50hz, 3 Phase A.C. supply. Power supply source will be provided by BHEL at a single point near the machine, as a layout recommendation by vender. All types of cables, connections, circuit breakers etc. required for connecting BHEL's power supply point to different parts of the machine/ control cabinets, shall be the responsibility of vender. |  | VENDOR TO CONFIRM |  |  |  |
| 2.13.2 | CONTROL PANEL:- A floor mounted sheet metal control panel to be provided complete with MCB for incoming power supply. PLC for turn control system, all contactors, relays, starters, timer, singe phasing preventor, MCB's and HRC fuses, Step down transformer for control voltage etc., indicating lamps for power supply. Status monitoring and fault enunciation with alarm etc. with desire interlocks with electrical safety provisions, indications and operation of all electro pneumatic valves. |  | VENDOR TO CONFIRM |  |  |  |
| 2.13.3 | PLC CONTROL SYSTEM :- Control system should ensure automatic winding of complete coil. A touch panel shall be used for general operation of machine and programming. |  | VENDOR TO CONFIRM |  |  |  |
| 2.13.3.1 | Programme shall be capable to complete winding programme of about 350 disk of coils with 20 stops and should be done by means of operator panel. |  | VENDOR TO CONFIRM |  |  |  |
| 2.13.3.2 | It should be possible to programme the data on the control panel. Data like stops, speed, wire tension on the break etc.shall be able to programme in the input field. |  | VENDOR TO CONFIRM |  |  |  |
| 2.13.3.3 | Parameters like actual no. of turns, graduation of the coil, next stop, operator information, error messages etc. shall be able to programme in the input field. |  | VENDOR TO CONFIRM |  |  |  |
| 2.13.3.4 | Complete operation shall be controlled through a PLC at siemens/ Allen Bradley make. |  | VENDOR TO CONFIRM |  |  |  |
| 2.13.4 | TROPICALISATION: All electrical/ electronic equipments shall be tropicalised. |  | VENDOR TO CONFIRM |  |  |  |
| 2.13.5 | All electrical & electronic control cabinets & panel should be dust and vermin proof with suitable cooling arrangement. Panel should be of reputed make. |  | VENDOR TO CONFIRM |  |  |  |
| 2.13.6 | All electrical components in the cabinets should be mounted on DIN Rail. |  | VENDOR TO CONFIRM |  |  |  |
| 2.13.7 | All electrical and electronic panel including operator's panel should be provided with fluorescent/ CFL lamp for sufficient illumination and power receptacles of 220 volts. 5/15 Amp. A.C. All adapters / receptacles should have compatibility with Indian equivalents. |  | VENDOR TO CONFIRM |  |  |  |
| 2.13.8 | Motors shall confirm to IEC of Indian standards. |  | VENDOR TO CONFIRM |  |  |  |
| 2.13.9 | Vender should ensure the proper earthing for the machine and its peripherals. |  | VENDOR TO CONFIRM |  |  |  |
| 2.13.10 | In- cycle hour counter with reset facility is to be included in the offer |  | VENDOR TO CONFIRM |  |  |  |
| 2.13.11 | Drive programming, optimization and programme software is to be provided with a interphase cable for LAPTOP arrangement. |  | VENDOR TO CONFIRM |  |  |  |
| 2.14 | SAFETY ARRANGEMENTS: |  |  |  |  |  |
| 2.14.1 | Following safety features in addition to other standard safety features should be provided on the machine: |  | VENDOR TO CONFIRM |  |  |  |
| 2.14.2 | Machine should have adequate and reliable safety interlocks/ devices to avoid damage to the machine, work piece and the operator due to the malfunctioning or mistakes. Machine functions should be continuously monitored and alarm/warning indications through lights/ alarm number with messages ( on panels ) should be available . |  | VENDOR TO CONFIRM |  |  |  |
| 2.14.3 | All the pipes. Cables etc . On the machine should be well supported and protected . |  | VENDOR TO CONFIRM |  |  |  |
| 2.14.4 | All the rotating parts used on machine should be statically & dynamically balanced to avoid undue vibrations . |  | VENDOR TO CONFIRM |  |  |  |
| 2.14.5 | ELCB to be provided in the the circuit to protect the equipment for Earth fault. |  |  |  |  |  |
| 2.14.6 | Emergency Switches at suitable locations as per International Norms are to be provided. |  | VENDOR TO CONFIRM |  |  |  |
| 2.15 | ENVIRONMENTAL PERFORMANCE TO THE MACHINE : |  |  |  |  |  |
|  | The Machine shall conform to following factors related to environment : |  | VENDOR TO CONFIRM |  |  |  |
| 2.15.1 | Maximum noise level shall be 85 dB (A) at normal load condition, this will be measured as per international standards like DIN 45635-16. Supplier to demonstrate compliance to noise level . if so required. |  | VENDOR TO CONFIRM |  |  |  |
| 2.15.2 | If any safety / environmental protection enclosure is required it should be built in the machine by the vendor |  | VENDOR TO CONFIRM |  |  |  |
| 2.15.3 | Paint of the machine should be oil / coolant resistant and should not peel off. |  | VENDOR TO CONFIRM |  |  |  |
| 3.0. | PNEUMATIC SYSTEM |  |  |  |  |  |
| 3.1 | COMPRESSED AIR POINTS : |  |  |  |  |  |
| 3.1.1 | Compressed Air Point with manual ON/OFF Valve and flexible of suitable for work piece cleaning. FRL, automatic dram trap valve condensate receiver for pneumatic system and suitable air compressor to give 8 bar pressure to operate machine . Compressor (reputed make ) shall be supplied by the vendor. (Capacity to be specified ). |  | VENDOR TO CONFIRM |  |  |  |
| 4.0. | TOOLING : |  |  |  |  |  |
| 4.1 | Bending tool :- As per Para 2.6 |  | VENDOR TO CONFIRM |  |  |  |
| 5.0. | LEVELLING & ANCHORING SYSTEM |  |  |  |  |  |
| 5.1 | Complete anchoring system including foundation bolts , anchoring materials ,fixators,leveling shoes etc, shall be supplied for the Machine . |  | VENDOR TO CONFIRM |  |  |  |
| 6.0. | TOOLS FOR ERECTION, OPERATION & MAINTENANCE : |  |  |  |  |  |
| 6.1 | Special tools and equipment required for erection of the machine shall be brought by the vendor. Necessary tools like Torque Wrench, Spanners,Keys, grease guns etc for operation and maintenance of the machine should be supplied. List of such tools should be submitted with offer . |  | VENDOR TO CONFIRM |  |  |  |
| 7.0. | ACCESSORIES : Conductor tensioning system, adaptor plate, bending tool , & hydraulic system as described in Para 2.4,2.6 &2.12 to be provided. |  | VENDOR TO CONFIRM |  |  |  |
| 8.0. | SPARES : |  |  |  |  |  |
| 8.1 | Itemised break - up of mechanical, hydraulic, electrical and electronic spares used on the machine in sufficient quantity as per recommendation of Vendor for 2 years of trouble free operation on three shifts continuous running basis should be offered by vender. The list to include following, in addition to other recommended spares :( Unit Price of each item of spare should be offered ) |  | VENDOR TO CONFIRM |  |  |  |
|  | a) Mechanical & Hydraulic Spares : All types of Valves, all types of pressure switches/transducers, all types of filters, all types of seals. |  | VENDOR TO CONFIRM |  |  |  |
|  | b) Electrical/Electronic Spares: All types of Relays , contactors, MCB's, Proximity Switches, Push Buttons, Indicating Lamps , Semiconductor Fuses, Special Fuses, Circuit Breaks, Main Power Switch, Encoders, MAC module, Operator's paned with Display Unit, Floppy Disk Unit . I/O Cards for PLC , Power Module & Control Cards for Drives . |  | VENDOR TO CONFIRM |  |  |  |
| 8.2 | All types of spares for total machine and accessories should be available for at least ten years after supply of the machine . If machine or control is likely to become obsolete in this period , the vendor should inform BHEL sufficiently in advance and provide drawings of parts/ details of spares & suppliers to enable BHEL to procure these in advance, if required |  | VENDOR TO CONFIRM |  |  |  |
| 8.3 | Recommended set of spares for all attachments are to be offered with details . |  | VENDOR TO CONFIRM |  |  |  |
| 8.4 | Vendor to confirm that complete list of spares for machine and accessories along with specification / type / model, and name & address of the spare supplier shall be furnished along with documentation to be supplied with the machine |  | VENDOR TO CONFIRM |  |  |  |
| 9.0. | DOCUMENTATION : Five sets of following documents ( hard copies) in English language should be supplied along with the machine. |  | VENDOR TO CONFIRM |  |  |  |
| 9.1 | Operating manual of machine |  | VENDOR TO CONFIRM |  |  |  |
| 9.2 | Programming manuals of machine |  | VENDOR TO CONFIRM |  |  |  |
| 9.3 | Detailed maintenance manual of machine with all drawings of machine assemblies / sub assemblies / parts including electrical/ pneumatic / hydraulic circuit diagrams. All assemblies / sub- assemblies drawings shall be supplied with the part list also. |  | VENDOR TO CONFIRM |  |  |  |
| 9.4 | Catalogues , O&M manuals of all bought out items including drawings where ever applicable in english lenguage. |  | VENDOR TO CONFIRM |  |  |  |
| 9.5 | Detailed specification of all rubber items and hydraulic / lube fittings. |  | VENDOR TO CONFIRM |  |  |  |
| 9.6 | PLC program print out with comments in English |  | VENDOR TO CONFIRM |  |  |  |
| 9.7 | PLC program and programming software on CD. |  | VENDOR TO CONFIRM |  |  |  |
| 9.8 | Complete master list of parts used in the machine shall be submitted by the vender |  | VENDOR TO CONFIRM |  |  |  |
| 10.0 | TRAINING |  |  |  |  |  |
| 10.1 | BHEL persons should be trained at supplier works for mutually agreed period in the area of a - Mechanical, Electrical and electronic maintenance for machine and other supplied equipments b- Operator for machine operation |  | VENDOR TO CONFIRM |  |  |  |
| 10.2 | Air- fare , boarding and lodging for the trainees shall be borne by BHEL. |  | VENDOR TO CONFIRM |  |  |  |
| 10.3 | Vender to quote for training on man/ week basis. |  | VENDOR TO CONFIRM |  |  |  |
| 10.4 | Vender should commit to organised and quote for training of electronic engineer at manufacturer works for advance features and specialized training if so required by BHEL. |  | VENDOR TO CONFIRM |  |  |  |
| 11.0 | FOUNDATION |  |  |  |  |  |
| 11.1 | vendor shall submit the preliminary layout drawing for getting BHEL"s approval within one month from the date of Letter of Intent ( LOI)/P.O. , whichever is earlier . Soil condition data will be furnished by BHEL along with the approval. Complete foundation Design including details, like Static/Dynamic load details etc . and final layout Drawings shall be submitted by the supplier within one month after getting BHEL"s approval . The Layout should consist of all requirements pertaining to complete machine and all accessories including space requirement. BHEL shall construct complete foundation for machine & supplier shall check & approve the same before erection. Vendor should arrange equipment required for the testing of foundation, if required. The vendor shall also indicate detailed specifications of grouting compound and grouting procedure etc. for foundation bolts of the machine. |  | VENDOR TO CONFIRM |  |  |  |
| 12.0. | ERECTION & COMMISSIONING |  |  |  |  |  |
| 12.1 | Supplier to take full responsibility for carrying out the erection, start up , testing of machine, it's control system & all types of other supplied equipment, machining of test pieces etc .Service requirement like power, air & water shall be provided by BHEL at only one point to be indicated by supplier in their foundation/ layout drawings . Other requirements like crane, lifting tackles etc. shall also be provided by BHEL. Details of these requirements should be informed by vendor in advance. |  | VENDOR TO CONFIRM |  |  |  |
| 12.2 | Successful proving of BHEL, components by the supplier shall be considered as part of commissioning .All tests, as mentioned at clause 16 ( Machine Acceptance shall form part of the commissioning activity . |  | VENDOR TO CONFIRM |  |  |  |
| 12.3 | Test mandrel for checking run-out/taper & alignment should be supplied by the Supplier. |  | VENDOR TO CONFIRM |  |  |  |
| 12.4 | Tools, Tackles, Test Mandrels, instruments and other necessary equipment including Laser equipment required to carry out all above activities should be brought by the supplier. |  | VENDOR TO CONFIRM |  |  |  |
| 12.5 | Commissioning spares required for commissioning of the machine within stipulated time, shall be brought by the supplier on returnable basis. |  | VENDOR TO CONFIRM |  |  |  |
| 12.6 | All Cover plates required for the machine and its peripherals including pits, if any, shall be supplied and installed by the vendor. The plates should be sourced form India |  | VENDOR TO CONFIRM |  |  |  |
| 12.7 | Portion, if any , of the machine, accessories and other supplied items where paint has rubbed off or peeled during transit or erection should be repainted and merged with the original surrounding paint by the vendor, For this purpose, the vendor should supply sufficient quantity of touch-up paint of various colours of paint used. |  | VENDOR TO CONFIRM |  |  |  |
| 12.8 | Schedule of Erection and Commissioning shall be submitted with the offer. |  | VENDOR TO CONFIRM |  |  |  |
| 12.9 | Charges, duration terms & conditions for E&C should be furnished in detail separately by vendor along with offer. |  | VENDOR TO CONFIRM |  |  |  |
| 13.0. | ACCURACY TESTS: |  |  |  |  |  |
| 13.1 | GEOMETRICAL ACCURACIES : |  |  |  |  |  |
| 13.1.1 | Geometrical Accuracies Tests shall be in accordance with standard chart or applicable standard. Detailed Test charts for the same, clearly showing the accuracies to be achieved on the machine, shall also be submitted with the offer. |  | VENDOR TO CONFIRM |  |  |  |
| 13.1.2 | All the above accuracies to be demonstrated to BEHL engineers during pre-acceptance tests at Suppliers works and during Erection & Commissioning at BHEL, works. |  | VENDOR TO CONFIRM |  |  |  |
| 14.0. | AMBIENT CONDITIONS & THERMAL STABILITY |  |  |  |  |  |
| 14.1 | Total machine including PLC system and all supplied items should work trouble free and efficiently under following operating conditions and should give specified accuracies. Power Supply: Voltage : 415 V +/-10%, Frequency : 50 Hz +/-1.5Hz . No . of phases = 4 wire system Ambient conditions : Temperature = 5 to 50 degree Celsius Relative Humidity = 95% max . (Vendor to confirm that machine is suitable for above and details of provisions on the machine for the same are to be furnished by Vendor ) |  | VENDOR TO CONFIRM |  |  |  |
| 14.2 | Weather conditions are tropical, Atmosphere may be dust laden during some part of the year. Machine shall be kept in the normal shop floor condition. Max.Temperature variation is up to 25 deg Celsius in 24 hours.( Vendor to confirm that machine is suitable for above and details of provisions on the machine for the same are to be furnished by Vendor |  | VENDOR TO CONFIRM |  |  |  |
| 14.3 | Thermal. Stability of the complete machine keeping in view specified Ambient Conditions and accuracy requirements of BHEL components and trouble free operation of the machine should be ensured by vendor . ( Vendor to confirm that machine is suitable for above and details of provisions on the machine for the same should be furnished by Vendor ) |  | VENDOR TO CONFIRM |  |  |  |
| 14.4 | The machine , including attachments and accessories, should be suitable for 24 Hrs. Continuous operation to its full capacity for 24 Hrs. a day and 7 days a week throughout. Vendor to ensure and confirm the same . |  | VENDOR TO CONFIRM |  |  |  |
| 15.0. | PROVE OUT OF BHEL COMPONENTS: |  |  |  |  |  |
| 15.1 | Complete prove out of transformer coils shall be done by Vendor at BHEL works to the specified design accuracy. Material for the prove out components shall be provided by BHEL. Vendor shall be fully responsible for prove out of transformer coil as per drawing and other requirements specified by BHEL to the full satisfaction of BHEL. Clarifications, if any required by vendor, regarding accuracy requirements of the prove out transformer coil, whether specified or not, should be discussed and cleared by vendor during initial technical discussions. |  | VENDOR TO CONFIRM |  |  |  |
| 15.2 | Vendor shall be responsible for any deviation/rejection in prove out transformer coil due to wrong operation or malfunctioning of the machine during prove out operation and also for the delay in operation due to improper recommended tooling etc. The cost of such deviation / rejection, if any , shall be refunded by the vendor to BHEL |  | VENDOR TO CONFIRM |  |  |  |
| 16.0. | MACHINE ACCEPTANCE: ( Tests / Activities TO be performed by Vendor ) |  | VENDOR TO CONFIRM |  |  |  |
| 16.1 | Tests/Activities should be carried out at supplier's works on the machine before dispatch : |  | VENDOR TO CONFIRM |  |  |  |
| 16.1.1 | Geometrical Accuracy Tests as per test chart . |  | VENDOR TO CONFIRM |  |  |  |
| 16.1.2 | The machine should be tested for continuous running of 8hrs. If any break down occurs during this test, the test should be repeated for 8 hrs from that time. |  | VENDOR TO CONFIRM |  |  |  |
| 16.1.3 | Demonstration of all features of the machine , PLC system and all Accessories. |  | VENDOR TO CONFIRM |  |  |  |
| 16.2 | Test to be carried out at BHEL works while commissioning the machine : |  |  |  |  |  |
| 16.2.1 | Geometrical Accuracy Tests as per test chart . |  | VENDOR TO CONFIRM |  |  |  |
| 16.2.2 | Full load test to demonstrate the maximum power & winding / loading capacity of the machine . |  | VENDOR TO CONFIRM |  |  |  |
| 16.2.3 | The machine should be tested for continuous running of 8 hrs. If any break down occurs during this test, the should be repeated for 8 hrs from that time . |  | VENDOR TO CONFIRM |  |  |  |
| 16.2.4 | Demonstration of all features of the machine & all accessories to the satisfaction of BHEL for their efficient and effective use. |  | VENDOR TO CONFIRM |  |  |  |
| 16.2.5 | Demonstration by actual use of all supplied attachments and accessorise to their full capacity . |  | VENDOR TO CONFIRM |  |  |  |
| 16.2.6 | Job prove out. Minimum three job trial will be taken as per BHEL specifications. |  | VENDOR TO CONFIRM |  |  |  |
| 16.2.7 | Two weeks supervision of independent operation of machine by BHEL after job prove out . |  | VENDOR TO CONFIRM |  |  |  |
| 16.2.8 | Training of BHEL machine operators in operation of complete machine & accessories etc.by the supplier's experts/ engineers during commissioning at BHEL works . |  | VENDOR TO CONFIRM |  |  |  |
| 17.0. | PACKING : |  |  |  |  |  |
| 17.1 | Standard & rigid packing for all items of complete machine, PLC System, all Accessories and other supplied items to avoid any damage/ loss in transit. When machine is despatched in containers, all small loose items shall be suitably packed in boxes |  | VENDOR TO CONFIRM |  |  |  |
| 18.0. | GUARANTEE : |  |  |  |  |  |
| 18.1 | 24 months from the date of acceptance of the machine. |  | VENDOR TO CONFIRM |  |  |  |
| 19.0. | GENERAL : The vendor should submit the following information : |  |  |  |  |  |
| 19.1 | Machine Model |  | VENDOR TO CONFIRM |  |  |  |
| 19.2 | Total connected load ( KVA ) : |  | VENDOR TO CONFIRM |  |  |  |
| 19.3 | Floor area required ( Length, Width, Height ) for complete machine & accessories. |  | VENDOR TO CONFIRM |  |  |  |
| 19.4 | Painting of machine / Electrical panels: RAL 6011 Apple green ( Polyurethane paint) |  |  |  |  |  |
| 19.5 | Total weight of the machine |  |  |  |  |  |
| 19.6 | Weight of heaviest part of the machine |  |  |  |  |  |
| 19.7 | Weight of the heaviest assembly / sub assembly of the machine |  |  |  |  |  |
| 19.8 | Dimensions of largest part / sub assembly / assembly of the machine. |  |  |  |  |  |
| 19.9 | Vender to submit , along with offer, the reference list of customers where similar machines have been supplied mentioning the customer, machine model, major specification of the supplied machine, control system , year of supply etc. |  |  |  |  |  |
| 19.10 | Detailed catalogues, sketch/ photograph of the machine and accessories/ attachments should be submitted with the offer. |  |  |  |  |  |
| 19.11 | Hydraulic, pneumatic and oil piping should be preferably metallic except places where flexible piping are essential. All the pipes required for the same shall be included in the standard scope of the machine. |  |  |  |  |  |
| 20.0 | QUALIFYING CONDITIONS |  |  |  |  |  |
| 20.1 | Only those vender, who have supplied and commissioned at least one 15 Tonnes vertical transformer coil winding machine of same or higher sizes ( table dia 2500mm, vertical travel 3000mm) for similar application in the past ten years and such machine is presently working satisfactorily for more than one year ( more than six month if supplied to BHEL).after commissioning should quote. The following information is to be submitted by the vender about the companies where similar machines have been supplied. This is required from all the vendors for qualifications of their offers. |  |  |  |  |  |
|  | 1. Name of the customer/ company where similar machine is installed |  |  |  |  |  |
|  | 2. Complete postal address of the customer |  |  |  |  |  |
|  | 3. Year of commissioning |  |  |  |  |  |
|  | 4. Application for which the machine is supplied with details of accuracies achieved on the job |  |  |  |  |  |
|  | 5. Name and designation of the contact person of the customer |  |  |  |  |  |
|  | 6. Phone , Fax No. and email address of the contact person of the customer |  |  |  |  |  |
|  | 7. Performance certificate from the customer regarding satisfactory performance of the machine supplied to them |  |  |  |  |  |

2.INSTRUCTIONS FOR SUBMISSION OF TENDER

- The offer against the enquiry shall be submitted in two parts.  The first part called Techno-commercial bid i.e. Part - A shall be un-priced and will be submitted in line with given details in commercial terms and conditions .  The second part called Price-Bid  i.e. Part-B, in this you will have to mention the basic prices, taxes duties etc for basic equipment & Standard / optional accessories if any.

- The vendors shall submit Part-A and Part-B of their bids in two different sealed envelopes duly super-scribing Enquiry No. and Part No. of the bid.  These two parts of the bid shall be kept together in a third envelope super-scribing Enquiry No., due date and time of opening as indicated in the tender document.

- Part-A of the bid alone shall be opened on the due date in presence of the tenderers in the first instance.   This part of the bid shall be examined in detail.  Part B of the bid shall be opened only for such of the tenderers whose Techno-Commercial offers are found acceptable by BHEL.  Such tenderers shall be advised the date and time of opening of Part B i.e. price part of their bids.  Tenderers must carefully go through all the instructions and conditions before filling in Part B of the bid.  Any type of mistake in submission of the bids will make the tender liable for rejection.

- The bids shall be submitted only in the format given in the tender documents. The tenderer is  required to sign and stamp each and every page of the tender document clearly bring out deviations, if any.

- Tenders not complying with the above and other requirements of the tender shall be liable for rejection.  Tenderers are advised to read all the documents carefully before submission of the bids and ensure the fulfillments of all the technical and commercial parameters.

- No over-writing in any Part-A or Part-B is permitted.  Corrections in the bid should normally be avoided. However, when these become essential, bidder shall score out the incorrect information and put the correct information duly endorsing each correction with his signatures.

- Tender Cost (if any) is to submitted in the Form of  DD in favour of BHEL-Jhansi  payable at

Jhansi along with Techno –Commercial offer. This DD should not be kept in Sealed Price Bid.

3.COMMERCIAL TERMS & CONDITIONS FOR PURCHASE ENQUIRY

- Offer is invited in two bids, outer cover, inner cover-I (Techno-Commercial bid) and inner cover II (Price-Bid) must bear our tender no., due date and bidders name.  Offers received in single bid will be bypassed.

- The offer should reach this office on or before 13.15 hrs on the due date.  The tender can also be dropped in the Tender Box kept at Administrative Building Gate.  The tenders will be opened after 1400 hrs on the due date in presence of all bidders who will be present.

- Offer should be free from overwriting or erasures.  Corrections or additions, if any should be attested.

- No enhancement of rate for whatever cause is allowed once the offer is accepted and the order is placed.  Withdrawal from the quotation after it is accepted or failure to make the supply within the scheduled period will entail cancellation of order, forfeiture of earnest money if any and/or imposition of risk & cost purchase.

- Material offered should be of required quality and conforming strictly to our specifications / drawings or to the specifications in the drawings.  Any deviation must be clearly stated in the offer with particular clause /detail in our specification / drawing for which deviation is quoted.  All required documents/details  e.g. QA Plan, type TCs, drawings & data sheet etc. must be submitted along with the offer.

- Irrespective of whether it is mentioned in our specification, all machines / equipments are required to be calibrated in metric units and certificates must be issued where applicable.

- Work test  certificates / inspection certificates/guarantee certificate/ compliance certificate, etc. will be required to be submitted along with supplies in case an order is placed.

- Required Delivery  time for affecting supplies should be indicated in the offer.

- Failure to supply by the time specified on the order will make the supplier liable to pay unconditional penalty of ½% of the prices of goods in arrear per week subject to a maximum of 10% for Indian suppliers and max 5% for Foreign suppliers of the order value unless agreed otherwise.

- The quantities for each item to be purchased may vary from the tendered quantity according to our actual requirement at the time of placing orders.

- Bharat Heavy Electricals Ltd does not bind itself to accept the lowest of any tender, but reserves to itself the right to accept or reject all or a part of any tender at its discretion following BHEL Purchase Policy & Government guidelines.

- In all cases of disputes, the matter shall be referred for arbitration.  The arbitrator  shall be nominated by the Unit Head, Bharat Heavy Electricals Ltd., Jhansi or any other officer nominated by him.  The award of Arbitrator shall be final and binding on both the parties.

- In case of any suit or other legal proceedings under or relating to this tender, the courts at Jhansi shall have full jurisdiction.

- Total delivered cost at BHEL Jhansi Railway siding/Works shall be the criterion for deciding the tender.

- Unless otherwise specified, offer should be on ex-works basis (for Indian Suppliers) with freight & insurance to be borne by BHEL.

- Your quotation against this enquiry must be valid for ordering up to minimum of 120 days from the date of tender opening.

- ADDITIONAL FOR INDIAN SUPPLIERS:

I)        If any material is booked to selves or if it is not consigned to BHEL Jhansi, any warfage or

demurrage occurring from these shall be to the account of supplier.  In case of road transport,

material shall be booked only through our rate contract transporters mentioned in the purchase

order.

- Where there is provision for payment of Sales Tax extra, it will be paid only if the registration number both under State Sales Tax and Central Sales Tax are specifically mentioned on the bill/invoice.  Your Sales Tax Registration number should be mentioned in the offer positively.

- Excise Duty Gate Pass will be required in original.  Applicable taxes & duties must be included in the offer.

- ADDITIONAL FOR FOREIGN SUPPLIERS:

i)	For documentation and correspondence language shall be English only.

- Rates quoted should be on FOB basis.  All despatches are to be made by sea  by IPBC conference vessels only unless otherwise stated in purchase order.

- Agency commission, if any, will be paid out of the FOB value after receipt & acceptance of material at BHEL Jhansi in Indian Rupees.  TT Buying exchange rate prevailing on the tender opening date or the letter of credit date or date of purchase order whichever is lowest will be considered for calculating the agency commission.

- The rates quoted should be inclusive of charges of packing and delivery of material up to the port of shipment on FOB terms.  The material is required to be packed suitably in all respects considering the peculiarity of the material for normal transport by Sea/air and subsequently by Rail/Road in India suitably protected against the effect of tropical salt laden atmosphere in the event of consignment being delayed at Indian Ports before outward clearance.  You should also furnish details of shipping specification, sizes, volume, weight, number of cases etc. When articles are packed with packaging material of plant origin, phytosanitary certificate will be required.

- Our standard terms of payment is 100% payment through bank against despatch documents or 100% through sight irrevocable & unconfirmed letter of credit.

19.   Security deposit (SD) cum Performance Bank Guarantee  (PBG) for 10% basic value of the order (excluding taxes &

duties) for  execution of the purchase order (PO) & satisfactory Performance of the equipment before release of  PO

will be required to submit which will be valid from the date of issue of LOI to 24 months after acceptance of material by

BHEL with a further claim period of 3 months. PO will be issued only after receipt of SD cum PBG.

20. For fully imported equipments / plant/ machinery (no part or sub-assembly of Indian origin) :

a. No SD cum PBG will be applicable for basic order value up to Rs. 25.0 Lacs on foreign supplier.

b. No SD cum PBG will be applicable for basic order value up to Rs. 10.0 Lacs on Indian stockist / agent of

foreign manufacturer.

21.   B.G. shall be from Scheduled Banks/Public Financial Institutions as defined in the Companies Act.

22. The equipment shall be guaranteed for 24 months from the date of acceptance by BHEL irrespective of date of supply,

erection and commissioning.

23. Our standard terms of payment (for Indian Suppliers) is 80% of P.O. value be paid after 60 days of acceptance of the

equipment at BHEL Jhansi works and balance 20% after successful commissioning & handing over of the equipment

(where applicable).

24. Amount of Service Tax / Education Cess / any other Govt. duty ( as applicable on the date of offer) for erection or any

other activities associated with the scope of work in the tender must be indicated in Techno-commercial bid of the offer.

25 . Blank Price Bid (After erasing prices) must be submitted along with Techno commercial bid.

26.  If any goods are rejected, BHEL shall be at liberty to take action as per following at the

risk and cost of supplier:-

    - Allow the supplier, wherever mutually agreed, to rectify the rejected goods at BHEL's works within reasonable time as fixed by BHEL.

    - Allow the supplier to make free replacement within a specified period, rejected goods can be lifted by the supplier thereafter.

OR

Allow the supplier to refund the full amount paid to him by BHEL by a Demand Draft  before lifting the rejected goods.  Fresh replacement shall be regularized as per terms & conditions of the original purchase order.

OR

Take alternative  procurement action from elsewhere and recover the difference in cost, if any incurred by BHEL in this regard from the supplier.  The supplier shall not be entitled to any gain on purchase.

    - Terminate the contract either in part or in whole at the  discretion of BHEL and recover the loss, if any, from the supplier.

27. For any clarification on our tender terms and conditions please contact Sr. Purchase Officer (Capital Items)

before due date.

28. Inspection may be done at supplier works by BHEL.

## 4. TECHNO COMMERCIAL BID  PART-A

( Supplier should fill up this bid by putting the tick in accepted / non accepted column)

SUPPLIER’S NAME & ADDRESS

1.          Offered Equipment with complete technical details  :                      As per attached details

2.           Packing & Forwarding Charges, FOB charges (if any)                                                               ….%

3.   	Present rate of Excise duty (if any)         		                                                              ….%

4.     	Present rate of sales tax			 		                                                   %

(UPTT/CST, Not applicable in case of Foreign suppliers)

5.	Delivery period in  No. of  days from  date of LOI / PO                                                                                                                    6.	Taxes and duties shall be extra as applicable on 		              Accepted / Not accepted

the date of actual delivery.

7.	Penalty clause as well as other conditions given			Accepted / Not accepted

in general Terms & conditions

and Standard Enquiry terms & conditions are

acceptable to us.

8.           Rates quoted in the price bid are on Ex Works / F.O.R. 		---------------------------------

Jhansi .basis.

9.	Validity of the rates quoted is 120 days from the 			Accepted / Not accepted

date of opening of price bid.

10.          Security deposit (SD) cum Performance Bank Guarantee                      Accepted / Not accepted

(PBG) for 10% basic value of the order (excluding taxes & duties)

for  execution of the purchase order (PO) & satisfactory

Performance of the equipment up to Guarantee period

of 24 months  before release of PO will be submitted.

11.        The equipment shall be guaranteed for 24 months from                            Accepted / Not accepted

the date of acceptance by BHEL irrespective of date of

supply, installation.

Note :- If any of the above conditions are not accepted

the offer shall be liable for rejection.