BHEL - JHANSI

TENDER DOCUMENT FOR 3 T SEMI GANTRY CRANE, Qty -01 No.

Enquiry No. E7160069, Tender Cost: Rs. 500/-

C  O  N  T  E  N  T  S

Page No.

1.         Item Description & Technical Details                                         2-7

2.	Instructions for submission of Tender		                          8

3.	Commercial Terms & Conditions		                                   9-11

4.	Techno-Commercial Bid          		                                    12

( A.K.Banerjee  )		     ( P.K.Pathak )                                  ( Y K Verma )

DGM(MM)	                           Sr. PO (MM)                                      Sr. PO (MM)

1. Item Description & Technical Details :

ITEM				:		3 Ton Semi Gantry Crane.

Class II duty as per I.S.3117/1985

ITEM No			:		SP/CRJ/9555/1

SPEC. NO			:		CRJ/9555

LOCATION			:		BAY - 6

QUANTITY		:		1 No.

- GENERAL

1.1	The traveling semi gantry crane floor operated is required for handling components weighing up to 3000 KG.

1.2	Electrically operated hoist will have cross travel along the horizontal member up to the maximum possible length.

1.3	One end of the semi gantry crane will run on gantry rails fixed on the column brackets at an appropriate height. The other end of the crane will run on the rail on floor level.

2.0	CRANE STRUCTURE :

2.1	HORIZONTAL & VERTICAL MEMBER:

These members are to be of steel welded construction made of rolled steel section of suitable size. One end of the horizontal member will be fixed to the vertical member with help of fastening bolts of suitable size end other end will be fixed to the end carriage. Similarly other end of the vertical member will be fixed to the end carriage moving at floor level.

  - END CARRIAGE :

The end carriages are to be mounted on double flange wheels made out from forged steel and are to machined to profile to suit the gentry rail. The end carriages fixed with horizontal member will run on the rails fixed on the brackets, where as the end carriage fixed with the vertical member will run on the rails at floor level.

3.0	LONG TRAVEL MECHANISM AND BRAKES

One wheel in each end carriage to be driven through machine cut forged steel gears enclosed in a gear box. The gears are to be of EN8 steel hardened and ground. The gear box to be coupled with suitable HP motor. Two motors must be connected to one switch, but the rotor circuit must be entirely independent to avoid synchronization and permit the slight variations of speed, necessary to let the crane freely on a track which deviates slightly from the perfect straightness. Driving motor on floor end carriage should not project out of end carriage width. Suitable size electromagnetic brakes are to be provided on both LT drive and size of the LT brakes will not be less then 150 mm.

- HOISTING AND CROSS TRAVEL MACHANISM:

The hoist drum to be of welded construction carried on four flanged forged steel runners. The lifting load is to be carried by a testing steel wire rope as per I.S. specification of suitable size wound on the hosting drum. The drum is to be rotated in any direction through machine cut spur/helical gear and closed in a gear box and coupled by a suitable coupling to an appropriate capacity electric motor.

The hoisting motor shall be fitted with an electromagnetic brake suitably designed and built to arrest and hold safely the full load capacity of the hoist. It is to be of solenoid operated.

The cross travel motion of the hoisting block shall be through a geared type traveling trolley being driven by means of electric motor.

Suitable type of brake shall be fitted in the cross travel mechanism and to be operated by solenoid.

- LEADING DIMENSIONS :

I)	Safe working load				:	3 Tonnes

II)	Duty required as per I.S.			:	Class II

III)	Clear lift of height above ground level	:	5 Meters

- Maximum height available above ground	:	5.25 Meters

Level ( Any projection of crane including

D.S.lead should not go above this height)

V)	Speed hoisting / lowering			:	5 mtrs/ minute

VI)	Long travel speed 				:	20 mtrs / minute

VII)	Cross travel speed 				:	18 mtrs / min.

VIII)	Span (between rail centers)			:	6.4 Meters.

IX)        Top of gantry rail on column

from ground floor                                          :          To be given by           Manufacturer

X)       Bay Length                                                     :	72 Meter

XI)	No. of falls for rope in hoist			:	Four

XII)	Operation					:	By remote control and

Pendent

XIII)	Electric supply				:	415 V, 3 Phase,										4 wires, 50 C/S

6.0	WIRING:

6.1	The crane shall be supplied complete with all wiring and materials. Collectors on the crane bridge and on the crab shall be provided where applicable. Moveable type flexible cable with necessary trolleys and festooned system shall be provided for supply of trolley.

6.2	All wiring shall be with PVC insulated and or armored cables having copper conductor. Wire and cable should be of 1100V grade in trunking. Trunking should be used instead of conduit pipe.

6.3	All electrical wiring and installation should strictly confirm to IS Act, 1910 and I.E.E. Rules. 1956 and other standard practice as adopted by the I.S.I.

6.4	All control terminations should be ferruled such that the numbers are clearly visible from out side, crimped terminations should be provided for control cables and as far as possible for main cables. A set of suitable current collector shoe should be provided to suit the angle iron down shop leads. Details of existing down shop lead and current collector system can be checked at site before finalizing the supply of current collector system.

7.0 	LONGITUIDNAL GANTRY LEAD

7.1	Longitudinal Gantry lead (Down shop Lead) to be provided suitable for 3 phase 4 wire system with earth connection.

7.2	All material to be supplied with the crane. Erection of DSL to be done.

8.0	RADIO REMOTE CONTROL  SYSTEM:

8.1	Radio remote control system shall be provided to operate the crane from shop floor in all motion and control. The radio remote control system shall consist of a portable transmitter and receiver along with the necessary relay logic for interfacing to the crane motors. The transmitter 	shall consist of push buttons.  A selector switch shall be provided in the operator cabin for remote / manual operation.

8.2	Details of Radio remote control system to be furnished with the offer.

8.3 	The system shall be following designated controls:

Main Power 		ON/OFF

Hoist 			UP/DOWN			1 -  0 -  1

CT			FORWARD/REVERSE	1 –  0 -  1

LT			LEFT / RIGHT (HIGH)	1 – 0 - 1

Alarm			ON/OFF

Emergency stop	ON/OFF

9.0	While quoting the following information should be furnished explicitly

- Dead weight of structure including LT drive.

- Dead weight of hoist

- Make of hoist with assembly details.

- Details of LT  & CT drive.

- Layout drawing of semi gentry crane showing all dimensions and clearances.

- Details of braking arrangement for all motion.

- Details of down shop lead.

- Hook approach of hoist on both ends.

- Electrical circuit diagram and remote controls system details.

- Any other details which may be relevant to the semi gantry crane supply and fixing

- Price for crane, down shop lead, erection of crane to be given separately. 	           I)	Erection commissioning to be done by suppliers own tools tackles and labours.

10.0	MAINTENANCE:

10.1	We would stress the need of adequate maintenance facility for all gearboxes and working mechanism which shall be located above platform level and shall be easily approachable for good maintenance.

11.0	SPARES :

11.1	Recommended spares for two years normal working of the equipment is to be furnished with the estimate, separately quoted for each items. Details of bearings, oil seals used should be supplied along with location of each item.

12.0	MISCELLANEOUS :

  - All gear boxes should have oil level indicator to show the level and condition of the oil in gear box.

  - All material used should be of the best of their respective kinds and should of the appropriate I.S. Specification & all workman ship should be of best quality.

  - The structural steel work in the bridge and the crab frame should be given one coat of suitable primer during fabrication. Gear boxes and similar parts to be given coat of oil paint. After erection at site, the crane should be given two coats of light orange synthetic enamel paint.

  - First fill oil, grease etc. for the commissioning of the crane shall be provided by the supplier.

  - Tool box with standard tools shall be supplied with the crane.

  - List of bought out items should be furnished with  names of makers and complete literature about the items should be supplied.

  - Crane to be fabricated for its satisfactory performance for a period of 24 months from the date of commissioning with guarantee certificate.

  - Lifting hook, wire rope and crane to be tested to IS standard and test certificate to be provided (4 copies each). Test certificate of hook should from a recognized Govt. Test House.

  - Four copies of manual of Operation and servicing to be with erection drawings. G.A. drawing, wiring diagram etc. to be supplied along with the crane.

  - Four copies of structural drawing, wheel assembly, maximum crane wheel load etc. H.P. rating of motor, hook approach, gear box details, weight of crane to be supplied.

  - Mechanical drawings, electrical wiring and schematic diagram should be sent in triplicate for approval before finalizing the design. Make and technical specification of all items is also to be submitted for approval.

  - Supplier shall inform BHEL in advance to witness assembly and load test.

  - The crane is to be erected by the supplier at site with their own labour, tools and tackles, winch. The crane is to be commissioned and tested before handing over.

  - All motions at various steps of speed shall be checked along with current being taken by respective motion motors.

  - After test but before crane is put to use, it shall be tested with overload of 125 percent of the S.W.L. when load is located at the center of span. During testing the overload test of each motion shall be measured in both direction and the crane shall sustain the load under full control.

  - Erection / commissioning charges to be quoted separately.

13.00	QUALIFYING CONDITIONS

Only those vender, who have supplied and commissioned at least 5 NUMBERS SEMI GANTRY CRANES  of mentioned specifications or higher Capacity  for similar application in the past five years and such cranes/machine is presently working satisfactorily for more than one year (more than six month if supplied to BHEL) after commissioning should quote. The following information is to be submitted by the vender about the companies where similar machines/cranes have been supplied. This is required from all technical bid venders for qualifications of their offers.

1. Name of the customer/ company where similar machine is installed

2. Complete postal address of the customer

3. Year of commissioning

5. Name and designation of the contact person of the customer

6. Phone , Fax No. and e.mail address of the contact person of the customer

7. Performance certificate from the customer regarding satisfactory performance of the

machine/crane supplied to them

2.INSTRUCTIONS FOR SUBMISSION OF TENDER

- The offer against the enquiry shall be submitted in two parts.  The first part called Techno-commercial bid i.e. Part - A shall be un-priced and will be submitted in line with given details in commercial terms and conditions .  The second part called Price-Bid  i.e. Part-B, in this you will have to mention the basic prices, taxes duties etc for basic equipment & Standard / optional accessories if any.

- The vendors shall submit Part-A and Part-B of their bids in two different sealed envelopes duly super-scribing Enquiry No. and Part No. of the bid.  These two parts of the bid shall be kept together in a third envelope super-scribing Enquiry No., due date and time of opening as indicated in the tender document.

- Part-A of the bid alone shall be opened on the due date in presence of the tenderers in the first instance.   This part of the bid shall be examined in detail.  Part B of the bid shall be opened only for such of the tenderers whose Techno-Commercial offers are found acceptable by BHEL.  Such tenderers shall be advised the date and time of opening of Part B i.e. price part of their bids.  Tenderers must carefully go through all the instructions and conditions before filling in Part B of the bid.  Any type of mistake in submission of the bids will make the tender liable for rejection.

- The bids shall be submitted only in the format given in the tender documents. The tenderer is  required to sign and stamp each and every page of the tender document clearly bring out deviations, if any.

- Tenders not complying with the above and other requirements of the tender shall be liable for rejection.  Tenderers are advised to read all the documents carefully before submission of the bids and ensure the fulfillments of all the technical and commercial parameters.

- No over-writing in any Part-A or Part-B is permitted.  Corrections in the bid should normally be avoided. However, when these become essential, bidder shall score out the incorrect information and put the correct information duly endorsing each correction with his signatures.

- Tender Cost (if any) is to submitted in the Form of  DD in favour of BHEL-Jhansi  payable at

Jhansi along with Techno –Commercial offer. This DD should not be kept in Sealed Price Bid.

3.COMMERCIAL TERMS & CONDITIONS FOR PURCHASE ENQUIRY

- Offer is invited in two bids, outer cover, inner cover-I (Techno-Commercial bid) and inner cover II (Price-Bid) must bear our tender no., due date and bidders name.  Offers received in single bid will be bypassed.

- The offer should reach this office on or before 13.15 hrs on the due date.  The tender can also be dropped in the Tender Box kept at Administrative Building Gate.  The tenders will be opened after 1400 hrs on the due date in presence of all bidders who will be present.

- Offer should be free from overwriting or erasures.  Corrections or additions, if any should be attested.

- No enhancement of rate for whatever cause is allowed once the offer is accepted and the order is placed.  Withdrawal from the quotation after it is accepted or failure to make the supply within the scheduled period will entail cancellation of order, forfeiture of earnest money if any and/or imposition of risk & cost purchase.

- Material offered should be of required quality and conforming strictly to our specifications / drawings or to the specifications in the drawings.  Any deviation must be clearly stated in the offer with particular clause /detail in our specification / drawing for which deviation is quoted.  All required documents/details  e.g. QA Plan, type TCs, drawings & data sheet etc. must be submitted along with the offer.

- Irrespective of whether it is mentioned in our specification, all machines / equipments are required to be calibrated in metric units and certificates must be issued where applicable.

- Work test  certificates / inspection certificates/guarantee certificate/ compliance certificate, etc. will be required to be submitted along with supplies in case an order is placed.

- Required Delivery  time for affecting supplies should be indicated in the offer.

- Failure to supply by the time specified on the order will make the supplier liable to pay unconditional penalty of ½% of the prices of goods in arrear per week subject to a maximum of 10% for Indian suppliers and max 5% for Foreign suppliers of the order value unless agreed otherwise.

- The quantities for each item to be purchased may vary from the tendered quantity according to our actual requirement at the time of placing orders.

- Bharat Heavy Electricals Ltd does not bind itself to accept the lowest of any tender, but reserves to itself the right to accept or reject all or a part of any tender at its discretion following BHEL Purchase Policy & Government guidelines.

- In all cases of disputes, the matter shall be referred for arbitration.  The arbitrator  shall be nominated by the Unit Head, Bharat Heavy Electricals Ltd., Jhansi or any other officer nominated by him.  The award of Arbitrator shall be final and binding on both the parties.

- In case of any suit or other legal proceedings under or relating to this tender, the courts at Jhansi shall have full jurisdiction.

- Total delivered cost at BHEL Jhansi Railway siding/Works shall be the criterion for deciding the tender.

- Unless otherwise specified, offer should be on ex-works basis (for Indian Suppliers) with freight & insurance to be borne by BHEL.

- Your quotation against this enquiry must be valid for ordering up to minimum of 120 days from the date of tender opening.

- ADDITIONAL FOR INDIAN SUPPLIERS:

I)        If any material is booked to selves or if it is not consigned to BHEL Jhansi, any warfage or

demurrage occurring from these shall be to the account of supplier.  In case of road transport,

material shall be booked only through our rate contract transporters mentioned in the purchase

order.

- Where there is provision for payment of Sales Tax extra, it will be paid only if the registration number both under State Sales Tax and Central Sales Tax are specifically mentioned on the bill/invoice.  Your Sales Tax Registration number should be mentioned in the offer positively.

- Excise Duty Gate Pass will be required in original.  Applicable taxes & duties must be included in the offer.

- ADDITIONAL FOR FOREIGN SUPPLIERS:

i)	For documentation and correspondence language shall be English only.

- Rates quoted should be on FOB basis.  All despatches are to be made by sea  by IPBC conference vessels only unless otherwise stated in purchase order.

- Agency commission, if any, will be paid out of the FOB value after receipt & acceptance of material at BHEL Jhansi in Indian Rupees.  TT Buying exchange rate prevailing on the tender opening date or the letter of credit date or date of purchase order whichever is lowest will be considered for calculating the agency commission.

- The rates quoted should be inclusive of charges of packing and delivery of material up to the port of shipment on FOB terms.  The material is required to be packed suitably in all respects considering the peculiarity of the material for normal transport by Sea/air and subsequently by Rail/Road in India suitably protected against the effect of tropical salt laden atmosphere in the event of consignment being delayed at Indian Ports before outward clearance.  You should also furnish details of shipping specification, sizes, volume, weight, number of cases etc. When articles are packed with packaging material of plant origin, phytosanitary certificate will be required.

- Our standard terms of payment is 100% payment through bank against despatch documents or 100% through sight irrevocable & unconfirmed letter of credit.

19.   Security deposit (SD) cum Performance Bank Guarantee  (PBG) for 10% basic value of the order (excluding taxes &

duties) for  execution of the purchase order (PO) & satisfactory Performance of the equipment before release of  PO

will be required to submit which will be valid from the date of issue of LOI to 24 months after acceptance of material by

BHEL with a further claim period of 3 months. PO will be issued only after receipt of SD cum PBG.

20. For fully imported equipments / plant/ machinery (no part or sub-assembly of Indian origin) :

a. No SD cum PBG will be applicable for basic order value up to Rs. 25.0 Lacs on foreign supplier.

b. No SD cum PBG will be applicable for basic order value up to Rs. 10.0 Lacs on Indian stockist / agent of

foreign manufacturer.

21.   B.G. shall be from Scheduled Banks/Public Financial Institutions as defined in the Companies Act.

22. The equipment shall be guaranteed for 24 months from the date of acceptance by BHEL irrespective of date of supply,

erection and commissioning.

23. Our standard terms of payment (for Indian Suppliers) is 80% of P.O. value be paid after 60 days of acceptance of the

equipment at BHEL Jhansi works and balance 20% after successful commissioning & handing over of the equipment

(where applicable).

24. Amount of Service Tax / Education Cess / any other Govt. duty ( as applicable on the date of offer) for erection or any

other activities associated with the scope of work in the tender must be indicated in Techno-commercial bid of the offer.

25 . Blank Price Bid (After erasing prices) must be submitted along with Techno commercial bid.

26.  If any goods are rejected, BHEL shall be at liberty to take action as per following at the

risk and cost of supplier:-

    - Allow the supplier, wherever mutually agreed, to rectify the rejected goods at BHEL's works within reasonable time as fixed by BHEL.

    - Allow the supplier to make free replacement within a specified period, rejected goods can be lifted by the supplier thereafter.

OR

Allow the supplier to refund the full amount paid to him by BHEL by a Demand Draft  before lifting the rejected goods.  Fresh replacement shall be regularized as per terms & conditions of the original purchase order.

OR

Take alternative  procurement action from elsewhere and recover the difference in cost, if any incurred by BHEL in this regard from the supplier.  The supplier shall not be entitled to any gain on purchase.

    - Terminate the contract either in part or in whole at the  discretion of BHEL and recover the loss, if any, from the supplier.

27. For any clarification on our tender terms and conditions please contact Sr. Purchase Officer (Capital Items)

before due date.

28. Inspection may be done at supplier works by BHEL.

## 4. TECHNO COMMERCIAL BID  PART-A

( Supplier should fill up this bid by putting the tick in accepted / non accepted column)

SUPPLIER’S NAME & ADDRESS

1.          Offered Equipment with complete technical details  :                      As per attached details

2.           Packing & Forwarding Charges, FOB charges (if any)                                                               ….%

3.   	Present rate of Excise duty (if any)         		                                                              ….%

4.     	Present rate of sales tax			 		                                                   %

(UPTT/CST, Not applicable in case of Foreign suppliers)

5.	Delivery period in  No. of  days from  date of LOI / PO                                                                                                                    6.	Taxes and duties shall be extra as applicable on 		              Accepted / Not accepted

the date of actual delivery.

7.	Penalty clause as well as other conditions given			Accepted / Not accepted

in general Terms & conditions

and Standard Enquiry terms & conditions are

acceptable to us.

8.           Rates quoted in the price bid are on Ex Works / F.O.R. 		---------------------------------

Jhansi .basis.

9.	Validity of the rates quoted is 120 days from the 			Accepted / Not accepted

date of opening of price bid.

10.          Security deposit (SD) cum Performance Bank Guarantee                      Accepted / Not accepted

(PBG) for 10% basic value of the order (excluding taxes & duties)

for  execution of the purchase order (PO) & satisfactory

Performance of the equipment up to Guarantee period

of 24 months  before release of PO will be submitted.

11.        The equipment shall be guaranteed for 24 months from                            Accepted / Not accepted

the date of acceptance by BHEL irrespective of date of

supply, installation.

Note :- If any of the above conditions are not accepted

the offer shall be liable for rejection.