TECHNICAL SPECIFICATION FOR DLP MULTIMEDIA PROJECTOR (WIRELESS)

Quantity – 19 Nos

Specifications:

| Projection System | DLP TM |
| --- | --- |
| Panel | DMD XGA x 1 |
| Resolution | 1024 pixels x 768 pixels |
| Brightness | 2000 Lumens minimum |
| Contrast Ratio | 2000:1 (Full on/off) |
| Aspect Ratio | 4:3, supports 16.9 |
| Projection Lens | F2.6 – 2.9, f=27.5 – 33.0 mm |
|  | 1:1.2 Manual Zoom, Manual Focus |
| Light Resource | 180 Watt High Performance Compact Lamp |
| Colour Palette | 16.7 million colors |
| Projecting Position | Front/Rear, Table/Ceiling |
| Projection Distance | 1.2 – 11.8m |
| Image size | 26” – 300” diagonal |
| RGB Signal (Input) | SXGA (Compression), XGA (True), SVGA/VGA (Expansion/True) |
| Video Signal | NTSC3.58, NTSC 4.43, PAL, PAL_N, PAL_M, PAL60, SECAM |
|  | YCbCr (NTSC, PAL), YPbPr (480p, 576p, 1080i, 720p)*1 |
| Horizontal Synch Range | 15 – 80 kHz |
| Vertical Synch Range | 50 – 85 Hz |
| Audio | Mono 0.5 – watt |
| Terminals (Input) | RGB (Analog/Digital)/YCbCr/YPbPr:DV1-1 x 1 |
|  | Composite Video: RCA x 1 |
|  | S-Video: Mini DIN 4 Pin x 1 |
|  | Audio (RGB/Video): Ø 3.5 mm Stereo Mini Jack x 1 |
|  | Remote Mouse |
|  | Wireless |
| Color | To be specified |
| Dimension excluding Lens and Stand | To be specified |
| Weight | To be specified |
| Power Supply | 100-240 V AC at 50/60 Hz |
| Power consumption | 240 Watts (Normal mode), 170 Watts (Eco-mode) |
| Environment | 32-95oF / 5 – 35 oC |
| On-screen Display Lan | English/German/French/Italian/Spanish/Swedish/Korean/Chinese/Japanese |
| Functions | Full-Auto Adjustment |
|  | Digital Keystone correction (Vertical: +17/-22 Degrees) |
|  | Digital Zoom (2x) |
|  | Freeze |
|  | Mute |
|  | Presentation Timer |
|  | Security Password |
| Accessories | All Standard Accessories – to be specified and spare bulb – 1no. and ceiling Mounting Kit for each projector, |
| Connectivity - OPTIONAL | Wireless Connectivity |
|  | PC Less presentation facility through CF Card |

TECHNICAL SPECIFICATION FOR THE DLP MULTIMEDIA PROJECTOR (WIRELESS)

Quantity – 3Nos

Specifications:

| Projection System | DLP TM |
| --- | --- |
| Panel | DMD XGA x 1 |
| Resolution | 1024 pixels x 768 pixels |
| Brightness | 3500 Lumes |
| Contrast Ratio | 2000:1 (Full on/off) |
| Aspect Ratio | 4:3, supports 16.9 |
| Projection Lens | F2.6 – 2.8, f=22.1 – 25.6 mm |
|  | 1:1.2 Manual Zoom, Manual Focus |
| Light Resource | 180 Watt High Performance Compact Lamp |
| Colour Palette | 16.7 million colors |
| Projecting Position | Front/Rear, Table/Ceiling |
| Projection Distance | 1.2 – 9.5m |
| Image size | 34” – 300” diagonal |
| RGB Signal (Input) | SXGA (Compression), XGA (True), SVGA/VGA (Expansion/True) |
| Video Signal | NTSC3.58, NTSC 4.43, PAL, PAL_N, PAL_M, PAL60, SECAM |
|  | YCbCr (NTSC, PAL), YPbPr (480p, 576p, 1080i, 720p)*1 |
| Horizontal Synch Range | 15 – 80 kHz |
| Vertical Synch Range | 50 – 85 Hz |
| Audio | Mono 0.5 – watt |
| Terminals (Input) | RGB (Analog/Digital)/YCbCr/YPbPr:DV1-1 x 1 |
|  | Composite Video: RCA x 1 |
|  | S-Video: Mini DIN 4 Pin x 1 |
|  | Audio (RGB/Video): Ø 3.5 mm Stereo Mini Jack x 1 |
|  | Remote Mouse |
|  | Wireless |
| Color | To be specified |
| Dimension excluding Lens and Stand | To be specified |
| Weight | To be specified |
| Power Supply | 100-240 V AC at 50/60 Hz |
| Power consumption | 240 Watts (Normal mode), 170 Watts (Eco-mode) |
| Environment | 32-95oF / 5 – 35 oC |
| On-screen Display Lan | English/German/French/Italian/Spanish/Swedish/Korean/Chinese/Japanese |
| Functions | Full-Auto Adjustment |
|  | Digital Keystone correction (Vertical: +17/-22 Degrees) |
|  | Digital Zoom (2x) |
|  | Freeze |
|  | Mute |
|  | Presentation Timer |
|  | Security Password |
| Accessories | All Standard Accessories – to be specified and spare bulb – 1no. and ceiling Mounting Kit for each projector, |
| Connectivity - Optional | Wireless Connectivity |
|  | PC Less presentation facility through CF Card |