SPECIFICATION & SCOPE FOR PORTABLE

WELDING FUME  EXTRACTOR

- Application:-

-	To extract welding fumes at source of welding in constricted  place,

absorb all harmful elements in filtration system & release clear air.

- Welding fumes are required to be captured inside Turbo Generator stator

body during welding in constricted space inside.

- Technical Specification :-

i)	Net Extraction Capacity 	-	1200 m3 /hour

ii)	Motor Power			-	0.75 KW /1 HP(minm)

iii)	Extraction Hose Length	-	10.0 m

- Cartridge type filtration system with proper surface treatment , having

filter surface efficiency	-	99.8%

- Automatic filter cleaning with compressed  air pulse & with set frequency of pulses.

- Automotive Start /stop  switching  system .

- Built in compressor  for independent  filter cleaning .

- Aluminum mesh (high grade) pre-filter to protect the cartridge  from sparks.

- Noise Level			-	within 69 dB(A).

- Audio alarm for filter saturation and  over heating.

- Input Supply 			-	220 V ± 10% Variation  single phase

& 50 HZ ± 3% Variation

xii)	Working Supply 		-	Ambient Temperature - 3°C to 48 °C

Relative  humidity	- Upto 95 %

during rainy

season.

3.0	Technical Feature /Condition :-

i)	Steel, Welded Construction painted with polyester –epoxy, Powder  paint .

- Door for  easy access to the filter and for waste drawer. It should be tightly sealed with latches.

- Pre-filter, Metal filter to protect cartridge filter from spark & for first step  of filtration.

- Cartridge –main filter on roll-out tray.

- Carbon filter .

- Handle

- Swivel Castor Wheel for mobility.

- Waste drawer mounted on roll-out rails.

- Built in Noise Abatement features.

- Built in safety  feature for safe use as per National / International norms.

- Rigid construction for day  to day shop floor use.

- Lifting  eye for handling by crane.

- Scope of supply:-

- Portable fume extractor as per specification  and technical features.

- 10.0 meter long heat resistance flexible metallic braided hose.

- Fume extraction hood  of suitable size to capture fumes at a distance of 30-40 cm from  the welding source.

- Fume extraction hood will be fitted with magnetic base with lever so that it can be set at required position by welder during welding in a constricted space  inside Turbo Generator Stator Body.

- Mounted on castor wheel base for portability.

- Input supply lead- 10.0 m long with plug.

- All connectors, clips etc.  required for hose and connection.

- Recommended consumables and spare for 2 year smooth operation .

- Commissioning  and trials by vendor at BHEL works.

- Training to BHEL personnel

- Guarantee for 3 years smooth and trouble free working after successful

commissioning .

- Prompt after sales service and response time within 24 hours.

- 3 sets of operation and maintenance manuals consisting of following documents

with supply :-

- Functional details with construction details .

- Specification and makes of main components .

- Operating  instructions.

- Maintenance instructions.

- Trouble shooting  and remedies.

- All clarification to be given point wise  as per above requirements and supporting technical literatures to be submitted with offer.

- Provide names & addresses of satisfied  customers using  similar welding  fume extraction system.