Specification for Electro Permanent Magnet

(EPM type lifting Magnet)

- Application:-

Handling of gas cut components, machined components, plates, flats, rounds.

- Technical Specification:-

- Rechargeable lead Acid Battery operated EPM lifter

Lifting Capacity – 1.0 MT

Testing Capacity – 3.0 MT.

- Rechargeable Lead Acid Battery operated EPM Lifter

Lifting Capacity        	-	2.0 MT

Testing capacity	-	6.0 MT

- Rechargeable Lead Acid Battery operated EPM Lifter

Lifting Capacity	-	5.0 MT

Testing capacity	-        15.0 MT

- Battery power should be needed only for switching on and off.

- With each full battery charge no. of switch on and off- about 300 times.

- Holding power should remain constant with time till it is switched off.

- Technical Features :-

- Light weight and robust construction for shop use in rough and tough working condition.

- No Battery power should be needed to keep magnet on.

- Design should be such that it can lift both flat and round components.

- Completely sealed, rechargeable, maintenance free Batteries should be provided.

- Built-in warning signal to indicate low charge and full charge of Batteries (Audio Alarm/charge indication).

- In built Battery charger with over charge protection.

- The magnets should not get heated during long duty cycle.

- Once the Battery is fully charged magnet should work for around 300 times switch on (Magnetize) and switch off (Demagnetize) with constant holding power.

- Fitted with lifting eye to be hooked to EOT crane for handling purpose.

- In built control; panel with all LED indicators, safe working voltage indicator, on/off switches etc. & suitably protected from day to day working hazards/damage.

4.0 	Scope :-

EPM Lifters should be in ready to use condition & complete with following: -

- EPM type magnet lifter of capacity 1.0 MT, 2.0 MT, 5.0 MT as per specification.

- Rechargeable & Maintenance free Lead Acid Batteries.

- Battery charging unit with cables/chord, sockets, Plugs etc.

- Battery condition monitor unit.

- Remote Control Pendant (chordless).

- Vendor to provide following informations with the offer: -

- Charts showing holding power w.r.t. thickness of components for all models of EPM Lifters.

- Charts showing change of holding power w.r.t. surface condition i.e. with mill scale, cleaned surface, hot surface etc.

- Charts showing Load Limit for flat and round surface (size minimum/ maximum).

- Calculation of lifting capacity of each model (i.e. 1.0 MT, 2.0 MT, 5.0 MT).

- Charts showing Load w.r.t. operative air gap.

- Safe working voltage, size of plate/round that can be lifted safely.

- Manufacture’s trade mark, serial nos. and date of manufacture to be provided on Magnets/Batteries.

- Test certificates for Safe load testing & Break load testing with the supply.

- 5 sets of operation and maintenance manuals including following documents with the

Supplying.

- Operating Instructions.

- Maintenance Instructions

- Battery charging frequency and instruction, life of battery.

- Electrical/Electronic circuit diagrams with proper wire nos.

- Detailed circuit diagrams of PCB cards and sequence of operation for control card.

- Load charts showing holding power w.r.t operative air gap.

- Do’s and Dont’s

- Trouble shooting chart with remedies

- Recommended spares for 2 years smooth and trouble free operation including PCB’s, Relays, Push button, Indication, Charging cable, Charging unit etc.

- Commissioning and trials by vendor at BHEL works.

- 18 month guarantee for smooth and trouble free working after commissioning / trials and after sales service.

- Names of customers with contact person and address to whom similar EPM Lifters have been supplied and working satisfactory.