| ###### SCOPE OF SUPPLY |  | Bidder’s Compliance Yes / No | Bidder’s Remarks |
| --- | --- | --- | --- |
|  | It is intended to procure Audio System at Administrative Building Conference hall BHEL RC Puram Hyderabad measuring 32’ X 60’. - Design, Supply, Installation, Configuring, Testing, Commissioning, demonstrating audio system to the satisfaction of BHEL as per specifications enclosed. - The scope includes supply of microphones , amplifiers , mixers , cables , recorder .speakers , etc and installation and demonstrating to the satisfaction of BHEL. |  |  |

| ###### Bidder Qualification Criteria |  |  | Bidder’s Compliance Yes / No | Bidder’s Remarks |
| --- | --- | --- | --- | --- |
|  |  | Bidder should be either OEM of Audio system who is willing to under take total scope of work OR an Authorized Dealer having direct purchase and support agreement with the OEM to quote for this tender. The system Integrator shall submit a letter of Authorization from OEM for this specific tender. The Certificate / Authorization Letter specific to this tender must be enclosed with the Technical Bid, without which the offer shall be liable for rejection. |  |  |
|  |  | The Bidder and the OEM should be able to support the Project site and the equipment for a period of 10 years. |  |  |
|  |  | The OEM / Dealer should have support infrastructure in spares stocking, Technical support and logistics in atleast one location in India. Specify the OEM Service / Spares logistics locations in India with address. |  |  |
|  |  | I) The Bidder should have experience in executing audio system solutions for similar conference hall. The Bidder should have successfully executed atleast TWO such solutions preferably PSUs / Government during the last 3 years as on 31st March 2006: (The executed work under consideration should be of total Solution and Infrastructure Setup, which consists cable laying, and system integration) Documentary evidence in the form of Performance certificates and copies of Purchase Orders to be enclosed. |  |  |
|  |  | The Bidder should submit financial statements for the last three years, duly audited by practitioner chartered accountant to assess the financial solvency. (A minimum turn over of Rs 30 Lakhs in the last financial year). A copy of PAN number to be provided. |  |  |
|  |  | Bidder should fulfill all statutory and safety requirements for personnel engaged while executing the contract. Due to non compliance of the applicable statutory provisions, if BHEL has to incur any expenditure in this regard the same will be compensated by the Bidder. |  |  |
| ###### Techno-Commercial Terms and Conditions |  |  | Bidder’s Compliance Yes / No | Bidder’s Remarks |
|  | The offer should be valid for 90 days from date of opening Price bid. |  |  |  |
| 2 | The price quoted shall be inclusive of the following : 1)Freight,handling and packing charges, transit insurance, installation. 2) All taxes & Duties. |  |  |  |
| 3 | The requirement, calls for a complete working audio system and not components thereof. Bids must be complete with all equipment and required accessories along with necessary power systems including supply of cables , laying them , routing in metal conduits, etc for the entire equipment, connectors, patch leads, mounting and fitting hardware, plugs, sockets, and any hardware, cables, etc. as required for complete installation of the audio System under this contract. |  |  |  |
| 4 | DELIVERY : Within 60 days of after receipt of confirmed order. |  |  |  |
| 5 | All the items, which are required for successful commissioning of the system, are to be included in the scope of supply. No men & material will be supplied by BHEL RC PURAM Hyderabad. |  |  |  |
| 6 | Guarantee : Eighteen months from date of commissioning. |  |  |  |
| 7 | Pre dispatch inspection (PDI) : Equipment to be offered for inspection along with test certificates before dispatch . Only after written clearance from BHEL the equipment can be shipped. |  |  |  |

D. TECHNICAL SPECIFICATIONS  - Audio conferencing system at BHEL

| Sr No | Equipment | Bidder’s compliance | Bidder’s remarks |
| --- | --- | --- | --- |
| I. | GENERAL SPECIFICATIONS |  |  |
| 1. | Complete Design, System Engineering, Supply, Installation, Testing & Commissioning of Auditorium Sound Reinforcement System at ABC Hall – BHEL RC Puram, Hyderabad of dimension 32’ X 60’ . |  |  |
| 2. | Supply & Installation of Equipment as per system design and performance requirements. |  |  |
| 3. | Inter-connections/termination of cables, supply of cables, routing of cables in metal conduits, installation of speakers in false roofing , etc are all in Bidder’s scope. |  |  |
| 4 | The cabling for Goose mikes should facilitate connecting 15 nos of goose mikes at different locations in the hall to be indicated at the time of commissioning. The vendor should include necessary cables and connectors accordingly. |  |  |
| 5 | Adjustments/customization of equipment for maximum speech intelligibility, however please note that this is a multipurpose hall. |  |  |
| 6 | Testing , installation and commissioning of complete system. |  |  |
| 7 | Training and documentation ( 3 sets). |  |  |
| 8 | The Bidder to include one complete set of batteries for operation. |  |  |
| 9 | Maintenance of the complete system during Guarantee period and thereafter on award of Annual maintenance contract. |  |  |
| 10 | The requirement calls for a complete working audio system and not components thereof. Bids must be complete with all equipment and required accessories along with necessary batteries for the entire equipment , mounting, routing of cables, installation of speakers ,etc and demonstrating working of the complete system to the satisfaction of BHEL. |  |  |
| 11 | STANDS : The scope includes 2 nos each of table top stands and long stands for remote microphones. |  |  |

E. Technical Specification

| Sl | Item | Qty | Y / N | Remarks |
| --- | --- | --- | --- | --- |
| 1. | Speakers: Frequency Range – 80 Hz – 16 KHz; 100V, 32W with active equalization; Sensitivity – 89dB-SPL, 1W, 1m (for speech) & 87dB-SPL, 1W, 1m (for music); Maximum Acoustic Output – 104dB-SPL(for speech) & 102dB-SPL (for music); Beam width (-6dB point) – 132˚ conical; Flange diameter – 7.72”; Hole Diameter – 7.19”; Depth-8”; weight – 1.68 Kgs; The speakers should false ceiling mountable. | 8 nos |  |  |
| 2. | Active electronics equalization for above 8nos speakers: Page Input Sensitivity At microphone: 5mV nominal At line: 100mV nominal Page Input Impedance Microphone: 3kW (balanced) Line: 200kW (balanced) Channel A and Channel B Input Sensitivity At 300mV switch position: 200mV nominal Input Impedance: 15kW (balanced) At 2V switch position: 1V nominal Input Impedance: 2kW (balanced) Line Output (Unequalized) Output level: 6V RMS max Output impedance: 600W (balanced) Frequency range: 40Hz – 20kHz Output (Equalized) Output level: 6V RMS max Output impedance: 600W (balanced) Frequency range: 80Hz – 16kHz Paging: Channel A page always on; Channel B page switch selectable Opti-voice Paging Circuitry Voice band-pass equalization for good speech intelligibility AGC circuit for automatic compensation of varying input levels Gradual return of music to original level after page Power Consumption 7 Watts Regulatory Compliance Complies with UL-813, UL-1711, CSA-C22.2, No. 1- M1990, EN-60065 and MITI | 1 no |  |  |
| 3. | Power Amplifier: Rated Output(THD 1%) 120W(22V/4,25V/5.2,70V/41,100V/83) Input Sensitivity/Impedance......1V/10K½ BAL Frequency Response(+1/-3dB).................70Hz~18KHz T.H.D(at 1KHz Rated Output)......Less than 1% S/N ..........................................Better than 95dB High Pass Filter ..........................-3dB at 400Hz Input Level Adjustment..................-12dB~0dB Power Source ..............AC 110V-240V, 50/60Hz, DC 24V Power Consumption .............................. 300W | 2 nos |  |  |
| 4. | Full diversity Receiver: FM CARRIER UHF for reduced interference FREQUENCY RANGE 630 - 960 MHz (Frequency Bands according to local regulation) CHANNEL FREQUENCIES 192 PLL selectable channels per frequency Band FREQUENCY STABILITY ± 0.005% S/N RATIO > 100dB AUDIO OUTPUT LEVEL -12dB AF OUTPUT IMPEDANCE 600 Ohms SQUELCH Pilot Tone and Noise Mute OUTPUT TERMINATIONS Balanced - 3 Pin XLR socket. Unbalanced - 6.3mm socket DISPLAY LCD DISPLAY INFORMATION Frequency; Receiver A/B; Mute Status; RF/AF levels. CONTROLS Power On/Off; Frequency Up/Down; Frequency Scan; Audio Level. SUPPLY POWER External 12V DC 500mA. RUGGED METAL CASE | 3 nos |  |  |
| 5. | Handheld Transmitter & stand with specifications: FM CARRIER - UHF FREQUENCY RANGE - 630 - 960 MHz (Frequency Bands according to local regulation); CHANNEL FREQUENCIES - 192 PLL Pre selectable Channels per Band RF OUTPUT - 10mW FREQUENCY STABILITY ± 0.005% FREQUENCY DEVIATION ± 48 KHz S/N RATIO > 102dB AUDIO FREQUENCY RESPONSE 50 Hz - 15 KHz DYNAMIC RANGE > +110dB CONTROLS Power On/Off, Frequency and AF Level set MICROPHONE Dynamic Cardioid LCD DISPLAY INFORMATION Frequency, AF level and Battery status POWER 2 x AA batteries giving more than 10 hours use FINISH Soft touch Black | 2 nos |  |  |
| 6. | Cordless lapel belt pack with specifications: FM CARRIER UHF FREQUENCY RANGE 630 - 960 MHz (Frequency Bands according to local regulation) CHANNEL FREQUENCIES 192 PLL Pre selectable Channels per Band RF OUTPUT 10mW FREQUENCY STABILITY ± 0.005% FREQUENCY DEVIATION ± 48 KHz S/N RATIO > 102dB AUDIO FREQUENCY RESPONSE 50 Hz - 15 KHz DYNAMIC RANGE > +110dB CONTROLS Power On/Off, Frequency and AF Level set LCD DISPLAY INFORMATION Frequency and Battery Status MICROPHONE Lavalier Cardioid Condenser POWER 2 x AA Batteries giving more than 8 hours use BELT CLIP Reverseable/removable | 2 nos |  |  |
| 7. | Goose neck microphones with base plate specifications: Type - Condenser ; Polar pattern - cardoid; FREQUENCY RESPONSE 50 Hz - 18 KHz, MAXIMUM SPL 125 dB 1% THD, S/ N ratio – 64dB(A); Power requirements – 9 to 48V phantom power; SWITCHABLE BASS FILTER Type - 4 dB per octave at 400 Hz to - 6 dB per octave at 100 Hz; Dimensions length 580mm, Shaft diameter 8.5mm | 10 nos |  |  |
| 8. | 6 channel automatic microphone mixer with specifications: INPUT IMPEDANCE 10kW;OUTPUT IMPEDANCE BALANCED 50W; UNBALANCED 600W; PRE-AMP OUT 600W; MAX INPUT LEVEL GAIN >25 dB -24 dBV GAIN <25 dB +16 dBV MAX OUTPUT LEVEL +16 dBV NOMINAL OUTPUT LEVEL AT 0 dBV ON METER PRE-AMP OUT -10 dBV BALANCED -60 dBV to +16 dBV (Level set on front panel) UNBALANCED -16 dB of Level set on front panel OUTPUT GAIN CONTROL 0 to -40 dB MAXIMUM GAIN 90 dB FREQUENCY RESPONSE 10Hz to 22kHZ THD+N -70 dB MAX NOMA ATTENUATION 20 dB PHANTOM POWER +12V POWER SUPPLY 115/230V AC (Switchable), 50/60 Hz | 3 nos |  |  |
| 9. | Digital signal processor with inbuilt automatic feedback reduction, equalizers & shelving, crossovers, compressors, limiters, gate & downward expander, delay and matrix mixer with specifications: Frequency Response 20 Hz to 20 kHz +/– 1dB Dynamic Range 110 dB minimum, A-weighted, 20 Hz to 20 kHz Digital-to-Analog, Analog-to-Digital Conversion 24 bit Impedance Input: 10 k_ Output 120 _ Input Clipping Level +24 dBu minimum Output Clipping Level +24 dBu +12 dBu with 12 dB pad engaged +6 dBu with 18 dB pad engaged Total Harmonic Distortion < 0.05%, +4 dBu, 20 Hz to 20 kHz Propagation Delay < 1.5 ms. Processing blocks add no latency Polarity Input to output: non-inverting (inverting optional) Operating Voltage 100-240 VAC, 50/60 Hz (auto-switching), 1 A, maximum Maximum Power Drain :45 W | 1 nos |  |  |
| 10 | Auto reverse tape deck cum CD professional recorder with following features and specifications: Heads:; Deck1: 1 Playback (rotating) Deck 2: 1 Erase and 1 Record/Playback (rotating). Motor:; 2 DC Servo motors. Wow & Flutter:; 0.09% (WRMS). Frequency Response:; 25 - 19,000 Hz (Metal) 25 - 18,000 Hz (CrO2) 25 - 17,000 Hz (Normal). Signal-to-Noise Ratio:; 59 dB (NR off, 3% THD level, Weighted) 69 dB (Dolby B NR on, over 5kHz) 79 dB (Dolby C NR on, over 1kHz). Power requirements:; 120/230V AC, 50/60 Hz. Key Features. • Twin IC logic control mechanisms • Dual auto-reverse tape transports • Variable pitch control ± 10% (Deck 1) • Bi-directional double-deck continuous Playback • Bi-directional Record and Playback (Deck 2) • Normal & High speed dubbing (Deck 1 to Deck 2) • Synchro reverse dubbing • Dolby B/C Noise Reduction • Dolby HX Pro (Deck 2) • Multi-function flourescent display • Left & Right peak program level meters • Dual 4-digit electronic tape counters • REC mute with auto spacer (Deck 2) • CPS Computomatic Program Search (Decks 1 & 2) • Intro Check & Blank Scan functions • Auto tape type selector (Normal/CrO2/Metal) • Headphone output • TEAC "UR" (Unified Remote) compatible * Professional CD recorder. | 1 no |  |  |
| 10. | Volume control appropriate for the provided speakers | 2 nos |  |  |
| 11. | High Quality rack for system electronics. | 1 no. |  |  |
| 12 | Suitable stand for all the microphones if not already indicated. | -- |  |  |
| 13 | Cables : Necessary control and power cables | -- |  |  |
| 14 | Connectors : Suitable connectors | -- |  |  |
| 15 | Cassettes / CDs : A) Pre recorded cassettes and CDs ( choice to be indicated later) . B) Empty cassettes and Writable CDs | 10 nos. each 10 nos. each |  |  |
| 16 | Batteries : Two sets of batteries for operation of each microphones. | 2 sets |  |  |