SPECIFICATIONS OF VERTICAL MILLING MACHINE WITH D.R.O.

- PURPOSE

The machine shall be required for machining of small components of various shape and sizes involving milling, drilling and boring operation with a high level of efficiency and accuracy.

- WORK PIECE MATERIAL

Non-ferrous metals like aluminium alloys, bronze and ferrous materials like carbon steels, alloy steels, tool steels etc.

- MACHINE SPECIFICATIONS

  - SPINDLE

    - No. spindle                      :                 One

    - Spindle position              :    Swiveling type Vertical

Swiveling angle +/- 45 deg.

    - Spindle taper                   :               ISO 40

    - Spindle motor power       :              5.5 KW (approx.)

    - Spindle speed range        :    20 to 2000 r.p.m. (approx.)

    - Vertical movement of quill    :        70 mm (approx.)

  - TABLE

    - Overall size / clamping area :     1350 mm X 310 mm

    - Max. permissible load on table :  500 kg (approx.)

    - The table top should have longitudinal T-slots. The dimension of T-slots is to be specified by supplier.

    - Longitudinal traverse :                800 mm or more

    - Vertical traverse :                        400 mm or more

    - Cross traverse :                            265 mm or more

  - FEED RATES

    - Rapid traverse rate Long & cross : 3200 mm/min (approx.)

Vertical       :   800 mm/min (approx.)

3.3.2    Feed rates         Long & cross       : 16 – 800 mm/min (approx.)

Vertical      :  4 – 200 mm/min (approx.)

  - FEATURES

    - The machine to be equipped with Digital Read-Out system of 0.001 mm for longitudinal, cross & vertical traverse. The DRO must be of Heidenhein (Germany) make.

    - The machine to be equipped with automatic table cycles.

- GENERAL FEATURES OF MACHINE

  - The specification of feed drive motors with make, model no., max. &             nominal torque should be specified.

  - Preferably the guideways of all axes should have telescopic type

protective covers with adequate sealing on joints to prevent seepage of

dust & coolant oil inside.

  - The machine should be capable of high stock removals with close

tolerances on work piece. Metal removing rate in cc/min to be specified.

  - The machine must have rigid streamlined and vibration free construction.

All gears must be case hardened and ground and all the slides should be

rigid to withstand heavy stock removal. It should have independent main

drive and feed drive motors.

  - A suitable, preferably centralised lubricating system for lubrication of all   moving / rotating elements should be there preferably with interlock / alarm system in case of it’s failure.

5.0          ATTACHMENT & ACCESSORIES

The following accessories should be quoted separately giving their

individual prices

Catalogs of all these accessories should be submitted by the supplier

along with the offer.

  - Machine vices with fixed base, width of jaws 160 mm.

  - Universal swivel vices with fixed base, width of jaw 200 mm.

  - Universal dividing head with tail stock 135 mm height, 3-jaw self centering chuck, dead centre & draw bolt.

  - Self-centering vice (50-150mm).

  - Manually operated rotary table Dia 300 mm or more. (Accuracies to be specified).

  - REDUCTION MILLING SOCKETS

5.10.1 TANG TYPE:                      ISO 40 / MT 1

/ MT 2

/ MT 3

/ MT 4

/ MT 5

5.10.2 DRAW BOLT TYPE:         ISO 40  / MT 1

ISO 40  / MT 2

ISO 40  / MT 3

ISO 40  / MT 4

ISO 40  / MT 5

  - STUB ARBORS WITH KEY

ISO 40 X 22 X 19 mm

ISO 40 X 27 X 21 mm

ISO 40 X 32 X 24 mm

ISO 40 X 40 X 27 mm

  - COLLET ADOPTER

    - Collet Adopter ISO 40

    - Set of collets (metric) of sizes (4,6,8,10,12,16,20,25 & 32 mm)

  - Recirculatory type coolant system.

  - Work piece illumination light.

  - Splash – guards.

  - Foundation parts.

  - Any other standard / optional attachment which will increase the productivity / performance of the machine should be quoted with their individual prices.

- POWER SUPPLY / ENVIRONMENT CONDITION

The machine should be suitable for operation in the following conditions: -

Power supply for the machine:  415 V  +10 % / -15 %, 3 phase, 50 Hz +/-3 %

For controls: 230 V +10 % / -15 %; 50 Hz + / -3 %

Temperature: 5 to 45 deg C

Relative humidity : 95 % max.

Compressed air : 4 to 6 Kg / sq. cm

Other conditions : Similar to tropical country.

Water supply industrial : 1.5 to 2.0 Kg / sq. cm

Dust laden atmosphere during some part of the year.

7.0     SPARES

For smooth and trouble free operation of machine and other equipment, a spare package for two years should be offered. It should include

- Mechanical spares & bought out items.

- Electrical spares.

- Hydraulic spares

- Spares transducer / EXE Unit must be quoted separately for DRO for X, Y & Z axis.

8.0    MANUALS / DOCUMENTS

  - 5 sets of documents for operation, maintenance and servicing manuals for machine and other equipments to be provided in English language only, out of which 1st set to be sent along with offer.

  - Foundation drawing is to be submitted by supplier within 4 weeks of placement of LOI.

- VOLTAGE TRANSFORMER & STABILIZER

The required voltage stabilizer and matching isolation transformer if

recommended by supplier, should be part of offer.

- GENERAL

Supplier to specify following in the technical offer.

  - Total power requirement of the machine.

  - Overall space required for installation of the machine.

  - Color of the machine.

  - The supplier is to submit a clause wise deviation statement against each

clause of this technical specification along with the offer.

************************************************************************