- TECHNICAL SPECIFICATIONS

TRAVELLING COLUMN RADIAL DRILL MACHINE

1.0 PURPOSE :

The machine is required for Drilling , Counter Boring , Counter Sinking , Reaming , Boring and Threading as applicable in HP/IP/LP Casings of Steam turbine . The material of the Casings shall be mild steel and  low alloy steel  having tensile strength  upto 780 N/mm².

2.0  SPECIFICATIONS :

2.01    Drilling capacity (in solid)        	        100 mm in Alloy steel

2.02    Tapping capacity                                          Min. M80 in Alloy steel

2.03    Spindle motor power (KW)                 	        To be informed by the supplier

2.04    Spindle RPM (min.-max.)                            To be informed by the supplier

2.05    Spindle taper	                                            MT 6 with cotter slot as per ISO1080

2.06    Spindle feed range (min.-max.)                    To be informed by the supplier

2.07    Spindle (Quill) diameter                               To be informed by the supplier

2.08    Spindle travel (Quill)                                    Min.475 mm

2.09    Max. torque (Nm)                                         To be informed by the supplier

2.10    Max. drilling thrust (N)                                To be informed by the supplier

2.11    Head traverse on arm                                    Min. 3400 mm

2.12    Min. and max. spindle radius                        To be informed by the supplier

2.13    Arm traverse on column                                Min. 1500 mm

2.14    Min. and max. distance between                   To be informed by the supplier

spindle face & base (spindle retracted)

2.15    Column travel                                                8000 mm

2.16    Column diameter                                           To be informed by the supplier

2.17    Range of swing of arm                                   0 – 360 deg.

2.18    Guideways width                                           To be informed by the supplier

2.19    Column travel on bed rapid traverse	          To be informed by the supplier

2.20    Standard traverse speed of  main saddle       To be informed by the supplier

2.21    Arm elevating motor (Kw)                            To be informed by the supplier

2.22    Arm clamping motor (Kw)                            To be informed by the supplier

2.23   Hydraulic clamping of main saddle                To be informed by the supplier

on base motor (Kw)

2.24   Main saddle traverse motor (Kw)                   To be informed by the supplier

2.25   Head automatic traverse motor (Kw)             To be informed by the supplier

2.26   Coolant pump motor (Kw)                             To be informed by the supplier

2.27   Coolant pump discharge at max.                    To be informed by the supplier

working height (litre/min)

2.28   Total connected power load                           To be informed by the supplier

2.29   Floor space requirement                                 To be informed by the supplier

2.30   Total weight of the machine                           To be informed by the supplier

2.31   Overall dimensions                                         To be informed by the supplier

2.32   Accuracy and Surface finish of the                To be informed by the supplier

holes produced by the machine as

per International standard

3.0  BASIC MACHINE :

3.00   Machine structure and drilling head should be rigid enough to withstand the cutting forces

while working at max. drilling capacity giving specified surface finish and accuracy. There

should not be any undue vibrations or noise .

3.01   The machine should be provided with telescopic covers for machine bed.

3.02   Machine should have provision for rapid and creep traverse of arm in horizontal and

vertical direction, and also of column along the bed. Limit switches should be

provided to prevent over-travel.

3.03   Machine should have provision of arm rotation by manual / Electrical motor drive.

3.04   Spindle feed movement should also be possible manually.

3.05   Suitable spindle braking system should be provided.

3.06   The electrical controls should be through PLC of Siemens make type S7 . All drives should

be AC thyristorised / transistorized type, of Siemens / ABB or Indramat make. Motors

should be protected against over-heating. In Electrical panel all switch gear elements /

accessories should be of Siemens make only.

  - Machine should have push-buttons controlling the different movements of the machine

and Emergency push buttons on a suitably located adjustable pendant and on the machine

head in front of the operator’s chair .

  - The lubrication system should be closed circuit type to the extent possible. However, in axes where closed circuit is not technically feasible, suitable recovery system should be  offered. Details to be submitted along with the offer.

  - Details of the Coolant system such as location of the coolant sump , its capacity , passage

of the coolant to the spindle etc. , to be submitted along with the offer.

  - Interlocking should be provided to prevent dry running of the machine in case of

lubrication failure.

  - Machine should have built-in provision to disallow wrong phase sequence connection.

  - Machine should have a platform / chair for the operator from where he can comfortably operate the controls and see the spindle operations on the job.

  - All hydraulic elements should be of Rexroth / Vickss / Parher make and tandem pumps should not be used.

  - Electrical devices and machine paint shall be suitably tropicalised.

  - Standard operation and service tools shall be supplied along with the machine.

  - Work light should be provided on machine.

  - Cast iron Bed plates of size 3m x 1.5m x 300mm thk. having T-slots , sufficient to cover a working area of 9m x 6m shall also be provided alongwith the machine . Fixing of the bed plates  as  per the given drawings shall be done by BHEL .

4.0  MANUALS :

4.01	5 sets of operation and maintenance manuals of machine and its control are to be supplied in English language along with the machine. The manuals must contain detailed assembly drawings of each subassembly / assembly of machine units with marking of each element alongwith their specification / part number .

4.02	Catalogues of all purchased items shall be provided in English language.

4.03	Complete Electrical , Hydraulic , Lubrication and Coolant circuit diagrams with marking of each element alongwith their specification / make / part number , shall be provided and included in O&M Manuals.

4.04	Service documentation of the AC drives (both feed and spindle) shall be provided by the vendor.

- COLOUR :

Colour of the machine shall be Apple Green.

- TRAINING AND PREDESPATCH INSPECTION :

  - The vendor shall impart training to two persons from BHEL for a period of five working

days / person in Operation and Maintenance of the machine at their works .

  - Geometrical accuracy tests as per relevant international standards shall be performed .

  - Proving on a test piece of suitable size made of alloy steel TS.780N/mm2  shall be done by

Drilling / tapping of several holes to verify max. capacity of the machine as per technical

specifications on the cutting parameters specified by the supplier . Arrangement of suitable

test pieces and required tools shall be the responsibility of the supplier .

6.04   Machine shall only be despatched after acceptance of the test reports by BHEL.

- INSTALLATION, COMMISSIONING AND ACCEPTANCE :

  - In case of P.O. being placed , the supplier shall provide the foundation drawings

sufficiently in advance before actual despatch of the machine . The  foundation bolts /

anchoring materials / fixators  / leveling shoes etc for the final erection of the  machine

shall be provided by the supplier . Civil works as per the given drawings shall be done

by BHEL .

7.02  The supplier shall take full responsibility for carrying out the installation , assembly, start

up, testing of machine and its controls . BHEL will provide the power connection of the

required rating , the overhead Crane of the required lifting capacity and auxiliary workers

for opening of the machine boxes and cleaning of the machine only . Supplier’s technicians

shall bring their own tools and tackles as required for the commissioning of the machine .

7.03  Geometrical accuracy tests conducted and their results as per relevant international

standards for testing the machine shall be supplied along with the Machine.

7.04   Supplier  is  to  provide  first filling of  all  oils & lubricants required for the machine.

Indian equivalents of these oils to be informed  by  the supplier.

7.05  Drilling / Tapping / Boring etc. of different sizes on  Steam turbine casings in actual

working conditions  for a mutually agreed period to ascertain the trouble free performance

of the machine , shall form part of commissioning and final acceptance of the machine by

BHEL .

- ESSENTIAL SPARES:

The offer should include following essential spare parts. Quantity offered should be indicated against each item. Also, the price bid should contain item wise prices.

8.01  1 no. of each type of hydraulic pump , lubrication pump and coolant pump used in the          machine .

8.02   1 no. of each type of hydraulic motors used in the machine .

8.03   2 nos. of each type of direction control valves used in the machine.

8.04  1 no. of each type of pressure relief valves , pressure reduction valves and flow control valves used in the machine.

8.05   1 set of lead nuts used in the machine.

8.06   1 complete set of clutch plates used in the machine.

8.07   1 set comprising of all types of seals/gaskets/wipers/hoses used in the machine.

8.08   1 set of end limit switches and relays etc. used in the machine.

8.09   1 set of control module and power module of each type of drive controller for axis

and spindle used in the machine.

8.10   5 nos. push-buttons / switches of each type used on machine.

8.11   5 nos. of each type of lamps used in the machine.

8.12   2 nos. proximity switches / sensors of each type used in the machine.

8.13   5 nos.of each type of Filter inserts used in the machine.

8.14   2 nos.of each type of relays / contactors used in the machine.

8.15   2 nos.of each type of input / output modules of PLC controller used in the machine.

- OPTIONAL  SPARES :

Spares for 2 years’ three-shift trouble-free operation of the machine shall be offered. The spares should cover hydraulic, mechanical and electrical systems. Item-wise prices shall be offered against the spares.

- GUARANTEE :

The vendor shall offer performance guarantee of the machine and all its peripherals for 24 months from the date of final acceptance of the machine. The guarantee shall also cover free replacement of all the faulty parts during  guarantee period.

- TECHNICAL SUPPLY CONDITION :

The machine should be able to operate in following technical conditions :

11.01   POWER:

Machine should be suitable to operate on 3-phase A.C. supply of

415 Volts ±  15%, 50 Hz ± 3%.

  - ENVIRONMENT:

Tropical environment consists of dust-laden atmosphere during some part of the year.

Temperature	             Min. 3°C – Max.45°C

Relative Humidity	 95% Max.

- GENERAL INFORMATION :

- Detailed catalogues and photographs of the machine shall be provided along with the technical offer .

- The vendor should reproduce the Specifications SERIALLY and give Agreement / Comment  against each and every Feature / Item. Any deviation in the offer shall be clearly highlighted against that particular Item / Feature.

13.0 QUALIFYING CONDITIONS :

- The supplier should be a reputed original Machine tool manufacturer.

- The supplier should have previous experience of manufacturing conventional travelling column Radial drilling machines having drilling capacity 100MM or more and should provide reference list of customers with full contact addresses to whom such type of machine has been  supplied in the past ten years. At least one performance certificate in original from  previous customer having similar capacity machine to be provided.