Annexure –A

Frame Spirit Level

1.	Provided with two prismatic measuring bases bottom horizontal and left hand vertical, and two flat measuring bases top horizontal and right hand vertical, contact faces hand scraped for sensitivity 0.02 mm/M and better, according to Standards DIN877 + DIN2276/1.

2.	The two prismatic measuring bases bottom horizontal and left hand vertical should be provided with magnetic inserts.

3.	Simple adjustment of the zero-point and of rollover errors (twist) of the main vial, by providing a suitable ingenious adjusting system.

4.	Should have excellent view on the vial from above and from the side

5.	The main vial should be fixed in a special mounting device to make the vial              absolutely free from any tensions.

6.	Transversal vial with a sensitivity of 2-5 mm / m for easier handling of the level when measuring on horizontal shafts or in the vertical.

7.	The prismatic bases suitable for shaft diameters Ø 19 ... 108 mm.

8.	Special plastic handles on 3 sides to eliminate the transfer and influence of the     body temperature.

9.	Packed in a wooden storage case.

10.	The FRAME SPIRIT LEVEL should be suitable for measurements on vertical and horizontal surfaces and shafts.

11.	Rollover stability ± 2 degrees.

12.	Sensitivity - 0.02 mm / meter.

13.	Dimensions :  200 x 200 x 40 mm.

14.	Wt. 2.900 kg- 3.500 kg.

15.	Calibration Certificate:

•	The FRAME SPIRIT LEVEL should be delivered with an internationally recognized Calibration Certificate

Annexure B

Electronic level Measuring Instrument

1.	The Electronic level measuring system consisting of electronic measuring instruments and an LCD digital indicating unit.

2.	The high precision electronic inclination measuring instrument should be specifically   suitable for the precision measurement of smallest angles/flatness of surfaces.

3.	Applications are in particular the measurement of flatness of surface plates or the   measurement of the geometry of machine tools.

4.	The sensor of the measuring instrument, to allow precise measurements even under   critical environmental conditions.

5.	Compact and pleasant design which is functionally optimized for precision measurement.

6.	Large and well readable LCD digital display which can be read from both sides by rotating the display unit handle.

7.	Sensitivities :

10 µm/m: Range ± 200mm/m

Linearity according to DIN 2276
Display range : ± 200 mm/m

8.	Limits of error < 0.5 Full-scale (DIN 2276) max. 1% of measured value / max. 1% Limits of error > 0.5 < Full-scale (DIN 2276) max. 1% of (2 x measured value - 0.5 x Full scale)

9.	Temperature error :       /°C max. 0.1% Full-scale / max. 0.1%

10.	Settling time, value available within 3 seconds

11.	Operating temperature range    0 ... +40 °C

12.	Storage temperature range    -20 ... +70 °C

13.	Net weight, including batteries   1200 g approx.

14.	Calibration Certificate:

•	The electronic LEVEL instrument should be delivered with an internationally recognized Calibration Certificate