# BHARAT HEAVY ELECTRICALS LIMITED					Date: ____________________

Heavy Power Equipment Plant, Ramachandra puram,			    Due Date: ____________________

Hyderabad-502 032. INDIA.						  Supplier Qtn. No.: ____________________

Phones: 23182250 / 23020173								Date: ____________________

# Purchase Dept.: 040 – 23020173 / 23021910 & 23021954

E-mail; burre@bhelhyd.co.in

## TENDER DOCUMENTS TO BE FILLED BY THE SUPPLIER FOR SUPPLY OF

PULL OFF ADHESION TESTER FOR LABS (M207CG7P0058/BRK)

| ### ADDRESS OF THE SUPPLIER: |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| TELEPHONE Nos: |  |  |  |  |  |  |
| FAX Nos.: |  |  |  |  |  |  |
| E-MAIL ADDRESS |  |  |  |  |  |  |
|  |  | Name of the Part | Description of BHEL Specification | Offered | Deviation | Remarks |
| 1.0 |  | Compliance | Shall be compliant to international standards. |  |  |  |
| 2.0 |  | Portable | Shall be easy to operate and portable. |  |  |  |
| 3.0 |  | Numerical value for adhesion | Numerical value for adhesion shall be indicated on the scale. |  |  |  |
| 4.0 |  | Range of pressure | 0 – 3000 PSI |  |  |  |
| 5.0 |  | Mode of Operation | It shall be hand operated for which power is not required. |  |  |  |
| 6.0 |  | Dolly dia | Dolly dia shall be min of 20 mm |  |  |  |
| 7.0 |  | #### Inspection | The supplier shall test the equipment as per the standard inspection and test procedure of the company and demonstrate the capabilities included in this specification in the presence of BHEL representatives before dispatch. |  |  |  |
| 8.0 |  | #### Experience | The supplier shall have a minimum experience of 5 years in manufacturing Pull off adhesion testers and should have supplied at least 10 equipment of the model quoted |  |  |  |
| 9.0 |  | #### No. of Installations | The supplier shall submit a users' list for the model quoted including the names of the user contact persons, telephone numbers and e mail addresses. |  |  |  |
| 10.0 |  | #### Training | Training on operation for a period of 1to 2 days for 2 persons shall be provided. |  |  |  |
| 11.0 |  | #### Service | The supplier shall explain in detail how the after sales service will be organized. The number of service engineers trained, qualified and experienced to attend service calls and details about ready availability of spares shall be mentioned. Time taken for service engineers to reach the site upon receipt of breakdown call shall be indicated and assured |  |  |  |
| 12.0 |  | #### Safety | The equipment shall fulfill safety regulation and guidelines |  |  |  |
| 13.0 |  | #### Commissioning | Installation and Commissioning shall be carried out at BHEL works |  |  |  |
| 14.0 |  | #### Spares | Spares for 2 years of trouble free operation shall be quoted. |  |  |  |
| 15.0 |  | #### Warranty | 18 months from date of commissioning |  |  |  |
| 16.0 |  | #### Manual | 2 sets of operation and maintenance manuals shall be provided |  |  |  |
| 17.0 |  | #### AMC | Supplier shall quote the AMC charges for 5 years after warranty period. |  |  |  |