[image: image1.png]


BHARAT HEAVY ELECTRICALS LIMITED

RAMACHANDRAPURAM :: HYDERABAD – 502032

MAINTENANCE & SERVICES DIVISION
10-REPAIR SHOP, AIRCONDITIONING

Phone No.: 040--23182554  Fax No.: 040–23020185  email : ramapra@bhelhyd.co.in
TENDER NOTICE NO.  HY/M&S/10-RS/RC/07-08/VRM-50/1006/1             Date : 06-10-2007
Tender in two parts (Techno-commercial & Price Bid) will be received by the undersigned in sealed cover from the parties recognized by BHEL Corporate CMT&IP Group. The following should be legibly written on the cover -- tender notice number, tender date, name of the work, due date of opening, techno commercial bid or price bid on top of cover. Tenders will be opened at 13.30 hrs on the due date in the presence of tenderer’s or their authorized representatives in BHEL, Ramachandrapuram, Hyderabad.

1. Name of work                    :  Reconditioning of MAS, CSSR make Portable Universal Radial 
  Drilling machine Type VRM-50 (Asset No.: 51-6028) as per 
  scope of work enclosed
2. Earnest Money Deposit
:   Rs. 25,000/-

3. Estimated Cost

:   Rs 8.99 Lakhs (excluding taxes & duties, jt.inspn.spares)
4. Cost of tender documents : NIL
(non-refundable)
5. Tender sale starts from
:  Not applicable
6. Tender sale closes on
:  Not applicable

7. Last date of receipt of tenders: 20 .10.2007 upto 13.00 Hrs.

8. Date and time of opening of tenders: 20.10.2007, 13.30 Hrs.

      (techno-commercial bid only)
Name & Address of the firm :

……………………………………
……………………………………

……………………………………

……………………………………

           TENDERERS ARE REQUESTED TO SIGN AND PUT SEAL ON ALL PAGES OF TENDER DOCUMENT AND SUBMIT.
for  BHARAT HEAVY ELECTRICAS LIMITED

      






 Sr. MANAGER (M&S)








10-REPAIR SHOP, A.C.
EMD Payment : An amount of Rs.25,000/- (Rupees Twenty Five Thousands only) towards EMD shall be Deposited  in cash at BHEL Cash office or by way of Demand Draft / Bankers cheque drawn in the name of M/s Bharat Heavy Electricals Limited payable at Hyderabad and shall be enclosed to the tender bid. No other means of payment will be accepted, no interest on EMD will be payable.
         Cash office timings:   09.30 AM to 11.30 AM &   1.30 PM to 3.30 PM

TENDERING PROCEDURE
Tenders may be dropped before the due date & Time in the M&S Tender box kept in 02-Annexe (Ground floor),  M&S (P&C Section) of BHEL, R.C.Puram, Hyderabad – 502 032

The bids shall be submitted in two parts.

1. Techno-commercial bid.      

2. Price   Bid
1. Techno-commercial Bid :  The envelop shall contain the Techno-commercial Bid as per the enclosed technical bid pro-forma with relevant documents like copies of ESI, PF code, Labour license, PAN No. and EMD.  The envelop shall be super scribed “Techno-commercial bid    ------  ( name of work)”.
2. The second envelope shall contain only the price bid to be quoted. Any other information in the price bid shall not be considered and the quotation is likely to be rejected.  Price bid document shall be signed by the tenderer at the bottom of the page. The envelope shall be  sealed and super scribed “Price Bid -----------    ( name of work)”.

Both the above two envelopes shall be kept in another sealed cover. The cover shall be super-scribed  with “Quotation for ------------- ( name of work )”   and shall be addressed to Sri Sri C.Suresh Kumar, Sr.Dy.Gen.Manager (M&S)/10-RS,A.C., 02 Annexe (Ground Floor), Bharat Heavy Electricals Limited, Ramachandrapuram, Hyderabad – 502 032 to reach at or before 13.00 hrs on the due date and it should also contain Tenderer address, Tender No. & due date.
Tender documents may be downloaded from BHEL website : http://www.bhel.com. BHEL is not responsible for any postal delay. The tenders will be opened in BHEL R.C.Puram Hyderabad- 32 on tender due date at 13.30 hrs in presence of those tenderers who choose to be present.


Date of opening of price bids will be intimated after techno-commercial suitability.


The technical bid consists of technical schedule requiring documentary proof.  In case the party has not satisfied all the conditions along with documentary proof, the tender will be rejected.


Tenders received mixed with both technical & Price bid documents without individual envelop will be rejected.

BHARAT HEAVY ELECTRICAS LIMITED

RAMACHANDRAPURAM :: HYDERABAD-502 032
MAINTENANCE & SERVICES DIVISION
10-REPAIR SHOP, AIR-CONDITIONING
TECHNO- COMMERCIAL BID
Tender Notice No :  HY/M&S/10-RS/RC/07-08/VRM-50/1006/1                     Date : 06-10-2007
NAME OF THE WORK : Reconditioning of MAS, CSSR make Portable Universal Radial 
       Drilling machine Type VRM-50 (Asset No.: 51-6028) as per scope of 
       work enclosed

               THIS IS AN INDIVISIBLE WORKS CONTRACT AND THE CONTRACTOR SHALL FILL REMARKS COLUMN AGAINST ALL ITEMS 

This tender schedule must be submitted along with the General Conditions, Special conditions etc. enclosed herewith duly signed and stamped on each page with your covering letter.  Quotations should be TYPED on tender schedules only.  Any deviations / deletions etc.  should be brought out separately on your letter pad and enclosed to the tender documents.
The following are to be filled up by the Bidder:
Name of the Contractor
:

Full Address

:

Contact person

:

Phone / Fax
:

Email Id.


: 

Mobile no.


: 






   .

	Sn.
	Description
	Contractor’s Acceptance / Remarks

	1
	PF CODE NO 
	

	2
	ESI CODE NO
	

	3
	LABOUR LICENCE NO.  under section12 (1) of   R&A 1970 & Validity ( either valid or under renewal should indicate)
	

	4
	PAN No.
	

	5
	Service tax regn. no.
	

	6
	TIN No.
	

	7
	CST No.
	

	8
	Validity : 90 days from technical bid opening date.
	

	9
	EMD : Rs.25,000/-, Cash receipt / bankers cheque  Details 
	

	10
	Security Deposit clause : acceptance (See terms and conditions for details)
	

	11
	Taxes and duties : mention whether included and firm during contract period OR extra as applicable
	

	12
	PAYMENT TERMS: Progressive   payments as mentioned in the scope of work enclosed for the work completed successfully in all respects.   
	

	13
	Annual turnover during last 3 years whether meeting prequalification.                                      2004-05
                                                                2005-06

                                                                2006-07
	Rs.


SPECIFICATIONS OF MAS, CSSR MAKE PORTABLE UNIVERSAL RADIAL DRILLING MACHINE, TYPE VRM-50 (ASSET NO.: 51-6028)
MAIN DATA OF MACHINE :

Machine type


:  Portable Universal Radial Drilling Machine for assembly 

   work
Type                                        : VRM-50A

Make



:  KOVOSVIT, National Corporation Sezimovo Usti

Production Year                
: 1982
Serial No.


: 153170040

Overall length                         : 3450 mm

Width                                      : 1720 mm

Height                                     : 2970 mm
Maximum working dimensions : 6120 X 4900 X 3440 mm
Total weight                            : Approx. 6100 Kg
Main voltage for electric motors : 3-phase 415 V AC, 50 Hz

Total power input

: 10 KW

Especially adapted for     
: Drilling of oblique holes in heavy work pieces or  

                                                  assembled machines

Supplied by


:  Strojimport, Prague

TECHNICAL DATA OF MACHINE

Maximum hole diameters:

Maximum diameter when drilling solid steel of 60kg/sq.mm tensile strength…..mm 50

Maximum thread diameter in steel of 60kg/sq.mm tensile strength……………..   M 48

Main dimensions:

Maximum distance of spindle from skirt………………………………………mm 2070

Minimum distance of spindle from skirt………………………………………mm 1170

Maximum and minimum radius in end positions of spindle………………mm  2270/1370

Largest and smallest distance of spindle head in vertical position from 

floor / at pushed in spindle.  Morse 4 ………………………..……………mm      1585 / 0
Largest and smallest distance of spindle axis in hor. position from floor    mm   2725 /295

Vertical travel of rails / stroke/  …………………………………………   mm           1250
Horizontal travel of rails ……………………………………………… .    mm             900
Revolving of rails………………………………………………………                       360º

Rotation of spindlestock……………………………………………….                        360º

Tilting of head with spindlestock………………………………………                       180º

Travel of saddle on bed…………………………………………………     mm          1200

Skirt diameter……………………………………………………………    mm            400

Spindle:

Diameter of spindle head, Morse  4 ……………………………………… mm              70
Taper in spindle……………………………………… …….                                  Morse 4 

Spindle stroke , drilling depth………………………………..                     mm            350

Spindle speeds: number of steps………………………………                                       15

Speed limits in both directions………………………...                               r.p.m.  16 –800

Spindle feeds:                                                                                                number          6

                        Limits……………………… …….                                 mm/rev. 0.05 –0.5

Drive:

Main motor: power/revs………………………..                     KW/r.p.m.              4.4/1425

Raising motor: power/revs……………………...                     KW/r.p.m.              2.2/1420

Saddle feed motor: power/reves…………………                   KW/r.p.m.             1.1/1420

Clamping motor for skirt: pow./reves…………                       KW/r.p.m.                1/2800

Motor for head tilting and for travel of rails  ……                   KW/r.p.m.                1/2800

Dimensions and weights:

Dimensions of bed / length , width, height /  …                          mm      2610 x 1050 x 260

Maximum working dimensions of machine  ….                          mm   5740 x 4540 x 3440

Weight of machine………………………………                       Kg                             6100

Maximum force of feed when drilling in horizontal position

 with rail in its extreme position ……………………………      Kg                             1000

SCOPE OF WORK FOR RECONDITIONING MAS, CSSR MAKE PORTABLE
UNIVERSAL RADIAL DRILLING MACHINE TYPE VRM-50A (ASSET NO.: 51-6028)

	Sn
	SCOPE OF WORK (BHEL REQUIREMENT)


	Contractor’s offer

	  A.
	MECHANICAL :


	

	    1.
	Measurement and recording geometrical accuracies as per manufacturer’s charts and inspection of condition of the machine before dismantling at BHEL by contractor with his personnel and measuring instruments.
	

	    2.
	Transportation of the machine to contractor’s works by BHEL at BHEL cost
	

	    3.
	Complete dismantling of machine into individual parts at contractor’s works, degreasing, chemically cleaning; pre-painting of structural parts, detailed inspection of parts to confirm accuracies, identifying of spares to be replaced, joint inspection to finalise the detailed list of 1) spares to be bought out, 2) spares to be manufactured. The list has got to be approved by BHEL. Drawings of manufacturing spares and specifications for bought-out items shall be prepared and submitted to BHEL for approval. BHEL may procure and supply these items to the contractor from any other sources as well. 
	

	   4.
	Replacement of all bearings (SKF/FAG/NTN make only) irrespective of their size and present condition. BHEL reserves right to supply available bearings from their stocks for this purpose. Balance bearings shall be procured by the contractor.
	

	   5.
	Replacement of all worn out gears / other parts with new ones to I.S. standards and specified quality materials complying to the original manufacturer’s specified tolerances.
	

	   6.
	Replacement of all seals, O-rings and rubber packings.
	

	   7.
	Vertical & horizontal arm movement screw rods and nuts to be replaced.
	

	   8. 
	Head tilting worm and worm wheel to be replaced
	

	   9.
	Clutch plates to be replaced
	

	 10.
	Deep scratches are observed on the column. Column to be replaced with new one.
	

	 11.
	All slide ways / guide ways of cross arms and saddle have to be ground and blue-matched with mating parts to obtain final alignment. Pasting of Turcite-B anti-friction bearing material on all slides, wherever required
	

	 12.
	All wedges and gibs are to be renewed, scraped and matched.
	

	 13.
	Providing centralised automatic lubrication system of reputed make for axes lubrication (column and cross arms).
	

	 14.
	Providing bellow covers for column and vertical screw rod.
	

	 15.
	Cross arms locking is not working properly, to be rectified
	

	 16.
	Counter weight chain to be checked and adjusted
	

	 17.
	Column lock & unlock mechanism repair / replacement of elements 
	

	 18.
	Spindle speeds gear shifting is not proper, to be rectified.
	

	 19.
	Load is not taking while drilling. Vibration is observed while drilling. These are to be rectified
	

	 20.
	Dials, name plates, instruction plates etc. shall be replaced with new ones.
	

	 21.
	All the missing handles, levers, cover plates to be renewed
	

	 22.
	All levers and handles shall be Nickel-plated.
	

	 23.
	General assembly of machine 
	

	 24.
	PAINTING :

Machine outside body shall be painted with two coats of Apcolite (Asian paints) Premium enamel opline green colour paint. Interior of the machine has to be painted with two coats of red enamel paint. Wherever oil comes in contact with interior of the body, oil resistant paint to be used. Letters, numerals etc. will have to be painted with a different colour as required / as indicated by BHEL at the time of final painting. For electrical panel boxes the inside is to be painted with two coats of white enamel paint. Painting should be carried out according to the standard procedure for painting including cleaning and removing old paint, applying metal primer putty etc. Good finish should be ensured. Overall Machine aesthetic view should be taken care of.
	

	  B.
	ELECTRICAL :


	

	   1.
	New control cubicle (IP41) to be provided. Location of control cubicle on the machine to be shifted slightly upwards to ease removal of cross arms screw rod during maintenance.
	

	  2.
	Replacement of operating station (pendant)
	

	   3.
	All L.T. switchgear items like MCBs, contactors, control relays, MPCBs, switches, pushbuttons and OLRs shall be replaced with SIEMENS make only.
	

	  4.
	All fuses and overload relays to be replaced with motor protection circuit breakers. Necessary modification to be done in the circuit.
	

	   5.
	Rotary / program switches should be of Jyothi / Kaycee / Salzer make. Other electrics should be of standard make approved by us.
	

	   6.
	Electrical control panel wiring is to be done as per the original drawings supplied by BHEL.
	

	   7.
	Wiring of the machine i.e. among electrical panel, pendant station, motors, magnet, limit switches etc.
	

	   8.
	Energy meter to indicate and record total consumption of electric power (cumulative consumption also) for the machine including auxiliary systems.
	

	   9. 
	All the conduits must be Steel reinforced PVC and of reputed make. Wiring accessories shall be reputed make.
	

	 10.
	Indication lamps and indicating instruments, wherever required, to be provided / replaced.
	

	 11.
	Alnico (spindle braking system) to be replaced. Change of alnico system to better system with necessary modifications is invited.
	

	 12.
	Replacement of machine lamp and its wiring
	

	 13.
	Replacement of Limit switches of reputed make
	

	 14.
	Repair of electromagnet and its mechanism
	

	 15.
	All motors are to be serviced and bearings to be replaced with new ones. Insulation values to be improved. If necessary the motors are to be rewound. Terminal blocks & covers, fans, end covers to be provided if missing. Damaged key ways on shafts to be rectified or new key ways to be made. Satisfactory no-load test shall be carried out on all motors.
	

	 16.
	All cables other than in-coming are in the scope of contractor and reputed make (IS standard) shall be used.
	

	 17.
	All equipment must be suitable to run with input voltage of 3 wire, 3 phase, 415 volts ± 10%, 50 Hz ± 2 Hz.
	

	 18.
	All equipment shall be tropicalised and suitable for operation in shop floor conditions, where temperature variation is between 10° to 45°C.
	

	 19.
	All the electrical equipment on the machine shall be properly earthed and the earth point shall be brought to the supply mains terminal box of the machine for further connecting to the earth wire of the incoming cable.

Earthing to be complied with as per IS for the complete machine.
	

	  C.
	Prove out of the reconditioned machine at Contractor works :


	

	    1.
	Checking of geometric accuracies as per original test chart supplied by BHEL. These accuracies shall be brought back to 100% of the original accuracies mentioned in the original test chart.
	

	    2.
	Supplier has to prove out all original functions of the machine to the satisfaction of BHEL.
	

	    3.
	Two standard test pieces have to be machined to check the accuracy of the job from reconditioned machine. All these tests should be witnessed by our representatives and certified.
	

	   D.
	Re-commissioning & Prove out at BHEL Works: 


	

	    1.
	Contractor has to dispatch the reconditioned and tested machine by BHEL approved transporter with proper care. Transportation cost to BHEL account. 
	

	    2.
	The complete machine is to be re-commissioned at BHEL works by  the contractor. The reconditioned machine shall be proved out as in clause C1—3. 
	

	   E.
	DOCUMENTATION :


	

	   1.
	Electrical drawings of the control panel, pendant station and Cabling drawings
	

	   2.
	Mechanical drawings of the replaced parts, modified parts and manufactured parts
	

	   F.
	GENERAL CONDITIONS :  


	

	    1.
	The machine is to be reconditioned at contractor works. Contractor personnel will be allowed to work inside BHEL premises only during checking of machine before taking for reconditioning, during re-commissioning and any maintenance work during the guarantee period.
	

	    2.
	Transport & transit insurance both ways in BHEL scope.
	

	    3.

  
	All works connected to guideways of all axes (eg. Slideways, gibs), repair by machining (grinding, milling, hand scraping), suiting with turcite material and matching are also covered under labour charges. However, bought-out items like turcite etc. are covered under spares.
	

	    4.
	SPARES :

a) The spare parts (electrical / mechanical) required to be replaced (manufacturing spares and bought-out spares) will be to BHEL account. Contractor has to quote for these spares along with manufacturing drawings for manufacturing items and specifications for bought-out items for approval. BHEL will supply spares available with them. BHEL may procure and supply these items to the contractor from any other sources as well. Spares not supplied by BHEL shall be purchased / manufactured by the contractor.                                                                                                                                                         

b) Spares required to be replaced other than those agreed to during joint inspection will have to be approved by BHEL before such replacement is effected. Cost of such spares shall be absorbed by the contractor. BHEL will not pay for these items. 

c) Prior intimation has to be given to BHEL by the contractor for deputing the team for joint inspection at contractor works.

d) Contractor must submit the quotation for joint inspection spares within a week of joint inspection


	

	   5.
	REWORK IN LIEU OF REPLACEMENT :

Reworks in lieu of replacement on all parts (guideways / sliding surfaces like spindle, bushes, box nuts, screw rods etc.) by machine grinding preferably/chrome plating / welding / machining etc. identified initially and also during joint inspection are covered under labour charges only. For such works BHEL will not pay extra.

Repair of spindle (if required) by machining, chrome plating etc. are also covered under labour only.
	

	   6.
	BANK GUARANTEE / ALL RISK INSURANCE COVERAGE AS SECURITY DEPOSIT (for safe custody of machine and material) :

The contractor has to submit Bank Guarantee / All risk insurance coverage for Rs.12.00 lakhs for safe custody and as security deposit for the period of machine and materials being stationed at contractor works. This bank guarantee will be released after receipt of reconditioned and tested machine at BHEL works. Any damage / loss shall be made good by contractor at their cost.


	

	   7.
	EMD (Earnest Money Deposit) :

Earnest Money Deposit of Rs.25,000/- shall be deposited to BHEL account by the bidder along with the offer. In the event of acceptance of full or part tender, and if the contractor/bidder fails to commence the work with in promised period,  BHEL shall without prejudice to any other right  or  remedy be at liberty to forfeit the said earnest money  absolutely. EMD does not carry any interest.
	

	   8.
	SECURITY DEPOSIT FOR WORK :

Successful bidder has to pay 10% of the total contract value to BHEL as security deposit before commencement of the work. This amount shall be returned after successful completion of the contract work. The security deposit shall not carry any interest


	

	    9.
	TIME OF COMPLETION : 
a) One week from the date of issue of work order for checking and recording the machine accuracies.

b) Two weeks for transporting the dismantled machine by BHEL

c) Twenty weeks for reconditioning the machine, from the date of receipt of the machine at contractor works to the date of inspection and acceptance of reconditioned machine at contractor works. 

d) Two weeks for transporting the reconditioned machine to BHEL works by BHEL.

e) One week for proving out the reconditioned machine at BHEL works by the contractor.


	

	 10.
	PAYMENT TERMS:

1. 30% of Part-A order (i.e. 30% of labour cost and 30% of initial spares cost) shall be paid after joint inspection of fully dismantled machine for 1) identification of spares and 2) deciding detailed scope of repair / rework on various components / parts and submission of quotation for joint inspection spares. This is subject to submission of Bank guarantee for like amount valid for the period till the reconditioned machine is handed over to Production at BHEL works.

2. 60% of labour cost and 60% of initial spares cost (both Part-A and Part-B together) along with 100% taxes and duties at actuals (against submission of documents to BHEL for reimbursement of the same from authorities concerned) shall be paid after prove out and handing over the reconditioned machine to Production at BHEL works. 

3. Joint inspection spares shall be ordered separately (Part-B). However, 90% of Part B order shall be paid after prove out and handing over the reconditioned machine at BHEL works

4. Balance 10% of Part-A & B orders shall be paid after submission of Performance Bank Guarantee of like amount valid for the guarantee period of 12 months (two-shift basis).  
	

	 11.
	PENALTY: 

0.5% of total order value per week or part thereof to a maximum of 10% of total order value shall be levied beyond the delivery time of twenty weeks (Clause F.9.c).
	

	  12.
	PERFORMANCE BANK GUARANTEE :

a) Satisfactory performance of the reconditioned machine shall be guaranteed for a period of 12 months (on two shift basis) from the date of handing over the machine to production at BHEL works. 

b) During the guarantee period contractor shall attend to the machine / replace failed parts free of charge as and when called upon to do so for defects / failures noticed and attributable to contractor’s work & supplies (both manufactured and bought-out by them)

c) The discarded parts (old) during the guarantee period shall be taken back by contractor at its expenses. Repairs, if any, shall be carried out either at BHEL / contractor works as agreed mutually but cost of transportation both ways shall be on contractor’s account. BHEL will not pay for this.
	

	 13.
	Bar chart showing the time required for the following elements to be given covering the period of delivery quoted

a) Complete dismantling, cleaning and Joint inspection of components at contractor works to identify parts for replacement and rework.

b) Manufacturing, rework / repairing etc. of the components, completion of scraping work and sub-assemblies

c) Complete reconditioning work

d) Assembly of the machine

e) Geometrical accuracy testing

f) Job trial and prove out

g) Re-commissioning at BHEL works

h) Geometrical accuracy prove out

i) Job trial and prove out
	

	   14. 
	General tools / instruments / gauges required for

a) Checking and packing of the machine at BHEL works for transporting the same to contractor works for reconditioning.

b) Re-commissioning and prove out of the reconditioned machine at BHEL works

shall be arranged for by the contractor. Special tools / instruments / gauges for these works will be provided by BHEL.

All these instruments / gauges should have valid calibration certificates from recognised agencies.
	

	 15.
	BHEL reserves the right to inspect the progress as well as workmanship at any stage of the work Decision of BHEL Representative shall be final in the matter of inspection at any stage. Contractor shall extend necessary co-operation to enable such inspection
	

	 16.
	All discarded parts shall be returned to BHEL
	

	 17.
	MACHINE will be kept in open condition. No air conditioner is provided for this machine. The machine shall be given specified accuracies in these conditions only.
	

	  18.
	General terms and conditions of BHEL as per enclosure shall be acceptable.
	

	  19.
	Any other activity (electrical and mechanical) not mentioned in this scope of work to bring back the machine to satisfactory working condition with all features and accuracies after reconditioning is in the scope of  contractor
	

	  20.
	The contractor is required to study the machine and the scope of work and satisfy himself about completeness of the scope in order to submit his offer. In case of any discrepancies in the scope of work or material supply the contractor is required to highlight the same to BHEL in his offer
	

	  21.
	The contractor may visit our works to see the machine, which is presently in working condition.
	

	  22.
	Lodging & Boarding facilities for the contractor personnel are at Contractor’s scope (during dismantling period, re-erection & re-commissioning period and for troubleshooting during performance guarantee period). BHEL is not responsible for this.
	

	  23.
	Material handling facilities such as crane, battery truck, slings, lifting tackles etc. along with operator, consumables like cleaning solvents, cotton waste etc. and utilities like electric power, compressed air etc. shall be provided by BHEL for the works performed at BHEL works
	

	  24.
	The enclosed Electronic Funds Transfer Format may be submitted along with the quotation, for ease of payments.
	


RECONDITIONING OF PORTABLE UNIVERSAL RADIAL

DRILLING MACHINE TYPE VRM-50 (BHEL ASSET NO.: 51-6028)

PRICE BID FORMAT

PART - A

	S.No.
	Description
	Price Rs. lakhs
	Total cost Rs. lakhs

	A.
	LABOUR : (Activities A to F of the scope of work including Engineering & Design charges)
	
	

	A.1.1
	Mechanical Engineers—       __ nos. x __ months
	
	

	A.1.2
	Electrical Engineers   ---       __ nos. x __ months
	
	

	A.1.3
	Mechanical Skilled Artisans – __nos. x __months
	
	

	A.1.4
	Electrical Skilled Artisans    -- __nos.x __ months
	
	

	A.1.5
	Unskilled labour           -- __ nos. x __ months
	
	

	
	Total
	
	

	
	Add Overheads @  ___%
	
	

	
	Total cost (A1 ): 
	
	

	
	Column sleeve machining charges including tooling (A2)
	
	

	
	Total labour cost (A) = A1 + A2
	
	

	B.
	Material :
	
	

	B.1
	Initial spares
	
	

	B1.1
	Mechanical
	
	

	B1.1.1
	Bearings as per list enclosed(A4)
	
	

	B1.1.2
	Centralised lubrication systems for column and cross arms lubrication, including piping, fittings etc.(A13)—2 sets
	
	

	B1.1.3
	Bellows for column & vertical screw rod (A14)
	
	

	B1.1.4
	Turcite-B material (A11, A12, F3)
	
	

	B1.1.5
	Column (A10)
	
	

	B1.1.6
	Cross arms screw rod and nut (A7)
	
	

	B1.1.7
	Vertical lead screw rod and nut (A7)
	
	

	B1.1.8
	Head tilting worm and worm wheel (A8)
	
	

	B1.1.9
	Clutch plates (A9)– 2 sets
	
	

	B1.1.10
	Others (painting material (A24), rubber items (A6), dials, name/instruction plates (A20), levers, handles, cover plates (A21), electroplating (A22), documentation (E1, E2)
	
	

	B1.2
	Electrical
	
	

	B1.2.1
	Control cubicle (B1) & panel incl. all electrical components(B3, B4),  operating station (B2), Alnico(B11), rotary/program switches (B5) etc.
	
	

	B1.2.2
	Wiring & wiring accessories (B6,B7,B9, B16,B19)
	
	

	B1.2.3
	Limit switches(B13)
	
	

	B1.2.4
	Others like machine lamp(B12), indication lamps & indicating instruments(B8, B10), etc.
	
	

	
	Total  initial spares cost (B 1) = B1.1 + B1.2
	
	

	
	Grand Total (A+B1)
	
	

	C
	Taxes & duties                                   (extra / inclusive)
	
	


Signature of the Bidder

Note :

1. References mentioned in parentheses in description column of the above format refer to clauses in the scope of work enclosed.

2. The bidder should undertake careful study of machine before offering rates under categories A to C of above format. 

3. L1 party (total lowest cost to BHEL) shall be decided based on quotations received in the above format only (part-A of price bid format both labour and initial spares together and joint inspection spares excluded).

4. Price validity shall be for ninety days from tender opening date.

5. Joint inspection spares shall be identified after total dismantling of machine at party’s works  and Part-B order will be issued for these  identified parts at mutually agreed values if found reasonable.

6. Excise duty and CST will be paid on Spares cost and service tax will be paid on Labour charges, at actuals. Education cess also will be paid. Documents concerned to be submitted along with bill.
7. Transportation and transit insurance both ways will be borne by BHEL.

PRICE BID FORMAT FOR BEARINGS TO BE REPLACED ON MAS, CSSR MAKE PORTABLE UNIVERSAL RADIAL DRILLING MACHINE, TYPE VRM-50
(ASSET NO.: 51-6028) DURING RECONDITIONING

	S.No
	Bearing No
	Qty(nos.)
	Unit rate, Rs.
	Total Cost, Rs.

	1
	6000
	2
	
	

	2
	6001
	3
	
	

	3
	6002
	3
	
	

	4
	6003
	2
	
	

	5
	6004
	1
	
	

	6
	6005
	6
	
	

	7
	6006
	2
	
	

	8
	6007
	4
	
	

	9
	6008
	2
	
	

	10
	6010
	1
	
	

	11
	6012
	1
	
	

	12
	6019
	1
	
	

	13
	6203
	2
	
	

	14
	6204
	6
	
	

	15
	6205
	16
	
	

	16
	6206
	7
	
	

	17
	6207
	2
	
	

	18
	6208
	1
	
	

	19
	6209
	3
	
	

	20
	6218
	1
	
	

	21
	6304
	1
	
	

	22
	6305
	1
	
	

	23
	6306
	1
	
	

	24
	6309
	1
	
	

	25
	7204
	1
	
	

	26
	7208
	1
	
	

	S.No
	Bearing No
	Qty(nos.)
	Unit rate, Rs.
	Total Cost, Rs.

	27
	30204
	2
	
	

	28
	30205
	5
	
	

	29
	30207
	1
	
	

	30
	30208
	3
	
	

	31
	32207
	2
	
	

	32
	32208
	1
	
	

	33
	51105
	1
	
	

	34
	51107
	1
	
	

	35
	51120
	1
	
	

	36
	51205
	3
	
	

	37
	51208
	1
	
	

	38
	51217
	1
	
	

	39
	51307
	2
	
	

	40
	6204ZZ
	4
	
	

	41
	6205ZZ
	2
	
	

	42
	6206ZZ
	1
	
	

	43
	6306ZZ
	3
	
	

	
	
	
	TOTAL Cost, (Rs.) 
	


Signature of the Bidder

ELECTRONIC FUND TRANSFER FORMAT

(To be furnished on Vendor's Letter head)
Ref  No. :
Date       :

To

DGM/Finance-Mon
Bharat Heavy Electricals Limited

Ramachandrapuram

Hyderabad 502 032

Dear Sir,




Sub: Details for Electronic Fund Transfer

The following information is furnished for crediting our Bank Account with the amounts payable against our bills.

Supcode (As per PO/SCO)
:

Beneficiary


:

City



:

Bank



:

Branch 



:

A/c Number


:

A/c Type (Savings or Current)
:

MICR Code of the branch

:

e-mail address


:

PAN



:










(Signature with Seal)









Authorised Signatory








Name :









Designation :
BHEL TERMS AND CONDITIONS
A) CONTRACTUAL :

1) The Contractor will ensure that the employees deployed by him in the premises of BHEL are physically and mentally fit and do not have any criminal record. 

2) The Contractor will maintain records of his employees deployed to carry out the job. The Contractor will provide employment card/Identity card with photograph duly attested by him to his employees.

3) In the event of termination of contract for any reason whatsoever, the contractor shall withdraw all his employees from the establishment of BHEL.

4) The Contractor will obtain insurance cover for his employees and take third party risk insurance coverage at his own cost.

5)
The Contractor will be solely entitled to dictate such workers about the manner of carrying out the work. 
6)
The Contractor or his authorized representative will supervise the work allotted to him and being carried out by his employees or will post a Supervisor for this purpose.

7) The Contractor will be responsible for the good conduct of his employees. In case of any misconduct /   misbehavior by any employee, contractor will replace such employee immediately.

8) The Contractor will keep watch on his employees and he will be liable for any pilferage / loss to BHEL due to acts of omission and commission by his employees. Similarly, liability for any compensation to outsiders on account of any act of omission and commission by the employees deployed by the contractor shall lie exclusively with him.

9)
The Contractor will ensure that all precautions are taken for the safety of his employees.

10)
The Contractor will provide to his employees all tools, tackles and equipments required to carry out the job under the contract at his own cost.

11)
The Contractor will provide safety appliances at his own cost which may be required under the statute or otherwise.

12)
The Contractor will provide all material at his own cost as mentioned in the contract to his employees for carrying out the job.
13)
Transportation facility for the employees to come to BHEL works is in the scope of the contractor.

14)         Consumables like air, water, power supply will be supplied by BHEL free of cost. 
STATUTORY :
14) The Contractor shall comply with all statutory requirements, rules, regulations, and notifications in relation to employment of his employees issued from time to time by the concerned authorities. 

15)   
 The Contractor shall provide PF passbook to his employees and ensure payment of PF, EDLI, and pension dues under EPF & MP Act, 1952 to the RPFC.  

16)          The Contractor shall produce proof of deductions as well as remittances of PF, EDLI, Pension, ESI 
               contribution, administrative charges etc. wherever applicable and shall maintain proper records.

17)     
 The Contractor shall furnish proper returns to the concerned statutory authorities. 

18)     
 The Contractor shall be solely responsible for non payment/delayed payment of wages, contributions 
               under EPF & MP Act, ESI Act etc. 

19) In case the contractor fails to make payment of wages to his employees or remittance of contribution to the concerned authorities, the security deposit/other dues under the contract can be utilized by BHEL to discharge the liability of the contractor.

20)
The Contractor shall indemnify BHEL against all claims and losses under various statutes or any civil or criminal law in connection with employees deployed by him.

21)
The liability for any compensation on account of injury sustained by an employee of the contractor will be exclusively that of the contractor.

22)
The Contractor shall obtain license under CL(R&A) Act, 1970. 

GENERAL :

1) The contractor shall pay minimum wages to workers as per minimum wages act. Contractor shall make PF, ESI contributions as per applicable rules. ESI Cards should be arranged to contract labour and their wards by the contractor. These records need to be preserved for a period of at least 3 years and made available even after the contract is over for any verification by the statutory authorities.
2) Contractor shall provide personnel protective equipment like safety uniform in blue colour, safety shoe in black colour and should ensure that they wear them daily while on work. 

3) Contract operates on Unit rates, contract labour should get at least Daily Unit rate as per the minimum wages act from the contractor. 

4) Contractor should pay double amount to the contract labour, if they are engaged more than 8 hours (normal).
5) Permissions for labour and materials shall be obtained in the prescribed formats. 
6) Break:  contract labour should be given break period for about 15 days once in every 6 months in case of long duration contracts, for one time/ short period contracts this clause will not apply

7) The contractor is wholly responsible for any loss of life or partial disability of any of their labour while on duty

8) In case of occurrence of any accident/ injury of contractor’s staff, BHEL will not pay any           compensation while they are on duty and contractor has to take care of health of workers under statutory obligation.

9) BHEL reserve the right to cancel the work order in part or in full without assigning any reason.

10) Upon awardal of the work, the contractor has to execute an agreement with BHEL on a non-judicial stamp paper worth Rs.100/-  as per General Agreement Conditions before commencement of work.
11) BHEL shall have the privities of the contract with the contractor only and will give instructions to the contractor or his authorized representative. BHEL will have nothing to do or be concerned with the employment of employees working for the contractor.  The relationship between BHEL and the Contractor will be that of independent entities and nothing herein contained will amount to joint venture, partnership or an employer-employee relationship.
12) The contractor shall maintain regular contact with the designated employee(s) of BHEL and will   interact on matters relating to the work awarded under this contract.
13) In case the Contractor does not carry out the Contractual/Statutory obligations or the services rendered by him are found to be unsatisfactory, BHEL shall bring the same to his notice and he will be obliged to discharge the obligations and rectify the efficiency/anomaly within three days time failing which, BHEL reserves the right to terminate the contract without assigning any reason whatsoever. In such an event, no damages will be payable for short closure of the contract. 
14) Not withstanding anything contained in the contract Agreement which will be entered after awardal of work, the contract may be terminated by BHEL without assigning any reason thereof by giving a notice of 30 days to the contractor. 
15) The Entry permits are to be issued to the Contract Labour by Assistant Commandant / CISF(Plant), based on the requisition submitted by Contract Executing Officer and forwarded by Dy.General manager / Personnel.

16) Every contractor shall submit a notice regarding commencement and completion of work in form – VI A&B [Rule 25 (VIII) & 81 (3)] to HR Department, IR section through his contract executing officer, for forwarding the same to State Labour Department. 

17) Non – compliance of any provisions under the act / rule / instructions / guidelines shall make the contractor liable for penal action including termination of contract.

18) Contractor shall furnish in a separate letter his place of residence and postal address. The delivery at the above named place or posting in a Post box regularly maintained by the post and telegraph Department or sending letters registered for acknowledgement of any notice,  letter or other communication to the contractor. Change in address shall come into force at any time by an instrument executed by the contractor and delivered to the Company official who has signed the contract.

19) The contractor must satisfy himself by personal study and examination of the drawings / specifications furnished and understand thoroughly the scope of proposed work in detail and all conditions affecting the work before entering into the contract. There shall not be at any time dispute / complaint of any misunderstanding with regard to scope of work and interpretation of specifications or any misunderstanding with regard to nature or omission of the work to be done not shall any application for compensation in terms of time and money shall be accepted by company regarding the above.

20) Contractor shall in his absence keep competent agent constantly on the works and any directions or explanations given by the “Contract Signing Officer” or his representative to such agent shall be held to have been given to the contractor himself.

21) It shall be contractor’s sole responsibility to protect the public and his employees against accident from any cause and provide required safety equipments and shall indemnify, the company against any claims for damages for injury to the person or property resulting from any such accidents and shall, where the provisions of the workmen’s compensation act apply, take steps to properly insure against any claims there under. 

22) In the event of any accident in respect of which compensation may become payable under the workmen’s compensation act VIII of 1923 whether by the contractor or by the company as principle employer, it shall be lawful for the company to retain out of monies due and payable to the contractor such sum or sums of money as may, in the opinion of the company shall be final in regard to all matter arising in this clause.

23) No work shall be done on Sundays or on other declared holidays of the company without the written permission of the company officer in-charge of the work. The contractor shall comply with the provisions of the Factories Act Rules framed there under if the same are applicable.

24) The contractor shall keep his work place clean and safe to avoid injuries to men and damage to finished products / equipments.

25) On the occurrence of an accident, which results in the death of any of the workmen employed by the contractor or which is so serious as to be likely to result in the death of any such workmen, the contractor shall intimate within 24 hours of the happening of such an accident intimate in writing to the company official in-charge of the work.

26) The contractor shall indemnify the company against all losses or damages sustained by the company resulting directly or indirectly from his failure to give intimation in the manner aforesaid including the penalties or dues if any and become payable by the company, as a consequence of failure, the company to give notice under the workmen’s compensation act or otherwise confirm to the provisions of the said act in regard to such accident.

27) The contractor shall ensure adherence to all statuary requirements applicable to BHARAT HEAVY ELECTRICALS LIMITED, RAMACHANDRA PURAM, HYDERABAD-502032.

28) Not withstanding the clause, in case of any financial loss incurred by the company on account of contravention of the Provident Fund regulations or any regulations of rule touching the same by the contractor, the contractor shall submit an undertaking to indemnify the company to the extent of the loss incurred by the company.

29) The contractor should engage only those labourers who shall be more than 18(eighteen) years of age.

30) The contractor shall not resort to subcontracting under any circumstances. If found subcontracting at a later date, BHEL reserves the right to take whatever action it deems fit, including cancellation of the contract.

31) The contractor shall be responsible to settle any grievances of the labour deployed by him. 

32) Whenever the term “CONTRACTOR” is used, it shall be understood to refer to the particular person, firm or corporation with whom an agreement has been made by the concerned company official for executing the work defined in the concerned agreement and for the purpose of instructions regarding compliance with contract conditions, it shall include the contractor’s authorised agents, who are entrusted with the work by contractor.

33) The quantities mentioned in the agreement schedule are worked out from the relevant drawings in the company and may not be the actuals required for execution. The company does not expressly or by implication agree that the actual amount of the work to be done shall correspond therewith, but reserves the right to increase or decrease the quantity of any class or portion of the work as he deems necessary.

34) All the works shall be carried out in accordance with the directions and to the satisfaction of the company official in accordance with the drawings, specifications and instructions.  Supplementing or explaining the same as may from time to time shall be done by the company official.

35) For all modifications, omissions or additions to the approved drawings and specifications, the company shall issue revised plans or written instructions and no modifications, omission or addition shall be made unless at any time before completion of the work. 

36) All materials, articles, and workmanship shall be the best of their respective kinds for the class of work described in the contract specifications and schedule and materials to be obtained by the contractor shall be from sources approved by the company.

37) Sample of materials shall be furnished by the contractor at his expense to the company when called for before execution of any work.

38) The company shall have power to reject at any stage, any work which is considered to be defective in quality of materials of workmanship and  shall not be debarred from rejecting wrong material by reasons of having previously passed them in an unworked condition.  Any portion of the work or materials rejected or pronounced to be inferior or not in accordance with specifications shall be taken down and removed from the work site at the contractor’s expenses.  

39) The decision of the company shall be final and binding on the contractor on all technical questions which may arise in the contract with respect to material, workmanship, removal; of improper work, interpretation of the working drawings, specifications, notes, procedures etc.

40) The decision of the “Contract Signing Officer” or any officer deputed by him duly authorised on his behalf, in respect of the rate of progress and the quality of work or material shall be final.

41) Contracts shall be deemed to have included in his tender price of all the plant, machinery and appliances required for the purpose of all operations connected to secure a satisfactory quality of work within the opinion of the company will ensure the completion of the work within the time specified. 

42) It is open to the company to lend or supply to the contractor any tools, implement, material and machinery that are needed by the contractor, which in the opinion of the company is essential to improve the quality or progress of work.  The contractor shall pay such deposit or charges that may be fixed or determined by the company for any such articles or machinery lent or hired to the contractor.

43) All rubbish as it accumulates shall be removed from the site to the point of disposal as indicated from time to time by the company. If this is not adhered to, the company shall remove the scrap and recover the cost of these operations from the bills of the contractor. 

44) Any sum due from the contractor on account of tools and plants, stores, or any other items provided by the company shall be deducted from the respective bill due to him.

45) In case the contractor does not execute the work awarded to him as per the terms and conditions of the agreement, the same shall be got executed by some other contractor and the excess cost incurred shall be recovered from the contractor’s security deposit/pending bills. 

46) In case of breach of any of the terms and conditions of the contract, a claim shall be lodged under the bank guarantee with the guarantor at the discretion of the competent authority of the company.  On successful completion of the contract to the satisfaction of the company, the bank guarantee shall be released after due claim period. 

47) In the event of any question or dispute or under this contract, the same shall be referred to a competent Authority in the company for sole arbitration and his decision shall be final and binding on the parties to the contract.

48) In case of any suit or other legal proceeding arising under this contract, the courts at Sangareddy (Medak Dist.) A.P only shall have the Jurisdiction. 

49) Disputes, grievances between the contractor and his labour, will have to be settled by the contractor only.

50) The labour employed by the contractor, if found in abetting and fellow labour or contractors or any BHEL employee, the same shall be considered as an act of indiscipline.  Such labourers shall be removed from the services of the contractor, on the advice of BHEL.  Further the contractor shall initiate every necessary action in accordance with the relevant Laws, Rules & Regulations and enactments of State and Central Government.

51) Wherever, BHEL/COMPANY standards are mentioned shall be strictly followed.

52) Nothing contained in the contract agreement and its conditions shall be deemed to preclude or render inoperative the service of any notice, letter or other communication upon the contractor personally. 

53) The decision of the “Contracting Signing Officer” shall be final and binding on the contractor on all technical questions which may arise touching the contract with respect to the quality of material, workmanship, removal of improper work, interpretation of the working drawings, specifications, notes, procedures etc.
54) All disputes arising in connection with the contract shall be settled by mutual    consultation.  If no agreement is reached the dispute shall be settled in accordance with the provisions of the Arbitration and Conciliation Act, 1996 and the rules made there   under.  The dispute shall be referred for arbitration to any arbitrator to be appointed by the   Head of the Unit.  The award of the arbitrator shall be final and binding on both the   Parties.  The venue of the Arbitration shall be Hyderabad in India.  The Award to be given   by the Arbitration shall be a speaking award.  All questions, disputes, differences arising under, out of or in connection with this contract shall be to the exclusive jurisdiction of Sangareddy Courts.

PAGE  
21/21

