|  | DESCRIPTION | BHEL SPECIFICATION | SUPPLIER OFFER | DEVIATIONS/COMMENTS |
| --- | --- | --- | --- | --- |
| A | MAIN PARAMETERS | INDENT NO: CG7P0144 |  |  |
| 1.0 | Type of Crane | Single girder box type EOT as per IS 3177:1999 for Indoor use (Hoist as per IS 3198) |  |  |
| 1.1 | Class & duty | M5 as per IS 3177:1999 |  |  |
| 1.2 | Main Hoist capacity | 1 Ton |  |  |
| 1.3 | Auxiliary Hoist capacity | Nil |  |  |
| 1.4 | Span | 16.75 Metres |  |  |
| 1.5 | Operation Speeds (maximum) |  |  |  |
| 1.51 | Main Hoist | 6 meters/minute |  |  |
| 1.52 | Inching Speed (Micro for main hoist) | 0.6 meters/minute |  |  |
| 1.53 | Auxiliary Hoist | NA |  |  |
| 1.54 | Inching Speed ( Auxiliary hoist) | NA |  |  |
| 1.55 | Long travel | 30 meters/minute |  |  |
| 1.56 | Cross travel | 15 meters/minute |  |  |
| 1.6 | Working clearance distance from centre of gantry rail to nearest obstruction (sideways) | 300 mm |  |  |
| 1.7 | Vertical distance from top of gantry rail to lowest overhead obstruction (maximum) | 1.0 Metre |  |  |
| 1.8 | Distance from floor to top of gantry rail | 5.0 Metres |  |  |
| 1.9 | Lift Height |  |  |  |
| 1.91 | Main Hoist (above ground) | 5.0 Metres |  |  |
| 1.92 | Main Hoist (below ground) | Nil |  |  |
| 1.93 | Auxiliary Hoist (above ground) | Not applicable |  |  |
| 1.94 | Auxiliary Hoist (below ground) | Not applicable |  |  |
| 2.0 | Hook Apprach |  |  |  |
| 2.01 | Main Hoist | DSL side 2000 mm, Opposite side 1500 mm |  |  |
| 2.02 | Auxiliary Hoist | Not applicable |  |  |
| 2.1 | Size of gantry rail (Long Travel) - Rail is part of BHEL scope | 90 lbs/yard |  |  |
| 2.2 | Size of CT Rail | CR 60 / Square bar |  |  |
| 2.3 | LT wheel load in Tonnes | Indicate |  |  |
| 2.4 | Number of Trolleys | One |  |  |
| 2.5 | Width of Crane | Supplier to indicate actual value in meters |  |  |
| 2.6 | Maximum clearance from rail to bottom of cabin | NA |  |  |
| B | CONSTRUCTION/STRUCTURAL DETAILS |  |  |  |
| 3.1 | Main Hoist hook type | Shank Hook |  |  |
| 3.2 | Auxiliary Hoist hook type | Not applicable |  |  |
| 3.3 | Hook latch for Main hoist | Required |  |  |
| 3.4 | Hoist wire rope | 6/36 or 6/37 construction as per IS:2266-1989 |  |  |
| 3.5 | Number of falls Main Hoist | 4 or as per manufacturer's standard |  |  |
| 3.6 | Number of falls Auxiliary Hoist | Not applicable |  |  |
| 3.7 | LT Drive | Twin corner drive with horizontal type gear box |  |  |
| 3.8 | Number of wheels for LT drive | 4 |  |  |
| 3.9 | Diameter of wheels for LT drive | > = 400 mm |  |  |
| 3.10 | Number of driven wheels for LT drive | 50% or more |  |  |
| 3.11 | CT Drive | With vertical gearbox/as per manufacturer's standard |  |  |
| 3.12 | Pump/motor with starter and timer for automatic lubrication of CT gearbox | Pump/motor to be provided |  |  |
| 3.13 | Platform for approach for maintenance of CT gearbox | Required |  |  |
| 3.14 | Number of wheels for CT drive | 4 |  |  |
| 3.15 | Diameter of wheels for CT drive | > = 400 mm |  |  |
| 3.16 | Type of bearings for LT/CT wheels | Anti-friction |  |  |
| 3.17 | Type of mounting of LT wheels | "L" type brackets |  |  |
| 3.18 | Variable speed control |  |  |  |
| 3.18.1 | LT Drive | Through VVVF drive |  |  |
| 3.18.2 | CT Drive | Through VVVF drive |  |  |
| 3.18.3 | AH Drive | NA |  |  |
| 3.18.4 | MH Drive | Through VVVF drive |  |  |
| 3.18.5 | MH micro speed | Through VVVF drive |  |  |
| 3.19 | Operations required through |  |  |  |
| 3.19.1 | Cabin | NA |  |  |
| 3.19.2 | Pendant | MH, LT & CT and micro speed for MH |  |  |
| 3.19.3 | Radio Remote Control | MH, LT & CT and micro speed for MH |  |  |
| 3.20 | Radio Remote Control | Should have 3 speed steps of operation. The unit should work with rechargeable batteries. Battery charger to be provided along with the unit. (Type of RRC - CC403 of SNT controls). |  |  |
| 3.21 | Number of speed steps through |  |  |  |
| 3.21.1 | Cabin | Not applicable |  |  |
| 3.21.2 | Pendent | Two steps |  |  |
| 3.21.3 | RRC | Three steps |  |  |
| 3.22 | Cabin details | Not applicable |  |  |
| 3.23 | Pendent | Shall be movable type |  |  |
| 3.24 | Duty cycle | 40% CDF all motions |  |  |
| 3.25 | Brakes | Electro Hydraulic operated brakes are to be provided for all MH and LT (Totally enclosed) conforming to IP:54. For CT as per manufacturer's standard |  |  |
| 3.26 | Gear Boxes for LT, CT and Hoists | Completely enclosed M.S fabricated and stress relieved split type |  |  |
| 3.27 | Couplings between motor and gearbox | Geared couplings |  |  |
| 3.28 | Couplings between gearbox and rope drum for hoists | Geared couplings |  |  |
| 3.29 | Mounting | All the gearboxes, motors, brakes, bearing pedestals, etc. shall be mounted on machined surfaces |  |  |
| 3.30 | Lubrication of Crane | Centralised lubrication system (with hand pump) should be provided at three areas - Two end carriages and trolley |  |  |
| 3.31 | Toeguards | Toeguards shall be provided for all openings including for trolley |  |  |
| 3.32 | Railings | Safety hand railings of tubular construction shall be provided across full length of the bridges and end carriages, staircases and in any other places where openings have been provided. |  |  |
| 3.33 | Jacking pads | Suitable jacking pads should be provided on end carriages and crab for jacking up the crane and crab for changing track wheels |  |  |
| 3.34 | Safety Pads | Suitable reinforced safety pads should be provided under the end carriages and crab to prevent the crane from falling more than 25mm in the event of breakage of a track wheel/bogie/axle. These safety stops shall not interfere with the removal of wheels |  |  |
| 3.35 | End buffers | Spring-loaded buffers are to be provided for long travel and cross travel. |  |  |
| 3.36 | Measurements | All components including fasteners should be in metric system only |  |  |
| 3.37 | Locking of Parts | Bolts and screws in rotating parts shall be locked. |  |  |
| 3.38 | Painting | The cranes should be painted with two coats of deep orange paint confirming to IS 2932 : 2003, ISC No 591 (as per IS 5) after giving primary coat. Girders and trolley frame shall be shot blasted/ Chemically treated before painting. |  |  |
| 3.39 | Plates used for fabrication | To be of rolled steel as per IS 2062 plates. Test certificates to be furnished |  |  |
| 3.40 | End carriages | As per manufacturer's design |  |  |
| 3.41 | Butt joints | All butt joints shall be 100% X rayed both compression and tension and both flanges / Webs |  |  |
| 3.42 | Girder | No splice joint – to be of single piece only. No of weldable joints along the total length of the girder should not exceed three |  |  |
| 3.43 | Platform | Full length chequered platform shall be provided on both sides of the bridge and wherever access is necessary for operation and maintenance. The platforms shall be bolt jointed. Width of the platform shall be >750 mm. |  |  |
| 3.44 | Ladders | NA |  |  |
| 3.45 | DSL | In BHEL scope |  |  |
| 3.46 | DSL guard | To be provided |  |  |
| 3.47 | DSL maintenance cabin | To be provided at suitable height |  |  |
| C | ELECTRICAL EQUIPMENT |  |  |  |
| 4.1 | Power Supply at DSL | 415 volts ± 10 %, 3 phase, 50, ± 2 Hz, 4 wires, one conductor earthed.Isolator switch should be provided for isolating the total supply to the crane |  |  |
| 4.2 | Control voltage | 110 Volts AC |  |  |
| 4.3 | Auxiliary voltage | 230 Volts AC |  |  |
| 4.4 | Sockets | The following sockets are to be provided in the protective panel a. 230 V AC work socket with on/off switch b. 24 V AC work socket for with on/off switch |  |  |
| 4.5 | Power distribution to trolley | Trailing cable supported on anti-friction wheel cable |  |  |
| 4.6 | Connection to pendent | Trailing cable supported on anti-friction wheel cable |  |  |
| 4.7 | Panels | 1. Individual panels to be provided as follows. a. Protective panel - one b. MH, CT & LT Panel - one 2. Ferrules to be provided for cable connections as per circuit diagram. 3. Individual isolating switch to be provided for each panel 4. Input contactor should be provided in the panel for each drive 5. Panels shall be of IP54 grade |  |  |
| 4.8 | Colour of panels | The panels should be powder coated. The colour of coating shall be light grey confirming to IS:2932:2003 (as per IS 5) |  |  |
| 4.9 | Mounting of panels and resistance boxes | To be mounted on supporting frames fabricated from 'L' angles |  |  |
| 4.10 | Electrical Motors | Crane duty, one hour rated, class B or F insulation, IP54 protection, 40 % CDF, 150 starts/hour squirrel cage motors for all motions/traverses |  |  |
| 4.11 | Contactors | Siemens make contactor 3 TA 22-30 Amps or equivalent rating to be provided as minimum size contactor in the power circuit. |  |  |
| 4.12 | Overload relays | To be provided for all the motors. |  |  |
| 4.13 | Cables | P.V.C armoured, copper multistrand conductors, minimum size 4 mm sq. in power circuit and 2.5 mm sq in control circuit. All inter connecting wiring to be done neatly through perforated trays fixed in suitable places. |  |  |
| 4.14 | Limit Switches for traverses | Limit switches to be provided for all the motions |  |  |
| 4.15 | Limit Switches for Main & Auxiliary hoists | One series limit switch with counter weight type and one rotary shunt limit switches for each hoist |  |  |
| 4.16 | Earthing | Double earthing are to be provided for all the electrical equipment. A common electrical earthing bus bars to be installed on the cranes covering the entire span and trolley etc. |  |  |
| D | MATERIALS |  |  |  |
| 5.1 | Gears | EN 9 Helical type |  |  |
| 5.2 | Hardness of Gears | 225 to 280 BHN |  |  |
| 5.3 | Pinions | EN 19/EN 24 helical type |  |  |
| 5.4 | Hardness of Pinion | 250 to 300 BHN |  |  |
| 5.5 | Heat treatment of gears/pinions | Hardened, tempered |  |  |
| 5.6 | Pulleys | Cast Steel IS: 1030 grade 4 |  |  |
| 5.7 | Gear Box | IS: 226/IS: 2062 fabricated casing duly stress relieved |  |  |
| 5.8 | Wheels | C 55 Mn 75 / forged steel with hardness of 300-350 BHN |  |  |
| 5.9 | Rope Drum | Rolled steel plate fabricated (IS-2062) duly stress-relieved with flanges at both ends. Rope drum to be machine grooved in hoisting direction. Dia of drum/rope ratio to be as per IS: 3177-1999. 100% X-ray quality welding for circumference joint to be done |  |  |
| 5.10 | Hooks | Made of mild steel/EN3A or wrought iron/20Mn2 to IS:6294/1971 with a hardness of 150-180 BHN and Nut to be forged from C45 material |  |  |
| 5.11 | Makes of other bought out components | To be as per annexure M&S/Cranes/Makes/2007, Rev 03, dated 25/01/2007 (enclosed) |  |  |
| E | GENERAL |  |  |  |
| 6.1 | Environment | All equipment shall be tropicalised to operate in normal shop floor conditions of temperature 10 to 40 ° C and maximum relative humidity of 95 %. |  |  |
| 6.2 | Guarantee | Supplier shall guarantee all equipment for a period of 18 months from the date of commissioning at BHEL works |  |  |
| 6.3 | Oils & grease | First filling of oils and grease are in supplier's scope |  |  |
| 6.4 | Manufacturer's reference list | The manufacturer should have manufactured, at least ten cranes of 1 ton or higher capacity and with span ≥16.75 meters (of Class M5 or higher as per IS 3177/1999 or equivalent), in the last 5 years as on date of enquiry. List of these cranes to be furnished with names of contact persons, telephone and FAX nos. and e-mail IDs. The supplier should furnish copies of the purchase orders or commissioning reports or performance certificates issued by the customers as documentary proof of the above. |  |  |
| F | DOCUMENTATION |  |  |  |
| 7.1 | List of documents to be submitted with offer (offer will be rejected unless these are submitted) | Point-wise clarifications to all points of this specification including deviations, if any |  |  |
| 7.2 |  | Sketches showing full details of cranes with dimensions / end clearances (between centre of LT rails to column face of building), overhead clearance from LT rails, hook approach limits, lift height, etc. |  |  |
| 7.3 | Total weight of crane in Tonnes | Indicate |  |  |
| 7.4 | Weight of assembled trolley in Tonnes | Indicate |  |  |
| 7.5 | Weight of crane girder in Tonnes | Indicate |  |  |
| 8.1 | List of drawings/documents for approval before manufacture of crane. Note: It is the responsibility of the supplier to get the drawings/documents approved. No extension of delivery will be accepted in case of any delay in submission/approval of drawings | General Arrangement (GA) drawing of crane |  |  |
| 8.2 |  | GA Drawing of Trolley |  |  |
| 8.3 |  | GA Drawing of individual mechanism |  |  |
| 8.4 |  | Drawings of bridges, end carriage, and their connections |  |  |
| 8.5 |  | Sub-assembly drawings for wheels, hook blocks, gearboxes, and rope drums |  |  |
| 8.6 |  | Calculation for selection of motor, gearbox and brake |  |  |
| 8.7 |  | Calculation for bridge girder, end carriage and selection of wheels |  |  |
| 8.8 |  | Total weight of structural, mechanical and electrical equipment separately |  |  |
| 8.9 |  | Weight of operator’s cabin (if any) together with all equipment mounted in it. |  |  |
| 8.10 |  | Quality Plan |  |  |
| 8.11 |  | Electrical/electronic circuit diagrams for total crane |  |  |
| 8.12 |  | GA drawing of electrical panels and layout of components within the panels |  |  |
| 9.1 | List of drawings/documents to be submitted at the time of inspection of crane at supplier's works and at BHEL | Test certificates of all boughtout items like hooks, steel plates, wire ropes, etc and for crane (certified by "Competent person" as per as per AP Factories Rules/Acts/Laws |  |  |
| 9.2 |  | 4 Nos. Cranes operation and maintenance manuals. |  |  |
| 9.3 |  | 1 No. spare parts manual |  |  |
| 9.4 |  | Drawings of all wheels |  |  |
| 9.5 |  | One set of drive manuals for each drive installed is to be provided |  |  |
| G | TESTING/INSPECTION |  |  |  |
| 10.1 | During manufacture | Will be checked/inspected by BHEL/third party appointed by BHEL |  |  |
| 10.2 | After manufacture | Will be checked/inspected by BHEL/third party appointed by BHEL. The crane will be checked for all functions and operations. |  |  |
| 10.3 | Crane span | Check of Crane span and diagonal - 10Kg pull should be applied while checking |  |  |
| 10.4 | Clearance for despatch | The equipment shall be despatched only after testing/inspection at supplier's works and on written clearance from BHEL |  |  |
| H | ERECTION & COMMISSIONING |  |  |  |
| 11.1 | At BHEL works | Part of supplier's scope. |  |  |
| 11.2 | Material (lifting equipment, lifting tackles, etc) required during erection and commissioning | Part of supplier's scope. All lifting machines/tackles to be inspected and certified by "Competent Person" as per AP Factories Rules/Acts/Laws before use. |  |  |
| 11.3 | Load test | To be done as per relevant IS standards. Load will be provided by BHEL |  |  |
| 11.4 | Test certificate for crane | Part of supplier's scope. To be inspected and certified by "Competent Person" as per AP Factories Rules/Acts/Laws before use. |  |  |
| 11.5 | Terms and Conditions during installation | During installation, the terms and conditions to be followed by the contractor are as per document M&S/Cranes/Terms/E&C enclosed. |  |  |

|  | # BHARAT HEAVY ELECTRICALS LIMITED ## RAMACHANDRAPURAM :: HYDERABAD – 502 032 | Ref: M&S/Cranes/Terms/E&C Rev: 00 Dt: 14/11/2003 Page: 12 of 17 |
| --- | --- | --- |
|  | TERMS & CONDITIONS DURING INSTALLATION OF LIFTING MACHINES AT BHEL WORKS |  |

TERMS AND CONDITIONS TO BE FOLLOWED

# Obligations Of The Contractors

  - The contractor shall fully comply with the following enactments:

- Contract Labour (R & A) Act, 1970 and rules formed there under in A.P.

- Wage Rates not less than that notified by State Labour Department/from time to time.

- Payment of Wages Act.

- ESI Act, 1948

- EPF Act, 1952

- Workmen’s Compensation Act, 1923.

- BHEL instructions as issued from time to time in regard to working hours, holidays, or any other statutory provision.

  - The contractor shall obtain License from the Assistant Labour Commissioner (Central), or appropriate government if he engages twenty or more workmen only.

  - The Contractor shall produce the following Registers and forms before commencement of work, for verification by the Executing Officer.

- Form XII		-	Register of contractors

- Form XIII		-	Register of workmen employed by contractor (Rule 75)

- Form XIV		-	Employment card issued by contractor (Rule 76)

- Form XVI		-	Muster Roll (Rule 78(1)(a)(i))

- Form XVII	-	Register of wages (Rule 78(1)(a)(i))

- Form XVIII	-	Register of wages-cum Muster Roll (in case of weekly

payment)

- Form XIX		-	Wage Slip (Rule 78(b))

- Form XX		-	Register of deduction for damages or loss

(Rule 78(1)(a)(ii))

- Form XXI		-	Register of files(Rule 78(1)(a)(ii))

- Form XXII	-	Register of advances(Rule 78(1)(a)(ii))

- Form XXIII	-	Register of overtime(Rule 78(1)(a)(iii))

- Form XXIV	-	Return to be sent by the contractor to licensing officer

(Rule 82(1))

The contractor shall maintain the above neatly, completely and legibly for inspection by various authorities even at short notice. The Executing officer of BHEL shall periodically inspect these registers and forms, and shall send his inspection note to Industrial Relations (IR) section of personnel department.

  - The contractor shall observe

## Weekly rest day

- BHEL List of Holidays.

  - Contractor shall obtain complete bio-data, of the labour, employment certificate and Antecedent verification in the prescribed form for each labour and supervisor engaged by him, and shall submit the same to the personnel department/IR section through the contract executing officers before commencement of the work.

  - The entry permits are to be issued to the contract labour by Assistant Commandant/Plant (CISF), based on the requisition submitted by Contract Executing Officer and forwarded by DGM / Personnel.

  - Every contractor shall submit a notice regarding commencement and completion of work in Form-VI A&B [Rule 25(viii) & 81(3)] to personnel department, IR Section through his contract executing officer, for forwarding the same to State Labour Department.

  - The contractor shall make himself or his representative available at the work spot everyday during execution of work, for effective supervision.

  - The contractor shall attend to all inspections notified/conducted by the personnel department, Labour department, P.F. authorities, Factory Inspectors, ESI inspectors, or any other such authorities.

  - Non-compliance of any provisions under the act/rule/instructions/guidelines shall make the contractor liable for penal action including termination of contract.

  - Contractor shall furnish in a separate letter his place of residence and postal address.  The delivery at the above named place or posting in a Post Box regularly maintained by the Post and Telegraph Department or sending letters registered for acknowledgement of any notice, letter or other communication to the contractor shall be deemed sufficient service thereof upon the contractor.  Change in address shall come into force at any time by an instrument executed by the contractor and delivered to the BHEL official who has signed the contract.

  - The contractor must satisfy himself by personal study and examination of the drawings/specifications furnished and understand thoroughly the scope of proposed work in detail and all conditions affecting the work before entering into the contract.  There shall not be at any time, dispute/complaint of any nature regarding scope of work and interpretation of specifications or any misunderstanding with regard to nature or omission of the work to be done nor shall any application for compensation in terms of time and money shall be accepted by BHEL regarding the above.

  - Contractor shall in his absence keep competent agent constantly on the works and any directions or explanations given by the "Contract Signing Officer” or his representative to such agent shall be held to have been given to the contractor himself.

  - Contractor on the advice of BHEL official shall immediately remove any person employed by him, who may in the opinion of the BHEL official, is incompetent or involves himself in misconduct. Such persons shall not be again employed on the works with out written permission of the BHEL official.

  - The contractor shall erect and maintain at his own cost, temporary weather proof sheds at such places approved by the BHEL for keeping materials under cover.

  - The contractor shall give all notices required by the acts, regulation, by laws, legal acts and pay all fees in connection therewith unless and otherwise arranged and decided in writing with BHEL.  He shall also ensure that no attachments are made against materials or work forming part of or for the use of the contractor.  In all such cases, contractor shall protect and indemnify BHEL against any claim or liability arising from or based on the violation of any such laws, ordinances, regulations, orders, decrees or attachment either by himself or by his employees.

  - It shall be contractor’s sole responsibility to protect the public and his employees against accident from any cause and provide required safety equipments and shall indemnify BHEL against any claims for damages for injury to the person or property resulting from

any such accidents and shall, where the provisions of the workmen's compensation Act apply, take steps to properly insure against any claims thereunder.

  - In the event of any accident in respect of which compensation may become payable under the workmen's compensation Act VIII of 1923 whether by the contractor or by BHEL as principal, it shall be lawful for BHEL to retain out of monies due and payable to the contractor such sum or sums of money as may, in the opinion of BHEL shall be final in regard to all matter arising in this clause.

  - No work shall be done on Sundays or on other declared Holidays of BHEL without the written permission of BHEL officer incharge of the work.  The contractor shall comply with the provisions of the factory act if the same are applicable.

  - The contractor shall keep his work place clean and safe to avoid injuries to men and damage to finished products/equipments.

  - On the occurrence of an accident, which results in the death of any of the workmen employed by the contractor or which is so serious as to be likely to result in the death of any such workmen, the contractor shall within 24 hours of the happening of such an accident intimate in writing to the BHEL official incharge of the work.

  - The contractor shall indemnify BHEL against all losses or damages sustained by BHEL resulting directly or indirectly from his failure to give intimation in the manner aforesaid including the penalties or dues if any and become payable by BHEL, as a consequence of failure, BHEL to give notice under the workmen’s compensation Act or otherwise confirm to the provisions of the said Act in regard to such accident.

  - The contractor shall ensure adherence to all statutory requirements applicable to BHEL.

  - The contractor shall ensure abidance by all the labour laws especially including contract labour (R&A) Act, payment of wages Act, workmen’s compensation Act, minimum wages Act, ESI Act and provident fund act as amended from time to time.

  - The contractor shall comply with Provident Fund Act either through PF code alloted to him or by the code provided by PF Department to work centres in the FACTORY/Township Civil of BHEL, R.C.Puram.

  - Notwithstanding the above clause, in case of any financial loss incurred by company on account of contravention of the Provident Fund regulations or any regulations of rule touching the same by the contractor, the contractor shall submit an undertaking to indemnify the company to the extent of the loss incurred by the company.

  - The contractor should engage only those labourers who shall be more than 18 (eighteen) years of age.

  - The contractor shall not resort to subcontracting under any circumstances.  If found sub-contracting at a later date, BHEL reserves the right to take whatever action it deems fit, including cancellation of the Contract.

  - The contractor shall provide the required safety equipment to the labours engaged by him.

  - Contractor shall issue "Employment Card" as per statute  to all labour and supervisors covered under the job work contract.

  - The contractor shall be responsible to settle any grievances of the labour deployed by him.