BHARAT HEAVY ELECTRICALS LIMITED

RAMACHANDRAPURAM::HYDERABAD – 502 032::502 032 INDIA

Phone No: 0091- 040- 23182250, 23020173

Fax No: 0091- 40- 23021910, 23020173

Technical specification for the Idly Cooking Plant

Idly Cooking Plant (Idly Box):

- Idly Cooking Plant (Idly Box) of 300 Idly Capacity without Steam Generator – Three Chamber model unit fabricated with 16 SWG AISI 304 Grade SS Sheet.   Each Chamber consists of 5 Nos., Idly trays of 20 idly each.

- The perforated idly tray and shutter tray shall be made of 20 SWG and 22 SWG SS Sheet of AISI 304 Grade.

- Each chamber is to be fitted with individual neoprene rubber gasket, brass door handle and automatic on/off controls for steam circulation.

- Unit to be mounted on SS stand legs made of 1 ¼ NB sec 10 x ¾;   NB sec pipe of AISI 304 grade quality.

- Size of the box : 625 x 600x 900 mm.

- Height of the stand : 600 mm.