	[image: image1.png]o vgE T )



                                                                                                        BHEL, R.C.PURAM, HYDERABAD.

T & C SUB CONTRACT

TECHNO-COMMERCIAL BID FORMAT    


	ENQUIRY NUMBER & DATE 

BHEL/HYD/OT/M-CING OF CASING-150MW   Dtd.14.07.12

	NAME OF THE VENDOR
	ADDRESS:

	DUE DATE FOR OPENING: 31.07.12

	TELEPHONE NO:
	E-MAIL

	TYPE OF BID: TWO PART


	CONTACT PERSON 
	Your Offer no.



	ITEM DETAILS : NO OF ENQUIRY ITEMS :01


	S.NO
	ITEM DESCRIPTION


	DRAWING NO
	QTY

(NOS)
	SCOPE OF WORK
	MACHINING REQUIREMENTS

	1
	HP-IP Outer casing for 150MW Steam Turbine.
	01050162008 00 02  

01060162955 00/01 04
330768 00031-00-00  

MATL SPEC:HY19571

(GS22Mo4)    

 Volumetric dimns            

LxBxH                            

in assembled  Conditions               5183x4360x2970mm 

Total weight of OC           47750 kgs (Gross)

both UP+LP                     44750 kgs (Net)
	02
	SCOPE OF WORK   CNC GANRTY /CNC HORIZONTAL BORING  MACHINE as per Drawing and Process.
1. Parting Plane rough and finish machining, both sides 

    gland bore rough m/cing &  chimney & gland steam 

    grooves rough  m/cing at both sides, inner casing     

    paws  seating pockets upper part.

2.  Parting Plane holes drilling and puncturing Upper 

     Part.

3.  Parting plane holes back counter boring Upper Part.

4.  Peripheral machining i.e. pockets, flanges, 

     bosses, resting paws, chimney & gland 

     stem flanges butting pockets for the Upper Part.

5.  Peripheral drilling & tapping for upper part.

6.  Parting Plane rough and finish machining, both sides 

     gland bore rough m/cing &  chimney & gland steam 

     grooves rough  m/cing at both sides, inner casing   

     paws  seating pockets of Lower Part.

7.  Parting Plane holes drilling and threading of Lower 

      Part.
8.  Peripheral machining i.e., Keyways, flanges, bosses, 

     boss for Wall Temperature   Measurement Holes for 

      Lower Part.

9.   Peripheral drilling & tapping for lower part.

10.  Parting planes PP holes mismatch checking, PP gap 

       checking & colour matching    (Scraping) & 

       assembly both the parts for taper pin holes drilling & reaming. 

11. Assemble and clamp both the parts with alternative studs and nuts for  taper pin holes drilling and reaming and also finish boring on CNC H boring  machine.

12. Carry out the taper pin holes drilling and reaming.

13. Rotate the table 180 degree  or re-clamp the job during finish boring  of  front  and rear glands  machining  as required .                                                               

14.  Front & rear gland finish m/cing, chimney & gland steam collection grooves  finish  m/cing & sealing fin grooves m/cing  at both sides glands in combined condition.                                                                                       

15. Dismantling the job.                                                               

16. Clean and deburr the sharp edges for  both the parts. 

17. Final inspection  for both the parts. 

                                                                                             NOTE:-                                                                                                                                                                                                                                                                                                                                                                                        1) Refer the technological process sheets during m/cing 
     for guidance and also follow the  enclosed drawings 
     during finish m/c'ing.                                                                                  2)  All standard tools and special tools which are 
     required for machining in vendor scope.                                                                              3) Collect the clamping fastners i.e., parting plane bolts, 
     nuts and taper pins  ie., (finished condition) as  free  
      issue  items from BHEL.                                                                                     4) All measuring equipments and checking gauges for 
     threaded holes  in vendor scope.                                                        5) Casing  upper  and lower parts castings are to be 

      collected from BHEL- Hyd.                 
6) Hydraulic pressure test will be carried out at BHEL 

     works.                                            


	Required machinery :

1.CNC  Horizontal boring WD-200   OR                                                                                                                                                                                  

   CNC Gantry Drilling & Milling.                 

2.Radial Drilling VR-10

3. Portable /Universal drilling machine  drilling upto dia 60 or more  
NOTES:

a) Machining should be done strictly 

    according to machining  drawings   

    and scope of work .

b) Dimensional tolerances, geometrical 

    accuracies like  perpendicularity,  

     parallality, concentricity and 

     cylindricity and runouts are to  be 
     maintained as  mentioned in the 
     machining  drawings and  scope of 
     work .

c) Parting Plane holes i.e. threaded 

     holes, through holes should 

     be  perpendicular to the Parting 

      Planes during machining as 

     mentioned   in the drgs.

d) Cap Nut seating surfaces i.e. spot 

    face diameters surfaces   

    should be  parallel to the parting 

     plane during machining as  

    mentioned in the  drawings.

e) Maintain the Parting Plane feeler 
    gap with in 0.03mm from  inside and   
    0.05mm from outside in free 
     condition with out  bolts tightening 
     before  taken up for finish boring in 
     combined  condition.

f) Maintain the colour contact 80-85% 
    area at Parting Planes   before   
    finish boring in combined 
     condition.

g) Taper pin holes should check with 
    taper pins after reaming for 

    colour   contact.

h) Half diameters differences should be 
    with in 0.05mm in both  halves.

i)  All the axial dimensions in specified 
     tolerances are to be  maintained   
    from reference plane.

j)  No flaws like blow holes, shrinkage, 
     inclusions etc. should open 

     up   during machining. However, if 
     any flaw opens up during 

     machining,  vendor shall 
     immediately intimate detail and 

     location of flaw and   corrective 
     action proposed. Further 

     machining will be suspended till  
    BHEL approves corrective 

     action.

k) After finish machining, inspection 
     report is to be submitted to 

     BHEL for  final clearance.

l)  Inspection of the job at supplier’s 
     works shall be carried out by 

    BHEL / agency authorized by BHEL 
    for which adequate prior 

    notice shall be    given by the sub-
     contractor.

m) After inspection protect the 
     machined surfaces with rust  
     washable  preventive paint.

n) Free   issue of raw- material  of Ring 
     Nut  S 360x9 (4 Nos )  of drg no: 
      33076800031-00 from BHEL . 
     Manufacture the Ring Nuts & suit  
      to Outer   casing during peripheral 
      Machining at vendor works.

o) Proper packing and clamping is to be 

     provided during transportation of 

     casings to BHEL – Hyd.



	2

	VENDOR SHOULD HAVE THE   MACHINES as mentioned above under machining requirements. 

(VENDOR SHOULD GIVE THE DETAILS OF THE MACHINE SUCH AS BRAND NAME, YEAR OF MANUFACTURE, SPECIFICATIONS, ACCURACIES & DETAILS OF MACHINE ACCESSORIES .
	

	3
	VENDOR SHOULD HAVE THE FOLLOWING TOOLS.

1. Suspension tools and straight grooving tools


	

	4
	VENDOR SHOULD HAVE THE FOLLOWING MEASURING INSTRUMENTS.

1. Suspension micrometers for suspension grooves checking, Outside Micrometers.


	


	  TERMS & CONDITIONS


	SUPPLIER OFFER /COMMENTS , CONFORMATIONS
FOR  COMMERCIAL TERMS



	5
	Scope of work and material supply
	Casing castings to be collected by  vendor from BHEL.  
Casing  machining is to be carried out as per drawing & Technology process. 

All the Tools/ Jigs/Fixtures and Measuring Instruments are in Vendor's scope.


	

	
	
	1. Vendor  shall  submit    detailed  Manufacturing  Process  Plan  (MPP)  containing  sequence  of  operations,  checkpoints, facilities  used,  special  tooling,   special  checking  gauges  and  templates,  etc.,  to   BHEL  for   review  and  approval  within  15  days  of  placement  of  order,  before  start   of  manufacturing.           


	

	
	
	2. Vendor  shall  submit    Vendor  Quality  Plan   containing  all  the   quality  checkpoints,  in  line  with   MPP,    to  BHEL   for  review  and   approval    within  15  days  of  placement  of  order,  before  start   of  manufacturing.


	

	
	
	3. Vendor  shall  submit    detailed  Dimensional  Inspection  Report  (DIR)  blank  formats (in  line  with  MPP)  to  BHEL   for  approval   within  15  days  of  placement  of  order,  before  start   of  manufacturing.


	

	
	
	4.  All  measuring  instruments and   checking  gauges  shall  be  in  the  scope  of  the  vendor


	

	
	
	5.  ISO Certification is desirable. Status of  ISO certification may be indicated.

	

	
	
	6. After  completion  of  all  operations, Vendor  shall  submit   detailed  Dimensional   inspection  Reports  (DIR)  in  the  agreed  formats  to  the  Inspection  Agency   for  final  clearance.


	

	6
	Bank guarantee towards value of BHEL material.
	In the event of placement of order, vendor is required to furnish a bank guarantee amounting to  Rs.5 lakhs.

The format of the bank guarantee indicating the amount, the validity period and other terms shall be sent before sending of the material.  The material will be dispatched after submission of bank guarantee.

	

	7
	Inspection
	Inspection to be done by BHEL approved inspecting agency  M/s. LLOYDS  OR BHEL .
Inspection Charges is  in BHEL  scope.


	

	8
	Delivery Terms
	The finished item should be sent to be BHEL R.C.Puram on FOR destination basis with transportation charges in vendors scope.  

	

	9
	Offer Validity
	 90 DAYS FROM THE DATE OF OPENING OF PRICE BID . 

	

	10
	Transportation
	TO & FRO is in Vendor Scope

	

	11
	Delivery Period
	Delivery should be

 90 days from the date of last free issue materials by BHEL for each casing .
In the event of placement of order, if the progress of work is not satisfactory, BHEL reserves the right to short close the order and with draw the material supplied as free issue.  No claim towards charges for partial completion of the work will be entertained. BHEL reserves the right to go for risk purchase in such cases


	

	12
	Payment terms
	1) Dispatch of Goods on door delivery basis only.  Goods dispatched with documents presented thro'Bank will not be accepted and the offer will be rejected. 

2) 100% payment will be made on receipt and acceptance of materials within  90 days from the date of dispatch / Invoice.  

3) Payment to SSI Units (Registered with BHEL) will be made on receipt and acceptance of materials within 45 days from the date of despatch / invoice. SSI certificate is to be enclosed along with offer.

4) Payment against proforma invoice will not be considered and the offer will be rejected.

5) In no case BHEL shall accept to pay advance / Liability towards interest.

6) Offers with deviation to the above payment terms will be LOADED at 18% .


	

	13
	Penalty
	Penalty will be applicable for delayed supplies @0.5% per week upto a maximum of 10% of total P.O. value for item wise.


	

	14
	Discretion clause


	BHEL reserves the right to accept or reject any offer partly or fully at its discretion without assigning any reason. 

	

	15
	Defects , after receipt of finished components
	On receipt of Finished items at destination, if any manufacturing defects are noticed, the same shall be attended  by the vendor at his own cost , without any delay.

	

	16
	Excise duty / CST/VAT
	Since it is job work, Excise Duty, Sales tax and Service tax is not applicable. Please confirm

	

	17
	Transit insurance


	Transit insurance is in BHEL scope.
	

	18
	Force majure clause
	Force majure clause will be applicable only in case of earthquakes, floods, riots  or any act of God beyond control of the contractor. Breakdown of machine  shall not be considered as force majure


	

	19
	Arbitration
	Arbitration : all disputes arising out of this contract shall be referred to the Executive director BHEL R.C.Puram Hyderabad 502032, or his authorized representative for arbitration and his decision shall be final and binding.  


	

	20
	Juridiction 
	Jurisdiction : In case of any suit or other legal proceedings arising under or relating to the contract, the courts at Sangareddy, Medak district Andhra Pradesh only shall have the jurisdiction and this will be resorted only after exhausting the arbitration clause as above.


	

	21
	INDEGINOUS VENDOR
	Offers are invited from indigenous vendors only on view of urgency. Hence to confirm required delivery period without any deviation.
	

	Note Drawings soft copies will be issued to the vendor by the Sub Contract department    
	Signature
	Signature

Stamp

Seal





�EMBED Word.Picture.8���








Page 6/6

[image: image2.png]o vgE T )



_1059289322.doc
[image: image1.png]






