| BHEL, R.C.PURAM, HYDERABAD. T & C SUB CONTRACT TECHNO-COMMERCIAL BID FORMAT |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ENQUIRY NUMBER & DATE BHEL/HYD/OT10/STEAM STRAINER ASSLY. DT.28.06.11 |  |  |  |  |  | NAME OF THE VENDOR |  | ADDRESS: |
| DUE DATE FOR OPENING: 21.07.11 |  |  |  |  |  | TELEPHONE NO: |  | E-MAIL |
| TYPE OF BID: TWO PART |  |  |  |  |  | CONTACT PERSON |  | Your Offer no. |
| ITEM DETAILS : NO OF ENQUIRY ITEMS :02 |  |  |  |  |  |  |  |  |
| S.NO | ITEM DESCRIPTION |  | DRAWING NO | QTY (NOS) | SCOPE OF WORK |  |  | MACHINING REQUIREMENTS |
| SH1 | Main Steam strainer housing assly |  | 21315062005-03(MS Steam Strainer Assly) 21315062007-00(MS housing m/c ing) 2-13150-62006-03(MS housing) | 10 | I. SCOPE OF WORK ON CNC HORIZONTAL & VERTICAL BORING MACHINES 1. Mark on the casting for overall dimension to ensure its suitability for ruther machining,. 2. preliminary machining to facilitate setting on vertical boring m/c 3. Finish machining of housing as per technological process 4.Peripheral machining as per drawing. NOTE: Refer Technological process for details and sequence of opns. during machining. II. Free issue of raw materials/castings/forgings/SD items (finish machining in vendor's scope ) by BHEL:- 21315062007-00 MS HOUSING 41315062026-00 REDUCTION PIECE 41315062026-00 REDUCTION PIECE 41315062001-00 SEGMENTAL SPLIT RING 41315062002-00 MTLC GASKET 41315062003-00 PROTECTIVE RING 41315062022-00 TIGHTENING COVER 31315062002-00 PLATE 41315062024-00 PIPE III.Procurement of the raw materials as per "part list details" for PGMAs in Vendor/supplier scope. Remaining materials in the part list are under the vendors scope |  |  | 1. Machining should be done strictly according to the relevant machining drawings and scope of work. 2. Geometrical accuracies like concentricity, pendicularity, parallality and cylindricity and surface finish are to be maintained as per drgs. 3. All the threaded holes on the valve casing peripheral are to be perpendicular to the Machined faces. 4. All the axial dimensions in specified tolerances are to be maintained from reference planes. 5. During hydraulic testing blanking device to be used. 6. After total housing assly and ensuring the clearances, the strainer has to be detached from assly and to be send to site separately |
| SH2 | HRH strainer housing assly |  | 1-13151-62014-00(HRH strainer assly) 2-13151-62007-00(HRH housing m/c ing) 2-13151-62005-00(HRH housing ) | 10 | I.SCOPE OF WORK ON CNC HORIZONTAL & VERTICAL BORING MACHINES 1. Mark on the casting for overall dimension to ensure its suitability for ruther machining,. 2. preliminary machining to facilitate setting on vertical boring m/c 3. Finish machining of housing as per technological process 4.Peripheral machining as per drawing NOTE: Refer Technological process for details and sequence of opns. during machining. II. Free issue of raw materials/castings/forgings/SD items (finish machining in vendor's scope ) by BHEL:- 21315162005-00 HRH HOUSING 41315162018-00 REDUCTION PIECE 41315162018-00 REDUCTION PIECE 31315162004-00 PLATE 41315062024-00 PIPE 41315162002-00 GASKET 41315162003-00 PROTECTIVE RING 41315162004-00 SEGMENTAL SPLIT RING 41315162016-00 TIGHTENING COVER III.Procurement of the raw materials as per "part list details" for PGMAs in Vendor/supplier scope. Remaining materials in the part list are under the vendors scope |  |  | 1. Machining should be done strictly according to the relevant machining drawings and scope of work. 2. Geometrical accuracies like concentricity, pendicularity, parallality and cylindricity and surface finish are to be maintained as per drgs. 3. All the threaded holes on the valve casing peripheral are to be perpendicular to the Machined faces. 4. All the axial dimensions in specified tolerances are to be maintained from reference planes. 5. During hydraulic testing blanking device to be used. 6. After total housing assly and ensuring the clearances, the strainer has to be detached from assly and to be send to site separately |
| - ELIGIBILITY CRITERIA - TECHNICAL REQUIREMENTS |  |  |  |  |  |  |  | VENDOR RESPONSE / COMMENTS |
| B1 |  | VENDOR SHOULD HAVE THE MACHINES as mentioned above under machining requirements. (VENDOR SHOULD GIVE THE DETAILS OF THE MACHINE SUCH AS BRAND NAME, YEAR OF MANUFACTURE, SPECIFICATIONS, ACCURACIES & DETAILS OF MACHINE ACCESSORIES . |  |  |  |  |  |  |
| (C) ELIGIBILITY CRITERIA - COMMERCIAL REQUIREMENTS |  |  |  |  |  |  | VENDOR RESPONSE / COMMENTS |  |
| C1 |  | VENDOR SHOULD HAVE ACHIEVED AN AVERAGE TURNOVER OF RS: 2 Crores DURING LAST 3 YEARS. |  |  |  |  |  |  |

| (D) TERMS & CONDITIONS |  |  |  | SUPPLIER OFFER /COMMENTS , CONFORMATIONS FOR COMMERCIAL TERMS |
| --- | --- | --- | --- | --- |
| D1 | Scope of work and material supply | Casing castings to be collected by vendor from BHEL. Casing machining is to be carried out as per drawing & Technology process. All the Tools/ Jigs/Fixtures and Measuring Instruments are in Vendor's scope. |  |  |
| D2 | Distribution of quantities based on tender priorities | This tender is for manufacturing and supplyof steam strainer housing assly. The order will be placed on technically suitable lowest vendor.(L1 vendor) for 60% of the quantity. Balance 40 % quantity will be ordered on L2 vendor, subject to acceptance of L1 rate. If the rate of L1 vendor is not accepted by the L2 vendor, other vendors will be considered in order of tender priority for ordering this 40% quantity. In case other vendors are not accepting the L1 rates, total quantity may be given to L1 vendor based on BHEL requirement. |  |  |
| D3 | Bank guarantee towards value of BHEL material. | In the event of placement of order, vendor is required to furnish a bank guarantee amounting to Rs.5 lakhs. The format of the bank guarantee indicating the amount, the validity period and other terms shall be sent before sending of the material. The material will be dispatched after submission of bank guarantee. |  |  |
| D4 | Inspection | Inspection to be done by BHEL approved inspecting agency (For indigenous vendors is by M/s. LLOYDS and for Overseas vendors by M/s.LLOYDS / BEREAU VERITAS / BNV / TUV ). Inspection charges shall be included in the price quoted by the vendor. |  |  |
| D5 | Delivery Terms | The finished item should be sent to be BHEL R.C.Puram on FOR destination basis with transportation charges in vendors scope. |  |  |
| D6 | Offer Validity | 90 DAYS FROM THE DATE OF OPENING OF PRICE BID . In the event of placement of Order, the order rate will be shall be valid for 12 months from the date of issue of rate contract order. |  |  |
| D7 | Transportation | TO & FRO is in Vendor Scope |  |  |
| D8 | Delivery Period | Delivery should be 90 days from the date of last free issue materials by BHEL for indigenous vendors. 150 days for the date of last free issue materials by BHEL for foreign vendors. In the event of placement of order, if the progress of work is not satisfactory, bhel reserves the right to short close the order and with draw the material supplied as free issue. No claim towards charges for partial completion of the work will be entertained. BHEL reserves the right to go for risk purchase in such cases |  |  |
| D9 | Payment terms | 1) Dispatch of Goods on door delivery basis only. Goods dispatched with documents presented thro'Bank will not be accepted and the offer will be rejected. 2) 100% payment will be made on receipt and acceptance of materials within 90 days from the date of dispatch / Invoice. 3) Payment to SSI Units (Registered with BHEL) will be made on receipt and acceptance of materials within 45 days from the date of despatch / invoice. SSI certificate is to be enclosed along with offer. 4) Payment against proforma invoice will not be considered and the offer will be rejected. 5) In no case BHEL shall accept to pay advance / Liability towards interest. 6) Offers with deviation to the above payment terms will be LOADED at 18% . 7) For foreign vendors 100% payment against IRREVOCABLE LC, which will be opened 30 days prior to despatch schedules. |  |  |
| D10 | Penalty | Penalty will be applicable for delayed supplies @0.5% per week upto a maximum of 10% of total P.O. value for undelivered portion of material for non acceptance / deviation in acceptance of the penalty clause, the charges shall be loaded to Basic Price accordingly. ( for example, if penalty clause is not accepted, a loading of 10% will be made on the basic price. If penalty is accepted to a maximum of 5% a loading of 5% will be made on the basic price) |  |  |
| D11 | Discretion clause | BHEL reserves the right to accept or reject any offer partly or fully at its discretion without assigning any reason. |  |  |
| D12 | Defects , after receipt of finished components | On receipt of Finished items at destination, if any manufacturing defects are noticed, the same shall be attended by the vendor at his own cost , without any delay. |  |  |
| D13 | Excise duty / CST/VAT | Excise Duty/CST/VAT applicable shall be cleary indicated. Further the TIN number ( in respect of VAT applicability ) also to be mentioned in your offers. BHEL will not consider any consequential taxes ( which are not mentioned) claimed by the vendors after submission of offer Price bid shall be evaluated on total cost to BHEL basis after considering all applicable taxes and duties. |  |  |
| D14 | Force majure clause | Force majure clause will be applicable only in case of earthquakes, floods, riots or any act of God beyond control of the contractor. Breakdown of equipment shall not be considered as force majure |  |  |
| D15 | Arbitration | Arbitration : all disputes arising out of this contract shall be referred to the Executive director BHEL R.C.Puram Hyderabad 502032, or his authorized representative for arbitration and his decision shall be final and binding. |  |  |
| D16 | Juridiction | Jurisdiction : In case of any suit or other legal proceedings arising under or relating to the contract, the courts at Sangareddy, Medak district Andhra Pradesh only shall have the jurisdiction and this will be resorted only after exhausting the arbitration clause as above. |  |  |
| Note Drawings soft copies will be issued to the vendor by the Sub Contract department |  |  | Signature | Signature Stamp Seal |

COMMITTEE MEMBERS

| R.SRINIVASAN AGM/TECH/T& C | T.M.RAO AGM/T & C PLANNING & SC | JAYANTA DAS AGM , QC | D.SRINIVASA RAO AGM/TURBINE ENGG |
| --- | --- | --- | --- |