| BHEL, R.C.PURAM, HYDERABAD. T & C SUB CONTRACT TECHNO-COMMERCIAL BID FORMAT |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ENQUIRY NUMBER & DATE BHEL/HYD/OT7/LP OUTER MODULE DATED : 28.06.2010 |  |  |  |  | NAME OF THE VENDOR | ADDRESS: |  |
| DUE DATE FOR OPENING: 21.07.11 |  |  |  |  | TELEPHONE NO: | E-MAIL |  |
| TYPE OF BID: TWO PART |  |  |  |  | CONTACT PERSON | Your Offer no. |  |
| ITEM DETAILS : NO OF ENQUIRY ITEMS : 5 |  |  |  |  |  |  |  |
| SL NO | NAME OF ITEM | DRAWING NO: | QTY (Nos) | SCOPE OF WORK |  |  | Major Machining Facilities required |
| LP1 | Longitudinal Girder Left side for 150MW Steam Turbine Fabricated | 01073662001-01FB 01073662001-01SR Total Weight: of the job : 7900 Kgs(Gross) 7400Kgs(Net) Volumetric dimns LXBXH : 5640x875x1210mm | 6 NOS | SCOPE OF WORK FOR FABRICATION 1. Item is to be supplied fully of supplier materials as mentioned in part list (M BOM) material specifications & drgs of BHEL. 2. Manufacture the individual parts as per their individual part drgs which are required for fabrication for all the parts. Rolling, bending, machining, drilling & threading to be carried out as per their part drgs which are required for before fabrication & fit-up & welding. 4. Pre-heat the plates above 50mm thickness during gas cutting. 5. Follow the WPS. Nos during welding as mentioned in individual drgs & assembly drgs. 6.Carry out the fabrication as per assembly drgs & individual Sub assemblies as per their relevant component drgs. 7.Get it inspect the items during major fitups & take the clearance for welding from BHEL-QC/its otherwise agency as mentioned in the drgs & technological Processes. 9. UT,DP & X-ray is to be done wherever required as per specifications/drgs/ processes/ welding stands. The required reports are to be submitted to QC inspector for certification. 10. All raw materials to be consumed for fabrication including electrodes, paints in vendor scope. |  |  | 1.Gas Cutting Machines upto 160mm thick 2.Bending /Rolling Machines upto 40mm/2000mm 3.Welding Machines 200mm width 4.Hand Grinders. 5.X-ray & UT Equipments 6. Stress relieving, Bogi 6 x 6 x 4mts 7. Shot Blasting |

|  |  |  |  | required test certificates & matl stampings/attestation to be obtained and submitted for approval to QC. 11Fabrication of pipe encasing is in vendor scope and to be delivered along with longitudinal girders. 12. Carry out the stress relieving as per technological processes after fabrication by QC Inspector clearance. 13. Carry out the grit blasting & painting as per process. 14. Send the item with all relevant documents/clearance from QC Inspector. NOTE : 1) Refer the technological process sheets for fabrication, Stress Relieving & painting along with the drawings and part list. 2) All consumables are in vendor scope. |  |
| --- | --- | --- | --- | --- | --- |
| LP2 | Longitudinal Girder Right side for 150MW Steam Turbine Fabricated | 01073662001-02FB01073662001-02SRTotal Weight :of the job :7900 Kgs(Gross) :7400Kgs(Net)Volumetric dimns LXBXH : 5640x875x1210mm | 6 | SCOPE OF WORK FOR FABRICATION 1. Item is to be supplied fully of supplier materials as mentioned in part list (M BOM) material specifications & drgs of BHEL. 2. Manufacture the individual parts as per their individual part drgs which are required for fabrication for all the parts. Rolling, bending, machining, drilling & threading to be carried out as per their part drgs which are required for before fabrication & fit-up & welding. 4. Pre-heat the plates above 50mm thickness during gas cutting. 5. Follow the WPS. Nos during welding as mentioned in individual drgs & assembly drgs. 6.Carry out the fabrication as per assembly drgs & individual Sub assemblies as per their relevant component drgs. 7.Get it inspect the items during major fitups & take the clearance for welding from BHEL-QC/its otherwise agency as mentioned in the drgs & technological Processes. 9. UT,DP & X-ray is to be done wherever required as per specifications/drgs/ processes/ welding stands. The required reports are to be submitted to QC inspector for certification. 10. All raw materials to be consumed for fabrication including electrodes, paints in vendor scope. | 1.Gas Cutting Machines 2.Bending /Rolling Machines 3.Welding Machines 4.Hand Grinders. 5.X-ray & UT Equipments 6. Stress relieving 7. Shot Blasting |

|  |  |  |  | required test certificates & matl stampings/attestation to be obtained and submitted for approval to QC. 11Fabrication of pipe encasing is in vendor scope and to be delivered along with longitudinal girders. 12. Carry out the stress relieving as per technological processes after fabrication by QC Inspector clearance. 13. Carry out the grit blasting & painting as per process. 14. Send the item with all relevant documents/clearance from QC Inspector. NOTE : 1) Refer the technological process sheeets for fabrication, Stress Releving & painting along with the drawings and part list. 2) All consumables are in vendor scope. |  |
| --- | --- | --- | --- | --- | --- |
| LP3 | LP Side Wall Turbine & Generator sides for 150MW Steam Turbine Fabricated | 03073862001-00FB 03073862001-00SR Total Wieght of the job :10646Kgs/2 Pieces(Net) :11576Kgs/2 Pieces(Gross) Volumetric dimns : 4960x850x2750 mm LxBxH | 12 | SCOPE OF WORK FOR FABRICATION 1.Item is to be supplied fully of supplier materials as mentioned in part list (M BOM) material specifications & drgs of BHEL. 2.Manufacture the individual parts as per their individual part drgs which are required for fabrication for all the parts. 3.Rolling,bending,machining,drilling & threading to be carried out as per their part drgs which are required for before fabrication & fit-up & welding. 4.Pre-heat the plates above 50mm thickness during gas cutting. 5.Follow the WPS.Nos during welding as mentioned in individual drgs & assembly drgs. 6.Carry out the fabrication as per assembly drgs & individual sub assemblies as per their relevant component drgs. 7.Get it inspect the items during major fitups & take the clearance for welding from BHEL-QC/its otherwise agency as mentioned in the drgs& technological processes 9.UT,DP & X-ray is to be done wherever required as per specifications/ drgs/ processes/ welding stands. The required reports are to be submitted to QC inspector for certification. 10.All raw materials to be consumed for fabrication required test certificates & matl stampings/attestation to be obtained and submitted for approval to QC. 11.Carry out the stress relieving as per technological processes after fabrication by QC Inspector clearance. 12.Carry out the grit blasting & painting as per process. 13.Send the item with all relevant documents/clearance from QC Inspector. NOTE : 1) Refer the technological process sheeets for fabrication, Stress Relieving & painting along with the drawings and part list. 2) All consumables are in vendor scope. | 1.Gas Cutting Machines 2.Bending /Rolling Machines 3.Welding Machines 4.Hand Grinders. 5.X-ray & UT Equipments 6. Stress relieving 7. Shot Blasting |

| LP4 | LP Outer Casing Upper Part(Turbine Side) for 150MW Steam Turbine Fabricated | 01074062003-01FB 01074062003-01 SR Volumetric dimns(LxBxH): 2517x5200x1966mm weight : 6686 kg | 6 | A. SCOPE OF WORK FOR FABRICATION 1.Item is to be supplied fully of supplier materials as mentioned in part list material specifications & drgs of BHEL. 2.Manufacture the individual parts as per their individual part drgs which are required for fabrication for all the parts .3.Rolling,bending,machining,drilling & threading to be carried out as per their part drgs which are required for before fabrication & fit-up & welding. 4.Pre-heat the plates above 50mm thickness during gas cutting.5.Follow the WPS. Nos during welding as mentioned in individual drgs & assembly 5.Follow the WPS. Nos during welding as mentioned in individual drgs & assembly drgs. 6.Carry out the fabrication as per assembly drgs & individual sub assemblies as per their relevant component drgs. 7.Get it inspected the items during major fitups & take the clearance for welding from BHEL-QC or 3rd party agency as mensined in the drgs & technological processes. 9.UT,DP & X-ray is to be done wherever required as per specifications/ drgs/ processes/ welding stands. The required reports are to be submitted to QC inspector for certification.10.All raw materials to be consumed for fabrication required test certificates & matl stampings/attestation to be obtained and submitted for approval to QC. 11.Sent the rougher diaphragm including finish machining as a loose item along with the outer dooms r for final assly during mock up assly at the time of machining. 12.Carry out the stress relieving as per technological processes after fabrication by QC Inspector clearance. 13. Carry out the grit blasting & painting as per process. 14. Send the item with all relevant documents/clearance from QC Inspector.NOTE : Refer the technological process sheets for fabrication, Stress Reliving & painting along with the drawings and part list. 2) All consumables are in vendor scope. | 1.Gas Cutting Machines 2.Bending /Rolling Machines 3.Welding Machines 4.Hand Grinders. 5.X-ray & UT Equipments 6. Stress relieving 7. Shot Blasting |
| --- | --- | --- | --- | --- | --- |

| LP5 | LP Outer Casing Upper Part(Generator Side) for 150MW Steam Turbine Fabricated | 01074062003-02FB 01074062003-02SR Volumetric dimns (LxBxH): 2573x5200x1966mm weight : 6692 Kgs | 6 | SCOPE OF WORK FOR FABRICATION 1.Item is to be supplied fully of supplier materials as mentioned in part list (M BOM) material specifications & drgs of BHEL. 2.Manufacture the individual parts as per their individual part drgs which are required for fabrication for all the parts. 3.Rolling,bending,machining,drilling & threading to be carried out as per their part drgs which are required for before fabrication & fit-up & welding. 4.Pre-heat the plates above 50mm thickness during gas cutting. 5.Follow the WPS. Nos during welding as mentioned in individual drgs & assembly drgs. 6.Carry out the fabrication as per assembly drgs & individual sub assemblies as per their relevant component drgs 7.Get it inspect the items during major fitups & take the clearance for welding from BHEL-QC or 3rd party agency as mentioned in the drgs& technological processes. 9.UT,DP & X-ray is to be done wherever required as per specifications/ drgs/processes/ welding stands The required reports are to be submitted to QC inspector for certification. 10.All raw materials to be consumed for fabrication required test certificates & matl stampings/attestation to be obtained and submitted for approval to QC. 11.Diaphragm including finish machining as a loose item along with the outer dooms for final assly. 12.Carry out the stress relieving as per technological processes after fabrication by QC Inspector clearance 13.Carry out the grit blasting & painting as per process. 14.Send the item with all relevant documents/clearance from QC Inspector. NOTE : 1)Refer the technological process sheets for fabrication, Stress Relieving & painting along with the drawings and part list. 2) All consumables are in vendor scope. | 1.Gas Cutting Machines 2.Bending /Rolling Machines 3.Welding Machines 4.Hand Grinders. 5.X-ray & UT Equipments 6. Stress relieving 7. Shot Blasting |
| --- | --- | --- | --- | --- | --- |

| (B) ELIGIBILITY CRITERIA - TECHNICAL REQUIREMENTS |  | VENDOR RESPONSE / COMMENTS |
| --- | --- | --- |
| B1 | VENDOR SHOULD HAVE THE FOLLOWING MACHINES. 1. Gas cutting facility upto 160mm thick CS sheets 2. 40mm thick x 2000 mm and above. 3. Suitable arc welding facility 4. Stress relieving furnace with BOGI size 6 x 6 x4 mts (VENDOR SHOULD GIVE THE DETAILS FOR THE MACHINE SUCH AS BRAND NAME YEAR OF MANUFACTURE. |  |
| B2 | VENDOR SHOULD HAVE THE FOLLOWING FACILITIES FOR HANDLING. 1. Crane 20tons and above (VENDOR SHOULD GIVE THE DETAILS FOR THE HANDLING FACILITIES SUCH AS BRAND NAME YEAR OF MANUFACTURE. |  |
| B3 | VEDOR SHOULD HAVE THE FOLLOWING MEASURING INSTRUMENTS. 1.Standard measuring instruments for fitup and assembly 2. Level bed for fitup work |  |
| (C) ELIGIBILITY CRITERIA - COMMERCIAL REQUIREMENTS |  | VENDOR RESPONSE / COMMENTS |
| C1 | VENDOR SHOULD HAVE ACHIEVED AN AVERAGE TURNOVER OF RS:2 crores DURING LAST 3 YEARS. |  |

| (D) TERMS & CONDITIONS |  |  | SUPPLIER OFFER /COMMENTS FOR COMMERCIAL TERMS |
| --- | --- | --- | --- |
| D1 | Scope of work and material supply | Fabrication and stress relieving of LP outer module for 150mw as per BHEL drawings and scope of work mentioned as above for each item . |  |
|  |  | - Vendor shall submit detailed Manufacturing Process Plan (MPP) containing sequence of operations, checkpoints, facilities used, special fixtures/tooling, special checking gauges and templates, etc., to BHEL for review and approval within 15 days of placement of order, before start of welding/fabrication. |  |
|  |  | - All raw material procurements shall be from BHEL approved sources. Required test certificates & material stampings/attestation shall be obtained and submitted to Inspection Agency for approval. |  |
|  |  | - Vendor shall submit Vendor Quality Plan containing all the quality checkpoints, in line with MPP, to BHEL for review and approval within 15 days of placement of order, before start of welding/fabrication. |  |
|  |  | - Material Attestations and transfer of material attestations shall be witness points by the Inspection agency. |  |
|  |  | - Vendor shall submit WPS & PQR for every welding process to BHEL for review and approval, after placement of order. Approval is to be obtained from BHEL before start of welding. |  |
|  |  | - Vendor shall maintain the Welder Qualification records, records pertaining to Calibration of furnaces, etc. Vendor shall submit details for the same. |  |
|  |  | 7.The vendor shall have qualified welders, qualified NDT personnel, required NDT facilities, etc. Vendor shall submit details for the same. |  |
|  |  | - Vendor shall submit detailed Dimensional Inspection Report (DIR) blank formats (in line with MPP) to BHEL for approval within 15 days of placement of order, before start of welding/fabrication. |  |
|  |  | - All measuring instruments, checking gauges and templates shall be in the scope of the vendor. |  |
|  |  | 10. ISO Certification is desirable. Status of ISO certification may be indicated. |  |
|  |  | - After completion of all operations, Vendor shall submit detailed Dimensional Inspection Reports (DIR), NDT Test Reports, etc., in the agreed formats to the Inspection Agency for final clearance. |  |
| D2 | Distribution of quantities based on tender priorities | The order will be placed on technically suitable lowest vendor.(L1 vendor) for 60% of the quantity. Balance 40 % quantity will be order on L2 vendor, subject to acceptance of L1 rate. If the rate of L1 vendor accepted by the L2 vendor, other vendors will be considered in order of tender priority for ordering this 40% quantity. Vendors should consider this aspect while submitting the offer. |  |
| D3 | Inspection | Inspection will be carried out by BHEL or any third party agency nominated by BHEL (For indigenous vendors is by M/s. LLOYDS REGISTER OF ASIA (LRA) and for Overseas vendors by M/s.LLOYDS / BEREAU VERITAS / BNV / TUV ). In the event of placement of order, detailed QAP has to be submitted to the vendor. |  |
| D4 | Delivery Terms | The finished item should be sent to be BHEL R.C.Puram on FOR destination basis with transportation charges in vendors scope. |  |
| D5 | Validity | 90 DAYS FROM THE DATE OF OPENING OF PRICE BID AND VALID FOR A PERIOD OF ONE YEAR FROM DATE OF RATE CONTRACT. |  |
| D6 | Payment terms | 1) Despatch of Goods on door delivery basis only. Goods despatched with documents presented thro'Bank will not be accepted and the offer will be rejected. 2) 100% payment will be made on receipt and acceptance of materials within 90 days from the date of despatch / Invoice. 3) Payment to SSI Units (Registered with BHEL) will be made on receipt and acceptance of materials within 45 days from the date of despatch / invoice. SSI certificate is to be enclosed along with offer. 4) Payment against proforma invoice will not be considered and the offer will be rejected. 5) In no case BHEL shall accept to pay advance / Liability towards interest. 6) Offers with deviation to the above payment terms will be LOADED at 18% . 7) For foreign vendors will be opened 30 days prior to despatch. |  |
| D7 | Penalty | Penalty will be applicable for delayed supplies @0.5% per week upto a maximum of 10% of total P.O. value for undelivered portion of material. For non acceptance / deviation in acceptance of the penalty clause, the charges shall be loaded to Basic Price accordingly. ( for example, if penalty clause is not accepted, a loading of 10% will be made on the basic price.If penalty is accepted to a maximum of 5% a loading of 5% will be made on the basic price) |  |
| D8 | Defects | On receipt of Finished items at destination, if any manufacturing defects are noticed, the same shall be attended by the vendor at his own cost , without any delay. |  |
| D9 | Defects , after receipt of finished components | BHEL reserves the right to accept or reject any offer partly or fully at its discretion without assigning any reason. |  |
| D10 | Excise duty / CST/VAT | Excise Duty/CST/VAT applicable shall be Cleary indicated. Further the TIN number ( in respect of VAT applicability ) also to be mentioned in your offers. BHEL will not consider any consequential taxes ( which are not mentioned) claimed by the vendors after submission of offer Price bid shall be evaluated on total cost to BHEL basis after considering all applicable taxes and duties. |  |
| D11 | Force majure clause | Force majure clause will be applicable only in case of earthquakes, floods, riots or any act of God beyond control of the contractor. Breakdown of equipment shall not be considered as force majure |  |
| D12 | Arbitration | Arbitration : all disputes arising out of this contract shall be referred to the Executive director BHEL R.C.Puram Hyderabad 502032, or his authorized representative for arbitration and his decision shall be final and binding. |  |
| D13 | Jurisdictions | Jurisdiction : In case of any suit or other legal proceedings arising under or relating to the contract, the courts at Sangareddy, Medak district Andhra Pradesh only shall have the jurisdiction and this will be resorted only after exhausting the arbitration clause as above. |  |
| D14 | Other General Terms and Conditions | Acceptance to all other BHEL general terms and conditions as mention in the enquiry / guidelines document. |  |
| D15 | Transportation | Transportation of finished assemblies is in vendor scope. |  |
| D17 | INDEMNITY BOND | IN THE EVENT OF PLACEMENT OF ORDER, VENDOR HAS TO SUBMIT INDEMNITY BOND TOWARDS THE VALUE OF THE ITEM BEING SENT BY BHEL FOR MACHINING |  |
| D18 | DELIVERY PERIOD | Within 90 days from the date of issue of Subcontract order. |  |
| Note Drawings soft copies will be issued to the vendor by the Sub Contract department Signature |  |  | Signature Stamp Seal |