| BHEL, R.C.PURAM, HYDERABAD. T & C SUB CONTRACT TECHNO-COMMERCIAL BID FORMAT |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ENQUIRY NUMBER & DATE BHEL/HYD/OT9/BFPDT ROTORS DT.28.06.11 |  |  | NAME OF THE VENDOR |  |  |  | ADDRESS: |  |
| DUE DATE FOR OPENING: 21.07.11 |  |  | TELEPHONE NO: |  |  |  | E-MAIL |  |
| TYPE OF BID: TWO PART |  |  |  | CONTACT PERSON |  |  | Your Offer no. |  |
| ITEM DETAILS : NO OF ENQUIRY ITEMS :01 |  |  |  |  |  |  |  |  |
| S.NO | ITEM DESCRIPTION | DRAWING NO |  |  | QTY (NOS) | SCOPE OF WORK |  | MACHINING REQUIREMENTS |
| R1 | BFP 500 MW ROTOR MACHINING | Forg. Drg :13030120009-00 Rotor drg : 03030100099-02 Bld. Plan : 03030100442-00 |  |  | 10 | 1) Finish turning as per rotor drg & blading drg(with 2mm allowance on journals and ends roughing beyond journals) including finish grooves turning as per blading plan drgs. 2) Blade Insertion groove machining & matching as per groove plan drg. 3) Sealing fin caulking & turning. |  | 1. CNC CENTRE LATHE (SOC: 700mm or more Wt: 5T or more). 2. CNC HORIZONTAL MACHINE |
| R2 | BFP 600 MW ROTOR MACHINING | Forg. Drg :13030120009-00 Rotor drg : 03030100567-00 Bld. Plan : 93030100592-00-Sht.01 & 03) |  |  | 30 | 1) Finish turning as per rotor drg & blading drg(with 2mm allowance on journals and ends roughing beyond journals) including finish grooves turning as per blading plan drgs. 2) Blade Insertion groove machining & matching as per groove plan drg. 3) Sealing fin caulking & turning. |  | 1. CNC CENTRE LATHE (SOC: 700mm or more Wt: 5T or more). 2. CNC HORIZONTAL MACHINE |

| - ELIGIBILITY CRITERIA - TECHNICAL REQUIREMENTS |  |  |  | VENDOR RESPONSE / COMMENTS |
| --- | --- | --- | --- | --- |
| B1 | VENDOR SHOULD HAVE THE FOLLOWING MACHINES. 1. CNC Centre lathe SOC 700 (for turning & Grooving) (VENDOR SHOULD GIVE THE DETAILS OF THE MACHINE SUCH AS BRAND NAME YEAR OF MANUFACTURE. |  |  |  |
| B2 | VENDOR SHOULD HAVE THE FOLLOWING FACILITIES FOR HANDLING. 1. Crane / Machine tool with 8 T for Rotors (VENDOR SHOULD GIVE THE DETAILS FOR THE HANDLING FACILITIES SUCH AS BRAND NAME YEAR OF MANUFACTURE. |  |  |  |
| B3 | VEDOR SHOULD HAVE THE FOLLOWING TOOLS. - Suspension tools and straight grooving tools |  |  |  |
| B4 | VEDOR SHOULD HAVE THE FOLLOWING MEASURING INSTRUMENTS. 1. Suspension mikes for suspension grooves checking, Outside Micrometers. |  |  |  |
| (C) ELIGIBILITY CRITERIA - COMMERCIAL REQUIREMENTS |  |  |  | VENDOR RESPONSE / COMMENTS |
| C1 | VENDOR SHOULD HAVE ACHIEVED AN AVERAGE TURNOVER OF RS: 2 Crores DURING LAST 3 YEARS. |  |  |  |
| C2 | Those who are interested in doing 10 or more jobs in a year need only apply. |  |  |  |
| (D) TERMS & CONDITIONS |  |  |  | SUPPLIER OFFER /COMMENTS , CONFIRMATIONS FOR COMMERCIAL TERMS |
| D1 |  | Scope of work and material supply | Forgings to be collected by vendor from BHEL. Rotor machining is to be carried out as per drawings and free delivery to BHEL,R.C.PURAM works. All the Tools/ Jigs/Fixtures and Measuring Instruments are in Vendor's scope. |  |
|  |  |  | - Vendor shall submit detailed Manufacturing Process Plan (MPP) containing sequence of operations, checkpoints, facilities used, special tooling, special checking gauges and templates, etc., to BHEL for review and approval within 15 days of placement of order, before start of manufacturing. |  |
|  |  |  | - Vendor shall submit Vendor Quality Plan containing all the quality checkpoints, in line with MPP, to BHEL for review and approval within 15 days of placement of order, before start of manufacturing. |  |
|  |  |  | - Vendor shall submit detailed Dimensional Inspection Report (DIR) blank formats (in line with MPP) to BHEL for approval within 15 days of placement of order, before start of manufacturing. |  |
|  |  |  | - All measuring instruments, special checking gauges and templates (required for inspection) shall be in the scope of the vendor. Design and Manufacture of special checking gauges and templates shall be in the scope of the vendor. Drawings for Special Gauges (like Suspension Micrometers, etc.) and Special Checking Templates (for checking radius and depth of Packing Grooves, Blade grooves bottom widths including corner radius, etc.) shall be submitted to BHEL for approval, before start of manufacturing. |  |
|  |  |  | - Vendor shall submit latest calibration report for the CNC Lathe (to be used for rotor machining) indicating Positional Accuracies, Repeatabilities, etc., performed within last two years. Positional accuracy for Z-axis of the CNC Lathe shall be better than 30 microns. (Note: Z-axis of CNC Lathe will be used for measurement of lengths on the machine.). |  |
|  |  |  | - Maximum Spindle Runout shall be within 0.02 mm. Maximum runout on the entire machined rotor shall be within 0.02 mm. Vendor to confirm. |  |
|  |  |  | - Year of Manufacture of the CNC Lathe shall be 1990 or later. |  |
|  |  |  | 9. ISO Certification is desirable. Status of ISO certification may be indicated. |  |
|  |  |  | 10 After completion of all operations, Vendor shall submit detailed Dimensional Inspection Reports (DIR) in the agreed formats to the Inspection Agency for final clearance. |  |
| D2 |  | Distribution of quantities based on tender priorities | This tender is for machining of Rotor Forgings. The order will be placed on technically suitable lowest vendor.(L1 vendor) for 60% of the quantity. Balance 40 % quantity will be ordered on L2 vendor, subject to acceptance of L1 rate. If the rate of L1 vendor is not accepted by the L2 vendor, other vendors will be considered in order of tender priority for ordering this 40% quantity. In case other vendors not accepting the L1 rates, total quantity may be given to L1 vendor based on BHEL requirement. |  |
| D3 |  | Bank guarantee towards value of BHEL material. | In the event of placement of order, vendor is required to furnish a bank guarantee amounting to Rs.5 lakhs. All overseas vendors to furnish a bank guarantee for full landed cost of raw material being issued by BHEL. The format of the bank guarantee indicating the amount, the validity period and other terms shall be sent before sending of the material. The material will be dispatched after submission of bank guarantee. |  |
| D4 |  | Inspection | Inspection to be done by BHEL approved inspecting agency (For indigenous vendors is by M/s. LLOYDS REGISTER OF ASIA (LRA) and for Overseas vendors by M/s.LLOYDS / BEREAU VERITAS / BNV / TUV ). Inspection charges shall be included in the price quoted by the vendor. |  |
| D5 |  | Delivery Terms | The finished item should be sent to be BHEL R.C.Puram on FOR destination basis with transportation charges in vendors scope. |  |
| D6 |  | Offer Validity | 90 DAYS FROM THE DATE OF OPENING OF PRICE BID . In the event of placement of Order, the order rate will be shall be valid for 12 months from the date of issue of rate contract order. |  |
| D7 |  | Transportation | TO & FRO is in Vendor Scope |  |
| D8 |  | Delivery Period | 90 DAYS FROM THE DATE OF DISPATCH OF ROTOR FORGINGS FOR INDIGINIOUS VENDOR AND SUBSEQUENT UNITS 30 DAYS THEREAFTER. 120 DAYS FROM THE DATE OF DISPATCH OF ROTOR FORGINGS FOR FOREIGN VENDOR AND SUBSEQUENT UNITS 30 DAYS THEREAFTER. In the event of placement of order, if the progress of work is not satisfactory, BHEL reserves the right to short close the order and with draw the material supplied as free issue. No claim towards charges for partial completion of the work will be entertained. BHEL reserves the right to go for risk purchase in such cases |  |
| D9 |  | Payment terms | 1) Despatch of Goods on door delivery basis only. Goods despatched with documents presented thro'Bank will not be accepted and the offer will be rejected. 2) 100% payment will be made on receipt and acceptance of materials within 90 days from the date of despatch / Invoice. 3) Payment to SSI Units (Registered with BHEL) will be made on receipt and acceptance of materials within 45 days from the date of despatch / invoice. SSI certificate is to be enclosed along with offer. 4) Payment against proforma invoice will not be considered and the offer will be rejected. 5) In no case BHEL shall accept to pay advance / Liability towards interest. 6) Offers with deviation to the above payment terms will be LOADED at 18% . 7) For foreign vendors LC will be opened 30 days prior to despatch. |  |
| D10 |  | Penalty | Penalty will be applicable for delayed supplies @0.5% per week upto a maximum of 10% of total P.O. value for undelivered portion of material . For non acceptance / deviation in acceptance of the penalty clause, the charges shall be loaded to Basic Price accordingly. ( for example, if penalty clause is not accepted, a loading of 10% will be made on the basic price.If penalty is accepted to a maximum of 5% a loading of 5% will be made on the basic price) |  |
| D11 |  | Discretion clause | BHEL reserves the right to accept or reject any offer partly or fully at its discretion without assigning any reason. |  |
| D12 |  | Defects , after receipt of finished components | On receipt of Finished items at destination, if any manufacturing defects are noticed, the same shall be attended by the vendor at his own cost , without any delay. |  |
| D13 |  | Excise duty / CST/VAT | Excise Duty/CST/VAT applicable shall be cleary indicated. Further the TIN number ( in respect of VAT applicability ) also to be mentioned in your offers. BHEL will not consider any consequential taxes ( which are not mentioned) claimed by the vendors after submission of offer Price bid shall be evaluated on total cost to BHEL basis after considering all applicable taxes and duties. |  |
| D14 |  | Force majure clause | Force majure clause will be applicable only in case of earthquakes, floods, riots or any act of God beyond control of the contractor. Breakdown of equipment shall not be considered as force majure |  |
| D15 |  | Arbitration | Arbitration : all disputes arising out of this contract shall be referred to the Executive director BHEL R.C.Puram Hyderabad 502032, or his authorized representative for arbitration and his decision shall be final and binding. |  |
| D16 |  | Jurisdiction | Jurisdiction : In case of any suit or other legal proceedings arising under or relating to the contract, the courts at Sangareddy, Medak district Andhra Pradesh only shall have the jurisdiction and this will be resorted only after exhausting the arbitration clause as above. |  |
|  |  |  |  | Signature Stamp Seal |