# Technical Specifications

Industrial 300 KV X-Ray Equipment

(Indent No.20089461, Dated 16.07.2009)

| Sl.No. Col.1 | #### Technical Parameters Col.2 | # BHEL Specification # Requirement Col.3 | Relevant technical detail of the offered equipment shall be filled against each row by the bidder. Col.4 |
| --- | --- | --- | --- |
| 1. | X-Ray Tube Head |  |  |
| 1.1 | #### Tube head weight without Guard Rings | 40 kg. (maximum) |  |
| 1.2 | Tube head type | Portable |  |
| 1.3 | High voltage generator | Inside the tube head |  |
| 1.4 | Output voltage range | Max. Tube voltage 300 KV or more |  |
| 1.5 | Tube current at 300KV | 2 mA (minimum) |  |
| 1.6 | Radiation geometry | panoramic |  |
| 1.7 | #### Beam angle | 360° X 40° (maximum) |  |
| 1.8 | Focal spot size | Up to 0.5 X 5.5 mm ( as per EN12543) or Up to Φ 5 X 1.3 ( as per IEC 336) |  |
| 1.9 | #### Anode cooling type | Air cooling |  |
| 1.10 | Penetration in steel | 48 mm. (At 700 mm FFD / D7 film / lead screens / D=1.5 / T=20 min./ standard film processing parameters) |  |
| 1.11 | Operating temperature | Up to 50°C |  |
| 1.12 | #### Duty cycle | 100% at 20°C |  |
| 1.13 | Guard Rings | Suitable guard rings for ease in handling and safety of the tube head shall be provided along with the equipment. |  |
| 1.14 | Exposure Chart | Exposure chart for steel to be provide along with the offer |  |
| 1.15 | Power Cable | Length of power cable shall be minimum 10 metres |  |
| 1.16 | Connection cable | Length of cable connecting tube head and control unit shall be at least 20 metres. |  |
| 1.17 | Input power supply | 230 Volts (+/-10% ), 50Hz(+/-3Hz) , Single phase, AC |  |
| 1.18 | #### Shock proof | The machine shall be adequate protection against electric shock. |  |
| 2. | Control Unit |  |  |
| 2.1 | Control unit type | Microprocessor based control unit with LCD display showing operating parameters, warming up parameters, memory of old shots , error / warning symbols etc. |  |
| 2.2 | Indicators on control unit | Indicators for Power ON , X-Ray ON , Stand By mode etc. |  |
| 2.3 | Warming up | Automatic selection of warming up parameters based upon equipment idle time. |  |
| 2.4 | Entry of exposure parameters | Digital number pad for entry of exposure parameters (KV, mA and exposure time). Auto machine off after completion of exposure time. |  |
| 2.5 | Overheat / Cooling failure protection | Inbuilt tube head overheat / cooling failure protection with auto machine OFF / message on control panel screen in case of overheat. |  |
| 2.6 | Flash lamp | Radiation warning flash lamp along with interconnection cable to the control unit shall be provided. |  |
| 2.7 | Gate limit switch | Interconnection cable between exposure hall gate limit switch and control panel shall be provided. |  |
| 2.8 | Menu language | English. |  |
| 2.9 | Operational accessories | Any other essential accessory required for the machine operation shall be offered along with the machine. |  |
| 3. | Other Requirements |  |  |
| 3.1 | Test certificate and Guarantee Certificate | Test Certificate and Guarantee Certificate in original bearing the test date shall be submitted. |  |
| 3.2 | Guarantee period | The supplier must give a guarantee for a period of at least one year from the date of commissioning of the equipment at BHEL, Hardwar. |  |
| 3.3 | #### Operation and Maintenance Manual | Two copies of Operation and Maintenance Manual shall be supplied by the supplier. |  |
| 3.4 | Commissioning and training | Commissioning shall be done by the supplier at BHEL, Hardwar free of cost. Necessary training shall be provided by the supplier to NDT laboratory operating staff for at least three days. |  |
| 3.5 | #### Authority letter | For the imported machines Indian agents must attach the authority letter from the principal in the quotation stage itself. |  |
| 3.6 | #### Other accessories and Spares | Other accessories and maintenance spares required for five years running shall be offered separately. |  |
| 3.7 | #### Lead belt for directional exposure | Lead belt for directional exposure may be offered separately as an optional accessory. |  |
| 3.8 | #### Tube head stand | Compatible Tube head stand shall be separately offered as an optional accessory. |  |
| 3.9 | #### Service Facility | Suppliers will clearly mention the service facility available in India for the offered equipment. |  |
| 3.10 | #### List of customers | A list of reputed Indian customers is to be sent along with the quotation containing the name of contact person , telephone number and full address where similar equipment has been supplied. |  |
| 3.11 | #### Reply to specification requirements | The supplier must give clear point wise reply with respect to individual specification points mentioned above in the space provided in Col.4 of the table. |  |

# Qualifying Conditions

Industrial 300 KV X-Ray Equipment

(Indent No.20089461, Dated 16.07.2009)

| Sl.No. Col.1 | #### Technical Parameters Col.2 | # BHEL Specification # Requirement Col.3 | Relevant technical detail of the offered equipment shall be filled against each row by the bidder. Col.4 |
| --- | --- | --- | --- |
| 1. | Experience | Offer of only those vendors will be considered who have supplied and commissioned at least one 300 KV panoramic or directional X-ray machine in the previous five years, for industrial radiography. |  |
| 2. | List of customers | A list of customers to whom 300 KV panoramic or directional X-ray machine has been supplied shall be provided mentioning the year of commissioning. |  |
| 3. | Name and address of the customer’s | Name / Designation of the customer’s contact person alongwith the address, contact phone / fax numbers, email address etc. shall be provided. |  |