MPR No. 20081280
Dt.12.1.09

SPECIFICATION 
OF
 20 T MOBILE CRANE
1.0 Type :

Rough - terrain diesel hydraulic mobile crane.

2.0 Lifting capacity:
· 20 T at 3m radius (85% rating) in blocked condition (360 deg. slew)

· 11 T at 3m radius (85% rating) free on wheels over front.

· The crane should be able to rotate 360 deg. with this load
3.0 Drive:
Four wheel drive, four wheel steer & hydraulic power steering. 

4.0 Axles:

Both front & rear axles should  be drive and synchronized  steerable.
5.0 Outriggers :

Independent hydraulic horizontal &  vertical movement & control from operator’s cabin.

6.0 Boom:
· Hydraulic  telescopic  fully powered boom with minimum extendable length  20.0 m.

· The boom should be suitable for rated lifting capacity.

7.0 Slew  System:
360 deg. continuous rotation powered by hydraulic motor & foot applied  multi disc  brake with locking device.

8.0 Brakes :

Pneumatic / hydraulic service brakes on all wheels & parking brakes.
9.0 Safety Device:

All in –built  safety devices like  pressure relief valves in hydraulic circuits, automatic safe load indicator, over hoist & overload  cut outs, etc.

10.0  Work lights:
360 deg. beacon  light, cabin light etc.

11.0 Operator’s Cabin:
Totally enclosed, full all round vision, windows fitted   with toughened safety glass, lockable sliding door, electric horn, interior light & fan, fully  adjustable upholstered seat for operator, controls for crane movements, travel & outrigger operations.

12.0 Paint finish:
High gloss finish cabin, super structure, chairs, safety markings on front & rear of chassis  & counter weight.
13.0 Speed :

32-35 kmph maximum with full counter weight.
14.0 Tool Kit:

Tool kit for normal  maintenance  of the crane, should be provided in a tool box and a list of tools should be given in the offer.

15.0 Spares:

· Recommended spares for 2 years  smooth working including.

      3 sets of hydraulic and engine filters.

16.0 i) Pre-dispatch inspection at supplier’s works.
ii) Maintenance training for BHEL  personnel at supplier’s works.
17.0 i) Commissioning  & job proving for rated load at BHEL works.

ii) The crane  should be got load tested  at BHEL works by an approved agency   of Govt. of Uttarakhand certificate to be submitted to BHEL.

18.0 Guarantee for 24 months after successful job proving. 
19.0  Pre- Qualifying Requirement (PQR):

i) Vendor should have been  manufacturing the offered or higher capacity diesel hydraulic rough terrain mobile crane for at least 5 years.

ii) The offered model  should be from current manufacturing range.

iii) Vendor to submit customers list for above mentioned supply with contact persons & telephone nos.

iv) Vendor to submit performance certificates from customers for offered or higher capacity diesel hydraulic cranes.
