SPECIFICATIONS

FOR

GOUGING SETS

- TYPE

Portable, compact, Transductor type D.C. Rectifier gauging set complete with Power Source,  Gauging Torch, output and earthing cable with Connectors.

2.0   TECHNICAL SPECIFICATIONS :

i)      Input supply                          -     415 V  +/- 10% variation

3 phase, 50 Hz ± 3% Variation

ii)     Current range                        -    150 to 1000 Amps.

iii)     Rated output current            -

At 100% duty cycle               -  1000  Amps (DC)

iv)     Type of Cooling                      -  Forced Air

v)      Power Consumption		-  To be mentioned by vendor.

vi)      Dimensions and Weight	-   To be mentioned by vendor.

3.0	TECHNICAL FEATURES :

It should have following in built features:

- Optimum VA characteristics for heavy duty arc gouging;

- Two widely overlapping ranges with broad current control;

- Automatic Thermal overload protection;

- Fitted with Ammeter and Voltmeter covering whole range on front

panel.

- Fitted with lifting hook and base frame.

- Quick and easy adjustment of controls.

- Front panel fitted with on-off switch,  Output Control Knob showing

overlapping ranges as per specification, Pilot lamps, Ammeter and Voltmeter etc.

- Compact, rigid and light weight and damage resistant cabinet to avoid

any damage during shipping and handling.

- Built-in-safety features as per National/International norms.

- All the wires should have proper wire nos. and should match with

Electric/electronic circuit diagrams given with maintenance manual.

- SCOPE  :-  All items should be separately priced.

- Power Source;

- Gouging torch suitable for 1000 Amps current at 100% duty cycle,.

- Output copper cable assembly with connectors (20.0 m long);

- Copper Earthing cable with connector   -  5.0 m long.

- Recommended spares for 2 years trouble free smooth operation with

identification no. and individual price.

- Commissioning and trials at BHEL Works by Vendor.

- Guarantee for 3 years smooth and trouble free operation.

- Prompt after Sales Services in case of any problem;

- 3 sets of maintenance manual giving complete technical details, spare parts with identification nos., complete electric/electronic circuit diagrams with proper wire nos.

- Addresses and names of customers to whom similar Gauging sets have been supplied and working satisfactorily.

10.0  IMPORTANT :

Vendor should clarify confirm all points mentioned above and provide technical literatures, leaflets of the offered model with the offer otherwise offers may not be considered.