TECHNICAL SPECIFICATIONS FOR HIGH BAY LIGHT FITTINGS

400W, 230V Metal Halide High Bay Luminaire suitable for IP 54 application.

High bay luminaire consists of housing with a anodized aluminium wide beam reflector which is chemically brightened for longevity and high photometric performance, profile to give optimum light distribution to make it suitable for higher height and wider spacing.

The housing shall make out of die cast alunimium and is powder coated in black.

The housing encases a copper wound ballast, power factor improvement capacitor, an electronic ignitor and mains connector.

The reflector shall have a heat resistant flat glass enframed in a retaining toggle with gasket.

An eye bolt with rubber bush shall be on top of the housing to facilitate vertical mounting with less damping vibrations.

Clip type removable gear tray shall be provided to facilitate easy maintenance and replacement.

A safety chain is to be provided from the housing eye bolt to the flat glass.

Cable gland shall be provided for cable/wire entry.

***