Specification for Surface Resistivity Tester for End corona 

protection coating of TG Bars
· It should be operative on mains supply 230V± 5%, 50 Hz  as well as on re-chargeable batteries.

· Resistance measuring voltage 10V,100 V & 1000 Volts,   Accuracy : ± 5%

· Resistivity  measuring range from 1.0 G.Ohm to 500 G.Ohm. (Min.)
· Accuracy : ± 5%

· Display : LCD/LED
· Arrangement should be for special marked distance template
      (Length 0-400mm) in order to fix the electrodes at different measured 
      Locations on TG Bar profile.

· Electrodes should be square in shape & can be easily shifted over the Generator Bar profile.
· It should be rugged, reliable & portable.
· Equipment shall be supplied with leads and other mandatory accessories
· Other requirements:

a) T/C & G/C including calibration certificate from approved Laboratory.

b) O &M manual in English

c) One set of spare leads

d) One spare rechargeable battery

e) Other recommended spares/ tooling
f) PC interface leads (USB)
  (S.K.Das)

                                         (Devanand)

                    AGM (QC-1)

                                       Sr.Manager (QC-1)

Other  requirement

Surface Resistivity Tester for End corona 

protection coating of TG Bars

· Party shall supply two copies of O &M manual in English along with internal circuitry.

· Equipment shall be commissioned at our premises free of cost by the party.

· TC & GC of the equipment are reqd.

· Warranty period shall be of min. one year.

· Calibration certificates are required.

· Party shall supply mandatory spares along with the equipment.

· Party shall offer for pre-inspection of the items before dispatch.

· Party shall submit previous experience of supplying similar equipment or higher rating equipment.

                                (S.K.DAS)

AGM (QC-1)

Pre-qualification requirement

Surface Resistivity Tester for End corona 

protection coating of TG Bars





     (Indent No.20090783)
· Party shall be regular manufacturer of Surface Resistivity Tester manufacturer

· Party shall submit previous experience of supplying similar equipment or better specified equipment. Party, whoever does not submit previous P.O. copy (priced /unpriced ) or order reference of at least one party  to whom they have supplied within last 6-7 years, shall not be considered. 

·  Party shall mention the date of commissioning of the supplied equipment to other party.

(S.K.DAS)

AGM (QC-1)

