TECHNICAL SPECIFICATIONS

CAST IRON SURFACE PLATE , SIZE 6M x 6M

- PURPOSE :

Cast iron surface plate shall be used as a precision reference for marking , lay out and inspection of

Steam turbine governing components .

- TECHNICAL SPECIFICATIONS :

2.1   Cast iron surface plate shall conform to IS:2285-2003 . It shall be made from plain grey iron

castings conforming to grade FG:220 of IS:210-1978 . Material test certificates as required by the

Indian standard shall be submitted along with the supply.

2.2   Overall assembled size shall be 6M x 6M consisting of 6 nos. of individual surface plates of size

3M x 2M .

2.3   Top working surface of each surface plate shall be provided with machined grooves in a grid of

250mm * 250mm .

  - Suitable number of cored holes on the peripheral ribbings shall be provided for lifting of each

surface plate . Provision shall also be made for covering of the cored holes when not in use.

  - Each plate shall be supported on suitable number of leveling screws which will be mounted on concrete supports grouted inside a pit . Adjustment of the leveling screws shall be done from underneath the surface plate. Number of leveling screws to be informed by the supplier .

2.6   Overall height of the assembled surface plate shall be  750+ 25 mm from average shop floor level.

However total height of the concrete supports and leveling screws should be sufficient to enable

easy entry of the personnel underneath the surface plate for leveling , cleaning etc. Supplier to

inform the basic dimensions and proposed lay-out alongwith the offer.

2.7   Load bearing capacity of each individual surface plate of size 3M x 2M  shall be 15MT or more.

Thickness of individual surface plate to be informed by the supplier.

  - The surface plates shall be finish machined on their top working surface as well as four sides at

their periphery . Top surface may be hand scrapped to achieve the specified flatness accuracy.

Parallelity and perpendicularity of the four sides of individual surface plates to be informed.

2.9   The flatness accuracy of individual surface plate of 3m * 2m size shall be

UP TO 0.04 mm and the overall accuracy of the assembled plate of size

6m * 6m shall be UP TO 0.25 mm .

2.10  Unmachined exterior surfaces at the periphery of the Castings & leveling screws shall be painted

with Green enamel paint . Interior surfaces of the castings shall be painted with red oxide primer.

  - Overall dimensions and Foundation details  shall be informed by the Supplier within two months after placement of the order. Civil work as per the details given by the supplier shall be done by BHEL.

- PRE-DESPATCH INSPECTION , COMMISSIONING AND ACCEPTANCE :

3.1   Provisional acceptance of the surface plate shall be made by BHEL at supplier's work prior to

dispatch . Test certificates , Specification compliance and flatness accuracies of the individual

surface plates shall be checked and verified .

  - Final acceptance shall be done after successful installation and commissioning at BHEL by the

Supplier to achieve specified flatness accuracy.

4.0  QUALIFYING CONDITION :

The supplier should have experience of manufacturing and commissioning of the Surface plates and should provide reference list of previous customers with details of the contact persons to whom individual surface plates of size 3M x 2M or larger having flatness accuracies up to 0.04 mm and load bearing capacity of 15MT or more , have been supplied . Satisfactory performance certificate from at least one previous customer to be submitted alongwith the offer.

5.0  Suppliers must furnish Point-wise replies in their offer along with  necessary Sketches / Drawings / Catalogues etc.