TECHNICAL SPECIFICATIONS FOR PERIPHERY/STREET LIGHT FITTINGS

Qtv-45 nos

150W, 230V SON Street light Luminaire suitable for IP 54 application.

Street lighting luminaire consists of housing with POT optics reflector which is chemically brightened for longevity and high photometric performance, profile to give optimum light distribution to make it suitable for higher height and wider spacing.

The housing shall make out of single piece die cast alunimium.

----

The housing encases a copper wound ballast, power factor improvement capacitor, an electronic ignitor and mains connector.

The reflector shall have a UV stabilized acrylic cover.

Street light fitting shall suitable to fix it on 120 degree bent 40mm dia GI pipe.

***                  ***

~ ""'"'