## Annexure-1

# TECHNICAL SPECIFICATIONS OF TOW MASTER 03 Nos.

Capacity					:	4000 Kgs.

Over all Length				:	1651 MM

Wheel Base					:	  965 MM

Turning Radius				:	1448 MM

Under Clearance				:	  203 MM

Over all Width					:	  965 MM
Over all Height				:	1422 MM

Rear Overhang				:	  406 MM

Front Overhang				:	  279 MM

Height of Seat					:	1067 MM

Front Wheel Track				:	Central

Rear Wheel Track				:	  914 MM

Height of Towing Bracket			:	  406 MM

Length of Towing Bracket			:	  111 MM

Towing Eye Clearance			:	    57 MM

Front Wheel 1 No. or 2 No. Suitably fitted 	:	Pneumatic 4” wide x 8” ID (4.00 X 8 inch)

Rear Wheels 2 Nos.				:	Pneumatic 6” wide x 9” ID (6.00 x 9 inch)

Service Weight (Equipment Weight)		:	1100 Kg. Approx.

Travel Speeds	(Toing speed Loaded)		:	8 Km/h

(Toing speed unloaded)	:	13KMPH

Capacity					:	4000 Kgs.

Head Lights					:	Lucas/Hella (G) Make –2 Nos.

Battery						:	Traction Battery Exide Make (Tubular)

Dry & Uncharged 48V, 156 AH-5 Hrs rate,

Suitable for above Tow Master.

Initial Charging				:	Shall be done at our works i.e. BHEL,

Hardwar at the time of commissioning.

Battery Charger				:	Constant Voltage Charger (conventional)

Suitable for above

### Hour Meter Control				:	Electric

Spare (Extra Items)

1. Speed Control (Step Control) Unit		:	2 Nos.

2. Battery Charger		:	1No

      - Any sound should not be come from Gear box and Differential, smooth movement is necessary.

      - Fully one year Guarantee required from the date of commissioning.

      - All bearing used in Tow Master should be SKF or NBC make.

      - Mechanical brake system will be preferred.